3GPP TSG-RAN2 Meeting #113e	R2-2101551

# Electronic meeting,  Jan 25th - Feb 5th  2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4582 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to timing alignment validation for transmission using PUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core, NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN2 made the following agreements on how pur-TimeAlignmentTimer is configured for PUR: - Configuration for TA validation criteria is provided in dedicated RRC signaling. - It should be possible to disable each or all of the optional TA validation criteria (i.e., TA timer, (N)RSRP change) via RRC signaling. - A new TA timer is defined for UEs configured with D-PUR in idle mode. - The (re)starting times for TA timer need to be aligned between UE and eNB. The details of the mechanism are FFS. - TA timer is restarted after TA is updated. - The value range for the TA timer is FFS. Value of “infinity” is possible. - PUR TA timer is configurable up to hour(s) level, disabled/infinity is possible. - Exact values FFS. - TA timer range is multiple of PUR periodicities, e.g. 1,…, 8. - FFS on exact values and whether offset is applied so that e.g. retransmissions are covered. - For both NB-IoT and eMTC, the value range of pur-TimeAlignmentTimer-r16 is INTEGER (1..8), i.e. 1~8 * PUR periodicity. Even though it was agreed that it should be possible to have the value “infinity” for the TA timer, i.e., pur-TimeAlignmentTimer, it is not clear whether/how this agreement has been captured in the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR clarifies how to make it possible to have the value “infinity” for the pur-TimeAlignmentTimer. Impact analysis Impacted functionality: Configuration of pur-TimeAlignmentTimer for PUR Inter-operability: If the UE is implemented according to this CR and the network is not, no interoperatibility issues are expected. If the network is implemented according to the CR and the UE is not, UE may assume that pur-TimeAlignmentTimer is not configured and the timing alignment value for transmission using PUR is not valid. This may lead to UE missing the PUR occasion(s) for transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreement that it should be possible to have the value “infinity” for the TA timer, i.e., pur-TimeAlignmentTimer, is not captured in the specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2, 6.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 6.3	RRC information elements

-- ASN1START

TypeFFS ::= NULL					-- To be removed

-- ASN1STOP

### 6.3.0	Parameterized types

#### –	SetupRelease

SetupRelease allows the ElementTypeParam to be used as the referenced data type for the setup and release entries. See A.3.8 for guidelines.

-- ASN1START

SetupRelease { ElementTypeParam } ::= CHOICE {

release			NULL,

setup			ElementTypeParam

}

-- ASN1STOP

### 6.3.1	System information blocks

#### –	SystemInformationBlockPos

The IE SystemInformationBlockPos contains positioning assistance data as defined in TS 36.355 [54].

SystemInformationBlockPos information element

-- ASN1START

SystemInformationBlockPos-r15 ::= SEQUENCE {

assistanceDataSIB-Element-r15		OCTET STRING,

lateNonCriticalExtension			OCTET STRING							OPTIONAL,

...

}

-- ASN1STOP

SystemInformationBlockPos field descriptions

assistanceDataSIB-Element Parameter AssistanceDataSIBelement defined in TS 36.355 [54]. The first/leftmost bit of the first octet contains the most significant bit.

…..

### 6.3.2	Radio resource control information elements

#### –	Alpha

The IE Alpha is used to indicate parameter α, see TS 36.213 [23], clause 5.1.1.1 and 5.1.3.1. Value al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1.

Alpha information element

-- ASN1START

Alpha-r12 ::= ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}

-- ASN1STOP

…..

#### –	PUR-Config

The IE PUR-Config is used to specify the PUR configuration.

PUR-Config information element

-- ASN1START

PUR-Config-r16 ::=		SEQUENCE {

pur-ConfigID-r16				PUR-ConfigID-r16			OPTIONAL,	-- Need OR

pur-ImplicitReleaseAfter-r16	ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	-- Need OR

pur-StartTimeParameters-r16		SEQUENCE {

periodicityAndOffset-r16		PUR-PeriodicityAndOffset-r16,

startSFN-r16					INTEGER (0..1023),

startSubFrame-r16				INTEGER (0..9),

hsfn-LSB-Info-r16				BIT STRING (SIZE(1))

}		OPTIONAL,	--Need ON

pur-NumOccasions-r16			ENUMERATED {one, infinite},

pur-RNTI-r16					C-RNTI						OPTIONAL,	-- Need ON

pur-TimeAlignmentTimer-r16		INTEGER (1..8)				OPTIONAL,	-- Need OR

pur-RSRP-ChangeThreshold-r16	SetupRelease {PUR-RSRP-ChangeThreshold-r16} OPTIONAL,	-- Need ON

pur-ResponseWindowTimer-r16		ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}		OPTIONAL,	-- Need ON

pur-MPDCCH-Config-r16			PUR-MPDCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PDSCH-FreqHopping-r16		BOOLEAN,

pur-PUCCH-Config-r16			PUR-PUCCH-Config-r16		OPTIONAL,	-- Need ON

pur-PUSCH-Config-r16			PUR-PUSCH-Config-r16		OPTIONAL,	-- Need ON

...

}

PUR-MPDCCH-Config-r16 ::=		SEQUENCE {

mpdcch-FreqHopping-r16			BOOLEAN,

mpdcch-Narrowband-r16			INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-PRB-PairsConfig-r16		SEQUENCE{

numberPRB-Pairs-r16				ENUMERATED {n2, n4, n6, spare1},

resourceBlockAssignment-r16		BIT STRING (SIZE(4))

},

mpdcch-NumRepetition-r16		ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

mpdcch-StartSF-UESS-r16			CHOICE {

fdd								ENUMERATED {v1, v1dot5, v2, v2dot5, v4, v5, v8, v10},

tdd							ENUMERATED {v1, v2, v4, v5, v8, v10, v20, spare1}

},

mpdcch-Offset-PUR-SS-r16	ENUMERATED {zero, oneEighth, oneQuarter,

threeEighth, oneHalf, fiveEighth,

threeQuarter, sevenEighth}

}

PUR-PUCCH-Config-r16 ::=			SEQUENCE {

n1PUCCH-AN-r16						INTEGER (0..2047)			OPTIONAL,	-- Need ON

pucch-NumRepetitionCE-Format1-r16	ENUMERATED {n1, n2, n4, n8}	OPTIONAL	-- Need ON

}

PUR-PUSCH-Config-r16 ::=		SEQUENCE {

pur-GrantInfo-r16				CHOICE {

ce-ModeA						SEQUENCE {

numRUs-r16						BIT STRING (SIZE(2)),

prb-AllocationInfo-r16			BIT STRING (SIZE(10)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

},

ce-ModeB						SEQUENCE {

subPRB-Allocation-r16			BOOLEAN,

numRUs-r16						BOOLEAN,

prb-AllocationInfo-r16			BIT STRING (SIZE(8)),

mcs-r16							BIT STRING (SIZE(4)),

numRepetitions-r16				BIT STRING (SIZE(3))

}

}	OPTIONAL,	-- Need ON

pur-PUSCH-FreqHopping-r16		BOOLEAN,

p0-UE-PUSCH-r16					INTEGER (-8..7),

alpha-r16						Alpha-r12,

pusch-CyclicShift-r16			ENUMERATED {n0, n6},

pusch-NB-MaxTBS-r16				BOOLEAN,

locationCE-ModeB-r16			INTEGER (0..5)	OPTIONAL -- Cond SubPRB

}

PUR-RSRP-ChangeThreshold-r16 ::= SEQUENCE {

increaseThresh-r16				RSRP-ChangeThresh-r16,

decreaseThresh-r16				RSRP-ChangeThresh-r16	OPTIONAL		--Need OP

}

RSRP-ChangeThresh-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config field descriptions

alpha Parameter: αc(3). See TS 36.213 [23], clause 5.1.1.1.

hsfn-LSB-Info Indicates the LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

locationCE-ModeB PRB location within the narrowband when PUSCH sub-PRB resource allocation is enabled for PUR grant in CE mode B.

mpdcch-FreqHopping Frequency hopping activation/deactivation for MPDCCH. See TS 36.213 [23].

mpdcch-Narrowband Indicates the index of a narrowband on which the UE monitors for MPDCCH, see TS 36.213 [23], clause 9.1.5. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..maxAvailNarrowBands-r13-1) as specified in TS 36.211 [21].

mpdcch-NumRepetition Maximum number of repetitions levels for UE-SS for MPDCCH, see TS 36.213 [23].

mpdcch-Offset-PUR-SS Starting subframes configuration of the MPDCCH search space for PUR, see TS 36.213 [23].

mpdcch-PRB-PairsConfig Indicates the configuration of physical resource-block pairs used for MPDCCH. See TS 36.213 [23]. mpdcch-PRB-Pairs indicates the number of PRB pairs. Value n2 corresponds to 2 PRB pairs; n4 corresponds to 4 PRB pairs and so on. resourceBlockAssignment indicates the index to a specific combination of PRB pair for MPDCCH set. See TS 36.213 [23], clause 9.1.4.4.

mpdcch-StartSF-UESS Starting subframe configuration for an MPDCCH PUR search space, see TS 36.213 [23]. Value v1 corresponds to 1, value v1dot5 corresponds to 1.5, and so on.

n1PUCCH-AN Indicates UE-specific PUCCH AN resource offset, see TS 36.213 [23], clause 10.1.

p0-UE-PUSCH Parameter: P0_UE_PUSCH,c (3). See TS 36.213 [23], clause 5.1.1.1, unit dB.

pucch-NumRepetitionCE-Format1 Number of PUCCH repetitions for PUCCH format 1/1a, see TS 36.211 [21] and TS 36.213 [23]. When pur-GrantInfo is set to ce-ModeA, value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. When pur-GrantInfo is set to ce-ModeB, actual value corresponds to 4 * indicated value.

pusch-CyclicShift Parameter: ncs,λ. See TS 36.211 [21] clause 5.5.2.1.1. Value n0 corresponds to 0 and n6 corresponds to 6.

pusch-NB-MaxTBS Activation of 2984 bits maximum PUSCH TBS in 1.4 MHz in CE mode A, see TS 36.212 [22] and TS 36.213 [23].

pur-GrantInfo Indicates UL grant for transmission using PUR. Field set to ce-ModeA indicates the PUR grant is for CE Mode A and the field set to ce-ModeB indicates the PUR grant is for CE Mode B. numRUs indicates DCI field for PUSCH number of resource units, see TS 36.213 [23] clause 8.1.6. prbAllocationInfo indicates DCI field for PUSCH resource block assignment, see TS 36.212 [22], clause 5.3.3.1.10 (CE Mode A) and clause 5.3.3.1.11 (CE Mode B). mcs indicates DCI field for PUSCH modulation and coding scheme, see TS 36.213 [23] clause 8.6. numRepetitions indicates DCI field for PUSCH repetition number, see TS 36.213 [23] clause 8.0. For CE Mode A, numRUs set to '00' indicates use of full-PRB resource allocation, otherwise sub-PRB resource allocation as defined in TS 36.213 [23], clause 8.1.6. For CE Mode B, subPRB-Allocation indicates whether sub-PRB resource allocation is used.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release, as specified in 5.3.3.20. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions and so on.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PDSCH-FreqHopping Frequency hopping activation/deactivation for PDSCH. See TS 36.213 [23].

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-PUSCH-FreqHopping Frequency hopping activation/deactivation for PUSCH. See TS 36.213 [23].

pur-ResponseWindowTimer PUR MPDCCH search space window duration. See TS 36.321 [6] and TS 36.213 [23]. Value in subframes. Value sf240 corresponds to 240 subframes, value sf480 corresponds to 480 subframes and so on.

pur-RSRP-ChangeThreshold Indicates the threshold(s) of change in serving cell RSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB and so on. When pur-RSRP-ChangeThreshold is set to setup, if decreaseThresh is absent the value of increaseThresh is also used for decreaseThresh.

pur-TimeAlignmentTimer Indicates the idle mode TA timer in seconds for TA validation. Actual value = indicated value * PUR periodicity. If the field is not present the value of infinity shall be used.

| Conditional presence | Explanation |
| --- | --- |
| SubPRB | This field is optionally present, need ON, if subPRB-Allocation is set to TRUE; otherwise the field is not present and UE shall delete any existing value for this field. |

#### –	PUR-ConfigID

The IE PUR-ConfigID is used to indicate the PUR configuration identity.

PUR-ConfigID information element

-- ASN1START

PUR-ConfigID-r16 ::= BIT STRING (SIZE(20))

-- ASN1STOP

Next change

## 6.7	NB-IoT RRC messages

### 6.7.1	General NB-IoT message structure

-- ASN1START

NBIOT-RRC-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

RRCConnectionReestablishmentReject,

SecurityModeCommand,

SecurityModeComplete,

SecurityModeFailure,

AdditionalSpectrumEmission,

ARFCN-ValueEUTRA-r9,

CarrierFreqsGERAN,

CellGlobalIdEUTRA,

CellIdentity,

C-RNTI,

DedicatedInfoNAS,

DRB-Identity,

InitialUE-Identity,

IntraFreqBlackCellList,

IntraFreqNeighCellList,

I-RNTI-r15,

LocationInfo-r10,

maxAccessCat-1-r15,

maxBands,

maxCellBlack,

maxCellInter,

maxCellIntra,

maxFBI2,

maxFreq,

maxMultiBands,

maxNrofS-NSSAI-r15,

maxPageRec,

maxPLMN-r11,

maxSAI-MBMS-r11,

maxSIB,

maxSIB-1,

MBMS-SAI-r11,

MBMS-SAI-List-r11,

MBMSSessionInfo-r13,

NextHopChainingCount,

NG-5G-S-TMSI-r15,

PagingUE-Identity,

PLMN-Identity,

PLMN-IdentityList2,

P-Max,

PowerRampingParameters,

PreambleTransMax,

PhysCellId,

Q-OffsetRange,

Q-QualMin-r9,

Q-RxLevMin,

ReestabUE-Identity,

RegisteredAMF-r15,

RegisteredMME,

ReselectionThreshold,

ResumeIdentity-r13,

RRC-TransactionIdentifier,

RSRP-Range,

SetupRelease,

ShortMAC-I,

S-NSSAI-r15,

S-TMSI,

SystemInformationBlockType16-r11,

SystemInfoValueTagSI-r13,

T-Reordering,

TimeAlignmentTimer,

TimeSinceFailure-r11,

TMGI-r9,

TrackingAreaCode,

TrackingAreaCode-5GC-r15,

UAC-AC1-SelectAssistInfo-r15,

DataInactivityTimer-r14

FROM EUTRA-RRC-Definitions;

-- ASN1STOP

#### …

### 6.7.3	NB-IoT information elements

#### 6.7.3.1	NB-IoT System information blocks

#### –	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

NOTE:	UE timers and constants related to functionality for which parameters are provided in another SIB are included in the corresponding SIB.

SystemInformationBlockType2-NB information element

-- ASN1START

SystemInformationBlockType2-NB-r13 ::=	SEQUENCE {

radioResourceConfigCommon-r13			RadioResourceConfigCommonSIB-NB-r13,

ue-TimersAndConstants-r13				UE-TimersAndConstants-NB-r13,

freqInfo-r13							SEQUENCE {

ul-CarrierFreq-r13						CarrierFreq-NB-r13			OPTIONAL,	-- Need OP

additionalSpectrumEmission-r13			AdditionalSpectrumEmission

},

timeAlignmentTimerCommon-r13			TimeAlignmentTimer,

multiBandInfoList-r13	SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission		OPTIONAL,	-- Need OR

lateNonCriticalExtension				OCTET STRING					OPTIONAL,

...,

[[	cp-Reestablishment-r14				ENUMERATED {true}				OPTIONAL		-- Need OP

]],

[[	servingCellMeasInfo-r14				ENUMERATED {true}				OPTIONAL,		-- Need OR

cqi-Reporting-r14					ENUMERATED {true}				OPTIONAL		-- Need OR

]],

[[	enhancedPHR-r15						ENUMERATED {true}		OPTIONAL,	-- Need OR

freqInfo-v1530						SEQUENCE {

tdd-UL-DL-AlignmentOffset-r15		TDD-UL-DL-AlignmentOffset-NB-r15

}	OPTIONAL,		-- Cond TDD

cp-EDT-r15							ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-r15							ENUMERATED {true}		OPTIONAL	-- Need OR

]],

[[	earlySecurityReactivation-r16		ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-EDT-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-EDT-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-PUR-EPC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-PUR-EPC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

cp-PUR-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

up-PUR-5GC-r16						ENUMERATED {true}		OPTIONAL,	-- Need OR

rai-ActivationEnh-r16				ENUMERATED {true}		OPTIONAL	-- Need OR

]]

}

-- ASN1STOP

….

#### –	PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

...

}

PUR-NRSRP-ChangeThreshold-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config-NB field descriptions

alpha Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1.

carrierConfig Carrier used for PUR.

hsfn-LSB-Info LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

npdcch-Config NPDCCH configuration for PUR.

npusch-CyclicShift Parameter: ncs. See TS 36.211 [21], clause 10.1.4.1.2. Value n0 corresponds to value 0 and value n6 corresponds to value 6.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for PUR.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for PUR.

npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1.1, unit dB.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release of PUR configuration. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions, and so on.

pur-NRSRP-ChangeThreshold Threshold(s) of change in serving cell NRSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB, and so on. When pur-NRSRP-ChangeThreshold is set to setup, if decreaseThrsh is absent the value of increaseThresh is also used for decreaseThresh.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

pur-TimeAlignmentTimer Value of the time alignment timer for PUR. Value in number of periodicity of PUR. If the field is not present the value of infinity shall be used.

#### –	PUR-ConfigID-NB

The IE PUR-ConfigID-NB is used to indicate the PUR configuration identity.

PUR-ConfigID-NB information element

-- ASN1START

PUR-ConfigID-NB-r16 ::= BIT STRING (SIZE(20))

-- ASN1STOP