3GPP TSG-RAN WG2 Meeting #113e	Tdoc R2-2101556

E-meeting, Jan 25th – Feb 5th, 2021

Agenda Item:	9.2.4

Source:	Ericsson

Title:	Connection density evaluation for IoT NTN devices

Document for:	Discussion, Decision

# 1	Introduction

In RAN#86, a SI was approved to determine and evaluate the minimum necessary specifications to introduce NB-IoT/eMTC support for non-terrestrial networks (NTN). The description for the SI was updated in RAN#90 [1] and it was agreed to use the existing work on NR NTN captured in TR 38.821 [2] as a baseline. In this contribution, we discuss the evaluation for connection density for IoT NTN devices.

# 2	Connection density

In [3], the connection density was defined as the number of devices per unit area that fulfil a target quality of service (QoS) requirement to ensure a packet drop rate of 1% for a certain packet size and packet arrival rate. For IMT-2020 connection density requirement for massive IoT devices [4], a target connection density of 1 million devices per km2 was to be supported for the specified set of scenarios. In these scenarios, the inter-site distances of 500 m and 1732 m were considered.

The cell sizes in NTN are significantly larger than those considered while evaluating eMTC/NB-IoT connection density requirement in terrestrial cellular networks.

In [5], a projected device density for IoT NTN based on the population data in England was presented. For a 40 km NTN cell diameter, it was estimated that around 500,000 devices within the satellite coverage area of 1257 km2 will need to be supported in rural areas. This translates to a target device density of less than 400 devices per km2. For dense urban areas, the target device density was estimated to be around 26,000 devices per km2. The average target device density was estimated to be around 6600 devices per km2.

At the RAN2#112-e meeting, it was agreed to include the table including IoT NTN device densities for the use case of fixed devices in a TP for TR 36.763, where the values in the table are directly copied from TR 38.821 as agreed for IoT connectivity in Rel-16 NR NTN SI. It is however not clear whether it is feasible to achieve the target device density and thus proper evaluation is needed in this regard.

Evaluate whether it is feasible to achieve the target IoT NTN device density, which is copied from TR 38.821 to TR 36.763.

# 4	Conclusion

In this contribution we have discussed the evaluation for connection density for IoT NTN devices. In the previous sections we made the following observation:

Observation 1	The cell sizes in NTN are significantly larger than those considered while evaluating eMTC/NB-IoT connection density requirement in terrestrial cellular networks.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Evaluate whether it is feasible to achieve the target IoT NTN device density, which is copied from TR 38.821 to TR 36.763.

# References

RP-202689, Study on NB-IoT/eMTC support for Non-terrestrial Network, RAN#90, Dec 2020.

TR 38.821, Solutions for NR to support Non-terrestrial Networks (NTN), 3GPP, V16.0.0, Jan 2016.

3GPP TR 38.913, “Study on scenarios and requirements for next generation access technologies,” version 16.0.0, July 2020.

ITU-R, Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, October 2017.

R2-1901404, IoT Device Density Models for Various Environments, Vodafone, RAN2#105, Athens 2019.