3GPP TSG-RAN WG2 Meeting #113-e	   R2-2101562

Online, Jan 25th - Feb 5th, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Clarification on the Intra-band and Inter-band EN-DC Capabilities

Agenda item:		5.4.3

Document for: 	Discussion and Decision

- Introduction

In the last plenary meeting, some intra-band combination capability issues were discussed, e.g. singleUL-Transmission [1], BandwidthCombinationSet [2]. In this paper, we try to clarify the other intra-band and inter-band (NG)EN-DC related capabilities, including:

intraBandENDC-Support/dualPA-Architecture/ul-TimingAlignmentEUTRA-NR/asyncIntraBandENDC/pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/simultaneousRxTxInterBandENDC.

- Discussion

According to [1], there are different types of the band combinations with Intra-band component:

- Type A: Intra-band combination without any additional inter-band component, e.g. DC 41A_n41A

- Type B: Intra-band combination with additional inter-band component and supporting the intra-band UL configurations, e.g. DC_25A_41A_n41A

- Type C: Intra-band combination with additional inter-band component but not supporting the intra-band UL configurations e.g. DC_25A_41A_n41A

Besides the Type A/B/C, another BC type that has the intra-band EN-DC requirement is:

- Type D: Inter-band EN-DC configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. [3][4][5]

According to the conclusion of [2], RAN4 will confirm the classification of intra-band EN-DC band combinations as below:

B) Resolve the general question of classification of intra-band EN-DC band combinations according to UL support. If the UE doesn't support UL on intra-band EN-DC part of a band combination, is band combination classified as "intra-band EN-DC band combination"?

Obviously, once this was clarified, the following parameters shall also be clarified.

intraBandENDC-Support/dualPA-Architecture/ul-TimingAlignmentEUTRA-NR/asyncIntraBandENDC/pa-PhaseDiscontinuityImpacts/asyncIntraBandENDC/simultaneousRxTxInterBandENDC.

In the following chapters, we will discuss these parameters one by one. For the discussion convenience, in this paper, we divide the (NG)EN-DC combination into 5 types as below:

- The intra-band (NG)EN-DC: (NG)EN-DC band combinations support intra-band UL, including

Type 1: Intra-band (NG)EN-DC without inter-band component.

Type 2: Intra-band (NG)EN-DC with inter-band component, e.g. DC_25A_41A_n41A.

- The inter-band (NG)EN-DC: (NG)EN-DC band combinations only support inter-band UL, including

Type 3: Inter-band (NG)EN-DC without Intra-band component.

Type 4: Inter-band (NG)EN-DC with Intra-band component, e.g. DC_25A_41A_n41A.

Type 5:Inter-band (NG)EN-DC where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g. DC_B42_n77 and DC_B42_n78.

Note 1: supportedBandwidthCombinationSetIntraENDC/supportedBandwidthCombinationSet needs RAN4 further input, thus in this paper, we didn’t touch these 2 elements.

Note 2: The band combination type definition here are only for the discussion convenience, it also depends on the RAN4 further feedback.

Observation 1: the (NG)EN-DC combination can be divided into 5 types as below:

- The intra-band (NG)EN-DC: (NG)EN-DC band combinations support intra-band UL, including

Type 1: Intra-band (NG)EN-DC without inter-band component.

Type 2: Intra-band (NG)EN-DC with inter-band component, e.g. DC_25A_41A_n41A.

- The inter-band (NG)EN-DC: (NG)EN-DC band combinations only support inter-band UL, including

Type 3: Inter-band (NG)EN-DC without Intra-band component.

Type 4: Inter-band (NG)EN-DC with Intra-band component, e.g. DC_25A_41A_n41A.

Type 5:Inter-band (NG)EN-DC where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g. DC_B42_n77 and DC_B42_n78.

2.1 IntraBandENDC-Support

The definition of the intraBandENDC-Support is as below:

| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

This capability was introduced in [6] to distinguish the signaling between contiguous and non-contiguous intra-band (NG)EN-DC. Obviously, as shown in the Table 1 and Table 2, this capability shall be adopted by the type 1 and the type 2 (NG)EN-DC.

Table 1: Example for the Type 1 EN-DC

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| BC 1:DC_(n)41AA | DC_(n)41AA |
| BC 2: DC_41A_n41A | DC_41A_n41A |

Table 2: Example for the Type 2 EN-DC

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| BC 3: DC_25A-41A_n41A | DC_41A_n41A |
| BC 4: DC_25A-(n)41AA | DC_(n)41AA |

For the Type 4 BC, according to the 38101-3, there were also contiguous (e.g. BC6 in Table 3) and non-contiguous EN-DC (e.g. BC5 in Table 3) for the intra-band Part, thus this capability shall also be adopted for the type 4 BC. As shown in the Table 3, for both of the BC 5/6, the UE would indicate 3 bands in the band combination, furthermore, the UE shall also indicate intraBandENDC-Support if the UE support BC5 or both BC 5/6, otherwise, the network will only take the BC 6 as the supported BC.

Table 3: Example for the Type 4 EN-DC

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| BC 5: DC_25A-41A_n41A | DC_25A_n41A |
| BC 6: DC_25A-(n)41AA | DC_25A_n41A |

For the type 5, it will adopt another parameter as in [3][4][5].

| interBandContiguousMRDC Indicates for an inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]), that the UE supports intra-band contiguous (NG)EN-DC/NE-DC requirements (see TS 38.101-3 [4]). If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements. | BC | CY | No | No |
| --- | --- | --- | --- | --- |

Proposal 1: The intraBandENDC-Support is for Type 1/2/4 BC, the interBandContiguousMRDC is for the Type 5 BC.

2.2 ul-TimingAlignmentEUTRA-NR/dualPA-Architecture/pa-PhaseDiscontinuityImpact

The definition of the ul-TimingAlignmentEUTRA-NR/dualPA-Architecture/pa-PhaseDiscontinuityImpact is as below:

| ul-TimingAlignmentEUTRA-NR Indicates whether to apply the same UL timing between NR and LTE for dynamic power sharing capable UE operating in a synchronous intra-band contiguous (NG)EN-DC. If this field is absent, UE shall be capable of handling a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous (NG)EN-DC network, as specified in TS 38.133 [5]. If this capability is included in an inter-band (NG)EN-DC BC with an intra-band (NG)EN-DC BC part, this capability is used to indicate the restriction to the intra-band (NG)EN-DC BC part. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| dualPA-Architecture For an intra-band band combination, this field indicates the support of dual PAs. If absent in an intra-band band combination, the UE supports single PA for all the ULs in the intra-band band combination. For other band combinations, this field is not applicable. | BC | No | N/A | N/A |
| pa-PhaseDiscontinuityImpacts Indicates incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band (NG)EN-DC/NE-DC, intra-band CA and FDM based ULSUP. | FS | No | N/A | N/A |

These 3 capabilities are for the intra-band UL, obviously, it would not be adopted for the type 4.

Proposal 2: The ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts is for the Type 1/2 BC, and not for the Type 4 BC.

For the type 5, according to the RAN4 LS [3], RAN4 has agreed that intra-band EN-DC requirements shall apply for inter-band EN-DC configurations where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g., DC_B42_n77 and DC_B42_n78. Thus we think this parameter shall also be indicated for the Type 5 BC. Anyway, the confirmation from RAN4 is also needed before any conclusion.

Proposal 3: Confirm with Ran 1/4 whether the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts shall be adopted for the Type 5 BC.

2.3 asyncIntraBandENDC

The definition of the asyncIntraBandENDC is as below:

| asyncIntraBandENDC Indicates whether the UE supports asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. If asynchronous FDD-FDD intra-band (NG)EN-DC is not supported, the UE supports only synchronous FDD-FDD intra-band (NG)EN-DC. | BC | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |

This capability is for asynchronous FDD-FDD intra-band (NG)EN-DC with MRTD and MTTD, thus it shall be only adopted for 1/2.

Proposal 4: The asyncIntraBandENDC is for Type 1/2 BC.

2.4 Inter-band EN-DC parameters

The definition of simultaneousRxTxInterBandENDC is as below:

| simultaneousRxTxInterBandENDC Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band (NG)EN-DC/NE-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Note: There is a reply LS [7] from RAN4 on this capability, but it focus on the NR-DC, to avoid confusion, we only focus on (NG)EN-DC/NE-DC in this paper.

This parameter was introduced for the case that the TDD band receiver can’t suppress the interface from the transmitter. Obviously, this parameter is not for the Type 1 BC, and it shall be adopted by the Type 3 BC. For the type 2 and type 4 BC, the transmission on one band may also introduce unacceptable interference to the receiving band, thus this parameter shall also be adopted for the type 2/4 BC

For the type 5 BC e.g. DC_B42_n77 and DC_B42_n78, considering that the same TDD DL_UL_pattern would be adopted for the two bands, thus this capability is not needed for the type 5 BC. Anyway an LS can be sent to RAN4 for the further confirmation.

Proposal 5: The simultaneousRxTxInterBandENDC is for Type 2/3/4 and send a LS to RAN4 to confirm whether it is needed also for the type 5 BC

Based on the proposal 1~5, we can find that the “intra-band or inter-band (NG)EN-DC/NE-DC” in the current field descriptions are not consistent, e.g. “intra-band (NG)EN-DC/NE-DC” in the field description of intraBandENDC-Support shall include Type 1/2/4 BC, while in the field description of the ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts it shall not include the type 4 BC. These issues can be solved by given more detail description in each field description. A draft CR was also provided for the clarification of the BC types for each Intra-band and Inter-band (NG)EN-DC capabilities.

Proposal 6: Agree the CR in [8][9].

For the type 5 BC, which kinds of Intra-band and Inter-band (NG)EN-DC capabilities shall be adopted, a LS can be sent to RAN4 first.

Proposal 7: Agree the LS to RAN1/4 in [10].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: the (NG)EN-DC combination can be divided into 5 types as below:

- The intra-band (NG)EN-DC: (NG)EN-DC band combinations support intra-band UL, including

Type 1: Intra-band (NG)EN-DC without inter-band component.

Type 2: Intra-band (NG)EN-DC with inter-band component, e.g. DC_25A_41A_n41A.

- The inter-band (NG)EN-DC: (NG)EN-DC band combinations only support inter-band UL, including

Type 3: Inter-band (NG)EN-DC without Intra-band component.

Type 4: Inter-band (NG)EN-DC with Intra-band component, e.g. DC_25A_41A_n41A.

Type 5:Inter-band (NG)EN-DC where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band, e.g. DC_B42_n77 and DC_B42_n78.

Proposal 1: The intraBandENDC-Support is for Type 1/2/4BC, the interBandContiguousMRDC is for the Type 5 BC.

Proposal 2: The ul-TimingAlignmentEUTRA-NR/ ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts is for the Type 1/2 BC, and not for the Type 4 BC.

Proposal 3: Confirm with Ran 1/4 whether the ul-TimingAlignmentEUTRA-NR ul-dualPA-Architecture/ pa-PhaseDiscontinuityImpacts shall be adopted for the Type 5 BC.

Proposal 4: The asyncIntraBandENDC is for Type 1/2 BC.

Proposal 5: The simultaneousRxTxInterBandENDC is for Type 2/3/4 and send a LS to RAN4 to confirm whether it is needed also for the type 5 BC

Proposal 6: Agree the CR in [8][9].

Proposal 7: Agree the LS to RAN1/4 in [10].

- Reference

- RP-202932    LS on single UL operation   3GPP Ran

- RP-202935   LS on BCS reporting and support for intra-band EN-DC band combinations  3GPP Ran

- R2-2000034 LS on UE capability of intraBandENDC-Support RAN4 to RAN2

- R2-2002349	UE capability of intra-band requirements for inter-band EN-DC/NE-DC	NTT DOCOMO, INC.	CR	Rel-15	38.331	15.8.0	1501	1	F	NR_newRAT-Core	Late

- R2-2002350	UE capability of intra-band requirements for inter-band EN-DC/NE-DC	NTT DOCOMO, INC.	CR	Rel-15	38.306	15.8.0	0259	1	F	NR_newRAT-Core	Late

- R2-1818899 CR on signaling contiguous and non-contiguous EN-DC capability  Intel Corporation, T-Mobile USA

- R4-2016988  Draft LS on simultaneous Rx/Tx capability  3GPP RAN4

- R2-2101563	CR on the Intra-band and Inter-band EN-DC Capabilities - R15	ZTE, Sanechips

- R2-2101564	CR on the Intra-band and Inter-band EN-DC Capabilities - R16	ZTE, Sanechips

- R2-2101565	Draft LS on the Intra-band and Inter-band EN-DC Capabilities	ZTE, Sanechips