3GPP TSG-RAN WG2 Meeting #113 electronic 		R2-2101649

Online Meeting, January 25th – February 5th, 2021

Agenda item:	8.1.2.1 Reliability

Title:	On reliability enhancement for NR multicast and broadcast

Source:	Convida Wireless

Document for:	Discussion

# Introduction

At RAN #86, a work item on supporting of multicast and broadcast services was approved [1] which was further updated at RAN #88-e as [2]. The objectives of the WI include:

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

In the RAN1 #102e & 103e meetings, the following agreements were reached on reliability enhancement [3][4]:

RAN1 #102e:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

RAN1 #103e:

- For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

  - ACK/NACK based HARQ-ACK feedback for multicast,

  - NACK-only based HARQ-ACK feedback for multicast,

  - To decide in RAN1#104-e whether or not to support only one or both of the above schemes

- Enabling/disabling HARQ-ACK feedback for MBS is supported

RAN2 #111e

- R2 expect that there may be HARQ with feedback (for PTM) and this is specified by R1

RAN2 #112e

- The reordering and in-order delivery function in PDCP is supported for NR MBS.

- The following PDCP functions are also supported for NR MBS: transfer of data; maintenance of PDCP SNs; duplicate discarding. Other PDCP functions are FFS.

- RLC AM is supported for PTP transmission of NR MBS.

In RAN2 #111e & RAN2 #112e meetings, following agreements related to reliability were made:

In this contribution, we provide our views on the potential reliability enhancement to support the NR multicast and broadcast.

# Discussion

According to the WID [1, 2], reliability is a significant requirement for NR MBS and reliability can be achieved via feedback and retransmission. Traditional feedback and retransmission mechanisms consist of: HARQ, ARQ and PDCP status reporting. During the RAN1 #111e meeting, it was agreed that HARQ is up to RAN1.

There is a clear majority view in RAN2 based on the email discussion Post111-e 904, that PDCP feedback and retransmission is needed for NR MBS. PDCP feedback and retransmissions is necessary to meet the reliability requirements of some use cases. Especially in situations resulting from mobility for example RLC AM handover, or situations resulting from PTP/PTM bearers switching for example lossless switching between PTM and PTP bearers.

Proposal 1: PDCP retransmission is supported for AM MRB.

Additionally, in support of reliability requirements, the following is proposed.

Proposal 2: The PDCP layer supports the following functions:

- timer based SDU discard;

- for split bearers, routing;

- duplication;

It is also our view  that in cases where the RAN delivery method is PTM, there will be use cases where the use of RLC UM and L1 HARQ will not be able to meet the reliability requirements, and the support for RLC AM is needed in order to correct HARQ residual errors in these cases.

Proposal 3: RLC AM is supported for PTM transmission of NR MBS.

In the RAN1 #102-e meeting, it has been agreed that HARQ feedback is supported for NR multicast. In rel-16 V2X, two HARQ feedback schemes, i.e., ACK-NACK based HARQ feedback and NACK only based HARQ feedback, were agreed for groupcast case. However, both schemes have pros and cons.

For the ACK-NACK based HARQ feedback scheme, each UE will have dedicated resource to send HARQ feedback. So, the gNB can know which UEs send ACK and which UEs send NACK. However, such benefit either comes with the price of low scheduling flexibility, e.g., as what has been agreed for NR V2X, the association between the PSSCH and PSFCH is fixed and the gNB can’t schedule the PSFCH resources for each UE freely; or comes with the price of high signaling overhead, e.g., if the gNB wants to flexibly schedule the resources for each UE to send feedback, it will require very large amount of bits in the DCI. Since each UE will be allocated with dedicated resource, the resources needed to send the feedback is much higher compared to the NACK only based scheme. Also, the ACK-NACK based scheme doesn’t work well when the gNB doesn’t know how many UEs are within the group.

For the NACK only based HARQ feedback scheme, a shared resource is allocated for all the UEs to send NACK. By doing this, the resource required is much lower and the signaling overhead is also very low. However, the gNB cannot know which UEs send the NACK. A more severe problem is if the UE missed the scheduling DCI or SCI, the UE will not send any feedback. From the transmitter side, since it doesn’t receive any feedback, it will assume the transmission was successful. As a result, there will be a misunderstanding between the transmitter and the receiver, which will cause trouble to the system.

Considering the pros and cons of the above two schemes, other HARQ feedback schemes may be also considered for NR multicast. For example, one candidate may be shared ACK-NACK based HARQ feedback. In the shared ACK-NACK based scheme, two resources are allocated to the UEs, one is shared by all the UEs to send ACK and the other one is shared by all the UEs to send NACK. Using this scheme, it can solve the misunderstanding issue occurring in NACK only based case without increasing the signaling overhead and the resource required a lot.

Proposal 4: Different HARQ feedback schemes should be considered for NR multicast for the UEs in RRC_CONNECTED state. Shared ACK-NACK based scheme can be considered as another alternative of the NACK only based scheme.

In some NR broadcast use cases, e.g., public safety, maintaining high reliability is also very important. Supporting HARQ feedback for NR broadcast to improve the reliability may be considered, where such procedure may be based on best effort.

Proposal 5: Supporting HARQ feedback for NR broadcast service to improve the reliability can be considered for the UEs in RRC_CONNECTED state.

In NR multicast and broadcast, different services with different reliability requirements may be provided. When the reliability requirement of a service is low, HARQ feedback may not need to be supported. Therefore, it has been agreed that gNB has the flexibility to enable and disable the HARQ feedback for different services. We propose that both static enabling/disabling and dynamic enabling/disabling should be considered.

Proposal 6: Both static and dynamic enabling and disabling of HARQ feedback should be supported in NR multicast and broadcast.

# Conclusion

In this contribution, we have the following proposals.

Proposal 1: PDCP retransmission is supported for AM MRB.

Proposal 3: The PDCP layer supports the following functions:

- timer based SDU discard;

- for split bearers, routing;

- duplication.

Proposal 3: RLC AM is supported for PTM transmission of NR MBS.

Proposal 4: Different HARQ feedback schemes should be considered for NR multicast for the UEs in RRC_CONNECTED state. Shared ACK-NACK based scheme can be considered as another alternative of the NACK only based scheme.

Proposal 5: Supporting HARQ feedback for NR broadcast service to improve the reliability can be considered for the UEs in RRC_CONNECTED state.

Proposal 6: Both static and dynamic enabling and disabling of HARQ feedback should be supported in NR multicast and broadcast.

# References

- RP-193248, “New Work Item on NR support of Multicast and Broadcast Services”, Huawei, RAN #86, Spain, December 9th – 12th, 2019

- RP-201038, “WID revision: NR Multicast and Broadcast Services”, Huawei, RAN #88-e, E-meeting, June 29 - July 3, 2020

- Chairman Notes, RAN1 #102-e, August 17th – 28th, 2020

- Chairman Notes, RAN1 #103-e, October 26th – November 13th, 2020