3GPP TSG-RAN WG2 Meeting #113 electronic					R2-2101650

Online Meeting, January 25th – February 5th, 2021

Agenda Item:	8.15.3 Resource allocation enhancements RAN2 scope

Title:	On Resource Allocation Mode 2 Enhancement for NR Sidelink

Source:	Convida Wireless

Document for:	Discussion

# Introduction

At RAN-86, a new work item on NR sidelink enhancement was approved. At RAN-89e, the revised WID for NR sidelink enhancement was approved [1]. The objectives of the WI include:

Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

- Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2]

  - Inter-UE coordination with the following.

    - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission.

  - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.

In this contribution, we consider and discuss the aspects of resource allocation for NR sidelink enhancement. Particularly, we consider the aspects of resource allocation mode 2 enhancement.

# Consideration on Reliability for Resource Allocation Mode 2

In Rel-16, NR sidelink resource allocation mode 2 performs autonomous resource selection. Such autonomous resource selection is based on resource sensing procedure. The resources that are sensed not to be available, for example, the sensed resources that have higher SL-RSRP and higher priority, could be excluded from the resource selection during resource selection procedure while the resources that are sensed to be available, for example, the sensed resources that have lower SL-RSRP and lower priority could be included in resource selection during resource selection procedure for the Tx UE. Different from resource allocation mode 1, the performance of resource allocation mode 2 is also determined by the performance of resource sensing. If resource sensing performance is not reliable enough, the total reliability of resource allocation mode 2 could be affected. Therefore, as compared to resource allocation mode 1, additional enhancement for resource allocation mode 2 should be considered.

When resource sensing procedures result in resource collisions for resource allocation mode 2, such collisions result in overall NR sidelink performance degradation for mode 2. If PSCCH decoding performance suffers, this could result in the UE not decoding PSCCH successfully. PSCCH decoding is part of resource sensing procedure. Failure of PSCCH decoding may cause inaccurate sensing results, which in turn may cause inaccurate resource selection. Inaccurate or incorrect resource selection may result in resource collisions. This could cause additional problem for reliability for resource allocation mode 2. In order to support URLLC-like use cases, reliability for resource allocation mode 2 should be evaluated and enhancement for resource allocation mode 2 should be considered.

Proposal 1: Reliability for NR sidelink resource allocation mode 2 should be evaluated and enhancement of reliability for resource allocation mode 2 should be investigated.

# Consideration on Latency for Resource Allocation Mode 2

NR sidelink resource allocation mode 2 performs autonomous resource selection which is based on resource sensing procedure. If resource sensing is not perfect or not accurate enough, collisions for resources during resource selection procedure may occur. When Tx UE selects the resources that are not appropriate for Rx UE, interference at the Rx UE could be high and detection of PSCCH and PSSCH could be poor and even fail. If too many collisions occur, not only reliability for NR sidelink communication using resource allocation mode 2 could be affected, but also there would be impact on latency for NR sidelink communication using resource allocation mode 2 as well. When resource collision occurs, the PSCCH and/or PSSCH may not be received successfully. This could have an impact on the detection of the 2nd stage SCI as well as PSSCH reception. This could result in additional retransmissions for HARQ procedure or blind retransmission. Therefore, resource allocation mode 2 should be evaluated whether there is significant impact on the latency especially for supporting URLLC-like latency. Latency for NR sidelink resource allocation mode 2 should be further evaluated. Reduction of latency for resource allocation mode 2 should be studied. Reduction for latency for NR sidelink resource allocation mode 2 should be studied considering URLLC-type sidelink use cases for low latency application.

Proposal 2: Latency for NR sidelink resource allocation mode 2 should be evaluated and reduction of latency for resource allocation mode 2 should be studied.

# Consideration on Inter-UE Coordination for Resource Allocation Mode 2

Since NR sidelink resource allocation mode 2 needs to perform additional procedures such as resource sensing procedures for resource (re-)selection and scheduling, there could be performance impact on either reliability, latency or both as described previously. Some methods may be needed to mitigate such impact on reliability or latency. Inter-UE coordination may be a way to mitigate the impact on reliability and latency. Inter-UE coordination could allow collaboration between UEs for resource sensing procedures and resource selection procedure as well as scheduling. It could potentially reduce the resource collisions and mitigate incorrect resource selection.

In one case Tx UE and Rx UE may have different interferences from other surrounding UEs or nodes, this may result in different resource sensing results. Therefore, inter-UE coordination may mitigate such interferences with the help or assistance of the other UE(s). For example, Rx UE may provide assistance information to Tx UE, or Tx UE may provide assistance information to Rx UE. By doing so, resource sensing performance may be improved, and resource selection performance could be enhanced.

In addition, Tx UE could schedule other UEs for transmitting or reception. In another case, if a UE can schedule another UE or UEs, e.g., a group of UEs, then there would be no collision at least between the UEs within that group. This is because UEs can be scheduled not to collide with one another within that group. In general, this could result in better reliability and lower latency for resource allocation mode 2 for NR sidelink. This could also result in lower power consumption for UEs since UEs that are scheduled may not need to perform sensing at all. Therefore, inter-UE coordination should be considered and studied for performance enhancement for reliability and/or latency as well as reduction for power consumption

For inter-UE coordination a set of resources determined at UE-A could be sent to UE-B. UE-B can take this information into account in the resource selection for its own transmission. We may consider a set of resources that is preferred for UE-B’s transmission, e.g., resource set which is preferred for UE-A’s reception. In addition, we may also consider resource set which is preferred not to be used by UE-B’s transmission, e.g., resource set that is not preferred for UE-A’s reception. In case of groupcast we may also need to consider a set of resources that is preferred for other intended receivers of UE-B’s transmission. In addition, we may also consider resource set that is not preferred for other intended receiver(s) of UE-B’s transmission.

The set of resources could be carried either at the physical layer or at higher layers. How to determine which/what channel to carry the information for a set of resource could depend on the payload size, priority and/or latency. For example, if payload size of the information for a set of resource is small, the set of resources could be carried at the physical layer, e.g., in SCI. On the other hand, if payload size is large, the set of resources could be carried at the higher layer. Alternatively, if payload size is small, the set of resources could be carried in PSCCH. If payload size is large, the set of resources could be carried in PSSCH.

Either periodic, aperiodic, semi-persistent methods to transmit the set of resources could be considered. The set of resources may be requested by UE B. UE B may request different types of resources or information. For example, UE B may request the set of resources that is good to avoid hidden-node problem, exposed-node problem, half duplex problem, or combination of them, etc. The coordination information may be requested by UE B. UE B may request different types of coordination information. For example, different types of coordination information could include scheduling type or assistance type information.

UE-B could take a set of resources into account in the sensing and resource (re-)selection procedures for its own transmission. Other assistance and/or coordinating information other than a set of resources could be supported if needed. UE B may request different type of information including the set of resources with and without other assistance information.

Inter-UE coordination can be supported in all cast types including unicast, groupcast and broadcast to enable the enhancement for reliability and latency for different cast types. At least unicast and groupcast should be supported. Different coordination signaling or information could be used for different cast types. A unified coordination signaling or information could also be developed and used.

When a set of resources determined at UE-A is sent to UE-B in mode 2, UE-B can take into account the information in the sensing and/or resource selection for its own transmission. UE-A could send information of a set of resources to UE-B based on explicit signaling, e.g., trigger or request signaling. UE-A could send information of a set of resources to UE-B based on implicit indication, e.g., based on a pre-defined or (pre)configured triggering condition(s). Combination of both implicit indication and explicit signaling could also be considered. UE A could send coordination signaling or coordination information based on triggering, request signal and/or predefined or (pre-)configured conditions.

Coordination signaling or coordination information may contain a set of resources and other assistance information. For example, UE A could send coordination signaling or coordination information including information for a set of resources periodically e.g., in PSCCH, aperiodically e.g., in PSSCH, or semi-persistently. Whether to use periodic or aperiodic could depend on payload size of coordination signaling or coordination information. For example, if payload size of coordination signaling or coordination information is large, then aperiodic transmission or reporting of coordination signaling or coordination information could be used. On the other hand, if payload size of coordination signaling or coordination information is small, then periodic transmission or reporting of coordination signaling or coordination information could be used.

Coordination information could be of different type(s) including scheduling type info or assistance type info for inter-UE coordination for mode 2. If assistance type inter-UE coordination is used, assistance type coordination information could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination. If scheduling type inter-UE coordination is used, scheduling type coordination information could include the exact resource that is used for scheduling UE B for inter-UE coordination. To support and enable scheduling type inter-UE coordination, an indicator could be included in SCI to distinguish between transmission and reception for scheduling the UE. If assistance type inter-UE coordination is used, assistance type coordination information could include other assistance info if needed in addition to a set of resources for inter-UE coordination

Proposal 3: Inter-UE operation with coordination information should be considered. Coordination information could be of different type(s) including scheduling type info or assistance type info for inter-UE coordination for mode 2.

Proposal 4: Assistance type info could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination.

Proposal 5: Assistance type info could include other assistance info if needed in addition to a set of resources for inter-UE coordination.

Proposal 6: Scheduling type info could include the exact resource that is used for scheduling UE B for inter-UE coordination.

Proposal 7: Which channel to carry a set of resources could be determined based on payload size of information.

Proposal 8: Whether to use periodic or aperiodic to transmit a set of resources could be determined based on payload size of information.

Proposal 9: An indicator could be included in SCI to distinguish between transmission and reception for scheduling the UE.

Proposal 10: At least unicast and groupcast should be considered for inter-UE coordination.

# Conclusions

In this contribution, we have the following proposals:

Proposal 1: Reliability for NR sidelink resource allocation mode 2 should be evaluated and enhancement of reliability for resource allocation mode 2 should be investigated.

Proposal 2: Latency for NR sidelink resource allocation mode 2 should be evaluated and reduction of latency for resource allocation mode 2 should be studied.

Proposal 3: Inter-UE operation with coordination information should be considered. Coordination information could be of different type(s) including scheduling type info or assistance type info for inter-UE coordination for mode 2.

Proposal 4: Assistance type info could include a set of resource that is provided by UE A and could be used for sensing and/or resource (re-)selection at UE B for inter-UE coordination.

Proposal 5: Assistance type info could include other assistance info if needed in addition to a set of resources for inter-UE coordination.

Proposal 6: Scheduling type info could include the exact resource that is used for scheduling UE B for inter-UE coordination.

Proposal 7: Which channel to carry a set of resources could be determined based on payload size of information.

Proposal 8: Whether to use periodic or aperiodic to transmit a set of resources could be determined based on payload size of information.

Proposal 9: An indicator could be included in SCI to distinguish between transmission and reception for scheduling the UE.

Proposal 10: At least unicast and groupcast should be considered for inter-UE coordination.

# References

RP-202846, WID revision: NR sidelink enhancement

“RAN1 Chairman’s Notes”, RAN1#102-e.