3GPP TSG-RAN WG2 Meeting #113 electronic							    	R2-2101680

E-Meeting, 25th Jan – 5th Feb, 2021

Source: 	Beijing Xiaomi Mobile Software

Title:  	Slow-moving PDCP reception window issue

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

# Introduction

According to discussion on the L2 architectures for the MBS in the RAN2#112e meeting, the reordering and the in-order delivery function in PDCP is supported [1]:

RoHC is located at PDCP. The reordering and in-order delivery function in PDCP is supported for NR MBS. The following PDCP functions are also supported for NR MBS: transfer of data; maintenance of PDCP SNs; duplicate discarding. Other PDCP functions are FFS.

In this contribution, we discuss the drawbacks of the current PDCP reordering/reception window.

# Discussion

## PDCP reordering window

According to the NR PDCP specification [2], the receiving PDCP needs to maintain the following constants variables for the reception of the PDCP PDU:

- Window_Size: indicates the size of the reordering window. The value equals to 2[pdcp-SN-SizeDL] – 1.

- RX_NEXT: indicates the COUNT value of the next PDCP SDU expected to be received.

- RX_DELIV: indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for.

- RX_REORD: indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

Then the lower bound of the PDCP reordering window equals to “RX_DELIV”, and the higher bound of the PDCP reordering window equals to “RX_DELIV + Window_Size - 1”. According to the PDCP reordering procedure, the PDCP PDU with COUNT value smaller than the lower bound of the PDCP reordering window (i.e. RCVD_COUNT < RX_DELIV) is discarded, and the PDCP PDU with COUNT value larger than the higher bound of the PDCP reordering window is not allowed due to the HFN desynchronization.

Observation 1: Due to the HFN desynchronization, the PDCP PDU with COUNT value larger than the higher bound of the PDCP reordering window is not allowed in the NR PDCP.

Figure 1: PDCP re-ordering window for PTM

For the PTM transmission as illustrated in Figure 1, multiple UEs use its own PDCP reordering window for the reception of the PDCP PDU. Then we could have one reordering window moving much slower than the others. In order to avoid the HFN desynchronization issue, the transmitter would have to send the PDCP PDU within the slowest reordering window. Then the slowest re-ordering window would slow down the transmission speed of all UEs receiving the same MBS service. For the unicast transmission, this is not an issue, as the gNB can provide a dedicated PDCP configuration to the UE. For example, the PDCP configuration with a lower value of the PDCP reordering timer would have the faster moving of the PDCP reordering window. However it is difficult to configure a faster moving PDCP reordering window for a specific UE (e.g. a UE at the cell edge) at least for the delivery mode 2 of PTM, as the PDCP configuration which is included in the MCCH configuration is applicable to all UEs in the PTM cell.

Observation 2: In order to avoid the HFN desynchronization issue for PTM, the transmitter would have to send the PDCP PDU within the slowest reordering window of multiple UEs receiving the same MBS service.

Thus we consider that RAN2 should consider some solutions for the UE to temporarily accelerate the moving of its reordering window, in order to avoid impacting the reception data rate of other UEs receiving the same MBS service.

Proposal: The UE can temporarily accelerate the moving of the PDCP reordering window.

# Conclusions

According to the analysis given above, we have the following Observations and Proposals:

Observation 1: Due to the HFN desynchronization, the PDCP PDU with COUNT value larger than the higher bound of the PDCP reordering window is not allowed in the NR PDCP.

Observation 2: In order to avoid the HFN desynchronization issue for PTM, the transmitter would have to send the PDCP PDU within the slowest reordering window of multiple UEs receiving the same MBS service.

Proposal: The UE can temporarily accelerate the moving of the PDCP reordering window.

# Reference

[1] RAN2#112e meeting minutes

[2] 3GPP TS 38.323, “NR: Packet Data Convergence Protocol (PDCP) specification”.