3GPP TSG-RAN WG2 Meeting #113-e	R2-2101683

E-meeting, 25 January– 05 February, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.340 | CR | 0014 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to 38.340 for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the section 5.5, as to the handling of unknown, unforeseen, and erroneous protocol data: When a BAP PDU that contains reserved or invalid values or contains a BAP address which is not included in the configured BH Routing Configuration, and is not the BAP address of this node is received, the BAP entity shall: - discard the received BAP PDU.. First, we need to clarify that “BH Routing Configuration” should be per BAP entity configuration, which may be different at IAB-DU and IAB-MT. So, there is some ambiguity on the “not included in the configured BH Routing Configuration”, which could mean both the BAP entities at IAB-MT and IAB-DU, or could means the collocated BAP entity of the receiving BAP entity. In addition to discard data with reserved or invalid values, the intended BAP behaviors to discard erroneous data should be below: Transmitting side discarding; Receiving side discarding. #1: For upstream data at IAB-node; BAP should discard data with BAP address not included in its routing table.; N/A. #2: For downstream data at IAB-node; BAP should discard data with BAP address not included in its routing table.; N/A. #3: For upstream data at IAB-donor-DU; N/A; BAP should discard data with BAP address not matching the donor-DU’s BAP address.. #4: For downstream data at IAB-donor-DU; N/A; N/A. However, in the current specification, following erroneous data are the missing cases: - Upstream data arrives at the donor-DU, with BAP address in the header NOT mathcing the IAB-donor-DU’s BAP address, but BAP address included in the DL routing table. - Upstream data arrivies at the inter-mediate IAB-node, with BAP address in the header NOT matching the IAB-node’s BAP address, but BAP address included in the the DL routing table. In summary, the section 5.5 should capture all cases in the above table: For IAB-donor DU, the BAP receiving side should discard data with BAP address in the header NOT matching the IAB-donor’s BAP address, regardless whether the BAP adress is included in the DL routing table or not. For IAB-node, the BAP transmiting side should discard data with BAP address NOT included in the DL routing table, regardless whether the BAP address in the header is matching the IAB-node’s BAP address or not. The procedures of BAP entity are described in clause 5.2, 5.3, 5.4, and 5.5, but all clauses except for 5.2 are missing when describing the procedures of BAP entity in the BAP entity establishment related part in section 5.1.1. |
| Summary of change: |  | In section 5.1.1, add clauses 5.3, 5.4, 5.5 as reference for the procedures of BAP entity. In section 5.5, change as “At the trasmitting side, when a BAP PDU contains a BAP address which is not included in the configured BH Routing Configuration, or;At the receiving side of IAB-donor-DU, when a BAP PDU that contains a BAP address which is not the BAP address of this node is received, the BAP entity shall: - discard the BAP PDU.” Impact analysis Impacted functionality: BAP PDU discarding |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IAB-node/IAB-donor-DU will not discard some unknown, unforeseen, and erroneous BAP data PDUs. The established BAP entity will not perform the flow control, the BH RLF notification and the abnormal packet discarding proceudres. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

5.1.1	BAP entity establishment

When upper layers request establishment of a BAP entity, the node shall:

-	establish a BAP entity;

-	follow the procedures in clause 5.2, 5.3, 5.4, and 5.5.

Next Change

5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP PDU contains reserved or invalid values, or;

At the trasmitting side, when a BAP PDU contains a BAP address which is not included in the configured BH Routing Configuration, or;

At the receiving side of IAB-donor-DU, when a BAP PDU that contains a BAP address which is not the BAP address of this node is received, the BAP entity shall:

-	discard the BAP PDU.

End of Change