3GPP TSG-RAN WG2 Meeting #113 electronic 	R2-2101716

Online, Jan 25 - Feb 5, 2021

Agenda Item:	8.10.2.2

Source:	CMCC

Title:	Outstanding Left-Issues for HARQ operation in NTN

Document for:	Discussion, Decision

Introduction

In previous RAN2 meeting, there were some extensively discussions about enabling/disabling HARQ feedback [1] [2], and had achieved some conclusions. However, there are still some key issues left. In this contribution, we will elaborate the above left issues, and provide our proposals.

Discussion

Issue 1

RAN2 111e meeting reached the following agreement [3]:

- From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL

Currently, only the granularity of HARQ process for enabling/disabling HARQ feedback had been regarded as potential consensus of participants during email discussion. Moreover, the aspect of the necessity of the enabling / disabling of HARQ feedback can be configurable on a per Configured CG/SPS (Semi-Persistent Scheduling) basis via RRC signalling haven’t been touched in both online and email discussion yet, which had been identified and captured as reasonable issue in the TR 38.821 after the email and online discussion in SI phase, as follows:

Semi-Persistent Scheduling should to be supported for HARQ processes with enabled and disabled HARQ feedback. Details can be decided in the WI phase.

As we know, in NTN, the maximum round trip delay is 541.46ms for GEO and 25.77ms for LEO. Hence, configured grant/SPS can reduce the PDCCH load for DCI scheduling and decrease the latency for uplink resource application. One of the main benefits for NTN with configured grant is due to the large delays associated with UE getting uplink resources when the propagation delays are high. In a normal case, the UE has to go through the SR->Grant(for BSR)->BSR->Grant(for UL data) procedure in order to get sufficient uplink resources.

Observation 1: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

Proposal 1: RAN2 need to consider enabling / disabling the HARQ feedback of SPS configurations via RRC signaling.

There are two different types of uplink grants: type 1 is the method where the configured grant parameters, such as periodicity, MCS, the start offset in time domain and other scheduling information are configured via RRC only. And for type 2, which is similar as DL SPS, the information such as periodicity and HARQ-processes are configured by RRC and the start offset in time domain and other is indicated via DCI scrambled by CS-RNTI.

At the other hand, for configured grant, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

In the downlink, for Semi-persistent, the HARQ-processes ID is calculated based on the following formula:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

According to above formula, we can observe that the HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain. On the other hand, in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Observation 2: The HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain.

Observation 3: in Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Therefore, for UL CG, since in Rel-15, HARQ feedback has been ruled out instead of automatically re-transmission in configured repetition number, this means the configured start offset of CG cannot be selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

Observation4: The configured start offset of CG cannot selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

However in NR-U, HARQ feedback has been supported for CG data transmission, therefore in this scenario, mechanism of HARQ feedback enabling/disabling still needs to be taken into consideration.

Observation 5: In NR-U scenario, HARQ feedback enabling/disabling mechanism still needs to be considered in case of UL CG configured.

Proposal 2: RAN2 need to consider enabling/disabling the HARQ feedback of UL CG configurations via RRC signaling in case of NR-U scenario.

Regarding DL SPS, since the HARQ feedback does still work, the situation is more awkward. Hence, if the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signaling. Furthermore, as the bottleneck is the lack of buffer size for HARQ soft combining in UE side, the application of DL SPS seems more important in NTN system for the reduction of the PDCCH load for DCI scheduling overhead and the latency for data transmission.

Observation 6: If the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling.

Proposal 3: RAN2 need to consider the conflict between RRC configuration of enabling / disabling of HARQ feedback and SPS/configured grant configurations via RRC signaling.

Issue2

Since HARQ disabling will introduce more delay of data re-transmission which only relies on ARQ re-transmission, it is not recommended to apply HARQ disabling to the service required low latency. When there is multiple services of various service requirements in a given UE, it is very possible that not all the HARQ processes is configured in disabling mode. However, the MAC layer can not aware of whether the RLC PDU is from a segmented RLC SDU or not, in the other word, the MAC layer can not aware of whether the RLC PDU1 and RLC PDU2 is segmented from one RLC SDU or not. Then it is possible that MAC PDU1 from RLC PDU1 is scheduled in HARQ process1 which HARQ feedback is enabled, while MAC PDU2 from RLC PDU2 is scheduled in HARQ process2 which HARQ feedback is disabled. In case of data re-transmission occurs e.g. the radio condition is deteriorated, MAC PDU1 can operate data re-transmission via HARQ re-transmission, conversely, MAC PDU2 can only perform data re-transmission via ARQ re-transmission, which costs different transmission delay, resulting in huge gap during data reception.

Observation 7: In case of data re-transmission occurs, MAC PDU1 can operate data re-transmission via HARQ re-transmission, conversely, MAC PDU2 can only perform data re-transmission via ARQ re-transmission, which are constructed from one same RLC SDU, which costs different transmission delay, resulting in huge gap during data reception.

One potential enhancement to address this issue is to enable the MAC be aware of which RLC PDUs are segmented from one RLC SDU or not and scheduled in HARQ process in same HARQ feedback mode.

Proposal 4: it is proposed to enable the MAC be aware of which RLC PDUs are segmented from one RLC SDU or not and scheduled in HARQ process in same HARQ feedback mode.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some further discussions on the HARQ feedback for NTN and have the following observations and proposals:

Observation 1: Configured grant/SPS can avoid DCI scheduling and uplink resource application, which will largely reduce the scheduling delay.

Observation 2: The HARQ-processes ID of both CG and SPS are calculated from parameters of radio resource allocation in time domain, e.g. configured periodicity, start off in time domain, configured HARQ process number. This is different from dynamic scheduling, where there is no relationship between HARQ ID and radio resource allocation in time domain.

Observation 3: In Rel-16 industrial IoT/URLLC WID, multiple SPS/configured grant configurations per cell can be configured to a UE has already been agreed in RAN1 and RAN2.

Observation4: The configured start offset of CG cannot selected randomly, which must be in the scope of configured disabled HARQ process number. Or an additional note in the specification is needed to clarify that the configured disabled/enabled HARQ process number has no impact on the HARQ number used in UL CG.

Observation 5: In NR-U scenario, HARQ feedback enabling/disabling mechanism still needs to be considered in case of UL CG configured.

Observation 6: If the HARQ ID is been reserved by RRC, this means the start offset of SPS cannot be selected randomly. Hence, the enable / disable of HARQ feedback on a SPS configurations cannot be implemented by enable / disable of HARQ feedback on a per HARQ process basis via RRC signalling.

Observation 7: In case of data re-transmission occurs, MAC PDU1 can operate data re-transmission via HARQ re-transmission, conversely, MAC PDU2 can only perform data re-transmission via ARQ re-transmission, which are constructed from one same RLC SDU, which costs different transmission delay, resulting in huge gap during data reception.

Proposal 1: RAN2 need to consider enabling / disabling the HARQ feedback of SPS configurations via RRC signaling.

Proposal 2: RAN2 need to consider enabling/disabling the HARQ feedback of UL CG configurations via RRC signaling in case of NR-U scenario.

Proposal 3: RAN2 need to consider the conflict between RRC configuration of enabling / disabling of HARQ feedback and SPS/configured grant configurations via RRC signaling.

Proposal 4: it is proposed to enable the MAC be aware of which RLC PDUs are segmented from one RLC SDU or not and scheduled them in HARQ process in same HARQ feedback mode.

Reference

- R2-2008188 Summary of [AT111][107][NTN] Pre-compensation and other MAC issues

- R2-2008214 Summary of [AT111][107][NTN] Pre-compensation MAC issues Phase 2

- RAN2-111e - R16 eMIMO-CLI-PRN-RACS - R17 NTN-REDCAP (Sergio)_2020-08-28_final