3GPP TSG-RAN WG2 Meeting #113-e	R2-2101769

Electronic, 25th Jan – 5th Feb, 2021

Agenda item:	5.3.1

Source: 	Huawei, HiSilicon

Title: 	Further discussions on DRX InactivityTimer operations

Document for:	Discussion and Decision

- Introduction

R2-2010621	Activation of CG and DRX Inactivity Timer	Ericsson	discussion	NR_newRAT-Core

[002] Topic is Postponed, companies to check their implementation (expected next meeting)

In the last RAN2 e-meeting, the issue of whether the drx-InactivityTimer should be started when the received PDCCH activates a configured grant (CG) Type 2 was discussed. There were different views on if this kind of PDCCH indicates a new transmission, so the topic was postponed to this meeting [1].

R2-2001626	Clarification on the Operation of DRX Inactivity Timer	Apple	CR	Rel-15	38.321	15.8.0	0700	-	F	NR_newRAT-Core

=> Revised in R2-2002065

R2-2002065	Clarification on the Operation of DRX Inactivity Timer	Apple	CR	Rel-15	38.321	15.8.0	0700	1	F	NR_newRAT-Core

[AT109e][002][NR15] Majority’s view is Alt2, and one company prefers Alt1; There seems to be support to have a clarification, Companies can check the internal implementation. Can come back next meeting.

In RAN2#109-e, there was a discussion on this issue [2]. The majority’s view is Alt2, i.e., PDCCH indicating activation of DL SPS/UL configured grant triggers drx-InactivityTimer to (re-)start. The corresponding discussion is extracted as follows,

In this contribution, we further analyse this issue, and propose that a PDCCH activating CG Type 2 or DL SPS indicates a new transmission and the drx-InactivityTimer should be (re-)started.

- Discussion

2.1 Configured grant Type 2

In the last RAN2 e-meeting, there were diverging views on the issue whether a PDCCH activating CG Type 2 indicates a new transmission. As described in the highlight part of the spec, if PDCCH contents indicate CG Type 2 activation, the NDI bit for the corresponding HARQ process is considered to have been toggled. Then in the HARQ process, a new UL transmission will be performed.

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall: 1> if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell: 2> set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration; 2> if the configuredGrantTimer for the corresponding HARQ process is not running: 3> consider the NDI bit for the corresponding HARQ process to have been toggled; 3> deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

Therefore, it is clear that a PDCCH activating a CG Type 2 indicates a new transmission. In this case, the drx-InactivityTimer should be (re-)started following the statements in the specification.

1> if the MAC entity is in Active Time: 2> monitor the PDCCH as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission; 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 2> if the PDCCH indicates a new transmission (DL or UL): 3> start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

Observation 1: According to the spec, a PDCCH activating CG Type 2 indicates a new UL transmission.

2.2 Downlink SPS

For the downlink semi-persistent scheduling (SPS), according to the current MAC specification [3], if PDCCH content indicates SPS activation, the NDI bit for the corresponding HARQ process is considered to have been toggled . With the same logic as CG Type 2, it can be concluded that a new DL transmission will be performed and the drx-InactivityTimer should be (re-)started.

Observation 2: According to the spec, a PDCCH activating DL SPS indicates a new DL transmission.

2.3 Analysis of consequence

Based on the above analysis, from the perspective of specification, we believe that a PDCCH activating CG Type 2 or DL SPS indicates a new transmission and the drx-InactivityTimer should be (re-)started. In this section, we will further analyse the problem from the aspects of impacts on both NW and UE.

In the MAC specification of LTE, there is no description that a PDCCH activating a UL/DL SPS configuration does not indicate a new transmission. Since DRX has been running stably on the LTE network from a long time ago, we do not have motivations to change the UE’s behaviour. If we changed the operation for drx-InactivityTimer in this late stage of R15, it may bring problems to the existing UEs. On the other hand, NW activates CG Type 2 or DL SPS mostly due to new data arrival for transmission for allocating semi-persistent resources. Therefore, it is possible that the network may further schedule dynamic resources for the (re-)transmission of the UE for the concerned traffic or others, and the network may also re-send the activation command blindly for reliability. In this case, it is beneficial for the UE to keep awake for a while and hence starting drx-InactivityTimer enables more flexible scheduling.

Although not starting the drx-InactivityTimer might save UE’s power, the saved power could be negligible since it can be only used to reduce the power for the duration of one drx-InactivityTimer at most. In a summary, we think not starting the drx-InactivityTimer in such case has very limited benefits with the cost of losing scheduling flexibility of NW. Therefore, we propose that a PDCCH activating Type 2 CG or DL SPS indicates a new transmission and the drx-InactivityTimer should be started or restarted.

Proposal 1: RAN2 to clarify that a PDCCH activating SPS or CG Type 2 indicates a new transmission (DL or UL), and the drx-InactivityTimer should be started or restarted, as in LTE.

There is no need to change the spec. We think a clarification in the chairman notes is enough.

- Conclusion

In this contribution, we discussed the FFS point of drx-InactivityTimer and have the following observations and proposal.

Observation 1: According to the spec, a PDCCH activating CG Type 2 indicates a new UL transmission.

Observation 2: According to the spec, a PDCCH activating DL SPS indicates a new DL transmission.

Proposal 1: RAN2 to clarify that a PDCCH activating SPS or CG Type 2 indicates a new transmission (DL or UL), and the drx-InactivityTimer should be started or restarted, as in LTE.

- Reference

[1] R2-2100001, Report of 3GPP TSG RAN2#112-e meeting, Online, 2 - 13 November, 2020.

[2] R2-2002501, Report of 3GPP TSG RAN WG2 meeting #109-eOnline, 24 February - 06 March, 2020.

[3] TS 38.321 v15.11.0, NR Medium Access Control (MAC) protocol specification.