3GPP TSG-RAN WG2 Meeting #113-e 	R2-2101777

Online, 25th Jan – 5th Feb 2021

Agenda Item:		6.16

Source:				Huawei, HiSilicon

Title:		Discussion on PHR reporting for PUSCH skipping

Document for:	Discussion and Decision

- Introduction

The issue of PHR reporting for PUSCH skipping was re-discussed in the last RAN2 meeting in [1], and the conclusion is to postpone the discussion.

R2-2009482	Clarification on PHR reporting for PUSCH skipping	Apple	CR	Rel-16	38.321	16.2.1	0929	-	F	NR_newRAT-Core, TEI16

[003] Postponed

[003] The issues can be discussed only for Rel-16

This contribution intends to further discuss the issue based on past RAN2 discussions and recent RAN1 progress.

- Discussion

For the CA case, as illustrated in the following Figure 1, A CG-PUSCH or DG-PUSCH (enabled by skipUplinkTxDynamic) can be skipped if there is no MAC PDU assembled, in this case, how to calculate the PH value for each serving cell based on virtual or real needs to be determined.

Figure 1. PHR virtual/real determination

At RAN2#103bis meeting, this issue was discussed with the following conclusion that the UE shall always assume it is “real” at the time of determination of PH value for a serving cell with a grant. Some proponents hold the view that the UE is able to determine whether the PUSCH of the grant will be skipped or not and hence the UE is able to calculate the PH value based on the outcome of UL skipping.

PHR for skipped PUSCH R2-1814804 PHR for uplink grant skipping Ericsson discussion Rel-15 NR_newRAT-Core R2-1812390 => Noted R2-1814871 Impact of Uplink Grant Skipping on PHR Huawei, HiSilicon discussion Rel-15 => Noted R2-1814872 CR for Impact of Uplink Grant Skipping on PHR Huawei, HiSilicon CR Rel-15 38.321 15.3.0 0481 - F NR_newRAT-Core => Not treated R2-1814453 PHR for UL grant skipping LG Electronics Mobile Research discussion NR_newRAT-Core => Noted Discussion - Lenovo doesn’t understand why it is not accurate. - Qualcomm thinks that this proposal has a serious impact on UE and the UE only knows if it will skip until last minute. - LG supports Huawei. - Nokia would like to have the real Agreements: - At the time of determination of PH value for a serving cell, the UE MAC assumes real transmissions for all cells with grants even if any grant is skipped

However, the exact time point of MAC PDU assembly (i.e. UL skipping) is up to UE implementation, so it could be case that the UE only knows if the grant shall be skipped at the last minute, and the consequence would be that it is impossible to re-calculate the PH value and re-run the MAC PDU assembly. From the UE perspective, it may have a serious impact on the UE implementation, and that is the main reason why RAN2 assumed it is always “real” at the time of determination of PH vale.

In the last RAN1 meeting, the issue was also discussed for Rel-16 in the context of URLLC inter-UE prioritization, but without any conclusion. According to the summary of the offline discussion in [2], it seems that the majority view thinks RAN1 could follow the RAN2 previous agreements and no additional RAN1 discussion is necessary.

Agreement

The TP for TS 38.214 Clause 6.1.4 is endorsed in R1-2009478 (TS38.214, Rel-16, CR#0137, Cat. F)

[103-e-NR-L1enh-URLLC-06] Email discussion/approval on remaining issues on inter-UE multiplexing enhancements – Xueming (vivo)

- Issue 1: Impact to PHR calculation due to UL CI in UL CA and/or UL skipping

- Issue 2: Impact to UE power scaling due to UL CI in UL CA and/or UL skipping

- Discussion and decision by 10/29, TPs by 11/5

The email discussion was closed without any agreements or conclusions.

We understand that the main concern from RAN1 is also about the timeline issue for UL skipping. Specifically, the “Tproc,2”-based timeline should refer to the point in time when the UE PHY figures out if a PUSCH is skipped due to lack of a MAC PDU from MAC layer. However, how to define such in RAN1 specs is not clear since such timing is internal to the UE. From RAN2 perspective, we therefore propose not to re-open the discussion in RAN2 and can be revisited if RAN1 has any further input.

Observation: PHR calculation with UL skipping needs to discuss timeline for UL skipping and it should be in the RAN1 scope.

Proposal: RAN2 confirms that the Rel-15 PHR reporting for UL skipping is applicable to Rel-16 without any RAN2 spec change.

- Conclusions

In this contribution, we further discuss the PHR reporting for PUSCH skipping and have the following observation and proposal.

Observation: PHR calculation with UL skipping needs to discuss timeline for UL skipping and it should be in the RAN1 scope.

Proposal: RAN2 confirms that the Rel-15 PHR reporting for UL skipping is applicable to Rel-16 without any RAN2 spec change.

- Reference

[1] R2-2009482	Clarification on PHR reporting for PUSCH skipping	Apple

[2] R1-2009462	Summary of email discussion [103-e-NR-L1enh-URLLC-06]	Moderator (vivo)