3GPP TSG-RAN WG2 Meeting #113e	R2-2101787

Online, Jan 25 – Feb 5, 2021

Agenda item:       8.10.3.2

Source: 	Convida Wireless

Title: 	NTN Cell reselection and idle mode enhancements

Document for:	Discussion and Decision

# 1  Introduction

For Rel-17, a work item for NR to support NTN (Non-Terrestrial Networks) has been approved with some initial progress in RAN2. One of the primary RAN2 objectives for such enhancements is to address idle mode procedures as highlighted below:

The following control plane procedures enhancements should be specified (see TR 38.821)

- Idle mode:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

  - Definition of NTN (satellite/HAPS) cell specific information in SIB

RAN2 has made some initial agreements in RAN2#111e and RAN2#112e for control plane idle mode procedures as shown below:

1.	Cell selection / reselection in NR is the baseline in NTN idle mode procedure.

2.	Satellite/HAPS ephemeris based cell selection and reselection should be defined for NTN (FFS what the term satellite/HAPS ephemeris actually means). FFS when this ephemeris based cell selection / reselection can be used. FFS whether UE location (and/or other information) based cell selection and reselection should be introduced for NTN.

3.	The satellite ephemeris should be provided to UE, at least for Satellite/HAPS ephemeris based cell selection and reselection (FFS what the term satellite/HAPS ephemeris actually means).

4.	The network type (i.e. TN or NTN) should be known to UE. FFS whether to achieve this in an implicit or explicit way.

5.	The existing cell reselection priority configuration can be taken as a baseline in NTN. FFS on any further enhancement.

6.	Postpone the discussion on whether to introduce a new SIB until we have more progress on the content of NTN specific system information.

7.	Existing cell reselection principles are considered as baseline and that information about when a cell is going to stop serving the area and information about new upcoming cell can be further considered. In which form and how this is exactly implemented in the cell reselection principles is FFS.

Based on the above agreements, the objective of this paper is to progress these agreements further and address some of the FFSs related to cell reselection.

# 2	 Discussion

#### 2.1 Rationale for NTN Cell Reselection Enhancements

In this section, we discuss the open issue regarding cell reselection enhancements over the existing NR idle mode procedures, which should be used as a baseline per RAN2 agreements.

Specifically, RAN2 has agreed that Cell selection / reselection in NR is the baseline in NTN idle mode procedure. Additionally, RAN2 has agreed Satellite/HAPS ephemeris-based cell selection and reselection should be defined for NTN and consistent language in the NTN WID [1]:

  - Definition of additional assistance information for cell selection/reselection (e.g. using UE location information, satellite Ephemeris information)

To address dissimilar characteristics for NTN platforms versus existing TN platforms, such as uniform received power across NTN cell coverage, and mixed NTN/TN coverage and deployment scenarios, TS 38.304 will likely require potential enhancements.

Characteristics of NTN platforms deviate from existing TNs, including the relatively small changes in UE received signal strength from cell center to cell edge. Based on the Figure below, using measured RSRP/RSRQ of the NTN cells would not be sufficient criteria for cell reselection and lead to a high number of reselections (and power consumption) especially in the presence of multiple NTN cells (e.g. HAPS, NGSO and GEO) and TN platforms.

Figure 2.1-1 NTN vs TN Near-far effects [38.821]

Re-selection priority handling should be enhanced to address these issues. Specifically, cell ranking (R-criterion) is strictly designed for TNs, and not optimized for fast-moving NTN cells or cells (and neighbor cells) that lack near-far signal strengths, making cell re-selection challenging.

Additionally, TN rules to limit intra- and inter-frequency measurements (relaxed measurements) are not valid for NTN platforms. Furthermore, the criteria to determine mobility states in earth-moving cells deployments should be redefined as there will be high number of re-selections with moving cells. Mobility aspects in these cases will have an impact on measurement rules in these deployments as defined for TN and should also be enhanced from the existing procedures.

It can be envisioned that the network will prefer to establish priorities for cell reselection for the UEs in the presence of multiple TN/NTN platforms. The UE may also have requirements to operate on each of the NTN cell types (timing and frequency compensation requirements), therefore enhancements from existing TN methods are required.

Observation 1: Per WID and RAN2 agreements, existing NR cell reselection procedures should be enhanced for NTN with additional assistance information.

Observation 2: The measured RSRP/RSRQ of the NTN cells is not sufficient criteria for cell (re)selection in mixed deployments and mixed coverage.

Observation 3: Existing Rel-16 measurement rules and mobility states for TN are not sufficient for NTN platforms.

#### 2.2 Potential enhancements

#### 2.2.1 Cell Reselection Priorities Handling for NTN

Per Reselection priorities handling for TNs, per TS 38.304 [4]:

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection.

In the case of NTN-capable UEs, NTN-specific priorities for reselection may be provided to the UE in system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. These priorities may depend on several factors that do not depend on absolute NR frequency prioritization.

#### 2.2.2 Measurement rules for cell re-selection

Following rules are used by the UE to limit unnecessary measurements [4]:

-	If the TN serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

- Otherwise, the UE shall perform intra-frequency measurements.

Specifically for NTN serving cells, TN measurement rules should be enhanced.

-	If the NTN serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose to evaluate the type of NTN platform and coverage area/time, associated satellite ephemeris data, location to determine when to perform intra-frequency measurements.

- Otherwise, the UE shall perform intra-frequency measurements.

If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements [4]. Extended/enhanced relaxedMeasurement may be useful for the network to configure for geo-stationary (GEO) NTN platform deployments, where the NTN cell footprint may be quite large.

Observation 4: The existing TN cell reselection procedures have shortcomings for mixed deployment scenarios/coverage and should be enhanced in TS 38.304. Additionally, enhancements related to measurement rules and mobility states for NTN cell reselection should be investigated.

Proposal 1: RAN2 agrees to enhance TS 38.304 with additional assistance information and enhancements, including cell reselection priorities handling and measurement rules.

# 3  Conclusion

To summarize, the following observations and proposals were examined:

Observation 1: Per WID and RAN2 agreements, existing NR cell reselection procedures should be enhanced for NTN with additional assistance information.

Observation 2: The measured RSRP/RSRQ of the NTN cells is not sufficient criteria for cell (re)selection in mixed deployments and mixed coverage.

Observation 3: Existing Rel-16 measurement rules and mobility states for TN are not applicable for NTN platforms.

Observation 4: The existing TN cell reselection procedures have shortcomings for mixed deployment scenarios/coverage and should be enhanced in TS 38.304. Additionally, enhancements related to measurement rules and mobility states for NTN cell reselection should be investigated.

Proposal 1: RAN2 agrees to enhance TS 38.304 with additional assistance information and enhancements, using TN cell selection as a baseline.

# 4  References

[1] 3GPP RP-201256, "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)," September, 2020.

[2] 3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), (Release 16).

[3] 3GPP MCC, “Draft Report of 3GPP TSG RAN WG2 #112-e”

[4] 3GPP TS 38.304, User Equipment (UE) procedures in Idle mode and RRC Inactive state, (Release 16).