3GPP TSG-RAN WG2 Meeting #113 electronic	                                      R2-2101823

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.10.2.2

Source:			Asia Pacific Telecom, FGI

Title:		UE calculated TA report

Document for:	 	Discussion and decision

Introduction

In RAN2#112e meeting, UE-calculated TA report related issue was preliminarily discussed in the email discussion but without consensus, which is left FFS.

RAN2#112e Agreement

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

- At least the following are FFS in Rel-17 NTN:

- Report UE-calculated TA in e.g., msg3/msg5/msgA

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

- LCP impact caused by disabling HARQ UL retransmission

In this contribution, we would like to share our views on the UE-calculated TA report.

Discussion

Background of UE-calculated TA

In RAN1#103e, RAN1 agreed that NTN UE at least supports UE-calculated TA based on its GNSS-acquired position and the serving satellite ephemeris.

RAN1#103e Agreement An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

Observation 1: UE-calculated TA can be at least acquired based on UE’s GNSS-acquired position and the serving satellite ephemeris.

With this assumption, NW can broadcast the real-time position of a satellite (i.e., satellite ephemeris) via SIB to the UE. The UE could derive the current satellite position based on its last acquisition of the satellite ephemeris and some basic propagator model. Then the UE could acquire the propagation delay between the satellite and the UE. That is, the UE-calculated TA can be acquired by UE’s GNSS receiver and satellite’s position provided by NW, as shown in Figure 1. On the other hand, the common TA is defined as the common component of propagation delay shared by all UEs within the cell coverage and corresponds to the RTD between the reference point (RP) and the satellite. (Note that it is zero if the RP is located at the satellite, or equal to NTN GW to satellite RTD if RP is located at the NTN GW depending on the implementation). The RTD experienced between the gNB and the RP shall be compensated by the network, which is transparent to the UE.

As a consequence, the full TA can be compensated by UE-calculated TA and Common TA to ensure the DL/UL frame boundary alignment at the NW side.

Full TA = UE-calculated TA + Common TA = 2*T_u + 2*T_c

Figure 1: TA acquisition in NTN

Observation 2: Full TA can be compensated by UE-calculated TA and Common TA to ensure the DL/UL frame boundary alignment at the NW side

UE-calculated TA report

In NR, the gNB is responsible for maintaining the TA to keep the L1 synchronized, i.e., to ensure the DL/UL frame boundary alignment at the NW side. In the current mechanism, gNB can signal an absolute TA via the RAR and signal a delta TA via TA command MAC CE to maintain the TA of the UE.

In our view, it is beneficial for the subsequent scheduling if the UE could moderately report the UE-calculated TA to gNB for assistance. The UE-calculated TA information helps gNB to adjust the TA of the UE more precisely.

Observation 3: UE-calculated TA information helps gNB to adjust the TA of the UE more precisely.

Observation 4: Reporting UE-calculated TA has been identified as beneficial..

On top of that, we may need to further discuss some issues for the UE-calculated TA report.

- Issue 1: UE-calculated TA should be reported by whichsignal?

- Issue 2: What information should be included in the UE-calculated TA report?

- Issue 3: When to report the UE-calculated TA?

For issue 1, we think MAC CE can be the baseline to report the UE-calculated TA. One reason is to utilize the same layer’s signaling (i.e., MAC signaling) as the absolute TA by RAR and the TA command MAC CE. Moreover, MAC CE provides higher reliability and more space of fields than PHY signaling.

Proposal 1: UE-calculated TA can be reported by MAC CE.

For issue 2, the discussion on detailed information of UE-calculated TA is in the scope of RAN1. So far, RAN1 only concluded that the position-based solution is the baseline for UE-calculated TA, but haven’t discussed the exact information, e.g., the content, the size, etc. Thus, we can wait for their discussion on this.

Proposal 2: Wait for RAN1 discussion on the exact information (e.g., the content, the size, etc.) to be included in the UE-calculated TA report.

For issue 3, we consider the baseline is to report the UE-calculated TA via the RA procedure since the RA procedure can be performed when the UE is in RRC_IDLE and RRC_INACTIVE, and also, would be performed when the UE would like to enter RRC_CONNECTED. On the other hand, the RA procedure can provide an UL grant for the UE to report it (i.e., via Msg3, Msg4, or MsgA).

On the other hand, we have agreed that both 2-step and 4-step RACH are supported in NTN.

RAN2#111e Agreement

- Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

Consequently, when the UE initiates a RA procedure, the UE will select one of 2-step and 4-step RA types to perform the RA procedure (It is noted that the RA type selection criteria specifically for NTN are still under discussion).

If the UE selects the 4-step RA type, a UL grant is provided via RAR, which is used for scheduling Msg3. However, the size of Msg3 is not quite huge. For some RA triggering events, e.g., initial access or RRC connection re-establishment, the Msg3 should include the CCCH SDU (with RRC message), which may consume almost all the space of the Msg3. In these cases, the Msg3 may not be able to carry the UE-calculated TA report. Thus, in order to report the UE-calculated TA as soon as possible, the gNB should schedule a UL grant via Msg4, i.e., for Msg5. Then the UE-calculated TA report can be transmitted via Msg5.  Similarly, if the UE selects the 2-step RA type, the MsgA PUSCH can be used if the size is enough, or the MsgB should schedule a UL-SCH resource for UE-calculated TA report transmission.

Proposal 3: For 4-step RA, the UE-calculated TA report should be multiplexed in Msg3 if the size of the Msg3 is enough. Otherwise, the UE-calculated TA reported should be transmitted via Msg5.

Proposal 4: For 2-step RA, the UE-calculated TA report should be multiplexed in MsgA PUSCH if the size of the MsgA PUSCH is enough. Otherwise, the UE-calculated TA reported should be transmitted via a UL-SCH resource scheduled by MsgB.

Except for the RA procedure, some enhancements could be discussed for the reporting of UE-calculated TA report as well. For instance, in an explicit way, the gNB could request the UE to report it when it is needed from the NW point of view. What kind of indication is feasible to indicate this request could be further discussed. While the UE receives the request of the UE-calculated TA report, the UE should trigger the UE-calculated TA report and indicate the up-to-date UE-calculated TA information to gNB. In addition, the UE-calculated TA report may be needed to update periodically, to keep the real-time status between UE and gNB.

Proposal 5: The UE-calculated TA report can be requested by gNB. FFS how to request by gNB.

Proposal 6: The UE-calculated TA can be reported periodically. FFS how to report periodically.

Conclusion

Based on the above analysis, we have the following observations and proposals.

Observation 1: UE-calculated TA can be at least acquired based on UE’s GNSS-acquired position and the serving satellite ephemeris.

Observation 2: Full TA can be compensated by UE-calculated TA and Common TA to ensure the DL/UL frame boundary alignment at the NW side

Observation 3: UE-calculated TA information helps gNB to adjust the TA of the UE more precisely.

Observation 4: Reporting UE-calculated TA has been identified as beneficial..

Proposal 1: UE-calculated TA can be reported by MAC CE.

Proposal 2: Wait for RAN1 discussion on the exact information (e.g., the content, the size, etc.) to be included in the UE-calculated TA report.

Proposal 3: For 4-step RA, the UE-calculated TA report can be multiplexed in Msg3 if the size of the Msg3 is enough. Otherwise, the UE-calculated TA reported should be transmitted via Msg5.

Proposal 4: For 2-step RA, the UE-calculated TA report can be multiplexed in MsgA PUSCH if the size of the MsgA PUSCH is enough. Otherwise, the UE-calculated TA reported should be transmitted via an UL-SCH resource scheduled by MsgB.

Proposal 5: The UE-calculated TA report can be requested by gNB. FFS how to request by gNB.

Proposal 6: The UE-calculated TA can be reported periodically. FFS how to report periodically.

Reference

[1] R1-2009748 FL Summary on enhancements on UL time and frequency synchronization for NR

[2] R2-2010764 Summary of [AT112-e][103][NTN] RACH and HARQ feedback aspects