3GPP TSG-RAN WG2 Meeting #113 electronic	                                R2-2101833

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.10.2.1

Source:			Asia Pacific Telecom, FGI

Title:		Enhancements on RACH in NTN

Document for:	 	Discussion and decision

- Introduction

In RAN2#111e, the following agreements were achieved.

RAN2#111e Agreement

- Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

- At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded)

In RAN2#112e, an agreement for the enhancements to the RSRP-based selection mechanism of 2-step vs. 4-step RACH was left FFS.

RAN2#112e Agreement

- If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

- At least the following are FFS in Rel-17 NTN:

- Report UE-calculated TA in e.g., msg3/msg5/msgA

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

- LCP impact caused by disabling HARQ UL retransmission

In this contribution, we would like to share our views on

- Enhancement on RA type selection

- Enhancement on BSR over 2-step RACH

- Discussion

Enhancements on RA type selection

In NTN, both 2-step RA type and 4-step RA type will be supported. If both 2-step and 4-step RA type RA resources are configured in a serving cell, the legacy RA type selection for CBRA is determined by UE based on the RSRP measurement. The UE can select 2-step RA type if the RSRP of the DL pathloss reference is above a threshold since higher channel quality is desired for MsgA PUSCH transmission. However, due to no near-far effect in NTN, the UEs in the cell center and the UEs in the cell edge may have similar RSRP levels. Hence, all the UEs in a cell may either select 2-step RACH RA type or select 4-step RACH RA type only.

Observation 1: Since no near-far effect in NTN, measurement-based RA type selection leads to a single RA type.

On the other hand, Figure 1 shows the impact of elevation angles and the corresponding RSRP in a cell. Strong RSRP occurs when a satellite is on the top of a UE, and weak RSRP for a small elevation angle. This is because the Line-Of-Sight (LOS) probability is smaller as the elevation angle decreases, which increases the chance of blocking. Especially, for an earth-fixed cell, the RSRP qualities for all UEs in a cell may highly depend on the elevation angle between the satellite and the UE. In other words, the NW would experience a burst request from the UEs in a cell for either a single RA type of 2-step or 4-step RA.

Figure 1: elevation angles of a serving cell and the RSRP quality

Observation 2: A burst from the UEs to select either a single RA type of 2-step or 4-step RA would occur due to the change of elevation angle.

Proposal 1: In NTN, the RSRP threshold for RA type selection is not applied.

A simple solution is to randomly select a RA type with equal probability. However, it shall be a better way.

In comparison with 4-step RA, the benefit of 2-step RA is to reduce the latency caused by multiple round-trip delays in NTN, since there is only one round trip cycle between MsgA and MsgB, instead of two round trip cycles between Msg1 and Msg4. It is also well-known that the transmission latency would be higher if the distance between the UE and the satellite is longer. Therefore, one criterion for RA type selection can rely on the distance. Note that the distance between the UE and the satellite can be derived by UE based on UE’s GNSS-acquired position and satellite ephemeris. The satellite ephemeris information can be indicated by NW, e.g., via SIB. Also, RAN1 has agreed that the UE’s GNSS-acquired position and the satellite ephemeris information can be used to derive the UE-calculated TA. Thus, the UE-calculated TA may be good information for RA type selection.

RAN1#103e Agreement

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

Proposal 2: In NTN, an RA type selection criterion can be based on UE-calculated TA.

Enhancements on BSR over 2-step RACH

For UL scheduling enhancement, we have agreed that BSR over 2-step RA is one of the methods to reduce RTD compared to the legacy SR-BSR procedure.

Based on the current mechanism, basically, if the UE determines that a BSR (i.e., Regular BSR) has been triggered and not cancelled, the UE will further trigger an SR in a case that there is no UL-SCH resource available.

TS 38.321 v.16.3.0 The MAC entity shall: 1> if the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled: 2> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the BSR MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the BSR MAC CE(s) as defined in clause 6.1.3.1; 3> start or restart periodicBSR-Timer except when all the generated BSRs are long or short Truncated BSRs; 3> start or restart retxBSR-Timer. 2> if a Regular BSR has been triggered and logicalChannelSR-DelayTimer is not running: 3> if there is no UL-SCH resource available for a new transmission; or 3> if the MAC entity is configured with configured uplink grant(s) and the Regular BSR was triggered for a logical channel for which logicalChannelSR-Mask is set to false; or 3> if the UL-SCH resources available for a new transmission do not meet the LCP mapping restrictions (see clause 5.4.3.1) configured for the logical channel that triggered the BSR: 4> trigger a Scheduling Request.

Observation 3: After triggering BSR, the UE will trigger SR if there is no UL-SCH resource.

In addition, while there is at least a pending SR, if the UE has a valid PUCCH resource configured for the pending SR, the UE will transmit the SR to NW to ask for UL-SCH resource, then the BSR MAC CE can be transmitted. In other words, the UE will not initiate a RA procedure if there is a valid PUCCH resource.

TS 38.321 v.16.3.0 As long as at least one SR is pending, the MAC entity shall for each pending SR: 1> if the MAC entity has no valid PUCCH resource configured for the pending SR: 2> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

Observation 4: After triggering BSR and SR, the UE will not initiate RA if there is a valid PUCCH resource.

It is noted that the intention of BSR over 2-step RACH is to avoid the large propagation delays of the SR-BSR procedure (i.e., at least 2 RTTs of delay). In that sense, if there is no available UL-SCH resource, it is beneficial that the UE can immediately send the BSR MAC CE via 2-step RACH (i.e., via MsgA PUSCH) even if the UE can trigger or transmit the SR. In other words, the UE should directly initiate a RA procedure and select the 2-step RA type after a BSR is triggered regardless of SR is triggered or not. However, it may cause the problem of resource congestion for 2-step RA. Some restrictions may need to limit the UE to use 2-step RA resources for BSR.

Proposal 3:  In NTN, if a BSR is triggered and there is no available UL-SCH resource to accommodate a BSR MAC CE, the UE should directly initiate a RA procedure and select a 2-step RA type (i.e., regardless of the SR). FFS if any limitation is needed.

Conclusion

Based on the above analysis, we have the following observations and proposals.

Observation 1: Since no near-far effect in NTN, measurement-based RA type selection leads to a single RA type.

Observation 2: A burst from the UEs to select either a single RA type of 2-step or 4-step RA would occur due to the change of elevation angle.

Proposal 1: In NTN, the RSRP threshold for RA type selection is not applied.

Proposal 2: In NTN, an RA type selection criterion can be based on UE-calculated TA.

Observation 3: After triggering BSR, the UE will trigger SR if there is no UL-SCH resource.

Observation 4: After triggering BSR and SR, the UE will not initiate RA if there is a valid PUCCH resource.

Proposal 3:  In NTN, if a BSR is triggered and there is no available UL-SCH resource to accommodate a BSR MAC CE, the UE should directly initiate a RA procedure and select 2-step RA type (i.e., regardless of the SR). FFS if any limitation is needed.

Reference

[1] R2-2010764 Summary of [AT112-e][103][NTN] RACH and HARQ feedback aspects

[2] Report of [Post112-e][152][NTN] UL scheduling enhancements [OPPO]