3GPP TSG-RAN WG2 Meeting #113 electronic	                                       R2-2101835

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.6.5

Source:			Asia Pacific Telecom, FGI

Title:		Discussion on CG-SDT configuration

Document for:	 	Discussion and decision

Introduction

In RAN2#112e, the following agreements for CG aspects were achieved.

RAN2#112e Agreement

- The configuration of configured grant resource for UE uplink small data transfer is contained in the RRCRelease message.  FFS if other dedicated messages can configure CG in INACTIVE CG. Configuration is only type 1 CG with no contention resolution procedure for CG.

- The configuration of configured grant resource can include one type 1 CG configuration.  FFS if multiple configured CGs are allowed

- A new TA timer for TA maintenance specified for configured grant based small data transfer in RRC_INACTIVE should be introduced.  FFS on the procedure, the validity of TA, and how to handle expiration of TA timer.  The TA timer is configured together with the CG configuration in the RRCRelease message.

- The configuration of configured grant resource for UE small data transmission is valid only in the same serving cell.  FFS for other CG validity criteria (e.g. timer, UL/SUL aspect, etc)

- The UE can use configured grant based small data transfer if at least the following criteria is fulfilled (1) user data is smaller than the data volume threshold; (2) configured grant resource is configured and valid; (3) UE has valid TA.  FFS for the candidate beam criteria.

- From RAN2 point of view:  An association between CG resources and SSBs is required for CG-based SDT.  FFS up to RAN1 how the association is configured or provided to the UE.  Send an LS to RAN1 to start the discussion on how the association can be made.   Mention that one option RAN2 considered was explicit configuration with RRC Release message

- A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

After RAN2#112e, we have an email discussion [1] to further discuss the unresolved issues. However, it can be seen that several issues may still need more discussion to reach the consensus. Thus, in this contribution, we would share our perspectives on these topics:

- Configuration of CG-SDT resource

- Store of CG-SDT configuration

- Single vs. Multiple CG configurations

- Initial vs. Dedicated BWP

Discussion

Configuration of CG-SDT resource

Last meeting, we have agreed that the configuration of CG-SDT resource can be contained in the RRCRelease, and FFS whether another dedicated message can be used. However, whether the full configuration of CG-SDT should be carried by RRCRelease is questionable. It is noted that there are quite many parameters and fields for CG type 1, which may cause signaling overhead for RRCRelease message. Moreover, if multiple CG configurations are allowed for SDT, higher overhead can be expected. Currently, CG type 1 is configured via RRCReconfiguration while the UE is in RRC_CONNECTED. Also known that before the UE transitions to RRC_INACTIVE, the UE should enter RRC_CONNECTED first, and then receives the RRCRelease message to enter RRC_INACTIVE. In that sense, the full configuration of CG-SDT resource can be configured as legacy (i.e., via RRCReconfiguration) while the UE is in RRC_CONNECTED. Then the RRCRelease only needs to indicate the delta configuration of CG-SDTwhile the UE enters RRC_INACTIVE.

Observation 1: Full configuration of CG-SDT leads to signalling overhead for RRCRelease.

Proposal 1: Full configuration of CG-SDT can be configured via RRCReconfiguration while the UE is in RRC_CONNECTED and delta configuration of CG-SDT can be indicated via RRCRelease while the UE enters RRC_INACTIVE.

Store of CG-SDT configuration

Based on current mechanism, CG type 1 configuration can be stored, and the UE will suspend the CG type 1, when the CG is not used. If this configuration is going to be reused, the UE can (re-)initialize the CG type 1 and apply the stored CG type 1 configuration again. Currently, we are discussing the criteria for SDT validity determination, e.g., based on TAT, RSRP change, beam quality, UL skipping, etc. If the CG-SDT is considered invalid, the UE needs to release or store the CG-SDT configuration to avoid using the invalid CG resource. One benefit to store the CG-SDT configuration is to reduce the signaling overhead for GG-SDT reconfiguration. On other hand, we think releasing the CG-SDT configuration when it becomes invalid may be overkilled, since the validity of CG-SDT is likely to recover soon. For example, if the CG-SDT becomes invalid due to TAT expiry, the UE can trigger the RA procedure to get the absolute TA, then the CG-SDT would be valid. For another example, if the CG-SDT is invalid due to beam blockage, the quality of beam may become qualified after a while, e.g., as the UE moves closer to the cell center. Since the traffic patterns for the UE in RRC_INACTIVE may not change frequently, it is not needed to reconfigure the full GG-SDT configuration each time while the CG-SDT becomes valid. Hence, to save the signaling overhead, the UE can store the CG-SDT configuration if the UE considers that the CG-SDT is not valid, and (re-)initialize the CG-SDT when it becomes valid. In addition, when a CG-SDT configuration should be released can be determined by NW, NW can send an explicit information, e.g., via SIB or RRCRelease, etc., to release the CG-SDT.

Observation 2: Storing the CG-SDT configuration when the CG-SDT is invalid can reduce signaling overhead for GG-SDT configuration when the CG-SDT becomes valid.

Proposal 2: UE stores the CG-SDT configuration when the CG-SDT is considered invalid in certain conditions. FFS the conditions (e.g., TAT expiry, RSRP change, Beam quality, UL skipping, etc.)

Proposal 3: UE (re-)initializes the stored CG-SDT configuration when the CG-SDT becomes valid.

Single v.s. Multiple CG configurations

In Rel-16, the mechanism of multiple CG configurations is supported. The benefit of multiple CG configurations is to support different services with different traffic patterns (e.g., instant messaging services, heart-beat/keep-alive traffic, traffic from wearables, sensors, smart meters, etc.). In WID, these applications have been identified as the use cases of small data traffic. Although some companies have concern about the issue of resource shortage, we think it can be well controlled by NW implementation. If the NW needs to serve lots of UEs, the NW can only configure one CG-SDT configuration to a UE for sure. Therefore, it is reasonable to give the network flexibility to configure more than one CG configuration for a UE in RRC_INACTIVE. As such, the data from each service could be mapped to the suitable CG configuration to ensure that the service requirement is met.

Observation 3: Multiple CG configurations are beneficial to support different services with different traffic patterns

Proposal 4: Multiple CG configurations for SDT can be supported.

Initial v.s. Dedicated BWP

At present, for UEs in RRC_INACTIVE, the UE should keep in initial BWP to receive paging, system information or perform RACH procedure. Regarding the BWP for CG-SDT, two options are raised for discussion:

- Option 1: The BWP for CG-SDT is the initial UL BWP

- Option 2: The BWP for CG-SDT is a dedicated UL BWP

The following table lists the pros and cons between two options.

Table 1: Pros and cons between initial BWP and dedicated BWP for CG-SDT

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: The BWP for CG-SDT is the initial UL BWP | Simplicity with minimum spec. impacts | Resource congestion |
| Option 2: The BWP for CG-SDT is a dedicated UL BWP | To Increase configurable time/frequency resource | Complexity on BWP related UE behaviors as well as initial DL BWP monitoring for paging/SI. |

In our perspective, we think initial BWP can be the baseline to configure CG-SDT from simplicity point view. However, since CG type 1 is the dedicated CG resource for a specific UE, it’s also reasonable to configure the dedicated CG resource on a dedicated BWP to reduce the resource congestion. It’s beneficial to support more UEs in a serving cell, to support multiple traffic patterns, and to alleviate the resource congestion in beam specific CG-SDT case. Since it’s not conflicted to support both option 1 and option 2 simultaneously, we are open to provide NW more flexibility and more time/frequency resources to configure the CG-SDT on a dedicated BWP.

Observation 4: As a baseline, CG-SDT resource can be configured on initial BWP.

Observation 5: CG-SDT resource configured on a dedicated BWP can reduce resource congestion on initial BWP and provide NW more time/frequency resource to configure CG-SDT.

Proposal 5: CG-SDT can be configured on a dedicated UL BWP.

Conclusion

Based on the above description, we have the following observations and proposals.

Observation 1: Full configuration of CG-SDT leads to signalling overhead for RRCRelease.

Proposal 1: Full configuration of CG-SDT can be configured via RRCReconfiguration while the UE is in RRC_CONNECTED and delta configuration of CG-SDT can be indicated via RRCRelease while the UE enters RRC_INACTIVE.

Observation 2: Storing the CG-SDT configuration when the CG-SDT is invalid can reduce signaling overhead for GG-SDT configuration when the CG-SDT becomes valid.

Proposal 2: UE stores the CG-SDT configuration when the CG-SDT is considered invalid in certain conditions. FFS the conditions (e.g., TAT expiry, RSRP change, Beam quality, UL skipping, etc.)

Proposal 3: UE (re-)initializes the stored CG-SDT configuration when the CG-SDT becomes valid.

Observation 3: Multiple CG configurations are beneficial to support different services with different traffic patterns

Proposal 4: Multiple CG configurations for SDT can be supported.

Observation 4: As a baseline, CG-SDT resource can be configured on initial BWP.

Observation 5: CG-SDT resource configured on a dedicated BWP can reduce resource congestion on initial BWP and provide NW more time/frequency resource to configure CG-SDT.

Proposal 5: CG-SDT can be configured on a dedicated UL BWP.

Reference

[1] [POST112-e][550][SDT]  Further details of CG aspects (Lenovo)