3GPP TSG-RAN WG2 Meeting #113 electronic	                                       R2-2101837

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.6.5

Source:			Asia Pacific Telecom, FGI

Title:		Beam operation for CG-SDT

Document for:	 	Discussion and decision

Introduction

In RAN2#112e, the following agreements were agreed for CG-SDT corresponding to beam aspects.

RAN2#112e Agreement

- The UE can use configured grant based small data transfer if at least the following criteria is fulfilled (1) user data is smaller than the data volume threshold; (2) configured grant resource is configured and valid; (3) UE has valid TA.  FFS for the candidate beam criteria.

- From RAN2 point of view:  An association between CG resources and SSBs is required for CG-based SDT.  FFS up to RAN1 how the association is configured or provided to the UE.  Send an LS to RAN1 to start the discussion on how the association can be made.   Mention that one option RAN2 considered was explicit configuration with RRC Release message

- A SS-RSRP threshold is configured for SSB selection. UE selects one of the SSB with SS-RSRP above the threshold and selects the associated CG resource for UL data transmission.

In this contribution, we will further discuss the issues on beam operation for CG-SDT.

Discussion

Beam operation for CG-SDT

Last meeting, we have agreed that the association between CG resources and SSBs is required for CG-SDT. An example in Figure 1 shows that the NW can configure multiple SSBs and the corresponding CG resource occasions. While the UE performs small data transmission via CG-SDT, the UE should select a qualified SSB (i.e., the SSB with SS-RSRP above a RSRP threshold) first, then perform transmission via a specific CG occasion that associates with the selected SSB. Since the association between SSB and CG is configured by NW, when the NW receives the UL data on the specific CG occasion, the beam alignment between UE and NW can be achieved. It is noted that how to configure the association is up to RAN1.

Figure 1: Association between SSBs and CG-SDT occasions

Observation 1: Beam alignment between UE and NW for CG-SDT can be achieved based on the association between SSB and CG-SDT resource.

It is noted that we have agreed that a SS-RSRP threshold can be configured for SSB selection for SDT. In RACH mechanism, the RSRP threshold (e.g., rsrp-ThresholdSSB) for SSB selection is configured in RACH configuration. Furthermore, two different RSRP thresholds are applied for NUL and SUL. The similar concepts can also be applied to CG-SDT.

RACH-ConfigCommon ::=               SEQUENCE {

[…]

rsrp-ThresholdSSB                       RSRP-Range        OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL              RSRP-Range        OPTIONAL,   -- Cond SUL

Proposal 1: RSRP threshold for SSB selection of CG-SDT is configured in the configuration of CG-SDT.

Proposal 2: For both NUL and SUL configured with CG-SDT resource, different RSRP thresholds for SSB selection of CG-SDT are separately configured.

In some situations, it is likely that the UE suffers from poor channel condition. That is, there may be no SSBs with SS-RSRP above the RSRP threshold. In the contention-based RA procedure, the UE may select any SSB to attempt the preamble transmission.

TS 38.321 v.16.3.0 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

However, for CG-SDT, it’s not appropriate to apply the same method. There are some differences between RACH transmission and CG-SDT transmission. For instance, the UE would transmit larger size of data via CG transmission, which needs better channel quality. Thus, if the DL channel condition is poor (i.e., no beam is qualified), it is very likely that the CG transmission will fail at the end. On top of that, from power saving perspective, the UE should avoid wasting the power to perform UL transmission via CG-SDT if no qualified SSB can be detected. That is, the UE should skip the UL transmission in this case.

Observation 2: The CG transmission is likely to be failed if the DL channel quality is poor. The UE will suffer from power consumption if the UE continuously performs CG-SDT transmissions.

Proposal 3: The UE should skip the CG transmission when there is no SSB with SS-RSRP above the RSRP threshold for SSB selection of CG-SDT.

It is well known that, for small data transmission, not only CG-SDT can be used, but also RACH-based SDT. Thus, the UE does not need to continuously attempt the CG transmission in poor beam situation without limitation. Moreover, the UE needs to perform PDCCH monitoring based on the selected beam for the DL response or for the scheduling of the subsequent data transmission. If there is no qualified beam, the PDCCH monitoring would be ineffective. Thus, the fallback to use RACH for SDT can be applied if the UE detects the DL channel quality is not qualified enough for a certain while. When and how to perform the fallback to RACH due to poor beam of CG-SDT can be further discussed.

Observation 3: PDCCH monitoring is ineffective in a case that no qualified beam can be selected.

Proposal 4: RAN2 studies when and how to perform the fallback to RACH due to poor DL channel condition (i.e., no SSB with SS-RSRP above the RSRP threshold) of CG-SDT.

Conclusion

Based on the above analysis, we have the following observations and proposals.

Observation 1: Beam alignment between UE and NW for CG-SDT can be achieved based on the association between SSB and CG-SDT resource.

Proposal 1: RSRP threshold for SSB selection of CG-SDT is configured in the configuration of CG-SDT.

Proposal 2: For both NUL and SUL configured with CG-SDT resource, different RSRP thresholds for SSB selection of CG-SDT are separately configured.

Observation 2: The CG transmission is likely to be failed if the DL channel quality is poor. The UE will suffer from power consumption if the UE continuously performs CG-SDT transmissions.

Proposal 3: The UE should skip the CG transmission when there is no SSB with SS-RSRP above the RSRP threshold for SSB selection of CG-SDT.

Observation 3: PDCCH monitoring is ineffective in a case that no qualified beam can be selected.

Proposal 4: RAN2 studies when and how to perform the fallback to RACH due to poor DL channel condition (i.e., no SSB with SS-RSRP above the RSRP threshold) of CG-SDT.

Reference

[1] [POST112-e][550][SDT]  Further details of CG aspects (Lenovo)