3GPP TSG-RAN WG2 Meeting #113 electronic	                                       R2-2101841

Online, Jan 25 – Feb 5, 2021

Agenda item:		8.9.2

Source:			Asia Pacific Telecom, FGI

Title:		Paging Enhancements for Power Saving

Document for:	 	Discussion and decision

- Introduction

In RAN2#112e, the following agreements were reached.

RAN2#112e Agreement

- Confirm that UE grouping is considered a candidate of paging enhancement for UE power saving

- RAN2 have discussed and considered “paging indication for UE subgroups using paging DCI”, “paging early indication or wake-up signal (WUS) for UE subgroups”, “cross-slot scheduling of paging for UE subgroups”.

- RAN2 understands that RAN1 have started to evaluate performance and complexity. RAN2 assumes that RAN1 continues with this evaluation, in order that decisions can be made regarding the paging indication/scheduling solution. As R2 is the leading group for this WI objective it is expected that final decisions are made by R2.

- Will send an LS to R1 (action to be discussed offline).

- The solution of PRNTI based group discrimination is deprioritized from RAN2 perspective

- The solution of “paging for UE subgroups using different time/frequency resources” is de-prioritized from RAN2 perspective.

After RAN2#112e, an email discussion [1] was triggered to discuss how to determine which paging subgroup a UE belongs to.

On top of that, we would share our views on the indications/signals for indicating at least UE subgroups in this contribution.

- Discussion

Paging mechanism

To reduce power consumption in RRC_IDLE/RRC_INACTIVE, a DRX mechanism is applied for paging monitoring, which implies that the RRC_IDLE/RRC_INACTIVE UE does not need to monitor the PDCCH continuously. As shown in Figure 1, a UE can be configured with DRX cycle(s) and several parameters for the determination of paging occasion (PO). It is noted that the UE may only monitor one PO per DRX cycle. But in some cases (e.g., multi-beam operation, operation with shared spectrum channel access), the UE would monitor multiple PDCCH monitoring occasions in one PO. Besides, a PO may be a set of ‘S’ consecutive PDCCH monitoring occasions, where ‘S’ is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1.

Figure 1: DRX mechanism for paging monitoring

Currently, paging allows the NW to reach UEs through Paging message and notify UEs of system information change and ETWS/CMAS indications through Short Message. As shown in Figure 2, a UE monitors the PDCCH to receive a Paging DCI which is transmitted by a DCI format 1_0 with CRC scrambled by P-RNTI. Upon the UE receives the paging DCI, the UE checks the Short Message Indicator to know if there is short message carried in the DCI and check if there is a scheduling information for paging message. If the short message indicator indicates there is paging message, the UE further receives the paging message on the PDSCH based on the scheduling information indicated by the DCI. Upon the UE receives the paging message while in RRC_IDLE/RRC_INACTIVE, the UE further checks if the UE ID included in the paging record matches the UE’s ID (allocated by upper layer or UE’s stored RNTI). If so, it means the NW would like to reach the UE.

Figure 2: Paging indication

Indications for UE subgroups

In [1], the majority of companies support considering UE grouping based on UE-ID. Other schemes are FFS. It is noted that the information of UE groups would be carried by an indication/signal for power saving in RRC_IDLE/RRC_INACTIVE. Three potential indications have been agreed to further discuss, i.e.,

- Paging DCI

- PEI

- Cross-slot scheduling

Paging DCI

As shown in Figure 3, if the UE subgroup information can be included in paging DCI, NW can indicate which subgroup(s) of UEs should further receive the paging message via PDSCH. In comparison with the legacy paging DCI, the benefit is to reduce the false alarm rate for receiving the paging message, e.g., some group(s) of UEs can skip receiving the paging message, which can save the power on PDSCH decoding. Another advantage is that minimum specification impact would be expected since the reserved bits in paging DCI can be used, i.e., no need to specify a new DCI format or new occasion for monitoring a new indication/signal. However, it is noted that only limited reserved bits in the current paging DCI can be used. For example, in a short message indicator, there is only one reserved index. In a short message, only five reserved indexes can be used. On the other hand, since paging DCI should be indicated closed to paging message in the time domain, the UE still needs to perform the SSB measurement for pre-synchronization before receiving the paging DCI. In other words, there is no power saving gain on “pre-synchronization” and “PDCCH monitoring”.

Figure 3: Paging DCI

PEI

As shown in Figure 4, if the UE subgroup information can be included in PEI, NW can indicate which subgroup(s) of UEs should or should not wake-up to monitor paging DCI in advance. For example, the PEI can be indicated before one to three SSB bursts before the PO. When the PEI indicates that the UE(s) don’t need to monitor the upcoming PO, the UE(s) can skip the following SSB measurements and skip monitoring the PO (also the paging PDSCH). The benefits are to save not only the power on paging monitoring but also on the “pre-synchronization” step. However, the drawback is complexity because we need to discuss how to model the PEI (e.g., PDCCH-based as DCP or sequence-based as I-WUS), how to define a new DCI format as well as new monitoring occasion(s) for PEI, etc.

Figure 4: PEI

Cross-slot scheduling

Regarding cross-slot scheduling (CSS), if NW could give UE some hints to let UE know a-priori the range/offset of symbols not carrying transmissions that the UE  expects to receive or transmit, the UE could be able to put its RF, portion of the front-end hardware and potentially other modem hardware into power saving mode (e.g., micro-sleep mode) for those symbols.

In our view, there are two possible alternatives for indicating CSS in RRC_IDLE/RRC_INACTIVE.

Alt 1, as shown in Figure 5, is to introduce a new DCI format that can not only indicate which subgroup(s) of UEs should further monitor the PO, but also indicate the time and frequency resources of the PDSCH in advance. This alternative is very similar to the function of PEI. Although the specification impact may be less compared to PEI, the shortage is the limitation on the range between the new DCI with CSS and PO (e.g., up to 32 slots). The power saving gain is not significant for “pre-synchronization” compared to PEI.

Alt 2, as shown in Figure 6, is to indicate the CSS information via paging DCI. The paging DCI and the corresponding paging PDSCH can be transmitted in different slots. The power saving gain comes from the that UE can avoid unnecessary DL signal buffering if there is no paging PDSCH transmitted at all. However, it is challenging for NW to decide the resource allocation and MCS for paging PDSCH when the scheduling offset between the paging DCI and the paging PDSCH is very large for power saving benefit.

Figure 5: Alt 1- New DCI with CSS

Figure 6: Alt 2- Paging DCI with CSS

Summary

Based on the above analysis, we summarize the pros and cons of these three different indications in Table 1. From our perspective, we think the method of Paging DCI leads to the least power saving gain but the smallest specification impact.  From the point of specification impact, paging DCI is a good approach. On the other hand, the method of PEI leads to the highest power saving gain but also the highest complexity. From a power saving perspective, PEI is another attractive approach. Thus, we think the compromise way is to support both mechanisms. Then NW can decide which method is better to use by NW implementation.  As for CSS, we think the method of CSS was discussed in RAN1 for RRC_CONNECTED in Rel-16, it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE in RAN1 first. Whether needs to support CSS in RRC_IDLE/RRC_INACTIVE should be determined by RAN1.

Observation 1: Paging DCI for UE subgroups incurs the least power saving gain but the smallest specification impact.

Observation 2: PEI for UE subgroups leads to the highest power saving gain but the highest complexity.

Observation 3: CSS was discussed in RAN1 for RRC_CONNECTED UEs in Rel-16, so it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE UEs in RAN1 first.

Table 1: Pros and Cons of the indications for UE subgroups

|  |  | Pros | Cons |
| --- | --- | --- | --- |
| Paging DCI |  | - Reserved bits can be used - Minimum specification impact | - Limited reserved bits support less UE groups - No power saving gain on “pre-synchronization” and “PDCCH monitoring” |
| Paging Early Indication (PEI) |  | - Save the power on PDCCH monitoring - High power saving gain on skipping “pre-synchronization” step | - Complexity of PEI modeling |
| Cross-Slot Scheduling (CSS) | Alt 1- New DCI with CSS | - Less specification impact than PEI | - Limitation on the range between the new DCI with CSS and PO. - Less power saving gain on “pre-synchronization” than PEI |
|  | Alt 2- Paging DCI with CSS | - Minimum specification impact | - Limitation on offset between paging DCI and paging PDSCH - No power saving gain on “pre-synchronization” and “PDCCH monitoring” |

Proposal 1: From RAN2 perspective, paging DCI for UE subgroups should be supported.

Proposal 2: From RAN2 perspective, paging early indication (PEI) for UE subgroups should be supported.

Proposal 3: Whether cross-slot scheduling is feasible for RRC_IDLE/RRC_INACTIVE UEs should be determined by RAN1.

Conclusion

Based on the above, we have the following observations and proposals.

Observation 1: Paging DCI for UE subgroups incurs the least power saving gain but the smallest specification impact.

Observation 2: PEI for UE subgroups leads to the highest power saving gain but the highest complexity.

Observation 3: CSS was discussed in RAN1 for RRC_CONNECTED UEs in Rel-16, so it’s better to discuss the similar concept for RRC_IDLE/RRC_INACTIVE UEs in RAN1 first.

Proposal 1: From RAN2 perspective, paging DCI for UE subgroups should be supported.

Proposal 2: From RAN2 perspective, paging early indication (PEI) for UE subgroups should be supported.

Proposal 3: Whether cross-slot scheduling is feasible for RRC_IDLE/RRC_INACTIVE UEs should be determined by RAN1.

Reference

[1] [POST112-e][064][Pow17] Group Determination