3GPP TSG-RAN WG2 Meeting #113-e	 R2-2101845

# eMeeting, 25th Jan – 5th Feb, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0396 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the simultaneousRxTxInterBandCA capability in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021/01/25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The capability simultaneousRxTxInterBandCA could be included in either ca-ParametersNR or ca-ParametersNR-ForDC. • BandCombination >> ca-ParametersNR >> simultaneousRxTxInterBandCA • BandCombination-v1560 >> ca-ParametersNR-ForDC >> simultaneousRxTxInterBandCA While it is included for a NR CA band combination, it is clear that this field indicates whether the UE could support simultaneous RX/TX in this CA band combination. However, it is unclear what does this filed mean while it is included for a NR DC band combination. The current description seems implies that it is used for NR CA only. We understand the intention of current ASN.1 define is trying to reuse the same filed to indicate the simultaneous RX/TX capability for NR-DC. And based on RAN4 LS R4-2016988, it is confirmed that the simultaneous RxTx UE capability is also needed for inter-band NR-DC (for TDD-TDD and TDD-FDD band combinations). Therefore, we propose to clarify in the field description that simultaneousRxTxInterBandCA is applicable to NR-DC while it is included in ca-ParametersNR-ForDC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 4.2.7.4, clarify that the capability simultaneousRxTxInterBandCA is applicable to NR-DC while it is included in ca-ParametersNR-ForDC. Impact analysis Impacted 5G architecture options: NR-DC Impacted functionality: UE capability reporting Inter-operability: If the UE implements the CR but the network does not or vice-versa, the consequences if not approved remains. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and the network may have different understanding of the simultaneousRxTxInterBandCA capability included for NR-DC band combination. It may result in configuration that does not fit UE capability and re-establishment is triggered. So, there is potential performance lost. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-2007887 |  |  |  |  |  |  |  |  |

1st change

#### 4.2.7.4	CA-ParametersNR

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| <Skip unrelated rows> |  |  |  |  |
| simulTX-SRS-AntSwitchingInterBandUL-CA-r16 Indicates whether the UE support simultaneous transmission of SRS on different CCs for inter-band UL CA. The UE indicating support of this feature shall include at least one of the following capabilities: - supportSRS-xTyR-xLessThanY-r16 indicates support transmission of SRS for xTyR (x<y) based antenna switching and SRS for CB/NCB/BM on different CCs in overlapped symbol(s) for inter-band UL CA. - supportSRS-xTyR-xEqualToY-r16 indicates support transmission of SRS for xTyR (x=y) based antenna switching and SRS for CB/NCB/BM on different CCs in overlapped symbol(s) for inter-band UL CA. - supportSRS-AntennaSwitching Indicates whether the UE support simultaneous transmission of SRS for antenna switching on different CCs in overlapped symbol(s) for inter-band UL CA. | BC | No | N/A | N/A |
| simultaneousRxTxInterBandCA If this field is included in ca-ParametersNR, it indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. | BC | CY | N/A | N/A |
| simultaneousRxTxSUL Indicates whether the UE supports simultaneous reception and transmission for a NR band combination including SUL. Mandatory/Optional support depends on band combination and captured in TS 38.101-1 [2]. | BC | CY | N/A | N/A |
| simultaneousSRS-AssocCSI-RS-AllCC Indicates support of CSI-RS processing framework for SRS and the number of SRS resources that the UE can process simultaneously across all CCs, and across MCG and SCG in case of NR-DC, including periodic, aperiodic and semi-persistent SRS. This parameter may further limit simultaneousSRS-AssocCSI-RS-PerCC in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination. | BC | No | N/A | N/A |

End of change