3GPP TSG-RAN WG2 Meeting #113-e	R2-2101849

# Online, January 25th – February 5th 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0824 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for inter-RAT cell selection triggered by SNPN selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Asia Pacific Telecom, FGI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN-Core |  |  |  |  |  | Date: |  |  | 2021-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Based on TS 23.122, it is expected that a UE camping on an E-UTRA cell may receive a SNPN selection reqeust from the NAS layer. Since SNPN is supported only by NR-RAN. RAT change (from E-UTRA to NR) would happen when the UE is implementing cell (re)selection procedure triggered by SNPN selection. - However, based on the cell reselection evaluation process in TS 36.304 subclause 5.2.4.1, the UE may still keep the following configurations and the (optinally) associated timer counting procedure during inter-RAT cell (re)selection procedure: - deprioritisationReq and the associated T325 counting procedure. - priorities provided by dedicated signalling and the associated T320 counting procedure. - In comparion, by refering to TS 38.304, it is already defined that the priorities and the deprioritisationReq provided by dedicated signaling would be deleted by the UE when a SNPN selection is performed on request by NAS. - Moreover, in TS 38.331, it is found that PLMN/SNPN selection is not included in the stop condition of T320 & T325. Therefore, T320/T325 in the NR protocols are still counting after the the priorities and the deprioritisationReq provided by dedicated signaling for cell reselection is deleted. - Therefore, to align the UE implementations in NR protocols and E-UTRA protcotols and different technical specifications in NR proctols/E-UTRA protocols respectively. The following Change Requests are proposed: - In TS 36.304, add ‘SNPN selection’ as one additional condition to delete the priorities and the deprioritisationReq provided by E-UTRA dedicated signaling. - In TS 38.331, add ‘SNPN selection’ to the stop condition of T320; add ‘PLMN or SNPN selection’ to the stop condition of T325 and further indicate that ‘UE shall store the deprioritisationReq until T325 expires or is stopped’ in NR protocols’. - In TS 36.331, add ‘SNPN selection’ to the stop condition of T320; add ‘PLMN or SNPN selection’ to the stop condition of T325 and further indicate that ‘UE shall store the deprioritisationReq until T325 expires or is stopped’ in NR protocols’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 3.3, add ‘SNPN’ in the Abbreviations. - In 5.2.4.1, Add ‘SNPN selection’ as one additional condition to delete the priorities and the deprioritisationReq provided by E-UTRA dedicated signaling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignments of between E-UTRA protocols and NR protocols while the UE receives the same SNPN selection request from NAS. - Misalignments of the validity time periods and the associated priorities and the deprioritisationReq provided by dedicated signaling stored by the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.304 CR XXXX TS 36.331 CR XXXX TS 38.331 CR XXXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

1xRTT	CDMA2000 1x Radio Transmission Technology

AS	Access Stratum

AC	Access Class (of the USIM)

ACDC	Application specific Congestion control for Data Communication

BCCH	Broadcast Control Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSS	Basic Service Set

CMAS	Commercial Mobile Altert System

CSG	Closed Subscriber Group

DRX	Discontinuous Reception

DL-SCH	Downlink Shared Channel

EHPLMN	Equivalent Home PLMN

EPC	Evolved Packet Core

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

FDD	Frequency Division Duplex

GERAN	GSM/EDGE Radio Access Network

GWUS	Group Wake Up Signal

HPLMN	Home PLMN

HSDN	High Speed Dedicated Network

H-SFN	Hyper System Frame Number

HRPD	High Rate Packet Data

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MBMS	Multimedia Broadcast-Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCC	Mobile Country Code

MCCH	Multicast Control Channel

MDT	Minimization of Drive Tests

MM	Mobility Management

MNC	Mobile Network Code

MPDCCH	MTC Physical Downlink Control Channel

MTCH	Multicast Traffic Channel

NAS	Non-Access Stratum

NB-IoT	NarrowBand Internet of Things

NR	NR Radio Access

NRS	Narrowband Reference Signal

PLMN	Public Land Mobile Network

ProSe	Proximity-based Services

PSM	Power Saving Mode

PTW	Paging Time Window

PWS	Public Warning System

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SAP	Service Access Point

SIBX	SystemInformationBlockTypeX

SNPN	Stand-alone Non-Public Network

TDD	Time Division Duplex

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

USIM	Universal Subscriber Identity Module

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle-to-Everything

WUS	Wake Up Signal

Next changes

5.2.4.1	Reselection priorities handling

<Partially omitted>

In case UE receives RRCConnectionReject with deprioritisationReq, UE shall consider current carrier frequency and stored frequencies due to the previously received RRCConnectionReject with deprioritisationReq or all the frequencies of EUTRA to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS TS 23.122 [5].

NOTE 4:	Connecting to CDMA2000 does not imply PLMN selection.

NOTE 5:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 36.133 [10] are still applicable.

The UE shall delete priorities or altFreqPriorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	the optional validity time of altFreqPriorities (T323) expires; or

-	a PLMN selection or SNPN selection is performed on request by NAS TS 23.122 [5].

NOTE 6:	Equal priorities between RATs are not supported.

<Partially omitted>

End of changes