3GPP TSG-RAN WG2 Meeting #113-e	R2-2101857

# Electronic Meeting, 25th Jan – 5th Feb 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1055 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Conditions to stop an ongoing RA procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Asia Pacific Telecom, FGI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The conditions to stop a RA procedure initiated due to a pending SR for BSR/BFR/LBT with no valid PUCCH resources configured may include: - Transmission of a MAC PDU on a UL grant other than a UL grant provided by RAR, and the MAC PDU includes a BSR MAC CE containing buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly - A UL grant other than a UL grant provided by RAR can accommodate all pending data available for transmission. Moreover, based on the post RAN2#108 email discussion, it was concluded that the UL grant shall also exclude the UL grant determined as specified in 5.1.2a for transmission of the MSGA payload. However, based on current text in clause 5.4.4 of TS 38.321, the type of UL grant that can possibly stop an ongoing RA procedure, which was initiated due to a pending SR for BSR/BFR/LBT, can be interpreted as the following: Type 1: a UL grant other than a UL grant provided by Random Access Response; or Type 2: a UL grant other than a UL grant determined as specified in 5.1.2a for transmission of the MSGA payload. As a consequence, two erroneous outcomes for stopping the RA procedure are introduced: - A UL grant determined as specified in 5.1.2a for transmission of the MSGA payload could cancel the RA procedure because it belongs to type 1 “a UL grant other than a UL grant provided by Random Access Response” - A UL grant provided by Random Access Response could cancel the RA procedure because it belongs to type 2 “a UL grant other than a UL grant determined as specified in 5.1.2a for transmission of the MSGA payload” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description in clause 5.4.4 of TS 38.321 so that a UL grant for stopping an ongoing RA procedure, which was initiated due to a pending SR for BSR/BFR/LBT with no valid PUCCH resources configured, shall exclude both the UL grant provided by Random Access Response and the UL grant determined as specified in 5.1.2a for transmission of the MSGA payload. Impact analysis Impacted functionality RA procedure due to a pending SR for BSR/BFR/LBT Inter-operability: If the network is implemented according to the CR and the UE is not, the UE may erroneously stop a ongoing RA procedure. If the UE is implemented according to the CR and the network is not, no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A RA procedure, which was initiated due to a pending SR for BSR/BFR/LBT with no valid PUCCH resources configured, could be stopped erroneously. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.4.4	Scheduling Request

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BSR, which was initiated by the MAC entity prior to the MAC PDU assembly and which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response and other than a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly; or

-	the UL grant(s) can accommodate all pending data available for transmission.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for BFR of an SCell, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response and other than a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU contains a BFR MAC CE or a Truncated BFR MAC CE which includes beam failure recovery information of that SCell; or

-	the SCell is deactivated (as specified in clause 5.9) and all triggered BFRs for SCells are cancelled.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR for consistent LBT failure recovery, which has no valid PUCCH resources configured, if:

-	a MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response and other than a UL grant determined as specified in clause 5.1.2a for the transmission of the MSGA payload, and this PDU includes an LBT failure MAC CE that indicates consistent LBT failure for all the SCells that triggered consistent LBT failure; or

-	all the SCells that triggered consistent LBT failure recovery are deactivated (see clause 5.9).

End OF CHANGE