3GPP TSG-RAN WG2 Meeting #113e	R2-2101906

25 January -05 Feburary 2021

Online

Agenda item:	8.11.2.1 (Latency analysis and latency enhancements)

Source:	Samsung

Title:	Latency reduction via configured grant for positioning

Document for:	Discussion & Decision

- Introduction

In the latest email discussion ([Post112-e][617][POS] Evaluation of latency enhancement solutions (CATT)‎) it covers the latency reduction on positioning mechanism for the possible study item. As one of RAN1 recommended item, using configured grant was listed up. Since we didn’t join the discussion and present our opinion, In this document, we would like to further discuss the direction of using configured grant for positioning measurement result report optimization.

- Discussion

As commented in the latency evaluation document, after UE measured the PRS, the UL availability and the procedures to get the UL grant, if not, is one factor to drag the E2E latency. The possible ways on this are listed up in the email discussion [Post112-e][617][POS] Evaluation of latency enhancement solutions (CATT) are as below:

- Option 1(summarized from companies’ comments): Using the existed CG-based transmission for a certain logical channel.

It is already supported by the configuration of logical channel in NR Rel-16 which is up to the network implementation to configure CG. However there is NRPPa impact. gNB may get the PRS period from LMF via NRPPa.

- Option2: New type or separate CG for positioning which is used to adapt the PRS period and positioning specific configured grant may be introduced in Rel-17.

This can be used as positioning use only uplink resources, so that periodic positioning measurement report could be sent without waiting any L1 signals.

Above directions are mostly feasible and simple solution. However regarding option 1 where additional introduction of NRPPa messages is necessary, it might be less costly in the latency perspective that LMF informs required CG characteristics of UE and UE determines the final required CG and requests to the gNB. Since processing and signaling time of NRPPa is greater than those of LPP, and Uu, there could be the merit on this direction too. Especially requested CG characteristics in LPP can be accompanied with other existing information in LPP message i.e., LPP LocationInformaitonRequest, then there is only latency of requesting CG from UE to gNB. This Uu signaling and processing can be further simplified by using lower layer signaling which can reduce more time.

So one more possible way to reduce the reduction w.r.t. CG is that LMF indicates the required CG or UL interval to UE and UE request to gNB.

Proposal 1. RAN2 study on latency reduction using CG where LMF first indicates the required CG info to UE and then UE request the CG to gNB.

- Conclusion

In this contribution, we discussed on the method to reduce the latency using CG, and has the following proposal:

Proposal 1. RAN2 study on latency reduction using CG where LMF first indicates the required CG info to UE and then UE request the CG to gNB.