3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2101958
Online, Jan 25 – Feb 5, 2021

Agenda Item:	10.8

Source: 		Session Chair (Samsung)

Title: 			Report from session on LTE V2X and NR SL

Document for:	Approval

## General

Please see the following TDocs for e-meeting guidance:

R2-2100000	Agenda for RAN2#113-e	Chairman	agenda

R2-2100351	3GPP TSG RAN WG2 Handbook (01/2021)	ETSI MCC	discussion

R2-2100352	RAN2#113-e Meeting Guidelines 	ETSI MCC	discussion

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## List and Status of Offline Email Discussions

[POST] Email discussion

[POST113-e][701][V2X/SL] Response LS to RAN1 LS (R2-2102328) (LG)

Scope: Prepare response LS to RAN1 based on the agreement made in the discussion of R2-2102190. We can confirm RAN2 can do the necessary spec update and indicate RAN2 will let RAN1 updated once spec update is completed.

Intended outcome: Approvable LS in R2-2102196

Deadline: Short email discussion until 3/1 11:00 (UTC)

[POST113-e][702][V2X/SL] Update of MAC CR (R2-2102186) (OPPO)

Scope: Update of MAC CR (R2-2102186)

Intended outcome: Agreeable MAC CR in R2-2102198

Deadline: Short email discussion until 3/1 11:00 (UTC)

[POST113-e][703][V2X/SL] Details of timers (InterDigital)

Scope: Discuss details of how to maintain the agreed timers (including exact definition of timers, how to set the timers, when to start/restart/stop the timers, additional consideration due to SL characteristics, considerations of both RX UE and its peer TX UE sides) and FFS parts related to timer operations.

Intended outcome: Discussion summary

Deadline: Long email discussion

[POST113-e][704][V2X/SL] TX UE centric or RX UE centric DRX configuration determination (OPPO)

Scope: Continue the discussion the remained issues regarding who will determine DRX configurations (including TX UE centric vs RX UE centric DRX configuration determination), covering both in and out of coverage scenario.

Intended outcome: Discussion summary

Deadline: Long email discussion

[POST113-e][705][V2X/SL] MAC impacts from the latest RAN1 decisions (LG)

Scope: Discuss MAC impacts from the latest RAN1 decisions.

Intended outcome: Discussion summary, corresponding CRs and response LS (if needed)

Deadline: Long email discussion

[POST113-e][706][V2X/SL] RRC impacts from the latest RAN1 decisions (Huawei)

Scope: Discuss RRC impacts from the latest RAN1 decisions.

Intended outcome: Discussion summary, corresponding CRs and response LS (if needed)

Deadline: Long email discussion

[POST113-e][707][V2X/SL] Spec update to level 3 logical slots (OPPO)

Scope: Discuss the update of CG equation and other spec changes according to level 3 logical slots (i.e. logical slots within a resource pool).

Intended outcome: Discussion summary and the corresponding CRs

Deadline: Long email discussion

[POST113-e][708][V2X/SL] How to handle DG for retransmissions? (OPPO)

Scope: Discuss option 1 (supported by the current spec) and option 2 (change of UE’s buffer flush behaviour).

Intended outcome: Discussion summary and CR (if needed)

Deadline: Long email discussion

[AT] Email discussion

[AT113-e][700][V2X/SL] Organisational (Session Chair)

Scope: Comments to session notes. Kick-off and management of email discussions for V2X/SL session. Coordination issues. Other organizational issues and announcements.

Intended outcome: Approval of Report from V2X/SL session.

Deadline: Feb 05 1100 UTC

[AT113-e][701][V2X/SL] Miscellaneous corrections (Huawei)

Scope: Discuss the need of changes and detailed wordings on the corrections in R2-2100786, R2-2100210, R2-2100231, R2-2100500, R2-2100502, R2-2101596, R2-2100919, R2-2100230, R2-2101767, R2-2101940, R2-2101655, R2-2100501, R2-2100785, and R2-2100923. Merge changes and prepare the agreeable CRs. Note for the changes which considered as non-backward compatible, we can prepare a separate CR (e.g. R2-2100230).

Intended outcome: Agreeable 38.331 CR in R2-2102171, R2-2102172 (if a separate CR is needed) and 36.331 CR in R2-2102173. Discussion summary in R2-2102174 (if needed). CRs will be approved by email.

Deadline: Feb 04 0430 (UTC)

[AT113-e][702][V2X/SL] T400 expiry in timer table and protection of RRC messages (Vivo)

Scope: Discuss the corrections in R2-101761, R2-2100788, R2-2100978, R2-2100790, R2-2100976, R2-2101760, and R2-2100977. Normative text may also need to be updated if adds some additional/different UE behaviour at T400 expiry. Merge changes and prepare the agreeable CR.

Intended outcome: Agreeable 38.331 CR in R2-2102175, 36.331 CR in R2-2102188, and discussion summary in R2-2102176 (if needed). CRs will be approved by email.

Deadline: Feb 04 0430 (UTC)

[AT113-e][703][V2X/SL] Discussion on detailed wording for a note (OPPO)

Scope: Discuss detailed wordings for a note to clarify inter-frequency operation and prepare the agreeable CR.

Intended outcome: Agreeable 38.331 CR in R2-2102177. CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

[AT113-e][704][V2X/SL] Left issue on reset configuration (OPPO)

Scope: Discuss if there is real problem with the current specification and what the best option is to solve it (if problem is justified). Prepare the agreeable CR (if needed).

Intended outcome: Agreeable 38.331 CR in R2-2102178 and discussion summary in R2-2102179 (if needed). CR will be approved by email (if needed).

Deadline: Feb 04 0430 (UTC)

[AT113-e][705][V2X/SL] RLC Re-establishment (Vivo)

Scope: Discuss the need of RLC re-establishment. Prepare agreeable CR (if needed).

Intended outcome: Agreeable 38.331 CR in R2-2102180 and discussion summary in R2-2102181 (if needed). CR will be approved by email (if needed).

Deadline: Feb 04 0430 (UTC)

[AT113-e][706][V2X/SL] Response LS to SA2 (LG)

Scope: Discuss detailed wordings and prepare the LS to be approved.

Intended outcome: Approvable response LS in R2-2102182. LS will be approved by email.

Deadline: Feb 02 1245 (UTC) => Done (LS was approved)

[AT113-e][707][V2X/SL] Who will decide SL DRX pattern? (OPPO)

Scope: Discuss who (TX UE, RX UE or gNB) will decide SL DRX pattern or configuration in various scenarios (scenario by scenario) considering whether SL DRX is for SL unicast, groupcast or broadcast, TX and RX UEs’ RRC state (including OOC), and whether TX and RX UE’s in the same or different serving cells (including IC and OOC).

Intended outcome: Discussion summary and proposals in R2-2102183.

Deadline: Feb 02 1245 (UTC) => Done (summary was handled in the session)

[AT113-e][708][V2X/SL] Granularity of SL DRX operation for groupcast/broadcast (Lenovo)

Scope: Discuss options identified above (including some level of understanding on how it works, e.g. what information can represent QoS level to differentiate SL DRX operation, how geo-location can work, etc., challenges, pros, and cons for each option) and check companies’ views. Note companies can add additional option if the option proposed in the contribution was missed.

Intended outcome: Discussion summary and proposals in R2-2102184.

Deadline: Feb 02 1245 (UTC) => Done (summary was handled in the session)

[AT113-e][709][V2X/SL] Mode 2 resource (re)selection (OPPO)

Scope: Discuss R2-2100099 based on the agreements from [POST112-e][701][V2X] and prepare agreeable 38.321 CR. Note this CR covers only mode 2 resource (re)selection related decisions.

Intended outcome: Agreeable 38.321 CR in R2-2102186, discussion summary in R2-2102187 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)

Scope: Discuss rapporteur’s suggestions in Table 1 for R2-2100212, R2-2100213, R2-2101741, and R2-2100503, and in Table 3 for R2-2100504, R2-2101068, R2-2101149, R2-2100323, R2-2101742, R2-2100861, R2-2100119, and R2-2100211. If changes are agreeable, merge them into a rapporteur’s miscellaneous corrections CR. Detailed wording can be further discussed.

Intended outcome: Agreeable 38.321 CR in R2-2102189, discussion summary in R2-2102194 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

[AT113-e][711][V2X/SL] SL CG related issues (OPPO)

Scope: Discuss SL CG related issues with details and attempt to make conclusions.

Intended outcome: Discussion summary in R2-2102190. If we have consensus, we can do email approval otherwise it will come-back next week.

Deadline: Feb 04 0430 (UTC)

[AT113-e][712][V2X/SL] LS to RAN1 (OPPO)

Scope: Discuss and prepare the approvable LS (including the discussion on detailed wordings).

Intended outcome: Approvable LS in R2-2102191. LS will be approved by email.

Deadline: Feb 04 0430 (UTC) => Suspended (comeback the issue online)

[AT113-e][713][V2X/SL] TX resource (re)selection w/ HARQ feedback consideration (Vivo)

Scope: Discuss what the problem is to reflect RAN1 decision and how to specify it (if no problem). Includes both single-shot case and multi-shots case. R2-2102260 CR can be baseline.

Intended outcome: Agreeable 38.321 CR in R2-2102192 and discussion summary in R2-2102193 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

## 4.3	V2X and Sidelink corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

## 6.4	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

Documents in this agenda item will be handled in a break out session

Tdoc Limitation: 9 tdocs. See also tdoc limitation for Agenda Item 6

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

### 6.4.1	General and Stage-2 corrections

Including incoming LSs, rapporteur inputs, etc.

R2-2100009	LS reply on SL CG handling (R1-2009460; contact: Ericsson)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2100010	LS on R16 V2X Mode-2 agreements to capture in MAC specification (R1-2009474; contact: Intel)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2100011	LS reply on RAN2 agreements and RAN1 related issues (R1-2009475; contact: Intel)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2100024	LS reply on RAN1 agreement on pre-emption (R1-2009661; contact: Intel)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

- All LSs above are noted. Issues are discussed in the Email Disc [POST112-e][701][V2X].

R2-2100017	LS on configurable values for sl-DCI-ToSL-Trans (R1-2009577; contact: Ericsson)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

[Huawei]: RRC has already captured it.

- Noted.

R2-2100022	Reply LS on UE capability for V2X (R1-2009635; contact: OPPO)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2	Cc:RAN4

R2-2100023	Reply LS on maximum data rate for NR sidelink (R1-2009643; contact: OPPO)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

[OPPO]: UE capability has already captured them.

- Two LSs above are noted.

R2-2100073	Reply to LS C1-206576 on the re-keying procedure for NR SL (S3-203483; contact: LGE)	SA3	LS in	Rel-16	eV2XARC	To:RAN2, CT1

R2-2100012	Reply LS on definition of NR V2X con-current operation (R1-2009491; contact: Huawei)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN4	Cc:RAN2

R2-2100061	LS on SL switching priority (R4-2017839; contact:Xiaomi)	RAN4	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN1	Cc:RAN2

[Session chair]: Do we need to respond to R2-2100061? [Xiaomi]: Question is on switching priority between LTE V2X and NR SL, which is mainly specified in RAN1. So we do not need to respond it. [ZTE]: Question also contains how to derive the priority, which is more RAN2 area. So it will be helpful if we respond it. [Session chair]: Suggest not to respond it right now since there is no requested action to RAN2. And we still can respond after RAN1 response.

- Three LSs above are noted.

R2-2100687	CR for TS 38.300 for NR V2X on miscellaneous issues	ZTE Corporation, Sanechips	CR	Rel-16	38.300	16.4.0	0335	-	F	5G_V2X_NRSL-Core

- Revised in R2-2102276.

R2-2102276	CR for TS 38.300 for NR V2X on miscellaneous issues	ZTE Corporation, Sanechips	CR	Rel-16	38.300	16.4.0	0335	1	F	5G_V2X_NRSL-Core

[OPPO]: Last change is not agreeable.

- Noted.

R2-2102328	LS on the resource reservation period (R1-2101922; contact: LGE)    RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

- Noted.

### 6.4.2	Control plane corrections

This agenda item may utilize a summary document on RRC (Huawei).

R2-2102240	Summary of RRC corrections in AI 6.4.2	Huawei, HiSilicon	discussion

Recommendation 1: Agree the Rapporteur’s recommendations in Table 1, and the detailed changes can be further discussed in the offline discussion for Rapporteur’s miscellaneous correction CR(s):

Agree the changes (at least the intention) proposed as agreeable in Table 1;

Disagree the changes proposed as not pursued/not needed in Table 1;

Further discuss the details of the changes proposed as to be further discussed in Table 1.

Recommendation 2: The intention of the CRs in R2-2101767 and R2-2101940 can be agreed. Merge the two CRs and agree the merged CR with necessary revision (i.e. changing remaining “UL” MAC PDU to “SL” MAC PDU in R2-2101767, removing the redundant part in R2-2101940).

Recommendation 4: For the corrections to SL measurement and reporting, the CRs in R2-2101655 and in R2-2100501 can be agreed.

Recommendation 5: RAN2 confirms the intent of the CR in R2-2100785, and further discuss whether the specific change in this CR is agreeable.

- Email discussion on miscellaneous corrections (including the discussion on the need of changes and detailed wordings) in R2-2100786, R2-2100210, R2-2100231, R2-2100500, R2-2100502, R2-2101596, R2-2100919, R2-2100230, R2-2101767, R2-2101940, R2-2101655, R2-2100501, R2-2100785, and R2-2100923.

Recommendation 3: Do not introduce new signalling in SUI for SL reset configuration as proposed in R2-2100115/R2-2100118.

Recommendation 3a: FFS whether the first two changes in R2-2100115 (corresponding to P1/2 in R2-2100118) are agreeable.

Recommendation 6: Do not introduce RLC entity reestablishment in the case of re-keying (which is a new feature not reaching consensus before).

Recommendation 7: Do not pursue the CRs listed in Table 7.

Recommendation 8: Discuss the Tdocs listed in Table 9 together with other related Tdocs under AI 6.1.1 in the main room.

- Will be discussed in separate based on contributions.

[AT113-e][701][V2X/SL] Miscellaneous corrections (Huawei)

Scope: Discuss the need of changes and detailed wordings on the corrections in R2-2100786, R2-2100210, R2-2100231, R2-2100500, R2-2100502, R2-2101596, R2-2100919, R2-2100230, R2-2101767, R2-2101940, R2-2101655, R2-2100501, R2-2100785, and R2-2100923. Merge changes and prepare the agreeable CRs. Note for the changes which considered as non-backward compatible, we can prepare a separate CR (including, e.g. R2-2100230).

Intended outcome: Agreeable 38.331 CR in R2-2102171, R2-2102172 (if a separate CR is needed) and 36.331 CR in R2-2102173. Discussion summary in R2-2102174 (if needed). CRs will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102174	Summary of [AT113-e][701][V2X] Miscellaneous corrections	Huawei, HiSilicon	discussion

Recommendation 1: RAN2 agree to add CT1 spec as reference in section 5.8.9.5 as proposed in R2-2100786.

Recommendation 2: RAN2 agree to add “reconfiguration” in the general description of sidelink RRC reconfiguration procedure as proposed in R2-2100210. Other proposed changes in R2-210210 are not pursued.

Recommendation 3: RAN2 agree with the clarification on the field description of sl-PreemptionEnable as proposed in R2-2100978.

Recommendation 4: RAN2 agree to change the expression ”perform sidelink DRB reconfiguration as specified in 5.8.9.1a;” to “perform sidelink DRB addition/modification/release as specified in 5.8.9.1a.1/5.8.9.1a.2 as proposed in R2-2100231.

Recommendation 5: RAN2 agree to remove “or the UE selects GNSS timing as the synchronization reference source” as proposed in R2-2100231.

Recommendation 6: RAN2 agree to remove the “of dynamic grant” restriction for DCI format 3_0 as proposed in R2-2100231.

Recommendation 7: RAN2 does not agree to the clarification on the field description of sl-PreemptionEnable as proposed in R2-2100231.

Recommendation 8: RAN2 agrees to add the field descriptions for the SL-related parameters as proposed in R2-2100231.

Recommendation 9: RAN2 agrees the changes 1, 2, 5, 6, and 9 as proposed in R2-2100500.

Recommendation 10: RAN2 does not agree to allow UE to use exception pool if the target cell provides exceptional pool in sl-ConfigDedicatedNR in RRCReconfiguration as proposed in R2-2101596.

Recommendation 11: RAN2 does not agree to add the description about connected UE uses configuration from system information as proposed in R2-2101596.

Recommendation 12: RAN2 agree to add UE operation if sidelink RLF occurs for a specific destination, i.e., release the PDCP entity, RLC entity and the logical channel of the sidelink DRB for the specific destination.

Recommendation 13: RAN2 agree to modify sl-ThresPSSCH-RSRP-List to sl-Thres-RSRP-List and clarify in the field description that PSCCH RSRP can be used to compare with the threshold as proposed in R2-2100919.

Recommendation 14: RAN2 agree to add clarification in the field description of sl-ConfigIndexCG, that the value range is 0..7, as proposed in R2-2100230.

Recommendation 15: RAN2 agree to the clarification on the field description of sl-AllowedCG-List and sl-ConfiguredGrantType1Allowed as proposed in R2-2101767/R2-2101940.

Recommendation 16: RAN2 does not agree to specify that a UE in RRC_CONNECTED sets the sl-MeasConfig according to stored NR sidelink measurement received from SIB12 as proposed in R2-2101655.

Recommendation 17: RAN2 agree to add the descriptions of removing measurement reporting entry from VarMeasReportListSL as proposed in R2-2100501.

Recommendation 18: RAN2 agree to correct the reference from SA3 TS 33.536 to CT1 TS 24.587 in section 5.8.1 as proposed in R2-2100785.

Recommendation 19: RAN2 agree to have a separate CR to add clarification in the field descriptions of sl-ConfigIndexCG and sl-HARQ-ProcID-offset.

- All recommendations above are agreed.

R2-2101761	Miscellaneous corrections on TS 38.331 (Rapportuer CR)	Huawei, Hisilicon	CR	Rel-16	38.331	16.3.1	2437	-	F	5G_V2X_NRSL-Core

R2-2102171	Miscellaneous corrections on TS 38.331	Huawei, Hisilicon	CR	Rel-16	38.331	16.3.1	2437	1	F	5G_V2X_NRSL-Core

- Agreed.

R2-2100230	Correction on value range of sl-ConfigIndexCG	OPPO	CR	Rel-16	38.331	16.3.1	2315	-	F	5G_V2X_NRSL-Core

R2-2102172	Correction on value range of sl-ConfigIndexCG and sl-HARQ-ProcID-offset		Huawei, Hisilcon, OPPO	CR	Rel-16	38.331	16.3.1	2315	1	F	5G_V2X_NRSL-Core

- Agreed.

R2-2100788	Correction on T400 expiry behavior	vivo	CR	Rel-16	38.331	16.3.1	2357	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][702].

R2-2100978	Corrections regarding sidelink impacting NR	Ericsson	CR	Rel-16	38.331	16.3.1	2373	-	F	5G_V2X_NRSL-Core

[Session chair]: CRs have different direction on the same issue (e.g. just correct the corresponding reference section (SL RLF at T400 expiry) in one CR while add some additional/different UE behaviour than SL RLF in others). So first we need to discuss which direction we should go towards. [Ericsson]: All changes are for informative text, so we can handle it as part of miscellaneous corrections. [OPPO]: Whether to consider T400 expiry as SL RLF or SL reconfiguration failure was discussed at RAN2#108. Later although it was agreed it as SL RLF, it was discussed and almost converged as SL reconfiguration failure. We also support to consider it as SL reconfiguration failure. [Intel, MediaTek]: We would like to keep the current UE behaviour, i.e. consider SL RLF at T400 expiry, so prefer just correcting the corresponding reference section in the informative timer table. [Vivo]: According to the proposals from the other side, if the UE does fallback to the prior configuration, it is not aligned with the current RRC because the current RRC specifies the UE needs to release all bearers since it is considered as SL RLF. [Apple, Samsung]: Share the view with Huawei/Ericsson. [Session chair]: UE behaviour in the informative table needs to be aligned with the UE behaviour specified in the formative text and companies need to share common understanding. If we agree with the additional or different UE behaviour, the normative text also needs to be updated accordingly.

- Treated in email discussion [AT113-e][702].

R2-2100790	Message protection for NR Sidelink	vivo	discussion

- Treated in email discussion [AT113-e][702].

R2-2100976	Protection of sidelinkUEInformation and ULInformationTrasferIRAT	Ericsson	CR	Rel-16	38.331	16.3.1	2372	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][702].

R2-2101760	Miscellaneous corrections on TS 36.331 (Rapportuer CR)	Huawei, Hisilicon	CR	Rel-16	36.331	16.3.0	4591	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][702].

R2-2100977	Protection of sidelinkUEInformation and ULInformationTrasferIRAT	Ericsson	CR	Rel-16	36.331	16.3.0	4558	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][702].

[AT113-e][702][V2X/SL] T400 expiry in timer table and protection of RRC messages (Vivo)

Scope: Discuss the corrections in R2-101761, R2-2100788, R2-2100978, R2-2100790, R2-2100976, R2-2101760, and R2-2100977. Normative text may also need to be updated if adds some additional/different UE behaviour at T400 expiry. Merge changes and prepare the agreeable CR.

Intended outcome: Agreeable 38.331 CR in R2-2102175, 36.331 CR in R2-2102188, and discussion summary in R2-2102176 (if needed). CRs will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102176	[Offline-702] [V2X] T400 expiry in timer table and protection of RRC message	Vivo	discussion	Rel-16	5G_V2X_NRSL-Core

Proposal1: Upon T400 expiry, UE will perform the same operations as SL RLF and no additional behaviour is needed.

Proposal 2: SidelinkUEInformationNR message shall never be sent unprotected before AS security activation if sl-CapabilityInformationSidelink information field is included in the message.

Proposal 3: The NR ULInformationTransferIRAT message carries others RRC messages. The protection of an instance of this message is the same as for the message which this message is carrying.

Proposal 4: As in legacy V2X, UE  can send SidelinkUEInformation message unprotected. before AS security activation

Proposal 5: The E-UTRA ULInformationTransferIRAT message carries others RRC messages. The protection of an instance of this message is the same as for the message which this message is carrying

- All proposals above are agreed.

R2-2102175	T400 expiry in timer table and protection of RRC messages	vivo	CR	Rel-16	38.331	16.3.1	2460	-	F	5G_V2X_NRSL-Core

- Agreed.

R2-2102188	Protection of sidelinkUEInformation and ULInformationTransferIRAT	vivo	CR	Rel-16	36.331	16.3.0	4602	-	F	5G_V2X_NRSL-Core

- Agreed.

R2-2100786	PC5-RRC connection release requested by upper layers	vivo	CR	Rel-16	38.331	16.3.1	2355	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100210	Correction on the Sidelink RRC Recofiguration Procedure	CATT	CR	Rel-16	38.331	16.3.1	2314	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100231	Miscellaneous Correction on RRC spec for NR SL communication	OPPO	CR	Rel-16	38.331	16.3.1	2316	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100500	Miscellaneous corrections to TS 38.331	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.3.1	2328	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100502	Editorial corrections in TS 38.331	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.3.1	2330	-	D	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2101596	Miscellaneous corrections on 38.331	Xiaomi communications	CR	Rel-16	38.331	16.3.0	2424	-	B	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100919	Clarficiations on RRC Parameter sl-ThresPSSCH-RSRP	CATT	CR	Rel-16	38.331	16.3.1	2364	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2101767	CR on LCP restriction parameters for configured SL grant type1	Huawei, HiSilicon	CR	Rel-16	38.331	16.3.1	2438	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2101940	Correction on SL LCP restriction of configured grant type 1	ASUSTeK	CR	Rel-16	38.331	16.3.1	2434	1	F	5G_V2X_NRSL-Core	R2-2101740

- Treated in email discussion [AT113-e][701].

R2-2101655	Correction on sl-MeasConfig configuration	Google Inc.	CR	Rel-16	38.331	16.3.1	2426	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100501	Corrections on the actions of measurement configuration in TS 38.331	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.3.1	2329	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100785	Lower layer indication in PC5 unicast link re-keying procedure	vivo	CR	Rel-16	38.331	16.3.1	2354	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2100923	Correction to UE actions related to reception of the UECapabilityEnquirySidelink	Samsung Electronics, OPPO	CR	Rel-16	38.331	16.3.0	2365	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][701].

R2-2101234	Correction on SL configured grant type 1 validity under Uu RLF	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, Qualcomm, CATT	CR	Rel-16	38.331	16.3.1	2391	-	F	5G_V2X_NRSL-Core

[Apple, InterDigital, MediaTek, ZTE, Intel]: Issue was discussed last meeting and companies considered the current specification covers the issue. However ok with adding the note (instead of changing any normative text). [Session chair]: RRC also covers until when the UE keeps the CG resources and when to release it? [Huawei]: Yes, it is covered. When to release CG resources is specified in the following bullet in the same section, i.e. the UE releases CG resources when T311 runs. So until T311 runs, MAC will not perform random resource selection. [LG]: Support the proposal since it can provide clearer understanding. [OPPO]: Do we need both normative text and note (both are included in CR)? [Nokia]: If we agree with proposal in the normative text, we do not need a note in addition.

- A note will be added only. Normative text changes will be removed.

R2-2102185	Correction on SL configured grant type 1 validity under Uu RLF	Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, Qualcomm, CATT	CR	Rel-16	38.331	16.3.1	2391	-	F	5G_V2X_NRSL-Core

[OPPO]: Note does not follow the appropriate format/font

- Note will be changed with the appropriate format/font.

- Revision number should be corrected.

- Agreed in R2-2102195 with the correction.

R2-2100116	Clarification on the inter-frequency operation	OPPO, Nokia, Nokia Shanghai Bell, Samsung Electronics, MediaTek Inc.	CR	Rel-16	38.331	16.3.1	2303	-	F	5G_V2X_NRSL-Core

[Ericsson]: 38.304 was updated last meeting and wonder if it is already clear to 38.304? Do we also need 38.331 CR? [Lenovo]: Do not see the real importance of CR. [Xiaomi]: Support the proposal and it makes the specification clearer. [Huawei]: It was already discussed last meeting. [Intel]: Support the proposal. [OPPO]: Yes, 38.304 CR was discussed last meeting and during the discussion it was not crystal clear what is really allowed UE behaviour according to the current specification. Companies looked back the history to see what discussed and why we had the related normative text in 38.304 in the past. In LTE, the related UE behaviour was specified in RRC however in NR we do not in RRC, which brought the difficulties to companies to understand whole picture. Motivation of 38.304 CR is missed in RRC.

- Agree with the intention, i.e. having a note for the allowed UE behaviour.

- Detailed wordings for a note will be discussed in the email discussion.

[AT113-e][703][V2X/SL] Discussion on detailed wording for a note (OPPO)

Scope: Discuss detailed wordings for a note to clarify inter-frequency operation and prepare the agreeable CR.

Intended outcome: Agreeable 38.331 CR in R2-2102177. CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102177	Clarification on the inter-frequency operation	OPPO, Nokia, Nokia Shanghai Bell, Samsung Electronics, MediaTek Inc.	CR	Rel-16	38.331	16.3.1	2303	1	F	5G_V2X_NRSL-Core

- Agreed.

R2-2100787	Clarification on SSB interval value 0	vivo	CR	Rel-16	38.331	16.3.1	2356	-	F	5G_V2X_NRSL-Core

[ZTE]: This should be discussed in RAN1. [MediaTek, Intel, Huawei, Qualcomm]: Agree with ZTE and consider it is not essential correction. [OPPO]: Tend to share the intention and ok to have this clarification. [Huawei]: This has not been resolved in RAN1 and it is ongoing RAN1 discussion, so we cannot make a decision in RAN2 now.

- Noted.

R2-2100118	Left issue on reset configuration	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2100115	Correction reset configuration	OPPO	CR	Rel-16	38.331	16.3.1	2302	-	F	5G_V2X_NRSL-Core

[AT113-e][704][V2X/SL] Left issue on reset configuration (OPPO)

Scope: Discuss if there is real problem with the current specification and what is the best option to solve it (if problem is justified). Prepare the agreeable CR (if needed).

Intended outcome: Agreeable 38.331 CR in R2-2102178 and discussion summary in R2-2102179 (if needed). CR will be approved by email (if needed).

Deadline: Feb 04 0430 (UTC)

R2-2102179	Summary of [AT113-e][704][V2X/SL] Left issue on reset configuration (OPPO)	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

Proposal 1	RAN2 confirm that during the re-set configuration, only the configuration received in the RRCReconfigurationSidelink (i.e., the configuration for Rx) is to be released, i.e., the configuration received from dedicated-RRC/SIB/Pre-configuration (i.e., the configuration for Tx) is not released.

- Agreed.

Proposal 2	RAN2 confirms that during the re-set configuration, after DRB release, the released bearers are to be re-added, based on the stored configuration received from dedicated-RRC/SIB/Pre-configuration.

- Agreed.

Proposal 3	Agree the CR in R2-2102178.

R2-2102178	Correction reset configuration	OPPO	CR	Rel-16	38.331	16.3.1	2302	1	F	5G_V2X_NRSL-Core

- “UE may” will be added before “perform”

- Agreed in R2-2102197 with the addition above.

R2-2100789	Support RLC Re-establishment	vivo	discussion

[AT113-e][705][V2X/SL] RLC Re-establishment (Vivo)

Scope: Discuss the need of RLC re-establishment. Prepare agreeable CR (if needed).

Intended outcome: Agreeable 38.331 CR in R2-2102180 and discussion summary in R2-2102181 (if needed). CR will be approved by email (if needed).

Deadline: Feb 04 0430 (UTC)

R2-2102181	Summary of [AT113-e][705][V2X/SL] RLC Re-establishment (vivo)	vivo	discussion

Proposal 1	RAN2 to confirm that, for sidelink, RLC entity is not re-established when PDCP re-establishment occurs.

- Agreed.

R2-2101232	Clarification with respect to validity of configured SL grant type 1 received in HO command	Nokia, Nokia Shanghai Bell	discussion	Rel-16	5G_V2X_NRSL-Core	R2-2009990

- Not treated.

R2-2100149	DAPS HO and NR Sidelink Communication	Samsung Electronics Co., Ltd	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2101702	Clarification on DAPS HO configuration	vivo	CR	Rel-16	38.331	16.3.1	2430	-	F	5G_V2X_NRSL-Core

- Two CRs above are moved to 6.1.1.

R2-2101740	Correction on SL LCP restriction of configured grant type 1	ASUSTeK	CR	Rel-16	38.331	16.3.0	2434	-	F	5G_V2X_NRSL-Core	Revised

R2-2100150	Corrections to SL Resource Configuration	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.3.1	2305	-	F	5G_V2X_NRSL-Core	Withdrawn

R2-2101703	Clarification on DAPS HO configuration	vivo	CR	Rel-16	38.331	16.3.1	2431	-	F	5G_V2X_NRSL-Core	Withdrawn

### 6.4.3	User plane corrections

Including [POST112-e][701][V2X] RAN1 related discussion (OPPO). This agenda item may utilize a summary document on MAC (LG).

R2-2100098	Summary of email discussion [701][V2X] RAN1 related discussion (OPPO)	OPPO	discussion	Rel-16	Late

Easy agreements which is also captured in the CR R2-2100099

Proposal8: MAC specification captures the timing for UE to perform evaluation or pre-emption.

Proposal 9: MAC trigger re-evaluation and pre-emption at moment “m-T3” and capture it in normative text.

Proposal10: to further clarify UE’s behaviour before “m-T3” or after “m-T3” till “m” is up to UE’s implementation in a Note.

Proposal11: to take rapporteur proposed text as baseline and further discuss the detail wording including the Note to clarify UE’s implementation in draft CR

Propsoal12: agree to capture UE’s behaviour w.r.t. sub-bullet 2 under question 3.2-1 as a Note.

Proposal13: take the proposed text under question 3.2-2 as baseline and discuss wording in draft CR.

Proposal14: to clarify UE’s behavior in a Note on setting reservation period in re-selected resource due to preemption. Detail wording can be discussed in draft CR.

Proposal15: To remove the text on resource reselection for dropped resource due to congestion control in current MAC spec.

Proposal 16: to remove the text on resource reselection for dropped resource due to prioritization, and add a NOTE to leave it to UE implementation.

Proposal 17: CR proposed by R2-2007918 is not pursued.

- All proposals above (listed from proposal 8 to proposal 17) are agreed.

Proposals to be discussed online which is not captured in CR yet:

Proposal1: to further discuss whether sharing between resource pool for mode 1 and mode2 is allowed in Rel-16.

Proposal2: to discuss online how to treat invalid CG resource slot if option 1 is concluded

Proposal3: if option1 is concluded, the accumulation granularity is changed from numberOfSLSlotsPerFrame to be parameter N and to replace “logical slot number in the frame” to be “logical slot number in two frames” in the equation.

Proposal4: if option1 is concluded, further clarify that the 1st frame of two radio frames where N is a constant value should be an even radio frame.

Proposal5: If option2 is concluded, the equation (1) , (2) and (3) under question 2.1-5 are agreed in principle. RAN2 can further discuss detail in CR phase.

Proposal6: it is confirmed that parameter CURRENT_slot and period of CG resource in the equation to calculate HARQ process ID for SL CG should be aligned with parameters in equation to calculate CG resource slot.

Proposal7: To further discuss online which option to go under question 2.2-2 about parameter sl-HARQ-ProcID-offset.

[OPPO]: At least we should decide the issue whether sharing between resource pool for mode 1 and mode2 is allowed. [Huawei, ZTE]: Should be discussed in RAN. [Session chair, OPPO, Lenovo, Apple]: We discuss Rel-16 CR, why we should consider non-existing Rel-16 functions in Rel-16 CR. Of course, whether to allow resource pool sharing in future release is up to RAN when new WID is made. [ZTE]: Resource pool sharing has nothing to do with the original issue. [Huawei]: Forward compatibility issue may be raised in future release if we do not design it w/o consideration in Rel-16. [LG]: Reservation period is set to 0 in mode 1, which means resource pool sharing between resource pool for mode 1 and mode 2 is not allowed in Rel-16. However, agree with ZTE in that it has nothing to do with the original issue, i.e. which SL logical slot will be considered in CG. [Ericsson]: It is crystal clear that resource pool sharing is not supported in Rel-16. We even do not need to capture the agreement as new one now.

- RAN2 reconfirms that resource pool sharing between resource pool for mode 1 and mode2 is not allowed in Rel-16. No standard effort to support it in Rel-16.

[AT113-e][711][V2X/SL] SL CG related issues (OPPO)

Scope: Discuss SL CG related issues with details and attempt to make conclusions.

Intended outcome: Discussion summary in R2-2102190. If we have consensus, we can do email approval otherwise it will come-back next week.

Deadline: Feb 04 0430 (UTC)

R2-2102190	Summary of email [AT113-e][711][V2X]SL CG related issues	OPPO	discussion	Rel-16

Based on the discussion, RAN2 intend to agree on option2 listed under question 1. The batch of proposals for option2 are:

Proposal1: The equation to define CG resource slot should be defined based on Level_3 logical slots i.e. logical slots within one resource pool

- Agreed.

Proposal5: if option2 is concluded, equation (2) and (3) are agreed with potential modification on parameter name in CR stage.

Proposal6: if option2 is concluded, scaling equation (1) of CG period could be revised once scaling equation for mode 2 period is concluded.

Proposal7: if option2 is concluded ,RRC CR is needed to introduce new parameter sl_TimeOffsetCGType1_RP and referenceSlot_RP and deal with parameter sl-TimeReferenceSFN-Type1 and sl-TimeOffsetCGType1 of existing equation.

If RAN2 conclude option1, then the batch of proposals for option1 can be agreed:

Proposal2: if option 1 is concluded, UE simply drops the invalid CG resource slot(s)

Proposal3: if option1 is concluded, the accumulation granularity is changed from numberOfSLSlotsPerFrame to be parameter N and to replace “logical slot number in the frame” to be “logical slot number in two frames” in the equation.

Proposal4: if option1 is concluded, further clarify that the 1st frame of two radio frames where N is a constant value should be an even radio frame.

[POST113-e][701][V2X/SL] Response LS to RAN1 LS (R2-2102328) (LG)

Scope: Prepare response LS to RAN1 based on the agreement made in the discussion of R2-2102190. We can confirm RAN2 can do the necessary spec update and indicate RAN2 will let RAN1 updated once spec update is completed.

Intended outcome: Approvable LS in R2-2102196

Deadline: Short email discussion until 3/1 11:00 (UTC)

[POST113-e][707][V2X/SL] Spec update to level 3 logical slots (OPPO)

Scope: Discuss the update of CG equation and other spec changes according to level 3 logical slots (i.e. logical slots within a resource pool).

Intended outcome: Discussion summary and the corresponding CRs

Deadline: Long email discussion

R2-2100099	CR on Correction on SL CG and mode2 operation	OPPO	CR	Rel-16	38.321	16.3.0	1001	-	F	5G_V2X_NRSL-Core	Late

[AT113-e][709][V2X/SL] Mode 2 resource (re)selection (OPPO)

Scope: Discuss R2-2100099 based on the agreements from [POST112-e][701][V2X] and prepare agreeable 38.321 CR. Note this CR covers only mode 2 resource (re)selection related decisions.

Intended outcome: Agreeable 38.321 CR in R2-2102186, discussion summary in R2-2102187 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102186	CR on Correction on SL CG and mode2 operation	OPPO	CR	Rel-16	38.321	16.3.0	1001	1	F	5G_V2X_NRSL-Core

[LG]: We need to clarify “current resource reservation interval” is non-initial resource reservation interval. [Session chair]: We can add further clarification for the next meeting. [LG]: Prefer removing “Note X2…” in this CR and revisit the Note X2 next meeting. [Ericsson]: Inter-operability part may need to be further enhanced.

- Remove Note X2

- Remove 5.22.1.1

- Check with Ericsson and enhance inter-operability issues (if needed).

- Further changes/updates are discussed in the short email discussion (including R2-2102187)

[POST113-e][702][V2X/SL] Update of MAC CR (R2-2102186) (OPPO)

Scope: Update of MAC CR (R2-2102186)

Intended outcome: Agreeable MAC CR in R2-2102198

Deadline: Short email discussion until 3/1 11:00 (UTC)

R2-2102222	Review Report on MAC CRs in AI 6.4.3	LG Electronics Inc. (Rapporteur)	discussion	Rel-16	5G_V2X_NRSL-Core

Proposal 1: Discuss rapporteur’s suggestions in Table 1 for R2-2100212, R2-2100213, R2-2101741, and R2-2100503, and in Table 3 for R2-2100504, R2-2101068, R2-2101149, R2-2100323, R2-2101742, R2-2100861, R2-2100119, and R2-2100211. If changes are agreeable, merge them into a rapporteur’s miscellaneous corrections CR. Detailed wording can be further discussed.

[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)

Scope: Discuss rapporteur’s suggestions in Table 1 for R2-2100212, R2-2100213, R2-2101741, and R2-2100503, and in Table 3 for R2-2100504, R2-2101068, R2-2101149, R2-2100323, R2-2101742, R2-2100861, R2-2100119, and R2-2100211. If changes are agreeable, merge them into a rapporteur’s miscellaneous corrections CR. Detailed wording can be further discussed.

Intended outcome: Agreeable 38.321 CR in R2-2102189, discussion summary in R2-2102194 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102194	[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)	LG Electronics Inc. (Rapporteur)	discussion	Rel-16	5G_V2X_NRSL-Core

Recommendation 1A: the change in R2-2100212 and R2-2101741 is reflected on 38.321.

Recommendation 2A: the change in R2-2100213 is reflected on 38.321

Recommendation 3A: the change in R2-2101741 is not pursued.

Recommendation 4A: the first change in R2-2100503 is reflected on 38.321

Recommendation 4B: the second change in R2-2100503 is not pursued.

Recommendation 4C: the third change in R2-2100503 is reflected on 38.321

Recommendation 5A: the change in 2100504 is not pursued.

Recommendation 6A: the change in R2-2101068 is not pursued.

Recommendation 7A: the change in R2-2101149 is not pursued.

Recommendation 8A: the change in R2-2100323 is not pursued.

Recommendation 9A: the change in R2-2101742 is not pursued.

Recommendation 10A: the change in R2-2100861 is not pursued.

Recommendation 11A: the first change in R2-2100119 is reflected on 38.321

Recommendation 11B: the second change in R2-2100119 is not pursued.

Recommendation 11C: the third change in R2-2100119 is not pursued.

Recommendation 11D: the fourth change in R2-2100119 is not pursued.

Recommendation 12A: the change in R2-2100211 is not pursued except “prioritization”.

- All recommendations are agreed.

R2-2102189	Miscellaneous MAC corrections	LG Electronics Inc.	CR	Rel-16	38.321	16.3.0	1061	-	F	5G_V2X_NRSL-Core

- Agreed.

R2-2100212	Modification on the Formula of Calculating the SL_RESOURCR_RESELECTION_COUNTER's Range	CATT	CR	Rel-16	38.321	16.3.0	1006	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100213	Correction on the UL Threshold and SL Threshold	CATT	CR	Rel-16	38.321	16.3.0	1007	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2101741	MAC Corrections for NR V2X	ASUSTeK	CR	Rel-16	38.321	16.3.0	1045	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100503	Miscellaneous corrections to TS 38.321	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.3.0	1018	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100504	Corrections on LCP in TS 38.321	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.3.0	1019	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2101068	Miscellaneous corrections to 38.321	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.3.0	1027	-	D	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2101149	Correction to Uu DRX with sidelink	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.3.0	1028	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100323	Clarification on the Notes for UL Prioritization	CATT	CR	Rel-16	38.321	16.3.0	1014	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2101742	MAC Corrections for sidelink BSR triggering	ASUSTeK	CR	Rel-16	38.321	16.3.0	1046	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100861	Correction for HARQ Options for SL groupcast	Apple	CR	Rel-16	38.321	16.3.0	1022	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100119	Miscellaneous Correction on NR-V2X	OPPO	CR	Rel-16	38.321	16.3.0	1002	-	F	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100211	Miscellaneous Correction on TS38.321	CATT	CR	Rel-16	38.321	16.3.0	1005	-	D	5G_V2X_NRSL-Core

- Treated in email discussion [AT113-e][710].

R2-2100117	Left issue on HARQ feedback for CG	OPPO, vivo, Apple, InterDigital, Qualcomm, ZTE Corporation, Sanechips, CATT	discussion	Rel-16	5G_V2X_NRSL-Core

Observation 1	Based on the current RAN2 spec, when HARQ FB being enabled and max re-transmission being reached yet NACK being received from Rx-UE, it is obviously a contradiction that Tx-UE auto-flushing but did not tell network via ACK in PUCCH.

Observation 2	Network cannot know whether sl-CG-MaxTransNumList has been reached or not because it has not information of the priority of the MAC PDU.

Observation 3	Based on RAN1 agreement, UE behaviour should not differ before and after maximum re-transmission being reached for CG, at least in case A/N being enabled and PUCCH being configured.

Proposal 1	Send LS to RAN1 to ask for clarification on UE behaviour if HARQ FB is enabled and max re-transmission is reached, yet NACK received from Rx-UE.

[OPPO]: Last meeting, some companies assumed max retransmission numbers is known in the gNB. However, it is defined per priority, so the gNB cannot be aware of it. It is new compared to last meeting discussion. [LG]: It was discussed last meeting, so we do not need to consider it. [Intel]: Agree with the problem. And it will be desirable if it can be solved in RAN2 instead of impacting RAN1 at this phase. [Huawei]: UE will ignore further grants for retransmission from the gNB if max number of retransmission is reached, which is already clear in MAC spec. Impact seems not so significant. [OPPO]: From UE point of view, it may be ok, however the resources for retransmissions provided by the gNB will be just wasted. Also it will be good to align this issue between RAN1 and RAN2. [Lenovo]: It will be clearer if the UE sends ACK if max retransmission is reached. [LG]: If RAN1 changes RAN1 specification, MAC will be also impacted. [Apple, Nokia, InterDigital, ZTE]: Good to send LS to RAN1 with the information what is specified in MAC and what is the consequence if we follow the existing RAN1 decision. Then it is up to RAN1 whether to change RAN1 specification or not. [Huawei]: disagree with that we send LS to RAN1. [LG]: It will be good to ask more clearly RAN1 to send ACK if max number of retransmissions are reached.

- From RAN2 point of view, it will be good to send ACK if max number of retransmissions are reached (See further discussion and decision in comeback session).

- LS will be sent to RAN1 to inform what is specified in MAC and what is the consequence with the current ACK/NACK transmission specified in RAN1. Ask RAN1 to take it into account (See further discussion and decision in comeback session).

[AT113-e][712][V2X/SL] LS to RAN1 (OPPO)

Scope: Discuss and prepare the approvable LS (including the discussion on detailed wordings).

Intended outcome: Approvable LS in R2-2102191. LS will be approved by email.

Deadline: Feb 04 0430 (UTC)

Further discussion on [AT113-e][712] in comeback session:

[OPPO]: First we need to discuss whether sl-CG-MaxTransNumList covers {only CG resources} or {CG resources + DG resources for retransmissions} [LG, Huawei, Samsung]: According to RAN1 decision, it is clear to cover {only CG resources}.

- RAN2 confirms sl-CG-MaxTransNumList covers {only CG resources}.

[OPPO]: With the confirmation above, motivation to send LS disappears. [Session chair]: Can gNB schedule DG resources for retransmission or not? [LG]: Yes, gNB can still do that with the appropriate configuration (e.g. set sl-CG-MaxTransNumList as larger value than 3, or not configure sl-CG-MaxTransNumList). In this case, the UE will not flush the buffer and we do not need to change the current specification. [Session chair]: Do we still need to send LS? Seems not.

- No need to send LS to RAN1.

How to handle DG for retransmissions needs to be further discussed:

- Option 1: No change of the current specification. gNB can schedule DG resources for retransmissions with the appropriate configuration (e.g. set sl-CG-MaxTransNumList as larger value than 3, or not configure sl-CG-MaxTransNumList).

- Option 2: UE does not flush the buffer when sl-CG-MaxTransNumList is reached.

[Session chair]: Since option 1 is already supported in the current specification, so option 2 needs good reason to change the spec. If not, we can rely on option 1.

[POST113-e][708][V2X/SL] How to handle DG for retransmissions? (OPPO)

Scope: Discuss option 1 (supported by the current spec) and option 2 (change of UE’s buffer flush behaviour).

Intended outcome: Discussion summary and CR (if needed)

Deadline: Long email discussion

R2-2100791	Left issues on TX resource (re-)selection	vivo, OPPO, Apple	discussion

[Intel, AsusTek, Nokia, Apple, OPPO, Ericsson]: Supports the proposal. [LG, Huawei]: Gap between RAN1 and RAN2 is already known, however LCP is dynamic, so the UE does not know what logical channel is to be selected in future, so if logical channel w/ HARQ enabled is selected in future while the UE assumed logical channel w/ HARQ disabled, it is not well supported. [Huawei]: If MAC handles it, we do not need to send any LS to RAN1. [OPPO]: LG’s explanation is more for multi-shots case, but for single-shot case, it will be good to follow RAN1 decision.

- Working assumption: RAN2 will update MAC to RAN1 decision at least for single-shot case (See further discussion in R2-2102193)

[AT113-e][713][V2X/SL] TX resource (re)selection w/ HARQ feedback consideration (Vivo)

Scope: Discuss what the problem is to reflect RAN1 decision and how to specify it (if no problem). Includes both single-shot case and multi-shots case. R2-2102260 CR can be baseline.

Intended outcome: Agreeable 38.321 CR in R2-2102192 and discussion summary in R2-2102193 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

R2-2102193	Summary of [AT113-e][713][V2X/SL] TX resource (re)selection w/ HARQ feedback consideration (vivo)	Vivo	discussion

Proposal 1: RAN2 confirms that there exists misalignment between RAN1 agreement and RAN2 specification on the condition to ensure minimum time gap in resource (re-)selection.

Proposal 2: RAN2 to confirm if followings are correct understanding for both single-shot and multi-shot transmission:

-	2a: The UE knows the HARQ attribute of the logical channel at the time for it to trigger resource (re-)selection

-	2b: The UE doesn’t know the HARQ attribute of the logical channel(s) to be finally multiplexed in MAC PDU(s) after LCP

[LG]: Do not understand why we need to agree with the proposals above.

[Session chair]: Skip the discussion on all other proposals except proposal 6 (most important question and lack of time now)

[need discussions online]

Proposal 5: if P2 is confirmed for both single-shot and multi-shot case, the multi-shot case is handled in the same way as single-shot case.

Proposal 6: RAN2 to make a decision which following option should be adopted for WF

-	Option 1: Keep the current specification and send a LS to RAN1 to explain the technical concern if we capture their agreement in MAC. Proposed CR in R2-2102260 is not pursued. FFS the exact content of the LS.

-	Option 3: Agree “in case that sl-HARQ-FeedbackEnabled has been set to enabled for the logical channel” for both single MAC PDU and multiple MAC PDUs, without changing the existing LCP. And this issue is closed.

[Qualcomm]: RAN1 agreement was for mode 1 not for mode 2. [OPPO]: Option 3 is preferred. Packet dropping is not new one. [Session chair]: Seems LG’s comment in the email discussion has valid point. If we go option 3, we need to change our specification and unnecessary packet dropping may happen, which is not good. Meanwhile if we go option 1, we still survive even w/o packet dropping and additional specification effort.

- Can be discussed next meeting. If we keep the current MAC specification, we may need to send LS to RAN1 to inform it.

[not reviewed by companies and can be discussed online]

Proposal 7:

If option-1 in P6 is agreed, the following information should be considered to be included in the LS:

-	The technical concern why we don’t align with RAN1 agreement (i.e. TB may be dropped for which HARQ FB is enabled but resource without ensuring minimum time gap is selected)

-	Ask RAN1 if they can revert their agreement to align with RAN2 specification

-	Ask RAN1 if they cannot revert their agreement, whether they have any concern on the current MAC specification

Proposal 8a:

If option-3 in P6 is agreed, agree the CR in R2-2102260 with changing all ‘logical channel(s)’ to ‘logical channel’.

Proposal 8b:

If option-3 in P6 is agreed, UE’s behavior is that if any HARQ enabled logical channel is mapped to a SL grant which does not meet the minimum time gap, related transmissions would be dropped. FFS whether/how we capture it in MAC.

R2-2100794	Draft LS to RAN1 on TX resource (re-)selection	vivo	LS out	To:RAN1

- Noted.

R2-2101925	Corrections on MCS selection	Huawei, HiSilicon	CR	Rel-16	38.321	16.3.0	1056	-	F	5G_V2X_NRSL-Core

[OPPO, LG]: It has some relation to the LS we sent to RAN1 so it will be good to wait for RAN1 response LS and prepare single CR to RAN1 decision.

[Huawei]: There was RAN1 decision not to introduce per MCS table for mode 2. [OPPO, LG]: Have different understanding than Huawei.

- Can be discussed once RAN1 decision is clear (or RAN1 response LS is received).

R2-2100412	Cancellation of triggered SL-CSI reporting	SHARP Corporation	discussion	5G_V2X_NRSL-Core

[LG, OPPO, ZTE, Huawei]: We can leave it to UE implementation regarding when exactly CSI report is triggered. In MAC, it is already specified it is reported per a pair of source and destination id, which sounds CSI reporting is sent after checking MAC HD. However, it is also true that it is not crystal clearly specified so it is left to UE implementation. [Sharp]: Ok with implementation option, but having a note would be clearer. [LG]: Will be good to have consistent approach to have a note (we did not agree with other note for something similar in the past).

- Noted.

R2-2100120	Left issue with RAN1 impact	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

[Huawei, LG]: How to send ACK/NACK are RAN1 discussions, so it will be good to directly submit the related contribution in RAN1. [OPPO]: It is to make sure what is missed in MAC specification.

- Noted.

R2-2100792	Clarification on sidelink process ID in SCI	vivo	discussion

R2-2100793	Draft LS to RAN1 on HARQ process number in SCI	vivo	LS out	To:RAN1

- Not treated.

R2-2100688	Correction on PDCP entity re-establishment	ZTE Corporation, Sanechips	CR	Rel-16	38.323	16.2.0	0063	-	F	5G_V2X_NRSL-Core

- Noted.

### 6.4.4	UE capabilities

This agenda item may utilize a summary document (OPPO).

R2-2100114	Update on V2X UE capability	OPPO	CR	Rel-16	38.306	16.3.0	0482	-	F	5G_V2X_NRSL-Core

- Agreed.

R2-2101244	On the peer UE capability transfer in unicast sidelink	Nokia, Nokia Shanghai Bell	discussion	Rel-16	5G_V2X_NRSL-Core

- Not treated.

## 8.15	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

Time budget: 2 TU

Tdoc Limitation: 6 tdocs

Email max expectation: 6 threads

### 8.15.1	Organizational

Including incoming LSs, rapporteur inputs, etc.

R2-2100019	Reply LS on new PQI support for PC5 communication (R1-2009621; contact: OPPO)	RAN1	LS in	Rel-17	FS_5G_ProSe	To:SA2	Cc:RAN2

[Session chair]: Do we need to respond it, e.g. any input from RAN2 point of view? [OPPO]: No action is needed from RAN2 side.

- Noted.

R2-2100105	Discussion on SA2 LS on sidelink DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

Proposal 1	For Q1, RAN2 reply AS layer can determine DRX parameters and no additional input from V2X layer other than the currently available QoS is needed.

[CATT]: AS layer should be able to determine DRX pattern by itself like Uu DRX, however the question is whether any additional information can be helpful. [Huawei]: How to configure DRX parameters is under email discussion. It is too early to say whether it is acquired from SIB and/or dedicated RRC to SA2. Anyway think we need QoS information from the upper layer. [ZTE]: DRX cycle can be determined by upper layer. For other parameters, they can be determined in AS layer. [LG, Samsung, Ericsson, InterDigital, Xiaomi]: Similar view as OPPO. DRX parameters can be determined based on the existing QoS information provided by the upper layer. No additional information is really required.

- Agreed.

Proposal 2	Before answering Q2, RAN2 confirm SA2 conclusion that “For unicast, the PC5 DRX may be negotiated between the UEs in AS layer, pending on the feedback from RAN2”.

[Nokia]: It seems obvious and we can confirm it.

- RAN2 confirms that for unicast, the PC5 DRX may be negotiated between the UEs in AS layer.

- We can also include this RAN2 confirmation into the response LS.

Proposal 3	For Q2, RAN2 further reply that for SL unicast, other than DRX parameter negotiation/sharing reason, AS layer can provide the PC5 DRX related information to the V2X layer, and RAN2 is working on the detailed DRX parameter that applies to each cast type. RAN2 would keep SA2 being update on the RAN2 progress.

[Vivo]: What “other than DRX parameter negotiation/sharing reason” means? [OPPO]: It is made based on top of proposal 2. It means for DRX parameter negotiation for unicast, it will be done in AS.

- Agreed.

Proposal 4	For Q3, RAN2 reply that RAN2 does not think it is beneficial for broadcast and groupcast to share the PC5 DRX related information amongst UEs in the vicinity in V2X layer.

[Huawei]: We can confirm it is not feasible to do that via PC5-RRC, but how RAN2 confirms it is not feasible to do that in V2X layer. [OPPO, Vivo]: The question in SA2 LS was about V2X layer.

- Agreed.

Proposal 5	For Q4, RAN2 reply that RAN2 is working on this aspects following the WID bullet of “Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE”, RAN2 would keep SA2 updated on related working progress.

- Agreed.

- We can add sentence to keep SA2 updated according to RAN2 progress in general (not in the answer to the specific question).

Agreements on SA2’s questions:

1: 	For Q1, RAN2 reply AS layer can determine DRX parameters and no additional input from V2X layer other than the currently available QoS is needed.

2:	RAN2 confirms that for unicast, the PC5 DRX may be negotiated between the UEs in AS layer. We can also include this RAN2 confirmation into the response LS.

3:	For Q2, RAN2 further reply that for SL unicast, other than DRX parameter negotiation/sharing reason, AS layer can provide the PC5 DRX related information to the V2X layer, and RAN2 is working on the detailed DRX parameter that applies to each cast type. RAN2 would keep SA2 being update on the RAN2 progress.

4:	For Q3, RAN2 reply that RAN2 does not think it is beneficial for broadcast and groupcast to share the PC5 DRX related information amongst UEs in the vicinity in V2X layer.

5:	For Q4, RAN2 reply that RAN2 is working on this aspects following the WID bullet of “Specify mechanism aiming to align sidelink DRX wake-up time with Uu DRX wake-up time in an in-coverage UE”, RAN2 would keep SA2 updated on related working progress.

[AT113-e][706][V2X/SL] Response LS to SA2 (LG)

Scope: discuss detailed wordings and prepare the LS to be approved.

Intended outcome: approvable response LS in R2-2102182. LS will be approved by email.

Deadline: Feb 02 1245 (UTC)

R2-2101726	(Draft) Reply LS on SA2 on PC5 DRX operation	LG Electronics France	LS out	Rel-16	NR_SL_enh-Core	To:SA2	Late

R2-2102182	Reply LS on SA2 on PC5 DRX operation	LS out	Rel-16	NR_SL_enh-Core	To:SA2	Cc: RAN1

- Approved.

R2-2100798	Draft Reply LS on PC5 DRX operation	vivo	LS out	To:SA2	Cc:RAN1

### 8.15.2	SL DRX

#### 8.15.2.1	SL DRX general

Including [POST112-e][702][SLe] High-level principles for SL DRX (LG), definition of on- and off- durations and the corresponding UE procedures, etc.

R2-2101727	Summary of [POST112-e][702][SLe] High-level principles for SL DRX	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core	Late

Proposal 2.2-2. For SL unicast (after SL unicast link is established), SL DRX configuration can be configured per a pair of source/destination. FFS whether SL DRX operates per direction or for both directions.

[Huawei, ZTE, OPPO, InterDigital]: Per a pair of source/destination already means per direction. That is what we assumed in Rel-16 SL. [Ericsson]: Prefer common DRX operates for both directions. [Apple, Qualcomm]: Prefer to open the possibility of common DRX for both directions [InterDigital]: Common DRX for both directions is not justified since traffic pattern is generated in TX UE.

- Agreed.

Proposal 2.1-2. For SL groupcast/broadcast, SL DRX configuration can be configured in common. FFS on granularity of SL DRX configuration.

- Agreed.

Proposal 2.4-1. Short DRX cycle is not introduced for SL unicast, groupcast and broadcast in Rel-17.

- Agreed.

Proposal 4.1-2. For data reception, RAN2 defines the behaviour for monitoring the SCI reception (i.e., PSCCH and 2nd SCI on PSSCH) during the SL active time for SL DRX. For data reception, the UE may skip monitoring of PSCCH and 2nd SCI on PSSCH during inactive time for SL DRX. Sensing aspect is not considered in this agreement.

[Vivo, Nokia, InterDigital, Apple]: ok with the proposal.

- Agreed.

Proposal 5.1-1a. At least, On-duration timer and Inactivity timer are supported in SL unicast.

- Agreed.

Proposal 5.1-1b. HARQ RTT is supported in SL unicast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

[Apple, Fujitsu, Qualcomm, LG]: HARQ RTT is quite short time and applied per HARQ process, so power saving gain is not clear. HARQ RTT configuration is not required since HARQ retransmission time is already signalled via SCI. Thus, single fixed HARQ RTT value may sound not good. [ZTE]: Agree with Apple and it is same for retransmission timer, so we need to put FFS on retransmission timer also. [Huawei, Vivo, LG]: For mode 1, the UE does not know when it will be scheduled, so a kind of timer would be required. [LG]: Also next reserved resource can be broken so we need at least some timer is required. [InterDigital, Intel, MediaTek]: Ok with the proposal. [Session chair]: HARQ retransmission timer may be helpful in case of resource re-evaluation and pre-emption.

- Agreed.

Proposal 5.2-1a. At least, on-duration timer is supported for SL groupcast. FFS for the need and detailed condition when inactivity timer is supported.

[OPPO, Huawei]: For groupcast, if SCI is missed, what will happen? Dynamic inactivity timer may not be good to groupcast. [Ericsson, Lenovo, InterDigital, LG, Vivo]: Think inactivity timer is needed and the situation is similar to Uu DRX. [Session chair]: For Uu DRX or SL unicast case, there is some mean that TX UE knows whether DCI/SCI is missed or not, e.g. based on HARQ A/N. [ZTE]: Prefer putting FFS on the need of inactivity timer. [MediaTek]: Benefit of inactivity timer is not clear and it seems quite complicated to support such a dynamic timer.

- Agreed.

Proposal 5.2-1b. HARQ RTT is supported in SL groupcast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

[Huawei]: To align with inactivity timer, prefer putting FFS on the need of HARQ RTT. [Apple, OPPO, MediaTek, InterDigital]: Ok with the proposal.

- Agreed.

Proposal 5.3-1. At least, on-duration timer is supported for SL broadcast.

- Agreed.

Proposal 6-1. SL DRX Command MAC CE is introduced for SL DRX operation in unicast. FFS on the need of groupcast. FFS on the detailed UE behaviour (including relation to inactivity timer).

[ZTE]: Functionality of SL DRX command MAC CE is not clear at the moment. [Intel, Lenovo, MediaTek]: Support the proposal. [Apple, Qualcomm]: It is not clear whether it is applied for all cast types or only for unicast. At least for broadcast and groupcast, it seems not working well because any UE can be TX UE. [Ericsson]: We do not favour having SL DRX command MAC CE for any cast type. [LG]: At least for unicast, we need to support it. [MediaTek]: It is required for RX UE to know when DRX operation is stopped and when (re)started. [Xiaomi]: Understand the problem for broadcast and groupcast, but it needs to be supported at least for unicast. [Intel, CATT, Vivo, MediaTek, Fujitsu, Lenovo]: Ok with the current proposal. [OPPO]: For broadcast, we did not agree to have inactivity timer yet, so it should be FFS only for groupcast.

- Agreed.

Proposal 7-1. In mode 1, when in RRC_CONNECTED, if DRX is configured, the MAC entity monitors the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX Active Time. MAC entity does not need to monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX in-active Time.

[Qualcomm]: Uu DRX active time is not clear what it means. [LG]: Uu DRX active time is to monitor PDCCH. [Ericsson, CATT]: Support the proposal.

- Agreed.

Proposal 2.5-1a. For RRC CONNECTED UE, gNB determines the SL DRX configuration.

Proposal 2.5-1b. For RRC IDLE/INACTIVE UEs, a UE can determine the SL DRX configuration. Which UE (e.g., Tx UE or Rx UE) determines SL DRX configuration is FFS.

Proposal 2.5-1c. For OOC UE, Pre-configuration parameters can be used for SL DRX configuration.

[AT113-e][707][V2X/SL] Who will decide SL DRX pattern? (OPPO)

Scope: Discuss who (TX UE, RX UE or gNB) will decide SL DRX pattern or configuration in various scenarios (scenario by scenario) considering whether SL DRX is for SL unicast, groupcast or broadcast, TX and RX UEs’ RRC state (including OOC), and whether TX and RX UE’s in the same or different serving cells (including IC and OOC).

Intended outcome: Discussion summary and proposals in R2-2102183

Deadline: Feb 02 1245 (UTC)

Proposal 5.1-2. Values of On-duration timer and Inactivity timer can be configured to different values per PC5 RRC Connection.

Proposal 5.1-3. The value of HARQ RTT timer and the Retransmission timer can be separately configured per PC5 RRC connection.

Proposal 5.2-2. Values of On-duration timer and Inactivity timer can be configured to different values per each SL groupcast session.

Proposal 5.2-3. The value of HARQ RTT timer and the Retransmission timer can be separately configured per each SL groupcast session.

Proposal 5.3-2. Values of On-duration timer can be configured to different values per each SL broadcast session.

[Session chair]: skip the above proposals (listed from proposal 5.1-2 to 5.3-2).

Agreements on high-level principles for SL DRX

1: 	For SL unicast (after SL unicast link is established), SL DRX configuration can be configured per a pair of source/destination. FFS whether SL DRX operates per direction or for both directions.

2:	For SL groupcast/broadcast, SL DRX configuration can be configured in common. FFS on granularity of SL DRX configuration.

3:	Short DRX cycle is not introduced for SL unicast, groupcast and broadcast in Rel-17.

4:	For data reception, RAN2 defines the behaviour for monitoring the SCI reception (i.e., PSCCH and 2nd SCI on PSSCH) during the SL active time for SL DRX. For data reception, the UE may skip monitoring of PSCCH and 2nd SCI on PSSCH during inactive time for SL DRX. Sensing aspect is not considered in this agreement.

5a:	At least, On-duration timer and Inactivity timer are supported in SL unicast.

5b: 	HARQ RTT is supported in SL unicast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

6a: 	At least, on-duration timer is supported for SL groupcast. FFS for the need and detailed condition when inactivity timer is supported.

6b: 	HARQ RTT is supported in SL groupcast. FFS for the detailed condition when it is supported. FFS whether HARQ RTT is explicitly configured or can be based on SCI. FFS on the need of HARQ retransmission timer.

7: 	At least, on-duration timer is supported for SL broadcast.

8: 	SL DRX Command MAC CE is introduced for SL DRX operation in unicast. FFS on the need of groupcast. FFS on the detailed UE behaviour (including relation to inactivity timer).

9: 	In mode 1, when in RRC_CONNECTED, if DRX is configured, the MAC entity monitors the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX Active Time. MAC entity does not need to monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX in-active Time.

R2-2102183	Summary of [AT113-e][707]	OPPO	discussion	Rel-17	NR_SL_enh-Core

Proposal 1	For broadcast/groupcast, for out-of-coverage case, TX-UE/RX-UE obtain DRX configuration from pre-configuration.

- Agreed.

Proposal 2	For broadcast/groupcast, for in-coverage case, RRC_IDLE/INACTIVE TX-UE/RX-UE obtain DRX configuration from SIB. It is up to network implementation how to coordinate active time between different cells.

[ZTE]: If TX and RX UEs are in the different cell, how to synchronize the active time? [OPPO]: It is up to network coordination. [LG]: Ask similar question to partial coverage case, between IC and OOC.

- Agreed.

Proposal 3	For broadcast/groupcast, for in-coverage case, for RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB. FFS on whether dedicated-RRC is also used.

[Ericsson]: Dedicated-RRC option may not be feasible since the gNB does not know the group members. [Lenovo]: Support the proposal. [ZTE]: DRX configuration for broadcast/groupcast is not UE specific, so we do not need dedicated-RRC. [Huawei]: gNB does not need to know the group members, it is just DRX configuration. Prefer keeping FFS. [Xiaomi]: Although the configuration may be common, it can be signalled by dedicated-RRC just as Uu. [Ericsson]: does dedicated-RRC mean to deliver the SIB or something else? [OPPO]: There was no common understanding in the email discussion. Some companies assumed more like UE specific configuration. [Lenovo]: It is up to network implementation.

- Agreed.

Proposal 4	For unicast, for OOC scenario, the UE who sends out the DRX configuration decides on the DRX configuration. FFS on whether pre-configuration and/or the assistance information from the peer UE is also taken into account when determining the DRX configuration.

[CATT, Ericsson, OPPO, Vivo, Huawei]: Support the proposal. [InterDigital, Nokia, Apple, Philips, Qualcomm]: Prefer removing “by implementation”. [OPPO]: 19 companies supported the proposal 4 while 8 companies do not.

- Agreed.

Proposal 5	For unicast, for OOC scenario, adopt per-direction DRX configuration is as baseline. FFS on whether it is TX-centric or Rx-centric, i.e. TX UE or RX UE decides it.

[Ericsson]: Do not prefer “per-direction” [OPPO]: Majority companies’ views are clear. 19 companies support the proposal while 3 companies do not like it. [QC]: With per-direction DRX configuration, we may have some issue in the delay point of view. Also it can preclude full duplex operation.

- Agreed.

Agreements on SL DRX configurations

1: 	For broadcast/groupcast, for out-of-coverage case, TX-UE/RX-UE obtain DRX configuration from pre-configuration.

2:	For broadcast/groupcast, for in-coverage case, RRC_IDLE/INACTIVE TX-UE/RX-UE obtain DRX configuration from SIB. It is up to network implementation how to coordinate active time between different cells.

3:	For broadcast/groupcast, for in-coverage case, for RRC_CONNECTED TX-UE/RX-UE can obtain DRX configuration from SIB. FFS on whether dedicated-RRC is also used.

4:	For unicast, for OOC scenario, the UE who sends out the DRX configuration decides on the DRX configuration. FFS on whether pre-configuration and/or the assistance information from the peer UE is also taken into account when determining the DRX configuration.

5: 	For unicast, for OOC scenario, adopt per-direction DRX configuration is as baseline. FFS on whether it is TX-centric or Rx-centric, i.e. TX UE or RX UE decides it.

Granularity of SL DRX operation for SL groupcast/broadcast?

- Option 1: single SL DRX operation for all

- Option 2: one SL DRX operation for all SL groupcast and another one for SL broadcast.

- Option 3: SL DRX operation per service type/destination id

- Option 4: SL DRX operation per QoS level

  - What will be used as QoS level?

- Option 5: SL DRX operation per geo-location

[AT113-e][708][V2X/SL] Granularity of SL DRX operation for groupcast/broadcast (Lenovo)

Scope: Discuss options identified above (including some level of understanding on how it works, e.g. what information can represent QoS level to differentiate SL DRX operation, how geo-location can work, etc., challenges, pros, and cons for each option) and check companies’ views. Note companies can add additional option if the option proposed in the contribution was missed.

Intended outcome: Discussion summary and proposals in R2-2102184

Deadline: Feb 02 1245 (UTC)

R2-2102184	Summary of [AT113-e][708]	Lenovo, Motorola Mobility	discussion	Rel-17	NR_SL_enh-Core

Proposal 1: RAN2 kindly agree that for a given DRX configuration for groupcast and broadcast communication a minimum deterministic time period is configured for SL communication; and in the remaining time the devices may sleep i.e. will not transmit data and will not wake up to receive data.

[Xiaomi]: We do not have clear agreement to consider transmission aspect yet, so it is proposed to remove “i.e. will not transmit data and” [Session chair]: It is not important proposal to spend much time to make a decision, so skip the discussion.

Proposal 2: RAN2 kindly agree that for groupcast and broadcast communication further granularity to multiple sets of DRX configurations (beyond just cast type) is required i.e. more than two DRX Cycle configurations should be supported in specification.

[Huawei]: What “beyond just cast type” does mean? Does it mean DRX configuration per cast type? [Lenovo]: Yes, main point is we need more granularities than DRX configuration per cast type.

- Agreed.

Proposal 3: RAN2 kindly de-prioritize “DRX cycles configurations per L2 destination ID” solution until it has been shown to be feasible.

[Session chair]: skip the discussion (it is discussed together with proposal 5). Re-wording may be better than “de-prioritize” considering number of supporting companies, e.g. putting FFS.

Proposal 4: RAN2 kindly agree that “service ID/ ITS-AID” is not used to derive groupcast and broadcast communication DRX cycles configurations.

Proposal 5: RAN2 will study/discuss how PQI and/or L2 destination ID is used to derive groupcast and broadcast DRX configuration.

[Huawei]: Support the proposal, and for RRC connected UE, we may consider per L2 destination id also. [Fujitsu]: How does TX UE and RX UE shares the same understanding of PQI? For the specified PQI, there are still many numbers. Does the UE need to monitor all DRX configurations according to all PQIs? [OPPO, ZTE]: Support the proposal. [Ericsson]: PQI is not possible to be shared among UEs. [Qualcomm]: RX UE does not know what PQI is associated to the packet, which means RX UE needs to monitor all DRX configurations to all possible PQIs. [LG]: To current SA2 specification, RX UE can know the PQI corresponding to the one for transmission in TX UE.

- Agreed.

Proposal 6: RAN2 kindly agree that “geo-location” is not used to derive groupcast and broadcast communication DRX cycles configurations.

Agreements on granularity of SL DRX operation for groupcast/broadcast

1: 	RAN2 kindly agree that for groupcast and broadcast communication further granularity to multiple sets of DRX configurations (beyond just cast type) is required i.e. more than two DRX Cycle configurations should be supported in specification.

2:	RAN2 will study/discuss how PQI and/or L2 destination ID is used to derive groupcast and broadcast DRX configuration.

Common approach for SL groupcast/broadcast as SL unicast, i.e. timer-based SL DRX, or different approach for SL groupcast/broadcast, i.e. separate resource pool based SL DRX?

R2-2100622	On general Sidelink DRX design	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core (Proposal 1 only)

[Session Chair]: On-duration timer for SL groupcast/broadcast was already agreed this meeting. Does it mean timer-based SL DRX is also applied to SL groupcast/broadcast (like SL unicast)? [LG, CATT, Apple, Ericsson, Intel, Convida, Qualcomm, Vivo, Xiaomi, Lenovo, Fraunhofer, Samsung, ZTE, MediaTek]: Agrees with the session chair’s observation.

- Timer-based SL DRX is also applied to SL groupcast/broadcast.

Agreements on SL DRX on groupcast/broadcast

1: 	Timer-based SL DRX is also applied to SL groupcast/broadcast.

Can we confirm working assumption to also consider sensing impact in SL DRX or should we wait for RAN1 decision or response LS?

R2-2100272	Left issues on definition of SL DRX functionality	OPPO	discussion	Rel-17	NR_SL_enh-Core (Proposal 2 only)

[Session chair]: No discussion due to lack of time. We can wait RAN1 response until next meeting.

Details of how to maintain the agreed timers (including exact definition of timers, how to set the timers, when to start/restart/stop the timers, additional consideration due to SL characteristics, considerations of both RX UE and its peer TX UE sides)

R2-2100514	Definition of the Active Time in SL DRX	InterDigital	discussion	Rel-17	NR_SL_enh-Core (Section 2.2 only)

[POST113-e][703][V2X/SL] Details of timers (InterDigital)

Scope: Discuss details of how to maintain the agreed timers (including exact definition of timers, how to set the timers, when to start/restart/stop the timers, additional consideration due to SL characteristics, considerations of both RX UE and its peer TX UE sides) and FFS parts related to timer operations.

Intended outcome: Discussion summary

Deadline: Long email discussion

Any backward compatibility issue?

R2-2101323	Backward Compatibility Issue of SL DRX with Rel.16 Sidelink	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core (Proposal 1 only)

[Session chair]: No discussion due to lack of time.

[POST113-e][704][V2X/SL] TX UE centric or RX UE centric DRX configuration determination (OPPO)

Scope: Continue the discussion the remained issues regarding who will determine DRX configurations (including TX UE centric vs RX UE centric DRX configuration determination), covering both in and out of coverage scenario.

Intended outcome: Discussion summary

Deadline: Long email discussion

R2-2100235	Sidelink DRX Granularity	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100236	Sidelink DRX Timer Maintainence and Active Time Definition	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100274	Discussion on granularity for sidelink DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

R2-2100496	Discussion on  principles for sidelink DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100497	Discussion on timer configuration for sidelink DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100515	Procedures for Handling the DRX Configuration	InterDigital	discussion	Rel-17	NR_SL_enh-Core

R2-2100536	General aspects for SL DRX	Ericsson	discussion	Rel-17	NR_SL_enh-Core	R2-2009231

R2-2100573	General Principle of NR SL DRX 	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2100637	Discussion on SL DRX	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core

R2-2100638	Discussion on SL DRX Timer	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core

R2-2100690	[draft]LS to RAN1 on SL DRX timer configuration	ZTE Corporation, Sanechips	LS out	Rel-17	NR_SL_enh-Core	To:RAN1	Withdrawn

R2-2100795	SL DRX remaining issues	vivo	discussion

R2-2100862	Discussion on remaining issues on SL DRX Configuration	Apple	discussion	Rel-17	NR_SL_enh-Core

R2-2101224	Discontinuous reception and transmission in SL	Lenovo, Motorola Mobility	discussion	NR_SL_enh-Core

R2-2101245	Discussion on Sidelink DRX	Qualcomm Finland RFFE Oy	discussion	Rel-17

R2-2101330	Granularity of SL DRX operation	Samsung Research America	discussion

R2-2101600	Discussion on sidelink DRX timer handling	Xiaomi communications	discussion

R2-2101723	Consideration on sidelink DRX for groupcast and broadcast	Huawei, HiSilicon	discussion

R2-2101725	General aspects of SL DRX for unicast	Huawei, HiSilicon	discussion

R2-2101756	Discussion on Sidelink DRX	ASUSTeK	discussion	Rel-17	NR_SL_enh-Core

#### 8.15.2.2	Mechanism to align wake-up time between TX and RX UEs

R2-2100237	Sidelink DRX Configuration Procedure for Sidelink Unicast	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100273	Discussion on configuration for sidelink DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

R2-2100421	Reservation Chain-based DRX Power Saving	Fujitsu	discussion	Rel-17	NR_SL_enh-Core

R2-2100422	Alignment of Wake-up Time between TX and RX UEs	Fujitsu	discussion	Rel-17	NR_SL_enh-Core	R2-2009133

R2-2100495	Discussion on  Mechanism to align wake-up time between TX and RX UEs	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100539	SL DRX alignment between two UEs	Ericsson	discussion	Rel-17	NR_SL_enh-Core

R2-2100574	NR SL DRX Alignment between UEs	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2100629	Alignment of DRX active time among sidelink UEs	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

R2-2100657	Inter-UE sidelink DRX wake-up time alignment	Spreadtrum Communications	discussion	Rel-17	NR_SL_enh-Core

R2-2100796	Mechanism to align wake-up time between TX and RX UEs	vivo	discussion

R2-2100863	Discussion on HARQ related timers in SL DRX	Apple	discussion	Rel-17	NR_SL_enh-Core

R2-2101117	Discussion on wake-up time alignment between Tx and Rx UEs	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2101192	Issue with SL DRX Inactivity Timer for SL groupcast 	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core

R2-2101207	SL DRX with pre-indicated resources 	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core

R2-2101209	On the discrepancy TX-centric vs. RX-centric in Sidelink DRX 	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_enh-Core

R2-2101246	On Wake-up alignment between Tx and Rx UEs	Qualcomm Finland RFFE Oy	discussion	Rel-17

R2-2101331	Alignment of wake-up time between TX and RX UEs	Samsung Research America	discussion

R2-2101598	DRX coordination between TX and RX UE	Xiaomi communications	discussion

R2-2101645	On aligning wake-up time between TX and RX UEs	MediaTek Inc.	discussion	Rel-17	NR_SL_enh-Core

R2-2101652	Sidelink DRX Considerations	Convida Wireless	discussion	Rel-17	NR_SL_enh-Core

R2-2101706	Discussion on SL DRX wake-up time alignment between inter-UEs	LG Electronics France	discussion	Rel-17	NR_SL_enh-Core

R2-2101762	Consideration on the sidelink DRX for unicast	Huawei, Hisilicon	discussion

R2-2101866	Methods for aligning SL DRX between UEs	Sierra Wireless, S.A.	discussion	Rel-17

#### 8.15.2.3	Coordination between Uu DRX and SL DRX

R2-2100538	DRX alignment between Uu and SL	Ericsson	discussion	Rel-17	NR_SL_enh-Core (Proposal 1, 2 and 6 only)

R2-2101763	Discussion on SL communication impact on Uu DRX	Huawei, Hisilicon	discussion (Proposal 1 only)

R2-2100275	Discussion on network involvement for SL related DRX	OPPO	discussion	Rel-17	NR_SL_enh-Core

R2-2100494	Discussion on  Coordination between Uu DRX and SL DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100575	NR SL DRX Uu and SL Wake-Up Time	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2100623	Alignment of Uu and SL DRX active time	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

R2-2100797	Coordination between Uu DRX and SL DRX	vivo	discussion

R2-2100864	Discussion on alignment of Uu DRX and SL DRX	Apple	discussion	Rel-17	NR_SL_enh-Core

R2-2100931	Coordination between Uu DRX and SL DRX 	Lenovo, Motorola Mobility	discussion	Rel-17	NR_SL_enh-Core

R2-2101247	On coordination between Uu DRX and SL DRX	Qualcomm Finland RFFE Oy	discussion	Rel-17

R2-2101306	On configuration and operation of SL DRX	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core	R2-2010058

R2-2101332	Coordination between DL DRX and SL DRX	Samsung Research America	discussion

R2-2101599	DRX coordination between Uu and sidelink	Xiaomi communications	discussion

R2-2101646	On coordination between Uu DRX and SL DRX	MediaTek Inc.	discussion	Rel-17	NR_SL_enh-Core

R2-2101764	Alignment between Uu DRX and SL DRX	Huawei, Hisilicon	discussion

R2-2101791	Alignment scheme for Uu DRX and SL DRX	LG Electronics Inc.	discussion	Rel-17	NR_SL_enh-Core

R2-2101855	Methods for configuring SL DRX relative to Uu DRX	Sierra Wireless, S.A.	discussion	Rel-17

R2-2100917	Discussion on Sidelink DRX and sensing	Sony	discussion	Rel-17	NR_SL_enh-Core

#### 8.15.2.4	Others

R2-2100238	Impacts of Sidelink DRX on the Other Procedures	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100499	Discussion on sensing and DRX	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100537	Interaction between partial sensing and DRX	Ericsson	discussion	Rel-17	NR_SL_enh-Core	R2-2009232

R2-2101333	Transmission UE behaviours for SL DRX 	Samsung Research America	discussion

R2-2101869	View on resource selection in mode 2	ITL	discussion

### 8.15.3	Resource allocation enhancements RAN2 scope

R2-2100240	Mixing Blind and Feedback-based HARQ Retransmissions	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100423	Dual-mode Configuration and Selection Mechanism for NR Sidelink	Fujitsu	discussion	Rel-17	NR_SL_enh-Core	R2-2009134

R2-2101335	Inter-UE coordination	Samsung Research America	discussion

R2-2101795	Power efficient resource allocation	LG Electronics Inc.	discussion	Rel-17	NR_SL_enh-Core

R2-2100239	Consideration on the Resource Allocation Enhancements	CATT	discussion	Rel-17	NR_SL_enh-Core

R2-2100276	Discussion on inter-UE coordination	OPPO	discussion	NR_SL_enh-Core

R2-2100498	Discussion on inter-UE coordination	ZTE Corporation, Sanechips	discussion	Rel-17	NR_SL_enh-Core

R2-2100516	Performing Mode 2 Resource Allocation when configured with SL DRX	InterDigital	discussion	Rel-17	NR_SL_enh-Core

R2-2100517	[DRAFT] LS on RAN1 impact on sidelink DRX	InterDigital	LS out	Rel-17	NR_SL_enh-Core	To:RAN1

R2-2100518	RAN2 Aspects of Resource Allocation with Inter-UE Coordination	InterDigital	discussion	Rel-17	NR_SL_enh-Core

R2-2100576	Inter-UE Coordination for Sidelink Mode 2 Resource Allocation	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2100577	Power Reduction for Sidelink Mode 2 Resource Allocation 	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2100613	Resource Allocation Enhancements for Power Saving	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

R2-2100659	Discussion on resource allocation enhancement for NR sidelink	Spreadtrum Communications	discussion	Rel-17	NR_SL_enh-Core

R2-2100799	Uu and SL DRX impact to resource allocation mode 1	vivo	discussion

R2-2100800	SL DRX impact to resource allocation mode 2	vivo	discussion

R2-2100865	Discussion on resource allocation for Pedestrian UE	Apple	discussion	Rel-17	NR_SL_enh-Core

R2-2100981	General principles of resource allocation enhacements for SL mode 2	Ericsson	discussion	Rel-17	NR_SL_enh-Core

R2-2100982	Way forward for resource allocation enhacements for SL mode 2	Ericsson	discussion	Rel-17	NR_SL_enh-Core

R2-2101116	Discussion on sidelink resource allocation enhancements	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2101299	Inter-UE Coordination for Enhanced Reliability	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

R2-2101303	Congestion control for Resource Allocation Schemes in NR Sidelink	Intel Corporation	discussion	Rel-17	NR_SL_enh-Core

R2-2101318	Coexistence of Sensing-based and Random Selection for Sidelink Mode 2 Resource Allocation	Nokia, Nokia Shanghai Bell	discussion	NR_SL_enh-Core

R2-2101334	Random selection and partial sensing 	Samsung Research America	discussion

R2-2101647	Transmission of assistance information for Mode 2 enhancement	MediaTek Inc.	discussion	Rel-17	NR_SL_enh-Core

R2-2101650	On Resource Allocation Mode 2 Enhancement for NR Sidelink	Convida Wireless	discussion	Rel-17	NR_SL_enh-Core	R2-2010144

R2-2101724	Consideration on resource allocation enhancement in Rel-17 NR SL enhancement	Huawei, HiSilicon	discussion

R2-2101796	Inter-UE coordination for NR V2X	LG Electronics Inc.	discussion	Rel-17	NR_SL_enh-Core

### 8.15.4	Other

R2-2100519	Discussion on Uu DRX for SL UE	InterDigital	discussion	Rel-17	NR_SL_enh-Core

R2-2101648	On SL sync search optimization	MediaTek Inc.	discussion	Rel-17	NR_SL_enh-Core