3GPP TSG-RAN WG2 #113-e	R2-2102039

Electronic meeting, Jan 25th – Feb 5th 2021

Agenda Item:	8.12.1

Source:	Rapporteur (Ericsson)

Title:	Summary of [AT113-e][108][REDCAP] UE identification and access restriction

Document for:	Discussion, Decision

# Introduction

The document summarizes the following offline discussion:

[AT113-e][108][REDCAP] UE identification and access restriction (Ericsson)

Scope: Continue the discussion on UE identification and access restriction based on the proposals in R2-2100985

The intention of this offline is to describe options in the TR and, whenever applicable/possible, also down-select some alternatives / provide some recommendations.

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement

    - List of proposals that require online discussions

    - Corresponding TP for the TR

Initial deadline (for companies' feedback): Monday 2021-02-01 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2102018): Monday 2021-02-01 22:00 UTC

Updated Scope: Continue the discussion on p13, p18 and detailed TP for p16 and p17 from R2-2102018.

Updated intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement

    - Corresponding TP for the TR

Deadline (for companies' feedback): Wednesday 2021-02-03 18:00 UTC

Deadline (for rapporteur's summary in R2-2102039): Wednesday 2021-02-03 22:00 UTC

This is 2nd round of the previous offline, summarized in R2-2102108. The relevant proposals in the previous summary are P13, P17 and P18:

Proposal 13 The legacy UAC principle for Access Categories is assumed for RedCap, that is, different access types are differentiated using Access Categories. - Proposal 17 Capture following text in 11.2.1 on RRC Connection reject: ”To save radio resources and limit negative impact on legacy network performance it is beneficial to bar or reject UEs as early as possible, preferably without additional signalling. Therefore, cell barring and UAC is beneficial compared to RRC connection rejection. However, if the network is aware the UE is a RedCap during initial access, it is possible for the network to reject RRC connection based on UE being a RedCap UE. There is no additional specification impact in case early indication is specified.” - Proposal 18 Discuss whether TR should capture RedCap access control by using separate RACH configuration.

Additionally, P16 in R2-2102108 was agreed and a Text proposal is provided in this offline for companies to comment:

Capture following options with descriptions in TR for RedCap UAC (first two have been agreed to be studied earlier): 1) Define new Access Identity or Identities for RedCap UE 2) Define new Access Category or Categories for RedCap UE 3) Broadcast a separate set of parameters for RedCap UEs 4) Use existing broadcasted UAC parameters for RedCap UEs without any changes

# Discussion

P13 in R2-2102108 was the result of discussion based on Proposal 3a as an attempt to clarify the original proposal which was misunderstood at least by some participants based on some of the company replies.

- Proposal 13	The legacy UAC principle for Access Categories is assumed for RedCap, that is, different access types are differentiated using Access Categories.

To clarify the intention of P13 further:

- For RedCap UAC (which may be some of the options discussed in the agreed P16), different access types are mapped to Access Categories, which is exactly the same principle as in the legacy UAC.

- As a corollary, if a new Access Category is defined for a RedCap UE (type), then likely we would need to define multiple new Access Categories to map the possible access types to Access Categories like in legacy.

- The proposal does not suggest to add any new Access Categories without further discussion – this is not proposed now (also subject checking with SA1/CT1).

- Also subject to further discussion is whether all existing ACs apply to RedCap.

Question 1: Do you agree to P13?

| Company | Agree to Q1 / P13 | Comments |
| --- | --- | --- |
| Qualcomm | See comment | We agree with the intention of the proposal, but we can’t agree with how it is worded because it clearly suggests a specific implementation for applying UAC to RedCap and contradicts what is captured in the TP (different options will be studied). We’d therefore like to suggest the following wording instead: The legacy UAC principle is assumed for RedCap. FFS how it is applied for RedCap using access identity(s) and/or access category(s). |
| Intel | No | In the TR, we already captured “The unified access control (UAC) framework is specified in TS 22.261 and it applies to all UEs in RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE. This mechanism should also apply to RedCap UEs to control RedCap UEs accesses to the network. “ We think the highlighted sentence already reflected the current status well. |
| OPPO | No | We agree with Intel. |
| vivo | Agree with the intention | We are fine with the wording suggested by Qualcomm. |
| Lenovo | See comment | We agree to apply the legacy UAC principle for RedCap. But how to apply the Access Category for RedCap needs more discussion. |
| LGE | Yes but | We agree with the intention but the wording should be enhanced. We are fine with Qualcomm’s suggestion. |
| MediaTek | Yes, but | We are ok with Qualcomm’s suggestion |
| Huawei, HiSilicon | Yes | We agree with the proposal and the intention listed by rapporteur. |
| Samsung | - | We are also fine with the wording from Qualcomm. |
| ZTE | Yes | We agree with the intention of this proposal. And we don’t think it is contradictory to the potential options. We understand this principle should be applied no matter which option is adopted. |
| Apple | No | Exactly the view of Intel |
| XIaomi | - | Qualcomm’s wording is fine to us. |
| Facebook | No | Share the same view as Intel |
| Sequans | - | Agree with Intel, but are fine to agree QC’s wording. |
| Ericsson | Yes | On QC’s comment, the intention is not really to say it needs to be different for RedCap. One of the options is clearly to use existing UAC so not sure what is the concern. |

SUMMARY 15 companies have replied: - 10 companies seem to agree with intention and Qualcomm’s wording - 4 companies think this is not needed and intention is already captured in the TR - 1 company thinks it is not needed but OK with Qualcomm’s wording Thus, the following is proposed according to Qualcomm’s suggested wording with clarification that details are to be discussed in the normative phase. The legacy UAC principle is assumed for RedCap. The details of how RedCap UEs are using access identity(s) and/or access category(s) are to be discussed during normative phase. (11/15)

The following proposal for TP for “RRC Connection Reject” was discussed online and agreed to be captured in the TR with further offline discussion on the wording:

- Proposal 17	Capture following text in 11.2.1 on RRC Connection reject: ”To save radio resources and limit negative impact on legacy network performance it is beneficial to bar or reject UEs as early as possible, preferably without additional signalling. Therefore, cell barring and UAC is beneficial compared to RRC connection rejection. However, if the network is aware the UE is a RedCap during initial access, it is possible for the network to reject RRC connection based on UE being a RedCap UE. There is no additional specification impact in case early indication is specified.”

#### RRC Connection Reject To save radio resources and limit negative impact on legacy network performance it is beneficial to bar or reject Ues as early as possible, preferably without additional signalling. Therefore, cell barring and UAC is beneficial compared to RRC connection rejection. However, if the network is aware the UE is a RedCap during initial access, it is possible for the network to reject RRC connection based on UE being a RedCap UE. There is no additional specification impact in case early indication is specified.

Question 2: Please provide suggestions on the wording of the TP, if any.

| Company | Comments / text proposal |
| --- | --- |
| Qualcomm | We are fine with this TP. |
| Intel | We are fine with this TP |
| OPPO | We are fine with this TP |
| vivo | We are fine with it in general. Besides, we suggest to modify the below sentence in a more general way as following: However, if the network is aware the UE type during initial access, it is possible for the network to reject RRC connection based on UE type. |
| Lenovo | We are fine with this TP |
| LGE | We are fine with this TP |
| MediaTek | We are fine with this TP |
| Huawei, HiSilicon | We are fine with this TP |
| Samsung | We are fine with this TP. |
| ZTE | We are fine with this TP |
| Apple | While we see better value from Vivo’s text, we are ok with either |
| Xiaomi | We are fine with this TP |
| Facebook | Fine with the TP. |
| Sequans | Fine with TP |
| Ericsson | We are fine with this TP |

SUMMARY 15 companies have replied and all are fine with the TP, 2 companies support to modify according to UE type. The suggestion is taken into account in the following proposal: Capture following text in 11.2.1 on RRC Connection reject: ”To save radio resources and limit negative impact on legacy network performance it is beneficial to bar or reject UEs as early as possible, preferably without additional signalling. Therefore, cell barring and UAC is beneficial compared to RRC connection rejection. However, if the network is aware of the UEs type during initial access, it is possible for the network to reject RRC connection based on the UE type. There is no additional specification impact in case early indication is specified.”

The following proposal related to access control by separate RACH configuration was agreed to be continued to be discussed offline.

During the previous round some companies considered this as not needed. However, this option has been discussed in some of the submitted tdocs since the beginning of RedCap SI. The rapporteur thinks that all reasonable options should be captured in the TR. Note, again, that capturing a solution in the TR does not mean endorsing such solution in any way, but if will show RAN2 has done its part on studying different possible options.

Therefore, the first question is about whether to capture the option at all or not:

Question 3: Do you agree to capture “Access control using separate RACH configuration” in the TR as one possible method in the study of access restrictions?

| Company | Reply to Q3 | Comments |
| --- | --- | --- |
| Qualcomm | See comment | We do not support using separate RACH configuration for access control. We are fine with having RedCap-specific RACH parameters through separate RACH configuration for RedCap. In our view, this separation is more about enabling differentiated RACH, instead of differentiated access control. RACH and access control serve different purposes and should not be mixed. So we do not think it should be captured as one of possible methods for access restriction or access control. Maybe it can be captured in a different section instead? |
| Intel | ok | Ok to capture candidate solutions in the TP. |
| OPPO | No | Access control should be implemented by cell barring and UAC. We don’t support to use separate RACH configuration to restrict access of RedCap UEs. |
| vivo | No | We think separate RACH configuration is not intended for access control. For access control, we already has UAC and cell barring by SI, which is enough. |
| Lenovo | See comment | We share similar view with QC. Access control is implemented by cell barring and UAC, and NOT in RACH procedure. We support to study separate RACH configurations for RedCap as in the updated text proposal, but maybe somehow put the text proposal under the coexistence between RedCap UEs and legacy UEs. |
| LGE | No | We don’t support access control using separate RACH configuration. As mentioned by other companies, we do not want implement access control using separate RACH configuration. |
| MediaTek | No | We share the view from others that Access Control is implemented by cell barring and UAC, and not by the RACH procedure. |
| Huawei, HiSilicon | Yes | Agree to capture it as one option in the TR. |
| Samsung | Yes | We are fine to capture it as an option in the TR. |
| ZTE | No | We share similar view as Qualcomm. We also think with RedCap specific RACH configuration (e.g. RACH configuration within a separate slice for RedCap), the NW can configure different RACH parameters for RedCap UE. However, separate RACH configuration is not intend for access control. As the separate RACH configuration is already captured in early identification part, we don’t need to capture this again. |
| Apple | No | While do agree that it is an option for the NW, we do not see the value in having access control done via RACH. |
| Xiaomi | No |  |
| Facebook | No |  |
| Sequans | Yes | We prefer to capture all options in the TR. If identification in Msg1 is agreed for other reasons, then a free byproduct would be the possibility to access control with it. |
| Ericsson | Yes | We have similar view as QC in general – however some parameter setttings and e.g. expanded range of some parameters could be, in effect, used to control the load from RedCap UEs so in that sense it would be access control. We would like to capture this as suggested, especially as we have not agreed a RedCap-specific control mechanism until now. SI indication can be used only for full barring, and not to control barring or access control in a more fine-grained way. |

SUMMARY 15 companies have replied: - 5 companies are OK to capture the option in TR - 8 companies do not think this should be captured, main argument is that this should not be used as access control mechanism - 2 companies also do not think this should be captures as access control mechanism, but agree that different RACH parameters could be configured for RedCap UEs (if the configuration is separated). No company seems to object of having possible RedCap-specific RACH configuration as such – and this might even be required depending on the need for early indication – therefore a proposal is made to capture this possibility in the TR but not listed as access control mechanism. However, it should be noted that e.g. RACH back-off is mentioned in TS 38.300 (and already in the TR) as possible overload or access control mechanism. The text proposal is shortened from the original version below to be more neutral, suggested to be captured per Lenovo proposal in coexistence section: Capture following text in clause 11.2.2 Analysis of coexistence with legacy UEs: “It is possible that separate RACH configuration is provided for RedCap UEs. In such case, it would be possible to configure different RACH parameters to RedCap and non-RedCap UEs, such as different maximum number for preamble transmission, different back-off timer after an attempt or a different power ramping step for RedCap UEs”.

During previous round of offline 108 a detailed text proposal was discussed. If companies agree to add the solution in the TR, the following is an update to the previous text proposal (actually, re-written TP):

#### Access control using separate RACH configuration One possibility to control RedCap UEs vs non-RedCap UEs access is to use separate RACH configuration for RedCap, that is, configure the parameters for RedCap UEs random access in a different way. Possible methods include not configuring RACH resources (if s separate resources for RedCap UEs are configured), different number of maximum attempts for preamble transmission, different back-off time after an attempt or a different power ramping step for RedCap UEs. If separate RACH resources are agreed to be used for RedCap UEs (e.g., due to need for early indication in Msg1), there would be limited specification impact. In case separate RACH resources are not defined for early indication, there would be larger impact as separate RACH configuration would need to be defined and used by the RedCap UEs.

The following question is conditional on the previous Question 3:

Question 4: Do you agree to include the above TP for “Access control using separate RACH configuration” in the TR (conditional on Q3)?

| Company | Reply to Q4 | Comments / Further TP suggestions |
| --- | --- | --- |
| Qualcomm | See comment | We are fine with capturing body of the above text in a more appropriate section in the TR (sorry we don’t have a good suggestion at the moment). |
| Intel | ok | Ok to capture candidate solutions in the TP. |
| OPPO |  | See our reply to Q3. |
| vivo |  | See above. |
| Lenovo |  | See our reply to Q3. Support separate RACH configurations for RedCap, but maybe somehow put it under coexistence between RedCap UEs and legacy UEs. |
| LGE | No | See our reply to Q3. |
| MediaTek | No | See our reply to Q3 |
| Huawei, HiSilicon | Yes | Agree to capture it as one option in the TR. |
| Samsung | Yes | We are fine to capture it as an option in the TR. |
| ZTE | No | See our reply to Q3. |
| Apple | No |  |
| Xiaomi | No |  |
| Facebook | No |  |
| Sequans | Yes |  |
| Ericsson | Yes |  |

SUMMARY 15 companies have replied – comments are similar as for Q3 and proposal can be found above in Q3 summary.

P16 related to UAC was agreed during the online session. There were comments from companies during the first round of offline 108 which are now taken into account in a revised text proposal.

Agreement:

Capture following options with descriptions in TR for RedCap UAC (first two have been agreed to be studied earlier): 1) Define new Access Identity or Identities for RedCap UE 2) Define new Access Category or Categories for RedCap UE 3) Broadcast a separate set of parameters for RedCap UEs 4) Use existing broadcasted UAC parameters for RedCap Ues without any changes

#### Unified Access Control The unified access control (UAC) framework is specified in TS 22.261 and it applies to all Ues in RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE. This mechanism should also apply to RedCap Ues to control RedCap Ues accesses to the network. In UAC each access attempt is associated with an Access Category and one or more Access Identities (defined in TS 24.501). The possible solutions for RedCap UAC that have been considered in the study are the following (the options do not need to be mutually exclusive): - Define one or more RedCap specific Access Identities. Access Identities are connected to the UE type and are (currently) used to lift the barring for certain identities, e.g. for special access classes or Ues configured for prioritized services. - Define RedCap specific Access Categories. Access Categories are related to the type of access attempt and is set per access attempt type depending on what triggered the access (set by NAS if NAS triggered, or by RRC if AS triggered). There can only be one Access Category per access attempt. To be able to treat different RedCap access attempt types differently, e.g. apply different barring two different access types, multiple Access Categories for RedCap could be defined. - Use some of the operator defined Access Categories for RedCap. The description of the previous solution applies also to this solution, the difference is that this solution has no specification impact but cannot be used for initial attach to the network since it depends to CN configuration of the UE. - Broadcast a different set of UAC parameters for RedCap Ues. This makes it possible for NW to flexibly and separately provide UAC parameters for RedCap Ues while avoiding impact on UAC configuration of non-RedCap Ues. - Use existing broadcasted UAC parameters for RedCap Ues with no changes, that is, the same UAC parameters apply for all Ues (non-RedCap Ues and RedCap Ues) and no new Access Categories and Access Identities are defined. This option requires no specification changes. UAC is defined in TS 22.261 and TS 24.501, and feasibility of the options (e.g. defining new Access Identities or Access Categories) should be consulted with SA1/CT1.

Question 5: Do you agree to include the text proposal on UAC in the TR? Please provide a text proposal if you prefer update to the suggested text.

| Company | Reply to Q5 / Agree TP | Comments / Text proposals |
| --- | --- | --- |
| Qualcomm | Agree |  |
| Intel | Agree |  |
| OPPO | Agree |  |
| vivo | Agree |  |
| Lenovo | Agree |  |
| LGE | Agree |  |
| MediaTek | Agree |  |
| Huawei, HiSilicon | Agree |  |
| Samsung | Agree |  |
| ZTE | Agree | With updating a typo: - Define RedCap specific Access Categories. Access Categories are related to the type of access attempt and is set per access attempt type depending on what triggered the access (set by NAS if NAS triggered, or by RRC if AS triggered). There can only be one Access Category per access attempt. To be able to treat different RedCap access attempt types differently, e.g. apply different barring to two different access types, multiple Access Categories for RedCap could be defined. |
| Apple | Ok with us |  |
| Xiaomi | OK |  |
| Facebook | OK |  |
| Sequans | Agree |  |
| Ericsson | Agree |  |

SUMMARY 15 companies replied and all agree to the text proposal. One typo is suggested to be corrected (thanks!). Update the text referring to UAC in clause 11.2.1 in the TR with following: ”In UAC each access attempt is associated with an Access Category and one or more Access Identities (defined in TS 24.501). The possible solutions for RedCap UAC that have been considered in the study are the following (the options do not need to be mutually exclusive): - Define one or more RedCap specific Access Identities. Access Identities are connected to the UE type and are (currently) used to lift the barring for certain identities, e.g. for special access classes or UEs configured for prioritized services. - Define RedCap specific Access Categories. Access Categories are related to the type of access attempt and is set per access attempt type depending on what triggered the access (set by NAS if NAS triggered, or by RRC if AS triggered). There can only be one Access Category per access attempt. To be able to treat different RedCap access attempt types differently, e.g. apply different barring to different access types, multiple Access Categories for RedCap could be defined. - Use some of the operator defined Access Categories for RedCap. The description of the previous solution applies also to this solution, the difference is that this solution has no specification impact but cannot be used for initial attach to the network since it depends to CN configuration of the UE. - Broadcast a different set of UAC parameters for RedCap UEs. This makes it possible for NW to flexibly and separately provide UAC parameters for RedCap UEs while avoiding impact on UAC configuration of non-RedCap UEs. - Use existing broadcasted UAC parameters for RedCap UEs with no changes, that is, the same UAC parameters apply for all UEs (non-RedCap UEs and RedCap UEs) and no new Access Categories and Access Identities are defined. This option requires no specification changes. UAC is defined in TS 22.261 and TS 24.501, and feasibility of the options (e.g. defining new Access Identities or Access Categories) should be consulted with SA1/CT1.” (15/15)

# Summary

- The following are proposed based on the discussion:

- The legacy UAC principle is assumed for RedCap. The details of how RedCap UEs are using access identity(s) and/or access category(s) are to be discussed during normative phase. (11/15)

Capture following text in 11.2.1 on RRC Connection reject: ”To save radio resources and limit negative impact on legacy network performance it is beneficial to bar or reject UEs as early as possible, preferably without additional signalling. Therefore, cell barring and UAC is beneficial compared to RRC connection rejection. However, if the network is aware of the UEs type during initial access, it is possible for the network to reject RRC connection based on the UE type. There is no additional specification impact in case early indication is specified.” (15/15)

Capture following text in clause 11.2.2 Analysis of coexistence with legacy UEs:
“It is possible that separate RACH configuration is provided for RedCap UEs. In such case, it would be possible to configure different RACH parameters to RedCap and non-RedCap UEs, such as different maximum number for preamble transmission, different back-off timer after an attempt or a different power ramping step for RedCap UEs”.

Update the text referring to UAC in clause 11.2.1 in the TR with following: 

”In UAC each access attempt is associated with an Access Category and one or more Access Identities (defined in TS 24.501). The possible solutions for RedCap UAC that have been considered in the study are the following (the options do not need to be mutually exclusive):

- 	Define one or more RedCap specific Access Identities. Access Identities are 	connected to the UE type and are (currently) used to lift the barring for certain 	identities, e.g. for special access classes or UEs configured for prioritized 	services.

-	Define RedCap specific Access Categories. Access Categories are related to the 	type of access attempt and is set per access attempt type depending on what 	triggered the access (set by NAS if NAS triggered, or by RRC if AS triggered). 	There can only be one Access Category per access attempt. To be able to treat 	different RedCap access attempt types differently, e.g. apply different barring to 	different access types, multiple Access Categories for RedCap could be defined.

-	Use some of the operator defined Access Categories for RedCap. The description 	of the previous solution applies also to this solution, the difference is that this 	solution has no specification impact but cannot be used for initial attach to the 	network since it depends to CN configuration of the UE.

-	Broadcast a different set of UAC parameters for RedCap UEs. This makes it 	possible for NW to flexibly and separately provide UAC parameters for RedCap 	UEs while avoiding impact on UAC configuration of non-RedCap UEs.

-	Use existing broadcasted UAC parameters for RedCap UEs with no changes, that 	is, the same UAC parameters apply for all UEs (non-RedCap UEs and RedCap 	UEs) 	and no new Access Categories and Access Identities are defined. This 	option requires no specification changes.

UAC is defined in TS 22.261 and TS 24.501, and feasibility of the options (e.g. defining new Access Identities or Access Categories) should be consulted with SA1/CT1.” (15/15)

# Delegate contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Rapporteur (Ericsson) | tuomas.tirronen@ericsson.com |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Intel | Yi.guo@intel.com |
| Apple | amsun.palle@apple.com |
| OPPO | Haitao Li (lihaitao@oppo.com) |
| vivo | Chenli (Chenli5g@vivo.com) |
| Lenovo | Jie Shi(shijie4@lenovo.com) |
| LGE | HyunJung Choe(stella.choe@lge.com) |
| MediaTek | Pradeep Jose (amsung[dot]jose[at]mediatek[dot]com) |
| Huawei, HiSilicon | baokun.shan@huawei.com |
| Samsung | Jaehyuk Jang (jack.jang@samsung.com) |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| Facebook | Yee Sin Chan (yeesinchan@fb.com) |
| Sequans | Noam Cayron (noam.cayrons@equans.com) |