3GPP TSG-RAN WG2 Meeting #113-e	R2-2102049

eLocation,  eLocation, 25 Jan – 05 Feb 2021	rev R2-2100229

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | NA | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support Non-Terrestrial Networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-core |  |  |  |  |  | Date: |  |  | 2021-03-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of Non-Terrestrial Networks support |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of specific NTN vocabulary, architecture and protocols aspects |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN is not supported in NR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.x and 16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: Stage 2 runningCR creation based on email discussion in R2-2010781 (RAN2#112-e), RAN2#113-e outcomes (chairman’s notes) and RAN3#111- outcomes (R2-2102057). This stg2 Running CR will completed with the RAN3 stg2 BL CR when completed. For information the current endorsed RAN3 stg2 BL CR can be found in R3-211344 |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< First Changes Begin >>>>>>>>>>>>>>>>>>>>

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCH	Broadcast Channel

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

GEO	Geostationary Earth Orbit

GFBR	Guaranteed Flow Bit Rate

HRNN	Human-Readable Network Name

HAPS	High Altitude Platform Station

IAB	Integrated Access and Backhaul

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MT	Mobile Termination

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

NB-IoT	Narrow Band Internet of Things

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGEO	Non-Geostationary Earth Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NTN	Non-Terrestrial Network

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TPC	Transmit Power Control

UCI	Uplink Control Information

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

BH RLC channel: an RLC channel between two nodes, which is used to transport backhaul packets.

CAG Cell: a PLMN cell broadcasting at least one Closed Access Group identity.

CAG Member Cell: for a UE, a CAG cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a CAG cell that is only available for normal service for CAG UEs.

Cell-Defining SSB: an SSB with an RMSI associated.

Child node: IAB-DU's and IAB-donor-DU's next hop neighbour node; the child node is also an IAB-node.

Conditional Handover (CHO): a handover procedure that is executed only when execution condition(s) are met.

CORESET#0: the control resource set for at least SIB1 scheduling, can be configured either via MIB or via dedicated RRC signalling.

DAPS Handover: a handover procedure that maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB.

Downstream: Direction toward child node or UE in IAB-topology.

Early Data Forwarding: data forwarding that is initiated before the UE executes the handover.

Feeder link: [to be provided by the RAN3 stg2 BL CR].

Geostationary Earth Orbit: a circular geosynchronous orbit 35,786 kilometres above Earth's equator and following the direction of Earth's rotation.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

High Altitude Platform Station: airborne vehicle embarking the NTN payload placed at an altitude between 8 and 50 km.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

IAB-DU: gNB-DU functionality supported by the IAB-node to terminate the NR access interface to UEs and next-hop IAB-nodes, and to terminate the F1 protocol to the gNB-CU functionality, as defined in TS 38.401 [4], on the IAB-donor.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviours specified for UEs unless stated otherwise. IAB-MT function used in 38-series of 3GPP Specifications corresponds to IAB-UE function defined in TS 23.501 [3].

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

Intra-system Handover: Handover that does not involve a CN change (EPC or 5GC).

Inter-system Handover: Handover that involves a CN change (EPC or 5GC).

Late Data Forwarding: data forwarding that is initiated after the source NG-RAN node knows that the UE has successfully accessed a target NG-RAN node.

MSG1: preamble transmission of the random access procedure for 4-step random access (RA) type.

MSG3: first scheduled transmission of the random access procedure.

MSGA: preamble and payload transmissions of the random access procedure for 2-step RA type.

MSGB: response to MSGA in the 2-step random access procedure. MSGB may consist of response(s) for contention resolution, fallback indication(s), and backoff indication.

Multi-hop backhauling: Using a chain of NR backhaul links between an IAB-node and an IAB-donor.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Non-CAG Cell: a PLMN cell which does not broadcast any Closed Access Group identity.

Non-terrestrial networks: [to be provided by the RAN3 stg2 BL CR].

NR backhaul link: NR link used for backhauling between an IAB-node and an IAB-donor, and between IAB-nodes in case of a multi-hop backhauling.

NR sidelink communication: AS functionality enabling at least V2X communication as defined in TS 23.287 [40], between two or more nearby UEs, using NR technology but not traversing any network node.

NTN-gateway: [to be provided by the RAN3 stg2 BL CR].

NTN payload: [to be provided by the RAN3 stg2 BL CR].

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PLMN Cell: a cell of the PLMN.

Satellite: a space-borne vehicle embarking the NTN payload, placed into Non-Geostationary Earth Orbit (NGEO) or GEO.

Service link: Wireless link between the NTN payload and UE.

SNPN Access Mode: mode of operation whereby a UE only accesses SNPNs.

SNPN-only cell: a cell that is only available for normal service for SNPN subscribers.

SNPN Identity: the identity of Stand-alone NPN defined by the pair (PLMN ID, NID).

Upstream: Direction toward parent node in IAB-topology.

V2X sidelink communication: AS functionality enabling V2X communication as defined in TS 23.285 [41], between nearby UEs, using E-UTRA technology but not traversing any network node.

Xn: network interface between NG-RAN nodes.

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

## 4.x	Non-Terrestrial Networks overview

[to be provided by the RAN3 stg2 BL CR]

Three types of service links are supported.

  - Earth-fixed: provision by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GEO satellites and HAPS)

  - Quasi-Earth-fixed: provision by beam(s) covering one geographic area for a finite period and a different geographic area during another period (e.g., the case of NGEO satellites generating steerable beams)

  - Earth-moving: provision by beam(s) which foot print slides over the Earth surface (e.g., the case of NGEO satellites generating fixed or non-steerable beams).

Non-geostationary earth orbit encompass circular orbits of constellation of satellites at altitude greater than or equal to 300 km. This includes Low-Earth Orbit at altitude approximately between 300 km and 1500 km and Medium Earth Orbit at altitude between 1500 km and 35786 km.

With NGEO satellites, the gNB can provide either quasi-Earth-fixed cells or Earth-moving cells, while gNB operating with GEO satellite can provide Earth fixed cells.

In this release, the UE supporting NTN is GNSS-capable.

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

## 16.x	Non-Terrestrial Networks

### 16.x.1 Overview

Support for Non-Terrestrial Networks (NTNs) is facilitated by the mechanisms described in the following clauses.

### 16.x.2 User Plane aspects

It is up to gNB implementation to ensure sufficient time on UE side for the random access Msg3 transmission. The UE shall apply an offset to the start of the response window and to the contention resolution timer. This offset shall take into account the UE-gNB Round Trip Time (RTT):

Editor’s note: FFS how UE obtains the RTT information

For downlink, HARQ feedback can be enabled/disabled, per HARQ process via RRC signalling.

### 16.x.3 System information

Editor’s note: The discussion on whether to introduce a new SIB for NTN has been postponed until more progress is made on the content of NTN specific system information.

The network may broadcast more than one TACs per PLMN in a cell. The UE determines the Tracking Area based on the system information.

Editor’s note: UE determination of the tracking area using other information than the broadcast TAC(s) is not excluded.

### 16.x.4 Mobility and State transition

#### 16.x.4.1 Intra-NTN

##### 16.x.4.1.1 Mobility in RRC_IDLE

The same principles as described in 9.2.1 apply to cell selection and reselection in NTN except for what is described below:

-	An NTN UE may perform cell selection and reselection based on the NTN (satellite/HAPS) ephemeris provisioned.

Editor’s note: FFS on the definition of satellite/HAPS ephemeris. FFS under what circumstance UE will perform cell selection/reselection based on satellite/HAPS ephemeris.

Editor’s note: FFS whether UE location (and/or other information) based cell selection/reselection should be introduced for NTN.

Editor’s note: FFS on any further enhancement on cell reselection priority configuration in NTN.

The information on when a cell is going to stop serving the area and/or the timing information about new upcoming cell is supported at least in quasi-Earth-fixed NTN scenario.

Editor’s note: FFS in which form and how this is exactly implemented in the cell reselection procedure. FFS if both types of information are needed. FFS if this is known from system information and/or the ephemeris. FFS on the utilization of such information.

The UE is made aware that a cell is provided by an NTN or a TN network.

Editor’s note: FFS on whether to make the UE aware of NTN in an implicit or explicit way.

The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris.

Editor’s note: The provision of NTN ephemeris can be provided through pre-configuration in uSIM, NAS, SIB and RRC signaling .

##### 16.x.4.1.2 Mobility in RRC_CONNECTED

###### 16.x.4.1.2.1 Overview

In the case of NGEO, service link switch refers to a change of serving satellite.

###### 16.x.4.1.2.2 Handover

The same principle as described in 9.2.3.2 applies to intra-NTN handover except for what is described below:

Editor's note: DAPS handover for NTN is deprioritized in this release.

###### 16.x.4.1.2.3 Conditional Handover

The same principle as described in 9.2.3.4 applies to intra-NTN conditional handover except for what is described below:

A4 event in addition to A3/A5 events is supported for CHO.

Editor’s note: FFS whether other triggers needs to be combined with this.

##### 16.x.4.1.3 Measurements

The same principle as described in 9.2.4 applies to measurements in NTN.

Editor’s note: For NTN, the location-based measurement event, in combination with the existing measurement event in NR, is supported. FFS on how to configure the location based measurement event.

#### 16.x.4.2 NTN/Terrestrial Network mobility

During mobility between NTN and Terrestrial Network, a UE is not required to connect to both NTN and Terrestrial Network at the same time.

Editor’s note: Triggers of Terrestrial Network/NTN mobility will be discussed in RAN2 once the intra-NTN mobility has been sufficiently progressed.

### 16.x.5	Switch over

#### 16.x.5.1 Definitions [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.2 Assumptions [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.3 Operations [FFS] [to be provided by the RAN3 stg2 BL CR]

#### 16.x.5.4 Procedures [FFS] [to be provided by the RAN3 stg2 BL CR]

### 16.x.6	Signalling [to be provided by the RAN3 stg2 BL CR]

### 16.x.7	O&M Requirements [to be provided by the RAN3 stg2 BL CR]

<<<<<<<<<<<<<<<<<<<< Next Changes Begin >>>>>>>>>>>>>>>>>>>>

# Annex B.x	Example implementation of Non-Terrestrial Networks (informative) [to be provided by the RAN3 stg2 BL CR]

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

The following appendices shall be removed from final CR

# A	Appendix: RAN2 agrements for WI NR-NTN-solutions

## A.1 Scope, requirements, scenarios, architecture

RAN2#111-e Agreements

RAN2 to stick to WI scenarios: Any restriction, e.g. on the LEO altitude (if needed) could come from other groups.

From RAN2 perspective, the table 4.2-2 of [TR 38.821] is used as a baseline for the normative work, with the removal of the regenerative payload option

(as the WI is restricted to transparent payload) we assume that the feeder link will use NR (how the satellite is controlled is out of the scope of the WI)

RAN2 confirms the assumptions on the UE ground speed in the handheld and VSAT cases

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Motion on the earth | Up to 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |

In Rel-17, only UEs with GNSS capabilities are supported

Both Earth fixed and earth moving beam scenarios are considered with NGSO constellation.

Both soft and hard feeder link switchover (e.g. for Non GSO) are supported.

Note: This requires satellite to be connected to one NTN GW at a time (hard switch) or at least two NTN GWs simultaneously (soft switch).

RAN2 to start discussing enhancements for soft feeder link switchover and then solutions for hard feeder link switchover.

As part of the NR-NTN WI, the following stepped approach is proposed:

- Step 1: Assessment of the Rel-16 LCS framework/application protocols (3GPP TS 23.273, TS 29.572, TS 38.455, TS 38.305, in particular but not excluding other TS) and its applicability to NTN

- Step 2: Assess whether changes to the existing network-based location methods are needed and define them if needed

The NTN network based positioning of UE should provide an accuracy comparable with the network based UE location accuracy of terrestrial networks.

For TN/NTN mobility, the UE is not required to connect to both TN and NTN at the same time.

RAN2 to discuss about trigger(s) of TN / NTN mobility, once the Intra NTN mobility has sufficiently progressed. Intra NTN mobility refers to idle and connected mode mobility between NTN cells (e.g. intra or inter satellite).

Transparent HAPS is assumed with the IMT BS on the ground and the HAPS is a relay.

The RAN2 work plan described in R2-2007565 should be considered as a basis for work

The work plan should be based on the following prioritization principles:

- 1st priority: user plane, control plane (idle and connected)

- 2nd priority: NTN-TN service continuity, network based UE location

Comeback to the discussion on capability of UE to support TN / NTN mobility in the future

## A.2 User plane – RACH & MAC aspects

RAN2#113-e Agreements

Both Type 1 and Type 2 configured grant are feasible in NTN.

From RAN2’s perspective, no need to modify parameter periodicity of IE ConfiguredGrantConfig to support NTN.

No need to modify maxNrofConfiguredGrantConfig-r16 and maxNrofConfiguredGrantConfigMAC-r16 to support NTN.

UE in NTN can have both 2-step RACH and configured grant configurations at the same time.

For HARQ processes with DL HARQ feedback disabled, drx-HARQ-RTT-TimerDL is not started.

FFS: method(s) to support blind retransmission for HARQ processes with HARQ feedback disabled.

From RAN2 perspective, for HARQ processes where gNB can sends UL grant without waiting for decoding result of previous PUSCH transmission, no new network scheduling restrictions are introduced to schedule subsequent grants (i.e. up to network implementation. (Can come back if we don't find an agreement on p8)

For HARQ processes with DL HARQ feedback enabled, drx-HARQ-RTT-TimerDL length is increased by offset (i.e. existing values within value range increased by offset). RAN2 working assumption: offset is equal to UE-gNB RTT (if RAN1 decides something that requires to change this we can revisit it)

RAN2#112-e Agreements

RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT.

If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated

From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission.

For UE with pre-compensation capability (at least for the HARQ-feedback enabled case. FFS for HARQ-feedback disabled, if supported), drx-HARQ-RTT-TimerDL is offset by UE-specific RTT (UE-gNB delay) in LEO/GEO. FFS if offset is applied to: 1) the start of the timers or 2) the timer value range (i.e. existing values within value range increased by offset)

From RAN2 perspective, for dynamic grant, one possibility for "enabling"/"disabling" HARQ uplink retransmission at UE transmitter is without introducing an additional mechanism (i.e. gNB can send grant with NDI not toggled/toggled without waiting for decoding result of previous PUSCH transmission). FFS on the handling of RTT timers. Other solutions for enabling/disabling HARQ UL reTX are not precluded

If the start of the ra-ResponseWindow and msgB-ResponseWindow is accurately compensated by UE-gNB RTT, ra-ResponseWindow and msgB-ResponseWindow are not extended in LEO/GEO.

At least the following are FFS in Rel-17 NTN:

•	Report UE-calculated TA in e.g. msg3/msg5/msgA

•	Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

•	LCP impact caused by disabling HARQ UL retransmission

RAN2 decision on starting ra-ContentionResolutionTimer, ra-ResponseWindow and msgB-ResponseWindow is postponed until further progress in RAN1 regarding UE pre-compensation method and TA estimation accuracy.

RAN2#111-e Agreements

From RAN2 perspective, an offset is applied to the start of ra-ResponseWindow in NTN for both LEO and GEO scenarios.

An offset to the start of the ra-ContentionResolutionTimer is introduced for both LEO and GEO scenarios.

Modification of drx-LongCycleStartOffset, drx-StartOffset, drx-ShortCycle, drx-ShortCycleTimer, drx-onDurationTimer, drx-SlotOffset and drx-InactivityTimer is not needed in Rel-17 NTN.

From a RAN2 perspective, for DL, HARQ feedback can be enabled/disabled in Rel-17 NTN, but HARQ processes remain configured. The criteria and decision to enable/disable HARQ feedback is under network control and is signalled to the UE via RRC in a semi-static manner. FFS for UL

At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded)

Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

## A.3 User plane – Other aspects (RLC & PDCP)

RAN2#112-e Agreements

RLC t-Reassembly timer needs to be extended in NR-NTN.

There is no need to extend t-PollRetransmit Timer in NR-NTN.

There is no need to extend t-statusProhibit Timer in NR-NTN.

There is no need to extend RLC SN length in NR-NTN

There is no need to extend PDCP SN length in NR-NTN

## A.4 Earth fixed moving beams related issues

RAN2#113-e Agreements

In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups

In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation.

RAN2#112-e Agreements

RAN2 to consider the case where gNB is co-located at the GW with higher priority.

RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Existing cell reselection principles are considered as baseline and that information about when a cell is going to stop serving the area and information about new upcoming cell can be further considered. In which form and how this is exactly implemented in the cell reselection principles is FFS.

## A.5 Control plane – Idle/Inactive mode aspects

RAN2#113-e Agreements

The NTN ephemeris is divided into serving cell’s ephemeris and neighbour’s ephemeris. FFS how would they differ regarding e.g. the required accuracy or signalling impact.

Consider pre-configuration in uSIM, NAS, SIB and RRC signalling for providing the NTN ephemeris. Further discussion depends on the agreed ephemeris contents.

RAN2 thinks that a UE needs to know whether the network is a TN or NTN no later than SIB1 reception

The information on when a cell is going to stop serving the area and/or the timing information (e.g. timer or absolute time) about new upcoming cell is supported at least in Earth-fixed NTN scenario. FFS if both types of information are needed. FFS if this is known from system information and/or the ephemeris.

RAN2#111-e Agreements

Cell selection / reselection in NR is the baseline in NTN idle mode procedure.

Satellite/HAPS ephemeris based cell selection and reselection should be defined for NTN (FFS what the term satellite/HAPS ephemeris actually means). FFS when this ephemeris based cell selection / reselection can be used. FFS whether UE location (and/or other information) based cell selection and reselection should be introduced for NTN

The satellite ephemeris should be provided to UE, at least for Satellite/HAPS ephemeris based cell selection and reselection (FFS what the term satellite/HAPS ephemeris actually means).

The network type (i.e. TN or NTN) should be known to UE. FFS whether to achieve this in an implicit or explicit way.

The existing cell reselection priority configuration can be taken as a baseline in NTN. FFS on any further enhancement.

Postpone the discussion on whether to introduce a new SIB until we have more progress on the content of NTN specific system information.

## A.6 Control plane – Connected mode aspects

RAN2#113-e Agreements

Support A4 event for NTN CHO. FFS whether other triggers needs to be combined with this.

RAN2#112-e AgreementsReconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

NTN specific CHO execution condition can be further discussed.

The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

Legacy SSB periods (as in TN) shall be supported in NTN

Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

DAPS HO for NTN is de-prioritized in this release.

Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

RAN2 understanding that UE shall not be forced to detect the SSB burst outside the corresponding configured SMTC window in NTN, just like the principle in TN.

SMTC and gap configuration in NTN are configured based on the timing of PCell

RAN2 can first identify the scenarios and discuss how serious the impact is before addressing any enhancement for SMTC configuration in NTN.

RAN2 can’t assume that the network will always have UE accurate location info for SMTC window configuration in NTN

UE along with the network in NTN should also have the same understanding of the timing, including the timing for measurement gap, to avoid any un-synchronized scheduling between UE and the network, just like the way we have in TN

## A.7 Control plane – LCS aspects

# B	Appendix: Who to contact about their comments

| Name | Company | Email |  |
| --- | --- | --- | --- |
| Nicolas Chuberre | Thales | Nicolas.chuberre@thalesaleniaspace.com |  |

END