3GPP TSG-RAN WG2 Meeting #113-e	R2-2102121

Electronic, 25nd Jan– 05th Feb, 2021

Agenda Item:	8.11.3.2

Source:	Huawei, HiSilicon

Title:	Text proposal for IDLE and INACTIVE positioning

Document for: Discussion

Discussion

During RAN2#113e, the following agreements have been made on R17 positioning. Agreements:

Proposal 1a: RAN2 confirms on the following recommendation of TSG RAN (17/17)

	Positioning in RRC_INACTIVE

	DL, UL and DL+UL positioning methods

	UE-based and UE-assisted positioning solutions

	Support of UE positioning measurements for UEs in RRC_INACTIVE state

	Options that can be considered include DL-PRS or DL-PRS and SSB

	Support of gNB positioning measurements for UEs in RRC_INACTIVE state

Proposal 1b: RAN2 confirms on the following (17/17)

	Positioning in RRC_IDLE

	It is feasible for a UE to perform DL positioning measurement in RRC_IDLE state

	It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs.

Proposal2: RAN2 recommends the following for normative work for DL positioning

	The report of PRS measurement performed in RRC_IDLE/INACTIVE when the UE is in RRC_INACTIVE is supported (10/12)

	PRS measurement report and/or location estimate are sent from the UE to the gNB in RRC_INACTIVE.  RAN2 generally agree to do this by enhancing small data transmission in RRC_INACTIVE (details of the use of SDT to be studied in the WI phase) (15/16)

Proposal4: For DL positioning in IDLE/INACTIVE, the followings are already supported for the current spec and can be reused:

	Current stage3 spec has already supported assistance data delivery for DL positioning during RRC_CONNECTED and on-demand SI request in RRC_IDLE/ INACITVE for IDLE/INACTIVE positioning. (14/14)

	Current stage3 spec already supports the transfer of RequestLocationInformation in RRC_CONNECTED for PRS measurement in IDLE/INACTIVE. (14/14)

Proposal5: Support RAT-Independent positioning in RRC_IDLE/INACTIVE. FFS the procedures that can be supported. (13/14)

In this TP, we capture the following agreement to the TR 38.857

Text proposal

================================FIRST CHANGE=====================================

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-193237: "new SID on NR Positioning Enhancements".

[3]	3GPP TR 38.855: "Study on NR Positioning (Release 16)".

[4]	R1-2009433	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

[5]	R1-2007665	Evaluation of NR positioning performance	vivo

[6]	R1-2007720	Evaluation of achievable positioning accuracy	BUPT

[7]	R1-2007754	Evaluation of achievable accuracy and latency	ZTE

[8]	R1-2007859	Discussion of evaluation of NR positioning performance	CATT

[9]	R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

[10]	R1-2009390	Update of Evaluation Results for NR Positioning Performance in I-IoT Scenarios	Intel Corporation

[11]	R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

[12]	R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

[13]	R1-2009555	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

[14]	R1-2009502 Discussion on Performance evaluation of Rel-17 positioning	Sony

[15]	R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

[16]	R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

[17]	R1-2009708	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

[18]	R1-2009428	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

[19]	R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CeWiT

[20]	R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

[21]	R1-2008765	Potential positioning enhancements	Ericsson

[22]	R1-2007666	Discussion on potential positioning enhancements	vivo

[23]	R1-2005380	Evaluation of achievable positioning accuracy and latency	vivo

[24]	3GPP TS 22.261	Service requirements for the 5G system; Stage 1 (Release 17)

[25]	RP-202094	Revised SID: Study on NR Positioning Enhancements	CATT, Intel Corporation

[26]	3GPP TS 38.901	Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

[xx]	3GPP TS 24.571	Control plane Location Services (LCS) procedures (Release 16)

======================================NEXT CHANGE===================================

- 10.1	NR positioning for UEs in RRC_INACTIVE state

NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting, which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

The following procedures are recommended for normative work for DL positioning methods in RRC_INACTIVE:

  - Reporting of DL-PRS measurement and/or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE.

    - The reporting of DL-PRS measurement and/or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE is enabled by enhancing small data transmission in RRC_INACTIVE. (Details of the use of SDT to be studied in the WI phase)

NOTE: The following procedures are considered to have already been supported and can be reused for DL positioning in RRC_INACTIVE

    - On-demand SI request in RRC_INACTIVE for assistance data delivery by broadcast in RRC_INACTIVE

    - ProvideAssistanceData in RRC_CONNECTED for DL-PRS configuration used in RRC_INACTIVE downlink positioning

    - RequestLocationInformation can be sent in RRC_CONNECTED for DL-PRS measurement or location estimate performed in RRC_INACTIVE

======================================NEXT CHANGE===================================

- 10.9	DL positioning measurement in RRC_IDLE state

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state.

  - Note: This does not imply that measurements have to be reported in RRC_IDLE state.

The following procedures are considered as feasible for DL positioning methods in RRC_IDLE:

  - Reporting of DL-PRS measurement and/or location estimate performed in RRC_IDLE when the UE is in RRC_INACTIVE/RRC_CONNETED.

NOTE: The following procedures are considered to have already been supported and can be reused for positioning in RRC_IDLE

    - On-demand SI request in RRC_IDLE for assistance data delivery by broadcast in RRC_IDLE

    - ProvideAssistanceData can be sent in RRC_CONNECTED for DL-PRS configuration used in RRC_IDLE downlink positioning

    - RequestLocationInformation can be sent in RRC_CONNECTED for DL-PRS measurement and/or location estimate performed in RRC_IDLE

- ================================NEXT CHANGE===================================

- 10.x Scope of RRC_IDLE/INACTIVE positioning

The following UE positioning procedures are under the scope of RRC_IDLE/RRC_INACTIVE positioning if any of them are performed when the UE is in RRC_IDLE/RRC_INACTIVE.

- NAS-transported positioning signalling

  - LCS messages defined in Clause 4.1.2 for location services in TS 24.571 [xx]

  - LPP signaling for positioning (e.g., Capability transfer, Assistance data transfer, Location information transfer)

- NRPPa

  - E-CID information transfer (UE-associated)

  - Positioning information transfer (UE-associated)

  - Measurement information transfer (non-UE-associated)

- Uu Signaling and procedure

  - RRC signaling for positioning (e.g.,  posSRS configuration)

  - MAC procedure/L1 signaling (e.g., activation/deactivation for semi-persistent/aperiodic posSRS)

  - Transmission of UL-PRS and reception of DL-PRS

  - Reception for assistance information broadcast

================================NEXT CHANGE=======================================

10.y RAT-Independent positioning

RAT-Independent positioning in RRC_IDLE/INACTIVE is recommended for normative work. The exact procedures that can be supported for RAT-Independent positioning in RRC_IDLE/INACTVE can be further studied.

- =======================================END OF CHANGES============================