3GPP TSG-RAN WG2 #113e	R2-2102154

Electronic meeting, Jan 25th – Feb 5th, 2021

Agenda Item:	9.1.2

Source:	Ericsson

Title:	Summary of Email Discussion [AT113-e][304][NBIOT/eMTC R17] 	Neighbour cell measurements before RLF

Document for:	Discussion, Decision

# 1	Introduction

- [AT113-e][304][NBIOT/eMTC R17] Neighbour cell measurements before RLF (Ericsson)

Scope:

Week 1: 1) What to ask in RAN4 LS. 2) Options for how to do measurements and trigger condition.

Week 2: 2) Approved LS 2) TBD online Monday 1 Feb

Intended outcome:

Week 1: Report in R2-2102154

Week 2: Approved LS in R2-2102156

Deadline:

Week 1: Jan 29 1100 UTC

Week 2: Feb 04 1100 UTC

The objective is to formulate an LS to RAN4 with questions that can help RAN4 to define Neighbor cell measurements to be performed in RRC Connected mode. For this purpose, a draft LS has also been provided.

# 2		How to do Measurements

In this section, companies are requested to provide their view on how UE may perform measurements.

Some assumptions have been provided in [R2-2101157] and R2-2100325 regarding the measurements; companies are requested to provide their view.

Question 1:

For NB-IoT, Can RAN2 assume any of the below?

- intra- and inter-frequency neighbor cell requirements defined for RRC_IDLE can be applied in RRC_CONNECTED state.

- The neighbor cell measurements performed by the UE in connected mode are intra-frequency measurements when anchor carrier of the current cell and anchor carrier of the neighbor cell operates on the same carrier frequency.

- The neighbor cell measurements performed by the UE in connected mode are inter-frequency measurements when anchor carrier of the neighbor cell operates on a different carrier frequency, compared to anchor carrier of the current cell.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This will be discussed anyway by RAN4. We should not spend time on this. |
| ZTE | Yes | Firstly, the inter-frequency and intra-frequency measurements for connected mode are totally new for NB-IoT. Secondly, multi-carrier cell (a cell configured with anchor carrier/non-anchor carriers and non-anchor carrier can be used as service carrier) is unique concept in NB-IoT that may have impacts on the assumption of inter-frequency and intra-frequency measurements. All these things may make it difficult to either directly reuse the concept of inter-frequency and intra-frequency measurements for NB-IoT UE in idle or directly reuse the inter-frequency and intra-frequency measurements for legacy LTE in connected mode. Moreover, the items cited in Question 1 are from the concepts defined in TS 36.300. That means RAN2 needs to involve in the defining work of inter-frequency and intra-frequency measurements for connected mode, e.g., to evaluate the metrics that may have impacts on UE behaviors and at least give the preference. RAN4 can mainly focus on the performance requirements. According to [R2-2100324], at least two possible assumptions for inter-frequency and intra-frequency measurements are mentioned. Option1 takes the anchor carrier as a reference (similar as that for NB-IoT idle mode). Option 2 takes the downlink dedicated carrier as a reference. Based on the analysis, proponent company prefers Option1. Even so, company also mention a difference from that in idle mode: such connected mode intra-frequency neighbor cell measurement in Option 1 looks a bit different from the one in general thinking as it still needs UE to retune its receiver between different carriers (in general thinking, intra-frequency neighbor cell measurement usually means UE doesn’t need to retune its receiver, e.g., UE can perform the neighbor cell measurement and receive the signals from the serving carrier simultaneously). Therefore, this may need more evaluation in RAN4. |
| Huawei. HiSilicon | No | At least bullet 1 does not make sense to us. We think bullet 2 and 3 do not represent the discussion/ contribution in RAN2 which were related to the UE being configured with a non anchor carrier. We are fine to let RAN4 decide what is intra- and inter-frequency measurements but we need to inform them of the different scenarios to be considered. |
| Sequans | No | It is better left to RAN4 to decide |
| Lenovo | No | The details on the definition of intra- and inter-frequency neighbor cell measurement and requiremetns is better to be deiceded by RAN4. |
| Spreadtrum | No | We should wait for the evaluation and decision from RAN4. |
| Qualcomm | No | Discussion needs to happen in RAN4 but we think it must be clear that in NB-IoT the impact of neighbour cell measurement on the on-going RRC connection depends on the configured downlink dedicated frequency and the neighbour cell frequency. |
| MediaTek | No | The decision should left to RAN4. |
| Nokia | Yes but | Eventhough we can agree on second and third points in the question it is not complete list. And also RAN4 needs to define complete requirements for each of them. So this assumption may not be needed or useful at RAN2 without RAN4 inputs |
| Thales | No | Need to be left to RAN4. Also we need to be careful if we have to decide or discuss among ourselves to not define such requirements which can deviate from NB-IoT principal. Anyway we also should clearly indicate that any measurements should be done based on current defined gaps and not degrade ongoing connection. |

From RAN2 perspective, whether Intra or Inter-Frequency measurements assumptions (same as Idle mode) cannot be concluded.

In R2-2100324 understanding, for the static or low mobility NB-IoT UE, the channel quality changes slowly, the measurement results can be thought as valid if not a long time has passed. In this way, for NB-IoT, it can be assumed a neighbor cell is known if it has been measured by the UE and not a long time has passed or the channel quality hardly changes.

Question 2:

For NB-IoT, can RAN2 agree whether a neighbor cell can be thought as known if it has been measured by the UE and not a long time has passed, or the channel quality hardly changes.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This will be discussed by RAN4. We should not spend time on this. |
| ZTE | Maybe yes | During the RRC re-establishement procedure, the searching time of the target cell can be considered as zero only when the target cell is known. In order to shorten the searching time, the purpose of introducing the measurement in connected mode is to determine a known cell as early as possible. Therefore, the measurement in connected mode should at least satisfy the requirements for determining a known cell. That’s why we need to discuss the concept of known cell for NB-IoT connected mode. RAN4 should finally decide the requirements for determining a known cell (how long or how much times to measure, quality criteria etc.). But before that, RAN2 can give some kind of high level thoughts on what’s the known cell (with consideration on UE mobility, power saving and so on). In [R2-2100324], company gives an example, e.g., a cell which has been measured and not a long time has passed or the channel quality seldom changes. Such example is rough and simple. RAN2 may or may not want to add more details, based on the discussion. |
| Huawei, HiSilicon. | No | This is RAN4 discussion. However, we think that the definition of a “known” cell is the only requirement we need from RAN4 if we don’t support early RLF (still to be decided) thus we need to ask RAN4 about this. |
| Sequans | No | It is better left to RAN4 to decide |
| Lenovo | No | This is RAN4 discussion. |
| Spreadtrum | No | It should be evaluated by RAN4. |
| Qualcomm | No | RAN4 needs to specify what is the period for which a neighbour cell can be considered is known after detection or re-confirmation. |
| MediaTek | No | Whether a neighbor cell can be considered as known and how long it can keep as known are out of RAN2’s scope, hence they should be defined by RAN4. |
| Nokia | No. But | The time between last measurement and the decision making point to consider the cell as known or unknown needs to be defined at RAN4. But RAN2 can assume that there is time requirement between measurement completion and re-establishment impact the overall re-establishment time. |
| Thales | No | Need to be decided by RAN4. |

From RAN2 perspective, whether a neighbor cell can be assumed as known cannot be concluded.

Question 3:

R2-2100324 provides analysis that in order to try to reduce the impacts of measurement on data transmission in connected mode, the interval between the available measurement occasions may also need to be considered and the value may be at least 20ms.  Does RAN2 agree?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This will be discussed by RAN4. We should not spend time on this. |
| ZTE | Maybe yes | The valid measurement results are thought to be obtained only if the measurement satisfies the requirements. RAN2 can give some general thoughts about that, e.g.,: - For neighbor cell measurement, at least the synchronization signals and reference signals need to be measured. - As it may be possible that the network can know the exact timing of the neighbor cells, the measurement length for measuring these signals may be further reduced. |
| Huawei, HiSilicon | No | This is RAN4 discussion. |
| Sequans | No | It is better left to RAN4 to decide |
| Lenovo | No | This is RAN4 discussion. |
| Spreadtrum | No | It should be evaluated and decided by RAN4. |
| Qualcomm | No | Not all measurements will take 20ms; some measurements may take less time while others may take longer. Coverage level will also affect how long it takes to do the measurement This will depend on what RAN4 decides. |
| MediaTek | No | The requirement of interval between the available measurement occasions needs to be defined by RAN4. |
| Nokia | No | RAN4 needs to define this requirement |
| Thales | No | Ran4 Discussion |

From RAN2 perspective, measurement occasions cannot be concluded.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This will be discussed by RAN4. We should not spend time on this. |

[R2-2101157] provide details that NB-IoT UE may require gaps in order to perform the measurement.

Question 5:

Does RAN2 agree with the observation that

UE may need to retune its receiver whenever it performs measurements on a neighbour cell with different centre frequency than the centre frequency of the downlink dedicated channels hence cause interruptions to data transfer.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This will be discussed by RAN4. We should not spend time on this. |
| ZTE | Yes | This is possible for connected mode inter-frequency measurement or even intra-frequency measurement in NB-IoT. In order to avoid this, RAN2 at least needs to discuss the following issues: - An explicit start and/or stop indications to network to make a continuous time period for measurement vs to utilize the existing transmission gaps to perform the neighbor cell measurement. - One-shot neighbor cell measurement vs Periodical neighbor cell measurement |
| Huawei. HiSilicon | No | We agree that the UE may need to retune but we have not agreed to interrupt data transfer. When the measurements are actually performed should be decided in RAN4. We could provide some hints where it should be feasible from RAN2 point of view e.g. - during DRX cycle outside active time - between two PDCCH occasions if the UE is not scheduled |
| Sequans | No | It is better left to RAN4 to decide. Since new gaps are outside the scope of the WI, this would rule additional interruption to data transfer. |
| Lenovo | No | The detail is better to be discussed in RAN4. |
| Spreadtrum | No | We have the simialr comments as Huawei. |
| Qualcomm | Yes | If the UE needs to re-tune it’s receiver then the opportunities for measurements can be fewer compared when a UE does not need to perform such re-tuning. Therefore, it can affect design even for ‘intra-frequency’ measurements. |
| MediaTek | No | UE need to retune it’s receiver to another frequency, only if the gap is foreseen and the length of gap is no too long to invoke a data loss. Interruption to data transfer needs to be avoid by all means. |
| Nokia | Yes | There will be scenario where UE may need to retune to different frequency for measurements and this will impact the data transfer. Impact of retuning on data interruption for single RX receiver is known and RAN2 can agree on this observation. The level of interruption depends on the actual gap which requires RAN4 discussion. For RAN2 to continue its discussion on minimizing the impact to ongoing data transfer due to measurements, this assumption is needed. |
| Thales | No | New periods or gaps should not be defined for NB-IoT RRC connected mode measurements, there are outside of the scope of the current WI especially keeping in mind the overhead and impact on NB-IoT devices. If certain measurements are not feasible without aforementioned changes or additional gaps, they are not supposed to be done. |

From RAN2 perspective, the receiver retuning and gap analysis cannot be concluded.

## 3	Trigger Condition

R2-2101113, R2-2100670 discusses the need of trigger condition.

Considering, the neighbor cell measurement to take place before RLF is beneficial to reduce the time taken to RRC reestablishment to another cell as in WID, the following options could be further discussed to trigger the neighbour cell measurement before the RLF procedure to assist the cell selection procedure to find suitable frequency or cell.

- Option1, the neighbour cell measurement could be trigger when the serving cell channel quality is lower than a threshold.

- Option2, the neighbour cell measurement could be trigger based on the RLM procedure. For example, after n number of consecutive "out-of-sync" indications for PCell is detected.

- Option 3: combination of option1 and option2; multiple triggers (e.g., a configured threshold of RSRP/RSRQ, T310) are applied, the neighbour cell measurement would be triggered whichever the configured threshold of RSRP/RSRQ is met or T310 starts.

Companies are invited to provide their view on which trigger condition is preferred.

| Company | Option | Comments |
| --- | --- | --- |
| Ericsson | 2 | We think Option 2 is optimum so that UE starts the neighbor cell measurement when it is really needed to be done. As such T310 is configured long in deployments (8000ms) and hence T310 should be adequate to perform the measurement. This will be a simpler trigger for NB-IoT device otherwise the UE may have to keep on comparing with the threshold which may consume additional power. |
| ZTE | 1 | The measurement within the time period of T310 would not be feasible in more cases, especially for inter-frequency measurement or measurement in enhanced coverage. Therefore, we think earlier measurement may be needed, e.g., according to kind of deterioration on serving cell quality. |
| Huawei, Hisilicon | 3? | Starting the measurements only when T310 is started may be too late. we think a combination of criteria is needed, as in option 3. We think additional conditions could be setup e.g. on UE mobility. e.g. criterion 1) RSRP < threshold (option 1) + (RSRP variation > threshold) before T310. criterion 2) RSRP < threshold (option 1) + T310 running |
| Sequans | 2 | Frequent consecutive out-of-sync indications are quick and simple indication that RLF may be close. Looking at it a different way – RLF will not happen if there are not enough out-of-sync indications, no matter the RSRP threshold. We think T310 time should be enough, but should consult RAN4 as well, |
| Lenovo | 2 | Prefer option.2, it is simple and efficient, and less power consumption comapred with the option.1 by comparision of measurement results. |
| Spreadtrum | 3 | We think option3 is better to avoid triggering the neighbor cell measurement after T310 starts and ensure that the related measurement can be completed as early as possible, which is beneficial to the reduction of time taken in RRC reestablishment. |
| Qualcomm | None | It’s too early to decide on triggers for neighbour cell measurement until there is some understanding of how much how the measurements itself can be done, especially where the receiver needs to be re-tuned. |
| MediaTek | 3 | The inter-frequency measurement needs a gap to perform, the chance of available gap may not be numerous. It’s better to start measurement early than out-of-sync indications occurs. |
| Nokia | 3 | Option should be primary condition for starting connected mode measurements. In case if T310 starts earlier, it can start early measurement prior to the condition. Having retuning for measurements along with serving cell RLM may also impact the RLM performance. Preferred to have measurements started earlier than T310 start. |
| Thales |  | Option 1 and a second criteria is fulfilled, i.e. non-delay tolerant traffic or short session i.e. session may end before RLF occurs. So question is not only whether functionality is (Optional) supported but also whether the device has evaluated to make use of it for this communication session. If session is about to end as buffer is empty then no need to start measurements. If traffic is delay tolerant or if static then same. Level may be below the threshold but may not drop further so RLF won’t occur in that scenario. A device cannot be set fixed to measurements supported or not supported, as depending on use case scenario (a device can have different use cases) as mentioned above it may make sense to perform these measurements or not. |

Measurement Trigger is based upon either "out-of-sync" indications or combination of "out-of-sync" indications and configured threshold of RSRP/RSRQ. Final decision is FFS.

## 4	Questions for RAN4

## 4.1 Inter-Frequency and Intra-Frequency Measurements

Inter-Frequency Measurements

One of the proposal that was discussed online was:

Inter-frequency measurement is supported for NB-IOT UEs in Connected mode for the purpose of reducing the reestablishment duration.

As part of online discussion, the agreement made was:

Formulate a question to RAN4 regarding the support of inter-frequency measurements.

We request companies to provide their input on the formulation of question to RAN4.

Input 1:	Please Provide your Input on whether the LS should have question on Inter-Frequency 			measurements and if yes please provide the input that can be put forward for RAN4

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | As such measurements are defined by RAN4 and hence RAN2 should leave the exact work for RAN4 to do and thus RAN2 should only inform RAN4 about the background that the measurements are intended so UE should perform Intra or inter-frequency measurements before RLF in order to shorten the duration from below point C (declaring RLF) to D (start of re-establishment procedure). And mentionign that WID prohibits definign any new measurement gap. Hence we intend to provide above figure and mention the WID and with below formulation Further, if any questions are really needed then we would like to have geneirc questions; i.e also include question for intra-frequecny measurements as below Figure 1: Illustrations of the reference points until RLF is declared - Is it possible for the UE perform the neighbor cell intra and/or inter frequency measurement during T310? - Is it possible for the UE to perform the neighbor cell intra-frequency measurements for cell search during ON duration time (when the UE is in active time) while data transmission and out-of-sync related measurements in the serving cell are ongoing? - Is it possible for the UE to perform the neighbor cell intra and/or inter-frequency measurements for cell search during DRX sleep cycles? Is this feasible even for the shortest possible DRX sleep cycle, i.e., 256ms – ON duration. - If DRX is not configured, can one assume that measurements can be performed during invalid subframes, subframes between search spaces, etc. would this duration be enough to do intra and/or inter-frequency measurements? |
| ZTE | Yes | For NB-IoT measurement in connected mode, RAN2 can have the following assumption on definitions for connected mode measurements: - The neighbor cell measurements performed by the UE in connected mode are intra-frequency measurements when anchor carrier of the current cell and anchor carrier of the neighbor cell operates on the same carrier frequency. - The neighbor cell measurements performed by the UE in connected mode are inter-frequency measurements when anchor carrier of the neighbor cell operates on a different carrier frequency, compared to anchor carrier of the current cell. RAN4 can check the feasibility of above definitions and define the necessary performance requirements. |
| Huawei. Hisilicon | yes | We think input 1 and input 2 (next question) can be merged. We should indicate to RAN4 the scenarios where the UE needs to perfom neigbour cell measurements from RAN2 point of view: 1 Intra –frequency neigbhour, UE configured with anchor carrier 2 intra-frequency neigbhour, UE configured with non-anchor carrier 3 inter-frequency neigbhour, UE configured with anchor carrier 4 inter-frequency neigbhour, UE configured with non anchor carrier. We should ask RAN4 the feasibility (and the required conditions, e.g. C-DRX cycle, SS configuration ...) for the UE to perfom the measurements in the different scenarios in absence of ’configured measurements gaps’ as indicated in the WID |
| Sequans | Yes | Agree with Ericsson. It would be good to add the relevant scenarios as suggested by HW. |
| Lenovo | Yes | Agree with above comment. |
| Spreadtrum | Yes | Agree with Ericsson and have no need for addtional supplement. |
| Qualcomm |  | In the LS identify the differnet scenerios that needs to be considered both for intra-frequency (i.e. A &B in the below table) and inter-frequency (i.e. C, D and E in the below table). Table: NB-IoT, scenarios require gap assistance Scenario; Serving & Neighbour cell anchor carrier frequency; UE connected mode operating Frequency. A; Intra-frequency; Anchor. B; Intra-frequency; Non-Anchor. C; Inter-frequency; Non-Anchor but same as Neighbour cell Anchor. D; Inter-frequency; Anchor. E; Inter-frequency; Non-Anchor but different from Neighbour cell Anchor. We don’t think it makes sense to ask RAN4 whether neighbour cell measurements can be done while T310 is runing. From physical layer/RF point of view there is no difference between the period Ref Point B to C and before Ref point B. That is, RRC configuration is the same upto Ref point C. |
| MediaTek | Yes | Agree with Ericsson’s comment. Additionally, we would like ask RAN4 to clarify that is there any difference on performing intra-frequency measurement and inter-frequency measurement in or out of T310 duration. Because during our precedent discussion, some assumptions are based on that UE cannot receive DL data during the T310. It may result in a conclusion that UE can fully utilize the duration to perform measurement with possible less side effect. We want this to be clarified. |
| Nokia | Yes | - We can indicate RAN4 to define intra and inter frequency neighbor cell measurements in RRC-CONNECTED mode for NB-IoT. - For these requirements the required gap for the connected mode operation for each type also needs to be investigated. - Also feasibility of using existing gaps of connected mode such as PDCCH configuration gap, C-DRX gaps for doing such measurements. |
| Thales | Yes | We should ask for which scenarios measurements could be provided without defining new gap requirements or especially impacting ongoing connection. I.e. ongoing connection should in general be configured efficient for transfer by the eNodeB and not with certain gaps created extra-long C-DRX cycles for potential need of mobility measurements. In many scenarios such configurations could be the root cause for RLF which may not have occurred otherwise. So again we see final trigger decision whether to do measurements should be left to UE. |

Intra-Frequency Measurements

Input 2:	Please Provide your Input on whether the LS should have question on Intra-Frequency 			measurements and if yes please provide the input that can be put forward for RAN4

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | - Is it possible for the UE perform the neighbor cell intra and/or inter frequency measurement during T310? - Is it possible for the UE to perform the neighbor cell intra-frequency measurements for cell search during ON duration time (when the UE is in active time) while data transmission and out-of-sync related measurements in the serving cell are ongoing? - Is it possible for the UE to perform the neighbor cell intra and/or inter-frequency measurements for cell search during DRX sleep cycles? Is this feasible even for the shortest possible DRX sleep cycle, i.e., 256ms – ON duration. - If DRX is not configured, can one assume that measurements can be performed during invalid subframes, subframes between search spaces, etc. would this duration be enough to do intra and/or inter-frequency measurements? |
| ZTE | Yes | For NB-IoT measurement in connected mode, RAN2 can have the following assumption on definitions for connected mode measurements: - The neighbor cell measurements performed by the UE in connected mode are intra-frequency measurements when anchor carrier of the current cell and anchor carrier of the neighbor cell operates on the same carrier frequency. - The neighbor cell measurements performed by the UE in connected mode are inter-frequency measurements when anchor carrier of the neighbor cell operates on a different carrier frequency, compared to anchor carrier of the current cell. RAN4 can check the feasibility of above definitions and define the necessary performance requirements. |
| Huawei, HiSilicon |  | see inputs 1 |
| Sequans | Yes | Similar to 1 |
| Lenovo | Yes | Same view to 1. |
| Spreadtrum | Yes | It has no essential difference between intra-frequency and inter-frequency measurements on the question to ask RAN4. The inputs 1 is enough for both intra-frequency and inter-frequency measurements. |
| Qualcomm | Yes | See reply to previous question. |
| MediaTek | Yes | Same like the last question. |
| Nokia | Yes | As indicated in Input 1 |
| Thales | Yes | See indicated answers above. |

## 4.2	Gap Analysis

[R2-2101157] provide details that NB-IoT UE may require gaps in order to perform the measurement. Companies are requested to review the paper and provide input as what can be asked to RAN4 on gap requirement from UE.

Input 3	Please Provide your Input on whether the LS should have question on gap analysis and 			if yes please provide the input that can be put forward for RAN4

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | We just need to mention that no new gaps can be defined for this purpose and we expect RAN4 to know the gap analysis |
| ZTE | Yes | RAN2 can firstly discuss which one is better, an explicit start and/or stop indications to network to make a continuous time period for measurement (option 1) or to utilize the existing transmission gaps to perform the neighbor cell measurement (option 2). From company’s view, we think it’s obvious option 1 involves more signaling overhead and also UE power consumption. So option 1 with less preference. It can be discussed only when other option is infeasible. For option 2, RAN2 can further discuss whether neighbor cell measurements can be performed during connected mode DRX, and/or PDCCH monitoring GAP. |
| Huawei, HiSIlicon | No | RAN4 to define. |
| Sequans | No | Let RAN4 define; just mention no new gaps can be defined. |
| Lenovo | No | It is in RAN4 discussion. |
| Spreadtrum | No | It should be left to RAN4 to define. |
| Qualcomm | No | Gap in legacy sense of the word are not part of this WID but it is important to inform RAN4 the different scenarios hence the modified table in out response to Input 1 is useful to share with RAN4. |
| MediaTek | No | RAN4 can define |
| Nokia | Yes | The question should include feasibility of use of existing gap for the required measurements |
| Thales | No | No, RAN4 to discuss. If measurements not feasible with existing gaps, they are out of scope of this WI. Since UMTS times Ran4 already handled such issues. |

## 4.3	Time Line

R2-2100325 provides some questions to check with RAN4

- the time duration in which the measurement results can be thought as valid and/or the acceptable channel quality variation range.

- One Shot measurement and associated measurement occasion

R2-2100324 provides analysis that in order to try to reduce the impacts of measurement on data transmission in connected mode, the interval between the available measurement occasions may also need to be considered and the value may be at least 20ms.  Does RAN2 agree? Should this be checked by RAN4.

Input 4:	Please Provide your Input on whether the LS should have question on time duration for one shot 		measurement and measurement occasion details; if Yes please provide the input that can be put 		forward for RAN4

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | The one-shot measurement appears to be detailed and this is something RAN4 should discuss. |
| ZTE | Yes | - As menioned in the answer to Question 2, from company’s view, we think RAN2 can assume a neighbour cell is known if it has been measured by the UE and not a long time has passed or the channel quality hardly changes - RAN2 also need to discuss which is better, one-shot neighbor cell measurement or periodical neighbor cell measurement. For issue 1, RAN2 can ask RAN4 to further determine the time duration in which the measurement results can be thought as valid and/or the acceptable channel quality variation range. For issue 2, we think RAN2 can further assume the following: - UE can perform one shot measurement, e.g., once the measurement is triggered, the measurement is performed until the measurement result is got. Unless the measurement results turn to invalid, the measurement wouldn’t be performed again. - For one shot measurement in connected mode, RAN2 assume continuous measurement in a one-shot measurement may be infeasible in some cases. - For one shot measurement in connected mode, RAN2 assume UE can perform measurement occasionally. Based on the above assumptions, RAN2 can ask RAN4: - RAN4 can check whether one-shot neighbour cell measurement is feasible/better for NB-IoT UE in connected mode. - RAN4 can check whether UE can perform measurement occasionally, especially for the neighbour cell that operates on a different carrier frequency compared with the downlink dedicated carrier. - RAN4 can further decide the minimum length of a measurement occasion. Moreover, whether interval between the available measurement occasions is needed. |
| Huawei, HiSilicon | No | Up to RAN4 to define. But we need to indicate to RAN4 the purpose of the measurements, i.e. target cell identification (rather that e.g. ranking/ comparison) as the requirements may be less stringent. we could refer to the interruption time definition below and indicate that the goal is to have Tsearch_NB1-NC = 0 TUE-re-establish_delay_NB-IoT = 100 ms + NNB-Iot-freq*Tsearch_NB1-NC + TSI_NB1-NC + TPRACH_NB-IoT |
| Sequans | No | Agree with HW, though would prefer to mention minimizing Tsearch_NB1-NC (i.e. not necessarily = 0) |
| Lenovo | No | It is interesting to discuss the one-shot measurment, but this needs to be discussed in RAN4. |
| Spreadtrum | No | Agree with Huawei. |
| Qualcomm | No | We think for RRC_CONNECTED state RAN4 need to provide information for at least the following two: - For how long the neighbour cell can be considered as known after it has been detected/re-confirmed. - How long does it take to perform cell detection both in normal and in extended coverage. - For how long the NRSRP/NRSRQ measurements can be consider as valid. - How long does it take to perform NRSRP/NRSRQ measurements. The above information is needed for both normal and extended coverage. |
| MediaTek | No | It is better to left to RAN4. RAN2 may agree on the preference of one shot measurement or periodical measurement before suggest to RAN4. It depends on how and when to use the measurement result. |
| Nokia | No | The type of measurements such as one shot or multiple measurements needs to be decided within RAN2. It is not concluded within RAN2. Hence this question is not needed now. |

## 4.4 Measurement Assumptions

R2-2100325 suggests checking feasibility of the following measurement definitions for NB-IoT to RAN4.

- The neighbor cell measurements performed by the UE in connected mode are intra-frequency measurements when anchor carrier of the current cell and anchor carrier of the neighbor cell operates on the same carrier frequency.

- The neighbor cell measurements performed by the UE in connected mode are inter-frequency measurements when anchor carrier of the neighbor cell operates on a different carrier frequency, compared to anchor carrier of the current cell.

Further R2-2101157 suggest that RAN2 assume intra- and inter-frequency neighbor cell requirements defined for RRC_IDLE can be applied in RRC_CONNECTED state

Input 5:	Please Provide your Input on whether the LS should have question on the measurement 			assumption; if Yes please 	provide the input that can be put forward for RAN4

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No |  |
| ZTE | Yes | See our answers in section 4.1, 4.2 and 4.3. |
| Huawei. HiSilicon | No | The requirement for Idle mode are based on UE eDRX cycle and UE DRX cycle, none of the two applies in connected mode. |
| Sequans |  | Defining the scenarios as HW suggests in 4.1 can be enough, but we would be OK with explicitely stating a quesion about what would count is inter/intra measurements |
| Spreadtrum | No | It is not necessary to provide such detailed explanation, which is easy to understand by RAN4. |
| Qualcomm | No | See response to Input 1. |
| MediaTek | No |  |
| Nokia | NO |  |
| Thales | No |  |

## 5 	draft LS

A draft LS is provided; companies are invited to provide their comments on the draft LS

”RAN2 would like to inform RAN4 that RAN2 is currently working on below objective where RAN4 is also marked as one of the responsible group.

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

The WI objective is to specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF. Based on the below Figure provided; the intention of this Rel-17 objective can be expressed as to reduce the time between reference points C and D through specifying signalling for neighbor cell measurements and corresponding measurement triggering between reference points A and C, i.e., until RLF is declared.

Figure 1: Illustrations of the reference points until RLF is declared

RAN2 would like to request RAN4 to define the NB-IoT measurements needed in RRC Connected mode for the purpose of reducing the time duration between point C and point D; i.e between RLF declaration to start of re-establishment procedure.

One of the requirements as specified in the WID is that no new gaps need to be defined.

Below are the agreements made in RAN2 so far.

- Enhancements to the random access procedure are not considered

- The solution includes reduction of the time between declaration of RLF and the start of the random access procedure (points C and D)

- Neighbour cells measurement (detection and measurements) are performed only on the anchor carrier

From RAN2 perspective both Intra-Frequency and Inter-Frequency measurements are desired.

2. Actions:

ACTION to RAN4:

RAN2 respectfully asks RAN4 to define the needed measurements for NB-IoT devices in RRC Connected mode for the purpose of reducing the time taken between RLF declaration to start of re-establishment procedure.”

| Company | Comments/ Questions for RAN4 |
| --- | --- |
| Ericsson | The above draft is adequate. RAN4 is marked responsible group for this WID and thus we should expect them to do this. |
| ZTE | We think it’s not enough. RAN2 needs to give more general/concept thoughts about connected mode measurement for NB-IoT and then ask RAN4 to check feasibility and decide performance requirements. Please see our answers in section 4.1, 4.2 and 4.3. |
| Huawei, HiSilicon | We think additional information are required as discussed in the previous questions: 1. Scenarios 2. Purpose (target cell known) We need to ask RAN4 about the feasibility for the different scenarios and under which conditions |
| Sequans | Agree with HW |
| Spreadtrum | The above draft is enough. |
| Qualcomm | RAN2 does not needed to instruct RAN4 to define measurements for NB-IoT RRC_CONNECTED state, the WID already does this. RAN2 needs some information from RAN4 in order to progress the design in RAN2 hence the LS needs to be specific to this information. Therefore, the above content is quite irrelevant. Based on our above responses, a draft LS is uploaded to the drafts folder. |

RAN2 to review and discuss the draft LS in R2-2102156.

# Participant Companies Name

| Company | Email Address |
| --- | --- |
| Ericsson | ritesh.shreevastav@ericsson.com |
| ZTE | lu.ting@zte.com.cn |
| Huawei, HiSilicon | odile.rollinger@huawei.com |
| Sequans | noam.cayron@sequans.com |
| Qualcomm | mdhanda@qti.qualcomm.com |
| MediaTek | Aaron.cai@mediatek.com |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	From RAN2 perspective, whether Intra or Inter-Frequency measurements assumptions (same as Idle mode) cannot be concluded.

Proposal 2	From RAN2 perspective, whether a neighbor cell can be assumed as known cannot be concluded.

Proposal 3	From RAN2 perspective, measurement occasions cannot be concluded.

Proposal 4	From RAN2 perspective, the receiver retuning and gap analysis cannot be concluded.

Proposal 5	Measurement Trigger is based upon either "out-of-sync" indications or combination of "out-of-sync" indications and configured threshold of RSRP/RSRQ. Final decision is FFS.

Proposal 6	RAN2 to review and discuss the draft LS in R2-2102156.