3GPP TSG-RAN WG2 #113-e	R2-2102194

# Online, Jan 25th - Feb 5th, 2021

Agenda item:	6.4.3

Source: 	LG Electronics Inc. (Rapporteur)

Title: 	[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)

Document for:	Discussion and decision

Introduction

This document is to trigger the following email discussion:

[AT113-e][710][V2X/SL] Miscellaneous MAC corrections (LG)

Scope: discuss rapporteur’s suggestions in Table 1 for R2-2100212, R2-2100213, R2-2101741, and R2-2100503, and in Table 3 for R2-2100504, R2-2101068, R2-2101149, R2-2100323, R2-2101742, R2-2100861, R2-2100119, and R2-2100211. If changes are agreeable, merge them into a rapporteur’s miscellaneous corrections CR. Detailed wording can be further discussed.

Intended outcome: agreeable 38.321 CR in R2-2102189, discussion summary in R2-2102194 (if needed). CR will be approved by email.

Deadline: Feb 04 0430 (UTC)

In this document, Rapporteur propose to discuss the CRs listed above.

Note that the some changes do not require inputs from companies in this document e.g. if the changes are obvious e.g. reference/editorial change or the changes are obviously not needed. For some of those changes, the rapporteur’s proposals were suggested.

Discussion

- R2-2100212 (CATT) and R2-2101741 (ASUSTek, First change)

3> randomly select, with equal probability, an integer value in the interval [5, 15] for the resource reservation interval higher than or equal to 100ms or in the interval 5×100max20, Prsvp_TX,15×100max20, Prsvp_TX for the resource reservation interval lower than 100ms and set SL_RESOURCE_RESELECTION_COUNTER to the selected value;

Question 1A:	Do you agree to reflect the above change in 38.321?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes |  |
| CATT | Yes |  |
| ASUSTeK | Yes |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| HW | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Apple | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Lenovo | Yes |  |

Summary 1A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 13 |
| No | 0 |

Recommendation 1A: the change in R2-2100212 and R2-2101741 is reflected on 38.321.

- R2-2100213 (CATT)

For Regular and Periodic SL-BSR, the MAC entity shall: 1> if sl-PrioritizationThres is configured and the value of the highest priority of the logical channels that belong to any LCG and contain SL data for any Destination is lower than sl-PrioritizationThres; and 1> if ul-PrioritizationThres is configured and the value of the highest priority of the logical channels that belong to any LCG and contain UL data is equal to or higher than ul-PrioritizationThres according to clause 5.4.5: 2> prioritize the LCG(s) for the Destination(s).

Note that first change is included in Q1A.

Question 2A:	Do you agree to reflect the above change in 38.321?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes |  |
| CATT | Yes |  |
| ASUSTeK | Yes |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| HW | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| ZTE | Yes |  |
| Apple | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Lenovo | Yes |  |

Summary 2A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 13 |
| No | 0 |

Recommendation 2A: the change in R2-2100213 is reflected on 38.321

- R2-2101741 (ASUSTek)

randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] ensuring the selected resources and all selected transmission opportunities occur in different slots, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier.

Question 3A:	Do you agree to reflect the second change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | We think that the second change is not needed. Regarding this slot selecting, RAN1 already had discussion and made following conclusion and agreements Conclusion RAN1 understands that the agreement in RAN1#100-bis-e / 101-e cited below already prevents selection of the resources in the same slot and no further clarification is necessary Agreements: - In Step 2, a UE shall select resources so that HARQ retransmission resources can be reserved by a prior SCI, except that - In case no resource can be found for reservation (e.g., based on the identified candidate set after Step 1) for a retransmission of a TB, the re-transmission can be transmitted on a resource that is not reserved After the resource selection is performed, HARQ retransmission on a resource not reserved by a prior SCI is allowed due to transmission dropping caused by prioritization, pre-emption and congestion control |
| CATT | No | We think for the first selection procedure, there is no need for the added description. |
| ASUSTeK | Yes | We thank rapporteur for providing the RAN1 agreement, but RAN1 only prevents selection of the same resource for a same TB (the re-transmission can be transmitted on a resource that is not reserved) since SCI cannot indicate retransmission on the same slot. That is, RAN1 does not prevent selection of the same resource for different TBs. The restriction should be indicated in RAN2 spec. |
| Qualcomm | No | We do not see a need for this additional text. |
| OPPO | No |  |
| HW | No |  |
| vivo | No |  |
| Samsung | No | No need of this change |
| ZTE | No |  |
| Apple | NO |  |
| Ericsson | No |  |
| Intel | No |  |
| Lenovo | No |  |

Summary 3A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 12 |

Recommendation 3A: the change in R2-2101741 is not pursued.

- R2-2100503 (ZTE, Sanechips)

Question 4A:	Do you agree to reflect the first change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | Suggest to change for brevity as below “for uplink ,downlink, and sidelink” |
| CATT | Yes |  |
| ASUSTeK | Yes | We are also ok with LG’s wording suggestion. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| HW | Yes | Same view as the rapporteur. |
| vivo | Yes | Agree with rapporteur. |
| Samsung | Yes |  |
| ZTE | Yes | We are ok with rapporteur’s rewording. |
| Apple | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Lenovo | Yes |  |

Summary 4A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 13 |
| No | 0 |

Recommendation 4A: the first change in R2-2100503 is reflected on 38.321

Question 4B:	Do you agree to reflect the second change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | The second change is not needed. According to MAC specification, the description of the SCI would not be distinguished between 1st stage SCI and 2nd stage SCI. |
| CATT | Yes | We agree with the intension, but the wording needs to be updated.=>1st 2nd |
| ASUSTeK | No strong view |  |
| Qualcomm | No | Agree with rapporteur |
| OPPO | Yes | but suggest some rewording like |
| HW | No | Should be reworded like in which transmission of SL-SCH and 2nd-stage SCI associated with the 1st-stage SCI occurs |
| vivo | No | We think the second change is duplicated with Section 5.22.2.1 as highlighted in yellow below. Keep the SCI definition in one place is enough. 5.22.2.1 SCI reception SCI indicate if there is a transmission on SL-SCH and provide the relevant HARQ information. A SCI consists of two parts: the 1st stage SCI on PSCCH and the 2nd stage SCI on PSSCH as specified in clause 8.1 of TS 38.214 [7]. |
| Samsung | Yes with comment | Agree with OPPO’s rewording |
| ZTE | Yes | Agree with OPPO and CATT’s rewording. To respond the rapporteur’s comment: We think the original description is not aligned with RAN1’s definition and prefer to capture it explicitly. |
| Apple | No | Not sure if the allocation of mode 1 resources in PDCCH cares about the nuance of 1st and 2nd stage SCI. It is better to level PHY details in a reference to PHY spec. |
| Ericsson | No | Agree with rapporteur |
| Intel | Yes | We think OPPO’s rewording makes sense |
| Lenovo | Yes with comments | Agree with the intention since “a set of PSCCH duration(s) in which transmission of SCI occurs” and “a set of PSSCH duration(s) in which transmission of SL-SCH” is not so accurate consider 2nd stage SCI is also in PSSCH. We agree with HW’s rewording in which transmission of SL-SCH and 2nd-stage SCI associated with the 1st-stage SCI occurs |

As a result, 6 companies not to agree this change, but 6 companies want this change with revision. As vivo pointed out, there is description of two parts of SCI in 5.22.2.1 (SCI reception section) in current MAC spec.

“SCI indicate if there is a transmission on SL-SCH and provide the relevant HARQ information. A SCI consists of two parts: the 1st stage SCI on PSCCH and the 2nd stage SCI on PSSCH as specified in clause 8.1 of TS 38.214 [7].”

Moreover, regarding SCI formats, detail description is specified in PHY spec (38.214). In a rapporteur’s perspective, there is no critical/technical motivation to include this CR. Nothing is broken.

Summary 4B:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 6 |
| No | 6 |
| No strong view | 1 |

Recommendation 4B: the second change in R2-2100503 is not pursued.

Question 4C:	Do you agree to reflect the third change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | The third change is not needed. We have an understanding that Prior to scheduling request is triggered, the MAC CE would not be generated. |
| CATT | Yes |  |
| ASUSTeK | Yes | We agree with the change since there’s no priority value for the trigger “SL-CSI reporting”, and other changes in the third change seem reasonable. Since there’s no MAC CE generated yet, perhaps additional change could be: The value of the priority of the triggered SR corresponds to the value of the priority of the a Sidelink CSI Reporting MAC CESL-CSI reporting. |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| HW | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| ZTE | Yes | To respond the rapporteur’s comment: The size of CSI report MAC CE is 8bits. It is not necessary that whether MAC CE is generated, UE can judge whether SL grant can accommodate the CSI report MAC CE before generating it. |
| Apple | Yes |  |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Lenovo | Yes |  |

Summary 4C:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 12 |
| No | 1 |

Recommendation 4C: the third change in R2-2100503 is reflected on 38.321

- R2-2100504 (ZTE, Sanechips)

###### 5.22.1.4.1.2 Selection of logical channels The MAC entity shall for each SCI corresponding to a new transmission: 1> select a Destination associated to one of unicast, groupcast and broadcast, having at least one of the MAC CE and the logical channel with the highest priority, among the logical channels that satisfy all the following conditions and MAC CE(s), if any, for the SL grant associated to the SCI: 2> SL data is available for transmission; and 2> SBj > 0, in case there is any logical channel having SBj > 0; and 2> sl-configuredGrantType1Allowed, if configured, is set to true in case the SL grant is a Configured Grant Type 1; and 2> sl-AllowedCG-List, if configured, includes the configured grant index associated to the SL grant in case the SL grant is a Configured Grant; and 2> sl-HARQ-FeedbackEnabled is set to disabled, if PSFCH is not configured for the SL grant associated to the SCI.

Question 5A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | According to RRC spec, description of sl-AllowedCG-List describes that this applies only when the SL grant is a configured grant. Hence, additional explanation on MAC spec is not needed. |
| CATT | No | A further explanation is not needed and the current description is aligned with the Uu interface’s description. |
| ASUSTeK | No |  |
| Qualcomm | No | Agree with rapporteur |
| OPPO | No |  |
| HW | No | Agree with rapporteur |
| vivo | No | There is no ambiguity in the original text. |
| Samsung | No |  |
| ZTE | Yes | We can follow the majority. |
| Apple | No |  |
| Ericsson | No | Agree with rapporteur |
| Intel | No | We think this clear from the context in the existing text, so do not see the need for this change |
| Lenovo | No |  |

Summary 5A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 12 |

Recommendation 5A: the change in 2100504 is not pursued.

- R2-2101068 (Nokia)

if there are both a sidelink grant for transmission of NR sidelink communication and a configured grant for transmission of V2X sidelink communication on SL-SCH as described in clause 5.14.1.2.2 of TS 36.321 [22] at the time of the transmission, and the MAC PDU includes any MAC CE prioritized as described in clause 5.4.3.1.3 or the highest priority value of the logical channel(s) in the MAC PDU is lower than ul-PrioritizationThres if ul-PrioritizationThres is configured; or

Question 6A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | No |  |
| ASUSTeK | No strong view | It could be good to align terminologies, but we can follow majority. |
| Qualcomm | No |  |
| OPPO | Yes | Good to align terminology in the spec. |
| HW | No | Seems not needed. |
| vivo | Yes | Although it’s editorial change but better to align the specification terminology. |
| Samsung | No |  |
| ZTE | No |  |
| Apple | Yes | The change is ok. |
| Ericsson | No |  |
| Intel |  | Seems like an editorial change. No strong view |
| Lenovo | No | No strong view |

Summary 6A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 3 |
| No | 8 |
| Not strong view | 2 |

Recommendation 6A: the change in R2-2101068 is not pursued.

- R2-2101149 (Nokia)

NOTE 1: In this release of specifications, UE behaviour if both Sidelink resource allocation mode 1 and Uu DRX are configured is left to UE implementation.

Question 7A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | From a rapporteur’s perspective “left UE implementation” means a UE has to be prepared for this DRX configuration by some implementation. However, rapporteur thinks that this configuration is invalid and if DRX is configured, UE behavior is unspecified according to agreements and the current NOTE. Hence, the change in this CR seems not correct. |
| CATT | No | We share the same view with LG . |
| ASUSTeK | No | We think the original note captures the intention of the agreement. If the concern is on the future-proof of this note, we can add: If Sidelink resource allocation mode 1 is configured by RRC, a DRX functionality is not configured in Rel-16. |
| Qualcomm | No | We find the original text clear in stating SL DRX is not configured for Mode 1, and do not see a compelling reason to modify. |
| OPPO | No |  |
| HW | No | The original text is aligned with the agreement. |
| vivo | No | We prefer to following the wording in the original agreement. |
| Samsung | No |  |
| ZTE | No |  |
| Apple | No |  |
| Ericsson | No | Share the same view as LG |
| Intel | No |  |
| Lenovo | No |  |

Summary 7A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 0 |
| No | 13 |

Recommendation 7A: the change in R2-2101149 is not pursued.

- R2-2100323 (CATT)

Among the UL transmissions where the MAC entity is able to perform the transmission of NR sidelink communication prioritized simultaneously, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether each UL transmission is performed.

Question 8A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | Yes | We would like to follow the majority’s view |
| ASUSTeK | No strong view |  |
| Qualcomm | No |  |
| OPPO | No |  |
| HW | No strong view | We are fine to follow the majority. |
| Vivo | No |  |
| Samsung | No |  |
| ZTE | No |  |
| Apple | No strong view |  |
| Ericsson | No strong view |  |
| Intel | No | Seems just a wording change to clarify but not essential |
| Lenovo | No | No strong view |

Summary 8A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 8 |
| No strong view | 4 |

Recommendation 8A: the change in R2-2100323 is not pursued.

- R2-2101742 (ASUSTeK)

2> sl-periodicBSR-Timer expires, in which case the SL-BSR is referred below to as 'Periodic SL-BSR'; 2> Sidelink resource allocation mode 1 is reconfigured by RRC and SL data is available for transmission in the RLC entity or in the PDCP entity, in which case the Sidelink BSR is referred below to as "Regular SL-BSR". 1> else: 2> Sidelink resource allocation mode 1 is configured by RRC and SL data is available for transmission in the RLC entity or in the PDCP entity, in which case the Sidelink BSR is referred below to as "Regular SL-BSR".

Question 9A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | No | Just follow the same processing mechanism of Uu interface. |
| ASUSTeK | Yes | According to the current specification, for a UE configured with allocation mode-1 in the source cell, upon performing handover to a target cell, since the UE will fall under the cyan condition check and will not trigger a SL-BSR if mode-1 is delta-configured by the target cell: - if the MAC entity has been configured with Sidelink resource allocation mode 1: (…) 1> else: 2> Sidelink resource allocation mode 1 is configured by RRC and SL data is available for transmission in the RLC entity or in the PDCP entity, in which case the Sidelink BSR is referred below to as "Regular SL-BSR". Therefore we think the current specification does not cover this case and should be modified. |
| Qualcomm | No |  |
| OPPO | No | During HO, we believe the following operation in mac reset can solve this issue 1> cancel, if any, triggered Buffer Status Reporting procedure; 1> cancel, if any, triggered Power Headroom Reporting procedure; 1> cancel, if any, triggered consistent LBT failure; 1> cancel, if any, triggered BFR; 1> cancel, if any, triggered Sidelink Buffer Status Reporting procedure; |
| HW | No |  |
| vivo | No | In NR Uu, there is no BSR trigger in HO. It is an optimization to introduce additional SL BSR trigger only for SL. |
| Samsung | No |  |
| ZTE | No |  |
| Apple | No |  |
| Ericsson | No |  |
| Intel | No | Agree with OPPO |
| Lenovo | No |  |

Summary 9A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 12 |

Recommendation 9A: the change in R2-2101742 is not pursued.

- R2-2100861 (Apple)

Question 10A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | According this CR, NACK only HARQ feedback is selected according to UE’s location availability, sl-TransRange, involving Zone_ID, before the number of candidate PSFCH resources is checked. However, there is no agreement on such selection procedure for the NACK only HARQ feedback. Moreover, RAN1’s understanding is below: Agreements: - RAN1 assumes that RAN2 will handle selection of appropriate groupcast HARQ feedback option. From RAN1 perspective, a TX UE can use GC HARQ feedback Option 2 only when the group size is not greater than the number of candidate PSFCH resources associated with the selected PSSCH resource. - LS (R1-2001426) is approved |
| CATT | No | The change in the CR doesn’t align with the agreement. |
| ASUSTeK | No |  |
| Qualcomm | No | We appreciate the intention of the CR, but perhaps a reference to the relevant RAN1 specification would be simpler. |
| OPPO | No | We have not fully understood the issue, i.e., to us one can interpret the spec in a way that until this step, the 2A/2B format is selected |
| HW | No | We don't need to specify which SCI format is selected by the UE, from MAC perspective, we only need to determine under which condition UE is only allowed to select nack only and under which condition UE is able to select either. Regarding to the detailed SCI format, we can leave it to RAN1 |
| vivo | No | Agree with QC’s comments. The details of the SCI format should be transparent to MAC and keeping a corresponding RAN1 reference is enough. In our understanding, it should be up to UE implementation to use SCI format 2A/2B in this situation. |
| Samsung | No |  |
| ZTE | No | L1 knows how to select SCI format. If communication range and zone ID are provided by L2, L1 will select SCI format 2-B, because only SCI format 2-B supports communication range and zone ID. |
| Apple | Yes | NACK-only groupcast mechamism is supported in both SCIU format 2-A and 2-B. If the location information (range, zone ID) is not passed down to PHY layer, our assumption is that SCI format 2A must be used by PHY layer for this groupcast. There is no L1 text on this assumption. Thus, we think it is necessary to clarfy this in current MAC spec. |
| Ericsson | No |  |
| Intel | No | Agree with HW |
| Lenovo | No | Agree with HW and we prefer to left SCI format selection to PHY |

Summary 10A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 12 |

Recommendation 10A: the change in R2-2100861 is not pursued.

- R2-2100119 (OPPO)

1) First change

1> if the BWP is activated: 2> transmit SL-BCH on the BWP, if configured; 2> transmit PSCCH on the BWP; 2> transmit SL-SCH on the BWP; 2> receive PSFCH on the BWP, if configured. 2> receive SL-BCH on the BWP, if configured;

Question 11A:	Do you agree to reflect the first change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | Yes |  |
| ASUSTeK | Yes | It’s good to align with the rest of the spec. |
| Qualcomm | No | Agree with rapporteur |
| OPPO | Yes | PSBCH is not something visible at MAC layer, i.e., only SL-BCH is the thing visible at MAC layer. We do not think “nothing is broken” is an excuse to sacrifice specification quality. |
| HW | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| ZTE | No | Share the view with rapporteur. |
| Apple | No strong view | No impact on the spec and UE implementation.. |
| Ericsson | Yes |  |
| Intel | Yes |  |
| Lenovo | Yes | Better to align with other bullet |

Summary 11A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 9 |
| No | 3 |
| No strong view | 1 |

Recommendation 11A: the first change in R2-2100119 is reflected on 38.321

2) Second change

Sidelink grant is received dynamically on the PDCCH, configured semi-persistently by RRC or autonomously selected by the MAC entity. The MAC entity shall have a sidelink grant on an active SL BWP to determine a set of PSCCH duration(s) in which transmission of SCI occurs and a set of PSSCH duration(s) in which transmission of SL-SCH associated with the SCI occurs. A sidelink grant addressed to SL-RNTI or SLCS-RNTI with NDI = 1 is considered as a dynamic sidelink grant. 5> consider the first set of transmission opportunities as the initial transmission opportunities and the other set(s) of transmission opportunities as the retransmission opportunities; 3> consider all transmission opportunities as the selected sidelink grant.

Question 11B:	Do you agree to reflect the second change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | Yes |  |
| ASUSTeK | No | We think the original wording does not exclude the sidelink grant addressed to SL-RNTI for being a dynamic grant. But changes can be adopted if there’s misunderstanding. |
| Qualcomm | No | Agree with rapporteur |
| OPPO | Yes | We are fine to rely on majority view. For the 2nd change above. But for the first one, it is necessary since there is no definition for the SL-RNTI based grant definition in the spec, so something is broken. |
| HW | No | For the first change, we don’t think it is needed. Instead we think the definition of the dynamic sidelink grant should be removed as in last meeting we agreed to delete the description about clearing the dynamic sidelink grant”. Since there is no other “dynamic sidelink grant” in use for SL, so the definition can be deleted. |
| vivo | No | For the 1st change, the same change has been proposed by R2-2007875 but RAN2 agreed not to have it. For the 2nd change, no strong view. |
| Samsung | No |  |
| ZTE | No |  |
| Apple | Yes for the 1st change | 2nd chage is not needed. |
| Ericsson | No |  |
| Intel | No |  |
| Lenovo | No | The 1st change is similar to legacy Uu UL grant description: “An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.”. we think this sentence is to explain why a grant associated with a SLCS-RNTI is a dynamic grant. And a grant associated with a SL-RNTI is a dynamic grant for sure, similar to C-RNTI which is also not included in the sentence |

Summary 11B:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 3 |
| No | 10 |

Recommendation 11B: the second change in R2-2100119 is not pursued.

3) Third change

1> else (i.e. retransmission): 2> if the HARQ Process ID corresponding to the dynamic sidelink grant, the configured sidelink grant or the selected sidelink grant is associated to a Sidelink process of which HARQ buffer is empty; or 2> if the HARQ Process ID corresponding to the dynamic sidelink grantis not associated to any Sidelink process: 3> ignore the sidelink grant. 2> else: 3> identify the Sidelink process associated with this grant, and for the associated Sidelink process: 4> deliver the sidelink grant of the MAC PDU to the associated Sidelink process; 4> instruct the associated Sidelink process to trigger a retransmission.

Question 11C:	Do you agree to reflect the third change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | No |  |
| ASUSTeK | No strong view | Can go with rapporteur’s view. |
| Qualcomm | No | Agree with rapporteur |
| OPPO | Yes | This for terminology alignment in the spec, it is strange that we use “dynamic sidelink grant” here and “sidelink grant received on PDCCH” there.. |
| HW | No | See our comments on Q 11B, with the definition removed, we can keep this part as it is. |
| vivo | No |  |
| Samsung | No |  |
| ZTE | No |  |
| Apple | No | There is no real difference between two wording choices. |
| Ericsson | No |  |
| Intel | No strong view |  |
| Lenovo | No | No strong view |

Summary 11C:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 1 |
| No | 10 |
| No strong view | 2 |

Recommendation 11C: the third change in R2-2100119 is not pursued.

4) Fourth change

> if sl-MaxTransNum corresponding to the highest priority of the logical channel(s) in the MAC PDU has been configured in sl-CG-MaxTransNumList for the configured sidelink grant by RRC and the number of transmissions of the MAC PDU has been reached to sl-MaxTransNum; or

Question 11D:	Do you agree to reflect the fourth change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | No | Nothing is broken |
| CATT | No |  |
| ASUSTeK | No | A sidelink grant must be a configured sidelink grant if it’s associated with sl-CG-MaxTransNumList. |
| Qualcomm | No | Agree with rapporteur |
| OPPO | Yes | We are fine to rely on majority view. |
| HW | No strong view | We are fine to follow the majority. |
| vivo | No strong view | Same understanding as ASUSTeK. |
| Samsung | No |  |
| ZTE | No | The field description indicates that this IE is used for CG. |
| Apple | No |  |
| Ericsson | No |  |
| Intel | No |  |
| Lenovo | No |  |

Summary 11D:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 0 |
| No | 11 |
| No strong view | 2 |

Recommendation 11D: the fourth change in R2-2100119 is not pursued.

- R2-2100211 (CATT)

Question 12A:	Do you agree to reflect the change?

| Company | Yes/No | Comment |
| --- | --- | --- |
| LG | Yes | “logical channel prioritization” Others parts are not needed. Nothing is broken |
| CATT | Yes | For others parts are really needed. Terminology unification is a necessary requirement for professional and high quality technical papers. |
| ASUSTeK |  | We can follow majority’s view for terminology changes. |
| OPPO | No with comment | If terminology alignment is something we tend to pursue (OPPO support that), we are fine to go for it. But by reading this CR, seems it is more about the usage of abbreviation and capital letter, which seem to be more cosmetic. |
| HW | No strong view | We are fine to follow the majority. |
| vivo | Yes | Follow rapporteur’s suggestion. |
| Samsung | No strong view |  |
| ZTE | No strong view |  |
| Apple | No | Only editorials. Nothing essential to correct. |
| Ericsson | No | The changes are tiny, and not needed. |
| Intel | No | Seems like an editorial change and not a necessary correction |
| Lenovo | Yes | We are fine to follow rapporteur’s suggestions |

Summary 12A:

| Answer | Number of supporting companies |
| --- | --- |
| Yes | 4 (only prioritization) |
| No | 4 |
| No strong view | 3 |

Recommendation 12A: the change in R2-2100211 is not pursued except “prioritization”.

Conclusion and recommendation

In conclusion, Rapporteur proposes the following recommendations as the outcome of this email discussion:

Recommendation 1A: the change in R2-2100212 and R2-2101741 is reflected on 38.321.

Recommendation 2A: the change in R2-2100213 is reflected on 38.321

Recommendation 3A: the change in R2-2101741 is not pursued.

Recommendation 4A: the first change in R2-2100503 is reflected on 38.321

Recommendation 4B: the second change in R2-2100503 is not pursued.

Recommendation 4C: the third change in R2-2100503 is reflected on 38.321

Recommendation 5A: the change in 2100504 is not pursued.

Recommendation 6A: the change in R2-2101068 is not pursued.

Recommendation 7A: the change in R2-2101149 is not pursued.

Recommendation 8A: the change in R2-2100323 is not pursued.

Recommendation 9A: the change in R2-2101742 is not pursued.

Recommendation 10A: the change in R2-2100861 is not pursued.

Recommendation 11A: the first change in R2-2100119 is reflected on 38.321

Recommendation 11B: the second change in R2-2100119 is not pursued.

Recommendation 11C: the third change in R2-2100119 is not pursued.

Recommendation 11D: the fourth change in R2-2100119 is not pursued.

Recommendation 12A: the change in R2-2100211 is not pursued except “prioritization”.