3GPP TSG RAN WG2 Meeting #113e	R2-2102211

Electronic meeting, 25th Jan – 5th Feb, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0540 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Cell Grouping UE capability for NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-03-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The release-15 standard only supports synchronous NR-DC configuration where all serving cells of the MCG are in FR1 and all serving cells of the SCG are in FR2. This restriction is removed in release-16 and it requires new UE capability signalling to convey the supported mapping of serving cells to Cell Groups. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New release-16 capability to indicate mapping of frequency bands of serving cells to cell groups (i.e. MCG or SCG) is introduced. Impact Analysis: Impacted 5G architecture options: NR-DC Impacted functionality: NR-DC Cell Grouping. Inter-operability: - If the network is implemented according to the CR and the UE is not; No inter-operability problem. The network would simply assume the UE only supports release-15 version of Cell Grouping. - If the UE is implemented according to the CR and the network is not; No inter-operability problem. The network would simply ignore the reported new UE capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-DC Cell Grouping beyond that of release-15 is not supported by the standard. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.331 CR xxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.2.7.12	NRDC-Parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| asyncNRDC-r16 Indicates whether the UE supports asynchronous NR-DC with MRTD and MTTD as specified in clause 7.5 and 7.6 of TS 38.133 [5]. A UE indicating this capability shall support asynchronous NR-DC configuration where all serving cells of the MCG are in FR1 and all serving cells of the SCG are in FR2. | BC | No | No | No |
| cellGroupingAsync-r16 Indicates the mapping of serving cells to cell groups (i.e. MCG or SCG) the UE supports in asynchronous NR-DC. The UE indicates this capability for a band combination with at least two frequency bands and the carriers corresponding to a frequency band shall belong to only one cell group. The UE includes this field when the UE supports additional mapping of serving cells to cell groups with respect to the mapping as indicated by asyncNRDC-r16. The value sameAsSync indicates the UE supports the same mappings of serving cells to cell groups as indicated in cellGroupingSync-r16. For a band combination with two or more frequency bands, the bitmap size is selected based on the number of frequency bands in the band combination, i.e., in case of two frequency bands, the bitmap corresponding to twoBands is selected and so on. A bit in the bit string set to 1 indicates that the UE supports asynchronous DC for the cell grouping option represented by the concerned bit position. Each bit position represents a different cell grouping option, as illustrated by the table, see NOTE 1. | BC | No | N/A | N/A |
| cellGroupingSync-r16 Indicates the mapping of serving cells to cell groups (i.e. MCG or SCG) the UE supports in synchronous NR-DC. The UE indicates this capability for a band combination with at least two frequency bands and the carriers corresponding to a frequency band shall belong to only one cell group. The UE includes this field when the UE supports additional mapping of serving cells to cell groups with respect to the mapping as indicated by ca-ParametersNRDC. The value anyGrouping indicates the UE supports all possible mappings of serving cells to cell groups for the band combination, with the restriction as stated above. For a band combination with two or more frequency bands, the bitmap size is selected based on the number of frequency bands in the band combination, i.e., in case of two frequency bands, the bitmap corresponding to twoBands is selected and so on. A bit in the bit string set to 1 indicates that the UE supports synchronous DC for the cell grouping option represented by the concerned bit position. Each bit position represents a different cell grouping option, as illustrated by the table, see NOTE 1. | BC | No | N/A | N/A |
| intraFR-NR-DC-PwrSharingMode1-r16 Indicates whether the UE supports intra-FR NR DC with semi-static power sharing mode1 between MCG and SCG cells of same frequency range as defined in TS 38.213 [11]. If this field is absent, the UE does not support intra-FR NR DC. | BC | No | No | No |
| intraFR-NR-DC-PwrSharingMode2-r16 Indicates whether the UE supports semi-static power sharing mode2 between MCG and SCG cells of same frequency range for synchronous intra-FR NR DC as defined in TS 38.213 [11]. The UE indicating the support of this also indicates the support of intraFR-NR-DC-PwrSharingMode1-r16. | BC | No | No | No |
| intraFR-NR-DC-DynamicPwrSharing-r16 Indicates the UE support of dynamic power sharing for intra-FR NR DC between MCG and SCG cells of same frequency range with long or short offset as specified in TS 38.213 [11]. The UE indicating the support of this also indicates the support of intraFR-NR-DC-PwrSharingMode1-r16. | BC | No | No | No |
| sfn-SyncNRDC Indicates the UE supports NR-DC only with SFN and frame synchronization between PCell and PSCell. If not included by the UE supporting NR-DC, the UE supports NR-DC with slot-level synchronization without condition on SFN and frame synchronization. In this release of the specification, the UE shall not report this UE capability. | UE | No | No | No |
| NOTE 1: The mapping of serving cells to cell groups (i.e. MCG or SCG), as indicated by cellGroupingSync-r16 and cellGroupingAsync-r16, is shown in the table below. A cell grouping option is represented by a number of bits, where the left-most bit corresponds to the frequency band listed first in the band combination, etc. Value 0 indicates that the carriers of the corresponding frequency band are mapped to MCG, while value 1 indicates that the carriers of the corresponding frequency band are mapped to SCG. The leading / leftmost bit of cellGroupingSync-r16 and cellGroupingAsync-r16 respectively corresponds to the Bit String Position 1. Nr of Frequency Bands:; 5; 4; 3; 2. Length of Bit-String:; 30; 14; 6; 2. Bit String Position; Cell grouping option (0= MCG, 1= SCG). 1; 00001; 0001; 001; 01. 2; 00010; 0010; 010; 10. 3; 00011; 0011; 011. 4; 00100; 0100; 100. 5; 00101; 0101; 101. 6; 00110; 0110; 110. 7; 00111; 0111. 8; 01000; 1000. 9; 01001; 1001. 10; 01010; 1010. 11; 01011; 1011. 12; 01100; 1100. 13; 01101; 1101. 14; 01110; 1110. 15; 01111. 16; 10000. 17; 10001. 18; 10010. 19; 10011. 20; 10100. 21; 10101. 22; 10110. 23; 10111. 24; 11000. 25; 11001. 26; 11010. 27; 11011. 28; 11100. 29; 11101. 30; 11110. |

End of change