3GPP TSG RAN WG2#113e	R2-2102212

Online meeting, 25 January – 05 February, 2021

# Title:	Introduction of Cell Grouping UE capability for NR-DC

# Response to:

# Release:	Release 16

# Work Item	LTE_NR_DC_CA_enh-Core

Source:	RAN2

To:	RAN4

Cc:	RAN1

Contact Person:

Name:	Masato Kitazoe

E-mail Address:	mkitazoe@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 technically endorsed CRs introducing Cell Grouping UE capability for NR-DC. RAN2 used “LTE-style” signalling (see Annex).

The key difference from LTE signalling though is that in the new NR signalling, each bit of the bitmap corresponds to a “frequency band” in the band combination, instead of “band entry” as in LTE signalling. Note that “band entry” corresponds to one contiguous band block in a band combination.

RAN2 has chosen this option because:

- RAN2 recognizes that the number of band entries is significantly increasing in NR, mainly because of FR2, e.g. CA_n5A-n260(8A).

- RAN2 however wanted to keep the UE capability signalling overhead to be comparable to LTE signalling.

This means that the new UE capability signalling for NR-DC covers NR-DC band combinations with up to 5 frequency bands, with the restriction that band entries of the same frequency band can be assigned to only one Cell Group.

For Sync NR-DC, RAN2 would like to note that RAN2 did not adopt the solution to reuse PUCCH grouping signalling, where grouping is done based on cell type {FR1 licensed TDD, FR1 unlicensed TDD, FR1 licensed FDD, FR2}, as indicated as a possible signalling solution by RAN1 in R1-2009570.

2. Actions:

To SA2:

ACTION:	RAN2 respectfully asks RAN4 to take the above information into account, and respond if they have any concern.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2#113bis-e	 April 12th – April 20th, 2021	Online meeting

4. Annex

A cell grouping option is represented by a number of bits, where the left-most bit corresponds to the frequency band listed first in the band combination, etc. Value 0 indicates that the carriers of the corresponding frequency band are mapped to MCG, while value 1 indicates that the carriers of the corresponding frequency band are mapped to SCG.

The UE signals the Bit-String where each bit corresponds to a cell grouping option. A bit in the bit string set to 1 indicates that the UE supports NR-DC with the cell grouping option.

| Nr of Frequency Bands: | 5 | 4 | 3 | 2 |
| --- | --- | --- | --- | --- |
| Length of Bit-String: | 30 | 14 | 6 | 2 |
| Bit String Position | Cell grouping option (0= MCG, 1= SCG) |  |  |  |
| 1 | 00001 | 0001 | 001 | 01 |
| 2 | 00010 | 0010 | 010 | 10 |
| 3 | 00011 | 0011 | 011 |  |
| 4 | 00100 | 0100 | 100 |  |
| 5 | 00101 | 0101 | 101 |  |
| 6 | 00110 | 0110 | 110 |  |
| 7 | 00111 | 0111 |  |  |
| 8 | 01000 | 1000 |  |  |
| 9 | 01001 | 1001 |  |  |
| 10 | 01010 | 1010 |  |  |
| 11 | 01011 | 1011 |  |  |
| 12 | 01100 | 1100 |  |  |
| 13 | 01101 | 1101 |  |  |
| 14 | 01110 | 1110 |  |  |
| 15 | 01111 |  |  |  |
| 16 | 10000 |  |  |  |
| 17 | 10001 |  |  |  |
| 18 | 10010 |  |  |  |
| 19 | 10011 |  |  |  |
| 20 | 10100 |  |  |  |
| 21 | 10101 |  |  |  |
| 22 | 10110 |  |  |  |
| 23 | 10111 |  |  |  |
| 24 | 11000 |  |  |  |
| 25 | 11001 |  |  |  |
| 26 | 11010 |  |  |  |
| 27 | 11011 |  |  |  |
| 28 | 11100 |  |  |  |
| 29 | 11101 |  |  |  |
| 30 | 11110 |  |  |  |