3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2102268

Elbonia, 25 January – 05 February 2021

Agenda item:	5.2

Source:	Nokia (Rapporteur)

Title:	Offline 001 on Stage 2 Corrections

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT113-e][001][NR15] Stage-2 (Nokia)

Scope: Treat R2-2100270, R2-2100271, R2-2101345, R2-2100091, R2-2100092, R2-2101478, R2-2101653

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule Thursday Jan 28 1200 UTC

# 2	UE Capabilities

The CRs on UE capabilities endorsed at the last meeting are resubmitted as such since no comments have been received in the meantime.

Question 1: Can we now agree the CRs R2-2100270 & R2-2100271?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated | Yes | Proponent. |
| Huawei, HiSilicon | Yes |  |
| Ericsson | Yes | Proponent. |
| Apple | Yes | Looks ok to us. |
| Nokia | Yes | Proponent. |
| vivo | Yes |  |
| Google | Yes |  |
| CATT | Yes |  |
| LG | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes |  |

Summary 1: all companies agree that the CRs can now be agreed.

Proposal 1: CRs R2-2100270 & R2-2100271 are agreed.

# 3	Data Forwarding

A CR on data forwarding upon intra-system HO using full configuration was submitted in R2-2101345. It is the rapporteur’s understanding that this CR was first discussed in RAN3, where it was felt that it would be more appropriate to handle it in RAN2. The CR argues that “it is unclear how to handle forwarded data upon intra-system HO using full configuration.”

Question 2A: Do you agree with the intention of the CR?

| Answers to Question 2A |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated |  | We would like to understand why the use of full configuration would affect retransmissions for loss-less data delivery in the target NG-RAN node. This also should be captured in the reason for change. |
| Huawei, HiSilicon |  | We understand in such a handover scenario, if only PDU session tunnel is setup, this does not apply. So this CR applies only in case of DRB level tunnel, and means that in this case, full configuration would clear the PDCP parameters and thus the original data from source gNB would not be forwarded anymore. We are wondering whether in this case it is essential to make such a change, as implementation wise, not all cases lead to data loss by full configuration. |
| Ericsson |  | We echo the comment from Qualcomm and Huawei. We believe that this is only one of the cases in which there is a data loss upon full configuration. However, this does not mean that full configuration is equal to data loss. Therefore, we are not sure we need to capture something in the spec. |
| Apple | Yes | We are fine to duplicate the 36.300 text in 38.300. |
| Nokia | Yes | Proponent. |
| vivo |  | We also wonder why PDCP SDU cannot be retransmitted in target NG-RAN if RLC-AM and FULL configuration are used? |
| Google | Yes | Proponent. As the target node has decided to perform full configuration, the PDCP SN is reset. We understand that the intention is to avoid wasting radio resources for retransmitting the PDCP SDU with SN which has been transmitted. Also with the proposed change, the possibly duplicated data received at the UE can be avoided. |
| CATT | Yes | Proponent. |
| LG | Yes | This is same as LTE U-plane handling. |
| OPPO |  | We have similar question as Qualcomm |
| Intel | Yes | Proponent. |
| ZTE |  | We are fine with the intention but not sure whether the CR is essential. |
| Samsung | Yes | Proponent >Question from Qualcomm: We hope this was sufficiently answered by Google. If desired, cover page can be updated to clarify >Regarding comment from Huawei: The data received from PDU session tunnel are fresh data i.e. SDAP SDU (no PDCP SN). So nothing needs to be clarified for this. However for DRB tunnel, there are two types of data including PDCP SDUs without SN and PDCP SDUs for which transmission was attempted to the UE. The latter should be ignored. Without any description, the target may have the same handling |

Summary 2A: Including the original co-signers, 10 companies agree with the intention, 5 asked for clarifications and 1 is not sure.

Proposal 2A: Clarify the problem on the cover sheet.

Question 2B: If you agree with the intention, are you happy with the wording or would you like to enhance it?

| Answers to Question 2B |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments & Possible Changes |
| Apple | Happy | The coversheet of the CR has some issues: - The inter-operabiltiy part: I think the change has impact on NW for DL delivery, so we do not understand the meaning of “duplicates might be delivered delivered to upper layers” The sentence in “Consequence if not approve” seems incomplete. |
| Nokia | Yes |  |
| Google | Yes |  |
| CATT | Yes |  |
| LG | Yes |  |
| Intel | Yes |  |
| Samsung | Yes |  |

Summary 2B: All companies having expressed an opinion are fine with the actual changes but issues were raised on the cover sheet.

Proposal 2B: clarify the intention and update the cover sheet of R2-2101345 to see if it can be agreed.

# 4	PDCP Change Indication

CRs on PDCP change indication were submitted in R2-2100091 & R2-2100092. The CR argues that the PDCP change indication can be included in the SN Modification Request message for the MN initiated SN modification.

Question 3A: Do you agree with the intention of the CRs?

| Answers to Question 3A |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Ericsson | No | If RAN3 specification is clear, we believe there is no need to update also stage 2. At the end of the story, current specification does not prevent the PDCP change indication to be included in the SN modification request message. We would like to not add over-clarifications in stage 2. |
| Apple | Yes |  |
| Nokia | Yes | PDCP Change Indication is indeed missing. |
| vivo | Yes |  |
| Google | Yes |  |
| CATT | Yes |  |
| LG | Yes | We think this is a missing aspect. |
| OPPO | Yes |  |
| Intel | Yes |  |
| ZTE | No | Since it is already clear in RAN3 specification and there is no stopper in stage2, we think nothing is broken without the CR. In addition, it seems not necessary to add everything in stage 2 specs, otherwise, lots of changes will be expected in the future. |
| Samsung | Yes | Agree with the reason for change. Prefer merging with other CR if possible. |

Summary 3A: Out of the 13 companies having expressed an opinion, 11 believe the CR is needed while 2 believe it is not.

Proposal 3A: Agree with the intention of the CR.

Question 3B: If you agree with the intention, are you happy with the wording or would you like to enhance it?

| Answers to Question 3B |  |  |
| --- | --- | --- |
| Company | Yes/No | Possible Changes |
| Qualcomm Incorporated | No | This is an alignment to stage-3, which does not seem very essential to warrant a release-15 CR. |
| Huawei, HiSilicon | Yes | The wording seems OK. |
| Apple | Yes |  |
| Nokia | Yes |  |
| vivo | yes |  |
| Google | Yes | No strong opinion as it is to align 38.423 and 37.340 |
| CATT | Yes |  |
| LG | Yes |  |
| OPPO | Yes |  |
| Intel | Yes |  |
| Samsung | Yes |  |

Summary 3B: all companies having supported the intention agree with the suggested wording.

Proposal 3B: CRs R2-2100091 & R2-2100092 are agreed.

# 5	Power Sharing

A CR on power sharing was submitted in R2-2101478. The CR argues that “the description of power sharing for (NG)EN-DC and NE-DC is missing although the description for NR-DC was introduced in Rel-16.”

Question 4A: Do you agree with the intention of the CRs?

| Answers to Question 4A |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated | Yes |  |
| Huawei, HiSilicon | Yes | Proponent |
| Ericsson | No | We are a bit hesitant to have this CR as the proposed change is not critical. The reason for having the description for NR-DC in Rel-16 is because the overall framework is more complex and so the signalling used. But for (NG)EN-DC and NE-DC probably there is no need for have the description. |
| Apple | Yes |  |
| Nokia | No | Not an essential correction |
| Google | Yes |  |
| CATT | Yes |  |
| LG | No | Agree with Ericsson. However, ff majority companies want to have such clarification, we think Rel-16 CR with simple change is enough. In (NG)EN-DC, NE-DC, and NR-DC, power sharing is performed within frequency band with either semi-static or dynamic power sharing. With semi-static power sharing, the UE transmission power is split between MCG and SCG through configuration. With dynamic power sharing, when determining the UL transmission power of a SCG transmission, the UE takes into account transmission(s) on MCG overlapping with any part of the SCG transmission as specified in TS38.213[21]. |
| OPPO |  | For the following added wording, just wondering for NE-DC/(NG)EN-DC case, whether we need to capture something in LTE side, i.e., TS 36.300? In (NG)EN-DC and in NE-DC, power sharing can be performed within a frequency range with either semi-static or dynamic power sharing. With semi-static power sharing, the maximum UE transmission power is split between MCG and SCG through configuration. With dynamic power sharing: - when determining the UL transmission power of an SCG transmission in (NG)EN-DC, the UE takes into account transmission(s) on MCG overlapping with any part of the SCG transmission; - when determining the UL transmission power of an MCG transmission in NE-DC, the UE takes into account transmission(s) on SCG overlapping with any part of the MCG transmission. |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Yes | We think LG’s wording can be a good compromise |

Summary 4: Out of the 12 companies having expressed an opinion, 8 believe the CR is needed, while 3 do not. An alternative wording was suggested.

Proposal 4: Agree with the intention.

Question 4B: If you agree with the intention, are you happy with the wording or would you like to enhance it?

| Answers to Question 4B |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments & Possible Changes |
| Qualcomm Incorporated | Yes | The text “through configuration” is not entirely clear and can be improved. |
| Huawei, HiSilicon | Yes | The intention of “through configuration” was trying to say semi-static power sharing is statically split by configuration. How about “…the maximum UE transmission power is split statically between MCG and SCG by RRC configuration”? |
| Apple | Yes | Wording is ok. |
| Google | Yes |  |
| CATT | Yes |  |
| Intel | Yes |  |
| ZTE | Yes |  |
| Samsung | Maybe | LGE’s simplified version look good |

Summary 4B: although a majority of companies are happy with the original wording, some concerns were raised. The LG alternative was also mentioned.

Proposal 4B: Update R2-2101478 to take the comments into account.

# 6	Data Forwarding

A CR on user plane handling for full configuration in SN Change was submitted in R2-2101653. It is the rapporteur’s understanding that this CR was first discussed in RAN3, where it was felt that it would be more appropriate to handle it in RAN2. The CR argues that “it is unclear how to handle the forwarded data in case of full configuration in SN change and duplicated data received at the UE should be avoided.”

Question 5A: Do you agree with the intention of the CR?

| Answers to Question 5A |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated |  | We would like to understand why the use of full configuration would affect retransmissions for loss-less data delivery in the target NG-RAN node. This also should be captured in the reason for change. |
| Huawei, HiSilicon |  | We understand this is a similar issue as described in Sec 3. So better conclude Sec 3 first and then apply the same principle for this one. |
| Ericsson |  | We have basically the same comment as in Q2.A as the change seems to be the same but for LTE. |
| Apple | Yes | We think this follows the same logic of HO case with full configuration. |
| Nokia | Yes |  |
| vivo |  | We also wonder why PDCP SDU cannot be retransmitted in target SN if RLC-AM and FULL configuration are used? |
| Google | Yes | Proponent. As the target Secondary node has decided to perform full configuration, the PDCP SN is reset. The intention is to avoid wasting radio resources for retransmitting the PDCP SDU with SN which has been transmitted. Also with the proposed change, the possibly duplicated data received at the UE can be avoided. |
| CATT | Yes |  |
| LG | Yes |  |
| Intel | Yes | Proponent |
| ZTE |  | This can be discussed together with NR CR. |
| Samsung | Yes | One of co-sourcing company |

Summary 5A: same views were expressed as for the NR counterpart.

Proposal 5A: Clarify the problem on the cover sheet.

Question 5B: If you agree with the intention, are you happy with the wording or would you like to enhance it?

| Answers to Question 5B |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments & Possible Changes |
| Apple | Yes |  |
| Nokia | Yes |  |
| Google | Yes |  |
| CATT | Yes |  |
| LG | Yes |  |
| Intel | Yes |  |
| Samsung | Yes |  |

Summary 5B: All companies having expressed an opinion are fine with the actual.

Proposal 5B: clarify the intention and update the cover sheet of R2-2101653 to see if it can be agreed.

# 3	Conclusion

The proposals are:

Proposal 1: CRs R2-2100270 & R2-2100271 are agreed.

Proposal 2B: clarify the intention and update the cover sheet of R2-2101345 to see if it can be agreed.

Proposal 3B: CRs R2-2100091 & R2-2100092 are agreed.

Proposal 4B: Update R2-2101478 to take the comments into account.

Proposal 5B: clarify the intention and update the cover sheet of R2-2101653 to see if it can be agreed.

And what is suggested to be captured in the minutes:

## 5.2	Stage 2 corrections

You should discuss your stage 2 CRs with the specification rapporteurs before submission.

### 5.2.1	TS 3x.300

Agreed in-principle

R2-2100270	UE Capabilities Description	Nokia (Rapporteur), Ericsson, Nokia Shanghai Bell, Qualcomm Incorporated, Sanechips, ZTE	CR	Rel-15	38.300	15.11.0	0301	2	F	NR_newRAT-Core	R2-2011034

R2-2100271	UE Capabilities Description	Nokia (Rapporteur), Ericsson, Nokia Shanghai Bell, Qualcomm Incorporated, Sanechips, ZTE	CR	Rel-16	38.300	16.4.0	0302	2	A	NR_newRAT-Core	R2-2011035

R2-2100270 & R2-2100271 are agreed.

Other

R2-2101345	Clarification of data forwarding upon intra-system HO using full configuration	Samsung, Intel Corporation, China Telecom, LGU+, Google Inc., CATT, Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility	CR	Rel-16	38.300	16.4.0	0339	-	F	NR_newRAT-Core	R3-207066

Update the cover sheet to clarify the issue.

### 5.2.2	TS 37.340

PDCP Change indication

R2-2100091	Correction on the PDCP Change Indication for 37.340	CATT	CR	Rel-15	37.340	15.11.0	0243	-	F	NR_newRAT-Core

R2-2100092	Correction on the PDCP Change Indication for 37.340	CATT	CR	Rel-16	37.340	16.4.0	0244	-	A	NR_newRAT-Core

R2-2100091 & R2-2100092 are agreed.

Power Sharing

R2-2101478	Corrections on UL power sharing	Huawei, HiSilicon, ZTE Corpoation (rapporteur)	CR	Rel-15	37.340	15.11.0	0247	-	F	NR_newRAT-Core

Update the CR to take the comments into account.

Data forwarding

R2-2101653	Correction on user plane handling for full configuration in SN Change 	Google Inc., Samsung, Nokia, Nokia Shanghai Bell, CATT, Lenovo, Motorola Mobility, Intel Corporation	CR	Rel-16	37.340	16.4.0	0249	-	F	NR_newRAT-Core

Update the cover sheet to clarify the issue.

# Annex – Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Benoist Sébire | benoist.sebire@nokia.com |
| Huawei, HiSilicon | Yang Zhao | zhaoyang@huawei.com |
| Ericsson (Tony) | Antonino Orsino | antonino.orsino@ericsson.com |
| vivo | Xiaodong Yang | Yangxiaodong5g@vivo.com |
| Google | Jing-Rong Hsieh | jinghsieh@google.com |
| CATT | Jing Liang | liangjing@catt.cn |
| Intel | Sudeep Palat | Sudeep.k.palat@intel.com |
| ZTE | Huang He | Huang.he4@zte.com.cn |