3GPP TSG-RAN WG2 Meeting #113-e 	R2-2102321

# online, online, 25 January 2020 – 5 February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1003 | rev | 1 | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. The term 'MAC header' can be updated to 'MAC subheader' for the consistency. Note that the changes were already implemented to Rel-16 version, but missed for Rel-15. Also, the term 'MAC header' was previously used to consider the full header since there might be multiple subheaders. So should be 'MAC subheader(s)' 2. 'a DRX cycle is configured' can be updated to 'DRX is configured' as for other instances in subclause 5.7 for the consistency. 3. Incorrect acronym: SRS should be 'Sounding Reference Signal', not 'Sounding Reference Symbols'. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR does not contain any functional change: 1. The term 'MAC header' is updated to 'MAC subheader' in subclauses 5.1.2 and 6.1.3.1. 2. In subclause 5.7, 'a DRX cycle is configured' is changed to 'DRX is configured'. 3. In subclause 3.2, acronym for SRS is corrected. Impact analysis Architecture options (NG)EN-DC, NR SA, NE-DC, and NR-DC Impacted functionality: None (merely to improve the text for the consistency) Inter-operability: If only the network is implemented according to the CR, no interoperability problems are foreseen. If only the UE is implemented according to the CR, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains with inconsistency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1.2, 5.7, and 6.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - |  |  |  |  |  |  |  |  |

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CSI	Channel State Information

CSI-IM	CSI Interference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

INT-RNTI	Interruption RNTI

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

MCG	Master Cell Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PHR	Power Headroom Report

PTAG	Primary Timing Advance Group

QCL	Quasi-colocation

RS	Reference Signal

SCG	Secondary Cell Group

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Signal-RNTI

UCI	Uplink Control Information

ZP CSI-RS	Zero Power CSI-RS

Next change

### 5.1.2	Random Access Resource selection

The MAC entity shall:

1>	if the Random Access procedure was initiated for beam failure recovery (as specified in clause 5.17); and

1>	if the beamFailureRecoveryTimer (in clause 5.17) is either running or not configured; and

1>	if the contention-free Random Access Resources for beam failure recovery request associated with any of the SSBs and/or CSI-RSs have been explicitly provided by RRC; and

1>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or the CSI-RSs with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the CSI-RSs in candidateBeamRSList;

2>	if CSI-RS is selected, and there is no ra-PreambleIndex associated with the selected CSI-RS:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7].

2>	else:

3>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB or CSI-RS from the set of Random Access Preambles for beam failure recovery request.

1>	else if the ra-PreambleIndex has been explicitly provided by PDCCH; and

1>	if the ra-PreambleIndex is not 0b000000:

2>	set the PREAMBLE_INDEX to the signalled ra-PreambleIndex;

2>	select the SSB signalled by PDCCH.

1>	else if the contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available:

2>	select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB.

1>	else if the contention-free Random Access Resources associated with CSI-RSs have been explicitly provided in rach-ConfigDedicated and at least one CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs is available:

2>	select a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS amongst the associated CSI-RSs;

2>	set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected CSI-RS.

1>	else if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if the Random Access Resources for SI request have been explicitly provided by RRC:

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	select a Random Access Preamble corresponding to the selected SSB, from the Random Access Preamble(s) determined according to ra-PreambleStartIndex as specified in TS 38.331 [5];

2>	set the PREAMBLE_INDEX to selected Random Access Preamble.

1>	else (i.e. for the contention-based Random Access preamble selection):

2>	if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available:

3>	select an SSB with SS-RSRP above rsrp-ThresholdSSB.

2>	else:

3>	select any SSB.

2>	if Msg3 has not yet been transmitted:

3>	if Random Access Preambles group B is configured:

4>	if the potential Msg3 size (UL data available for transmission plus MAC subheader(s) and, where required, MAC CEs) is greater than ra-Msg3SizeGroupA and the pathloss is less than PCMAX (of the Serving Cell performing the Random Access Procedure) – preambleReceivedTargetPower – msg3-DeltaPreamble – messagePowerOffsetGroupB; or

4>	if the Random Access procedure was initiated for the CCCH logical channel and the CCCH SDU size plus MAC subheader is greater than ra-Msg3SizeGroupA:

5>	select the Random Access Preambles group B.

4>	else:

5>	select the Random Access Preambles group A.

3>	else:

4>	select the Random Access Preambles group A.

2>	else (i.e. Msg3 is being retransmitted):

3>	select the same group of Random Access Preambles as was used for the Random Access Preamble transmission attempt corresponding to the first transmission of Msg3.

2>	select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB and the selected Random Access Preambles group.

2>	set the PREAMBLE_INDEX to the selected Random Access Preamble.

1>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]); and

1>	if ra-AssociationPeriodIndex and si-RequestPeriod are configured:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB in the association period given by ra-AssociationPeriodIndex in the si-RequestPeriod permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] corresponding to the selected SSB).

1>	else if an SSB is selected above:

2>	determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured or indicated by PDCCH (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected SSB).

1>	else if a CSI-RS is selected above:

2>	if there is no contention-free Random Access Resource associated with the selected CSI-RS:

3>	determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6], corresponding to the SSB which is quasi-colocated with the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-colocated with the selected CSI-RS).

2>	else:

3>	determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS).

1>	perform the Random Access Preamble transmission procedure (see clause 5.1.3).

NOTE:	When the UE determines if there is an SSB with SS-RSRP above rsrp-ThresholdSSB or a CSI-RS with CSI-RSRP above rsrp-ThresholdCSI-RS, the UE uses the latest unfiltered L1-RSRP measurement.

Next change

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in clause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

1>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

2>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH.

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if drx-onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not report CSI on PUCCH.

NOTE:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource outside DRX Active Time, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

Next change

#### 6.1.3.1	Buffer Status Report MAC CEs

Buffer Status Report (BSR) MAC CEs consist of either:

-	Short BSR format (fixed size); or

-	Long BSR format (variable size); or

-	Short Truncated BSR format (fixed size); or

-	Long Truncated BSR format (variable size).

The BSR formats are identified by MAC subheaders with LCIDs as specified in Table 6.2.1-2.

The fields in the BSR MAC CE are defined as follows:

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose buffer status is being reported. The length of the field is 3 bits;

-	LCGi: For the Long BSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to 1 indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to 0 indicates that the Buffer Size field for the logical channel group i is not reported. For the Long Truncated BSR format, this field indicates whether logical channel group i has data available. The LCGi field set to 1 indicates that logical channel group i has data available. The LCGi field set to 0 indicates that logical channel group i does not have data available;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC headers and MAC subheaders are not considered in the buffer size computation. The length of this field for the Short BSR format and the Short Truncated BSR format is 5 bits. The length of this field for the Long BSR format and the Long Truncated BSR format is 8 bits. The values for the 5-bit and 8-bit Buffer Size fields are shown in Tables 6.1.3.1-1 and 6.1.3.1-2, respectively. For the Long BSR format and the Long Truncated BSR format, the Buffer Size fields are included in ascending order based on the LCGi. For the Long Truncated BSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

NOTE:	The number of the Buffer Size fields in the Long BSR and Long Truncated BSR format can be zero.

Figure 6.1.3.1-1: Short BSR and Short Truncated BSR MAC CE

Figure 6.1.3.1-2: Long BSR and Long Truncated BSR MAC CE

Table 6.1.3.1-1: Buffer size levels (in bytes) for 5-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 8 | ≤ 102 | 16 | ≤ 1446 | 24 | ≤ 20516 |
| 1 | ≤ 10 | 9 | ≤ 142 | 17 | ≤ 2014 | 25 | ≤ 28581 |
| 2 | ≤ 14 | 10 | ≤ 198 | 18 | ≤ 2806 | 26 | ≤ 39818 |
| 3 | ≤ 20 | 11 | ≤ 276 | 19 | ≤ 3909 | 27 | ≤ 55474 |
| 4 | ≤ 28 | 12 | ≤ 384 | 20 | ≤ 5446 | 28 | ≤ 77284 |
| 5 | ≤ 38 | 13 | ≤ 535 | 21 | ≤ 7587 | 29 | ≤ 107669 |
| 6 | ≤ 53 | 14 | ≤ 745 | 22 | ≤ 10570 | 30 | ≤ 150000 |
| 7 | ≤ 74 | 15 | ≤ 1038 | 23 | ≤ 14726 | 31 | > 150000 |

Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field

| Index | BS value | Index | BS value | Index | BS value | Index | BS value |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 64 | ≤ 560 | 128 | ≤ 31342 | 192 | ≤ 1754595 |
| 1 | ≤ 10 | 65 | ≤ 597 | 129 | ≤ 33376 | 193 | ≤ 1868488 |
| 2 | ≤ 11 | 66 | ≤ 635 | 130 | ≤ 35543 | 194 | ≤ 1989774 |
| 3 | ≤ 12 | 67 | ≤ 677 | 131 | ≤ 37850 | 195 | ≤ 2118933 |
| 4 | ≤ 13 | 68 | ≤ 720 | 132 | ≤ 40307 | 196 | ≤ 2256475 |
| 5 | ≤ 14 | 69 | ≤ 767 | 133 | ≤ 42923 | 197 | ≤ 2402946 |
| 6 | ≤ 15 | 70 | ≤ 817 | 134 | ≤ 45709 | 198 | ≤ 2558924 |
| 7 | ≤ 16 | 71 | ≤ 870 | 135 | ≤ 48676 | 199 | ≤ 2725027 |
| 8 | ≤ 17 | 72 | ≤ 926 | 136 | ≤ 51836 | 200 | ≤ 2901912 |
| 9 | ≤ 18 | 73 | ≤ 987 | 137 | ≤ 55200 | 201 | ≤ 3090279 |
| 10 | ≤ 19 | 74 | ≤ 1051 | 138 | ≤ 58784 | 202 | ≤ 3290873 |
| 11 | ≤ 20 | 75 | ≤ 1119 | 139 | ≤ 62599 | 203 | ≤ 3504487 |
| 12 | ≤ 22 | 76 | ≤ 1191 | 140 | ≤ 66663 | 204 | ≤ 3731968 |
| 13 | ≤ 23 | 77 | ≤ 1269 | 141 | ≤ 70990 | 205 | ≤ 3974215 |
| 14 | ≤ 25 | 78 | ≤ 1351 | 142 | ≤ 75598 | 206 | ≤ 4232186 |
| 15 | ≤ 26 | 79 | ≤ 1439 | 143 | ≤ 80505 | 207 | ≤ 4506902 |
| 16 | ≤ 28 | 80 | ≤ 1532 | 144 | ≤ 85730 | 208 | ≤ 4799451 |
| 17 | ≤ 30 | 81 | ≤ 1631 | 145 | ≤ 91295 | 209 | ≤ 5110989 |
| 18 | ≤ 32 | 82 | ≤ 1737 | 146 | ≤ 97221 | 210 | ≤ 5442750 |
| 19 | ≤ 34 | 83 | ≤ 1850 | 147 | ≤ 103532 | 211 | ≤ 5796046 |
| 20 | ≤ 36 | 84 | ≤ 1970 | 148 | ≤ 110252 | 212 | ≤ 6172275 |
| 21 | ≤ 38 | 85 | ≤ 2098 | 149 | ≤ 117409 | 213 | ≤ 6572925 |
| 22 | ≤ 40 | 86 | ≤ 2234 | 150 | ≤ 125030 | 214 | ≤ 6999582 |
| 23 | ≤ 43 | 87 | ≤ 2379 | 151 | ≤ 133146 | 215 | ≤ 7453933 |
| 24 | ≤ 46 | 88 | ≤ 2533 | 152 | ≤ 141789 | 216 | ≤ 7937777 |
| 25 | ≤ 49 | 89 | ≤ 2698 | 153 | ≤ 150992 | 217 | ≤ 8453028 |
| 26 | ≤ 52 | 90 | ≤ 2873 | 154 | ≤ 160793 | 218 | ≤ 9001725 |
| 27 | ≤ 55 | 91 | ≤ 3059 | 155 | ≤ 171231 | 219 | ≤ 9586039 |
| 28 | ≤ 59 | 92 | ≤ 3258 | 156 | ≤ 182345 | 220 | ≤ 10208280 |
| 29 | ≤ 62 | 93 | ≤ 3469 | 157 | ≤ 194182 | 221 | ≤ 10870913 |
| 30 | ≤ 66 | 94 | ≤ 3694 | 158 | ≤ 206786 | 222 | ≤ 11576557 |
| 31 | ≤ 71 | 95 | ≤ 3934 | 159 | ≤ 220209 | 223 | ≤ 12328006 |
| 32 | ≤ 75 | 96 | ≤ 4189 | 160 | ≤ 234503 | 224 | ≤ 13128233 |
| 33 | ≤ 80 | 97 | ≤ 4461 | 161 | ≤ 249725 | 225 | ≤ 13980403 |
| 34 | ≤ 85 | 98 | ≤ 4751 | 162 | ≤ 265935 | 226 | ≤ 14887889 |
| 35 | ≤ 91 | 99 | ≤ 5059 | 163 | ≤ 283197 | 227 | ≤ 15854280 |
| 36 | ≤ 97 | 100 | ≤ 5387 | 164 | ≤ 301579 | 228 | ≤ 16883401 |
| 37 | ≤ 103 | 101 | ≤ 5737 | 165 | ≤ 321155 | 229 | ≤ 17979324 |
| 38 | ≤ 110 | 102 | ≤ 6109 | 166 | ≤ 342002 | 230 | ≤ 19146385 |
| 39 | ≤ 117 | 103 | ≤ 6506 | 167 | ≤ 364202 | 231 | ≤ 20389201 |
| 40 | ≤ 124 | 104 | ≤ 6928 | 168 | ≤ 387842 | 232 | ≤ 21712690 |
| 41 | ≤ 132 | 105 | ≤ 7378 | 169 | ≤ 413018 | 233 | ≤ 23122088 |
| 42 | ≤ 141 | 106 | ≤ 7857 | 170 | ≤ 439827 | 234 | ≤ 24622972 |
| 43 | ≤ 150 | 107 | ≤ 8367 | 171 | ≤ 468377 | 235 | ≤ 26221280 |
| 44 | ≤ 160 | 108 | ≤ 8910 | 172 | ≤ 498780 | 236 | ≤ 27923336 |
| 45 | ≤ 170 | 109 | ≤ 9488 | 173 | ≤ 531156 | 237 | ≤ 29735875 |
| 46 | ≤ 181 | 110 | ≤ 10104 | 174 | ≤ 565634 | 238 | ≤ 31666069 |
| 47 | ≤ 193 | 111 | ≤ 10760 | 175 | ≤ 602350 | 239 | ≤ 33721553 |
| 48 | ≤ 205 | 112 | ≤ 11458 | 176 | ≤ 641449 | 240 | ≤ 35910462 |
| 49 | ≤ 218 | 113 | ≤ 12202 | 177 | ≤ 683087 | 241 | ≤ 38241455 |
| 50 | ≤ 233 | 114 | ≤ 12994 | 178 | ≤ 727427 | 242 | ≤ 40723756 |
| 51 | ≤ 248 | 115 | ≤ 13838 | 179 | ≤ 774645 | 243 | ≤ 43367187 |
| 52 | ≤ 264 | 116 | ≤ 14736 | 180 | ≤ 824928 | 244 | ≤ 46182206 |
| 53 | ≤ 281 | 117 | ≤ 15692 | 181 | ≤ 878475 | 245 | ≤ 49179951 |
| 54 | ≤ 299 | 118 | ≤ 16711 | 182 | ≤ 935498 | 246 | ≤ 52372284 |
| 55 | ≤ 318 | 119 | ≤ 17795 | 183 | ≤ 996222 | 247 | ≤ 55771835 |
| 56 | ≤ 339 | 120 | ≤ 18951 | 184 | ≤ 1060888 | 248 | ≤ 59392055 |
| 57 | ≤ 361 | 121 | ≤ 20181 | 185 | ≤ 1129752 | 249 | ≤ 63247269 |
| 58 | ≤ 384 | 122 | ≤ 21491 | 186 | ≤ 1203085 | 250 | ≤ 67352729 |
| 59 | ≤ 409 | 123 | ≤ 22885 | 187 | ≤ 1281179 | 251 | ≤ 71724679 |
| 60 | ≤ 436 | 124 | ≤ 24371 | 188 | ≤ 1364342 | 252 | ≤ 76380419 |
| 61 | ≤ 464 | 125 | ≤ 25953 | 189 | ≤ 1452903 | 253 | ≤ 81338368 |
| 62 | ≤ 494 | 126 | ≤ 27638 | 190 | ≤ 1547213 | 254 | > 81338368 |
| 63 | ≤ 526 | 127 | ≤ 29431 | 191 | ≤ 1647644 | 255 | Reserved |