3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2102448

# E-Meeting, 25th Jan – 5th Feb 2021                                               Revision of R2-2102406

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0346 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DAPS HO configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-2-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE behavior in case the network configures DAPS and Sidelink together is undefined and it may lead to the system to not work properly. Therefore, there should be restriction on co-configuration of DAPS and Sidelink in this release, as discussed in RAN2#113-E [AT113-e][015]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in section 9.2.3.1 that DAPS HO is not configured when UE is configured with NR sidelink communication or V2X sidelink communication. Impact analysis Impacted 5G architecture options: NR SA Impacted functionality: DAPS, NR SL, V2X Inter-operability: - If UE implements according to the CR and the network does not, the UE behavior is undefined when configured with DAPS and Sidelink at the same time. - If the network implements according to the CR and the UE does not, the is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is not clear whether DAPS HO can be configured when UE is configured with NR sidelink communication or V2X sidelink communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ### 9.2.3.1 Overview |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------[Start of Change] ------------------------------------------------

# 9	Mobility and State Transitions

## 9.2	Intra-NR

### 9.2.3	Mobility in RRC_CONNECTED

#### 9.2.3.1	Overview

<Text omitted..>

In case of DAPS handover, the UE continues the downlink user data reception from the source gNB until releasing the source cell and continues the uplink user data transmission to the source gNB until successful random access procedure to the target gNB.

Only PCell is kept during DAPS handover. All other serving cells, multi-DCI/single-DCI based multi-TRP, NR sidelink configurations and V2X sidelink configurations are released by the network before the handover command is sent to the UE.

--------------------------------------------------[End of Change] ------------------------------------------------