3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2102464

# Elbonia, 25 January – 05 February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2341 | rev | 1 | Current version: | 16.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RA report and Logged MDT Info extendibility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The IE “LogMeasInfo-r16” is not extendable in Rel-16 for MDT. This will cause problem to include new measurements in the logged measurement report. The IE “RA-Report-r16” is not extendible in the Rel-16 for RA report. This will cause problem to include new measurements in the RA report in the future. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ellipis “…” is added to the SEQUENCE of the LogMeasInfo-r16 IE. Ellipis “…” is added to the SEQUENCE of the RA-Report-r16 IE and to the SEQUENCE of the RA-InformationCommon-r16 IE. The field ra-InformationCommon-r16 is made OPTIONAL. This CR is considered mandatory to support the impacted functionality. Impact analysis Impacted functionality: Logged MDT, RA report. Inter-operability: - If the network is implemented according to the CR and the UE is not, the network may ignore the Logged MDT Report and RA report message, as not valid according to an expected (different) abstrax syntax version. - If the UE is implemented according to the CR and the network is not, the Logged MDT Report and the RA report message can’t be comprehended, as the network may treat it as not valid according to an expected (different) abstrax syntax version. Logged MDT and RA report can’t be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to add in a simple way new measurements to Logged MDT report and the RA report. Considering early stage of Rel-16, any future realleases extensions of logged information and RA reort information will require special handling and extra effort to get them implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev -0: R2-2100608 Rev-1: Changes related to RAReport as agreed in the RAN2#113 meeting online session are included. |  |  |  |  |  |  |  |  |

First Modified Subclause

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...

}

ConnEstFailReport-r16 ::=            SEQUENCE {

measResultFailedCell-r16             MeasResultFailedCell-r16,

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultList2NR-r16               OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

numberOfConnFail-r16                 INTEGER (1..8),

perRAInfoList-r16                            PerRAInfoList-r16,

timeSinceFailure-r16                 TimeSinceFailure-r16,

...

}

MeasResultServingCell-r16 ::=        SEQUENCE {

resultsSSB-Cell                      MeasQuantityResults,

resultsSSB                           SEQUENCE{

best-ssb-Index                       SSB-Index,

best-ssb-Results                     MeasQuantityResults,

numberOfGoodSSB                      INTEGER (1..maxNrofSSBs-r16)

}                                                                        OPTIONAL

}

MeasResultFailedCell-r16 ::=         SEQUENCE {

cgi-Info                             CGI-Info-Logging-r16,

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList

}

}

}

RA-ReportList-r16 ::= SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16

RA-Report-r16 ::=                    SEQUENCE {

cellId-r16                           CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        SEQUENCE {

physCellId-r16                       PhysCellId,

carrierFreq-r16                      ARFCN-ValueNR

}

},

ra-InformationCommon-r16             RA-InformationCommon-r16				OPTIONAL,

raPurpose-r16                        ENUMERATED {accessRelated, beamFailureRecovery, reconfigurationWithSync, ulUnSynchronized,

schedulingRequestFailure, noPUCCHResourceAvailable, requestForOtherSI,

spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1},

...

}

RA-InformationCommon-r16 ::=         SEQUENCE {

absoluteFrequencyPointA-r16          ARFCN-ValueNR,

locationAndBandwidth-r16             INTEGER (0..37949),

subcarrierSpacing-r16                SubcarrierSpacing,

msg1-FrequencyStart-r16              INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-FrequencyStartCFRA-r16          INTEGER (0..maxNrofPhysicalResourceBlocks-1)     OPTIONAL,

msg1-SubcarrierSpacing-r16           SubcarrierSpacing                                OPTIONAL,

msg1-SubcarrierSpacingCFRA-r16       SubcarrierSpacing                                OPTIONAL,

msg1-FDM-r16                         ENUMERATED {one, two, four, eight}               OPTIONAL,

msg1-FDMCFRA-r16                     ENUMERATED {one, two, four, eight}               OPTIONAL,

perRAInfoList-r16                    PerRAInfoList-r16,

...

}

PerRAInfoList-r16 ::= SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16

PerRAInfo-r16 ::=                    CHOICE {

perRASSBInfoList-r16                 PerRASSBInfo-r16,

perRACSI-RSInfoList-r16              PerRACSI-RSInfo-r16

}

PerRASSBInfo-r16 ::=                 SEQUENCE {

ssb-Index-r16                        SSB-Index,

numberOfPreamblesSentOnSSB-r16       INTEGER (1..200),

perRAAttemptInfoList-r16             PerRAAttemptInfoList-r16

}

PerRACSI-RSInfo-r16 ::=              SEQUENCE {

csi-RS-Index-r16                     CSI-RS-Index,

numberOfPreamblesSentOnCSI-RS-r16    INTEGER (1..200)

}

PerRAAttemptInfoList-r16 ::=         SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16

PerRAAttemptInfo-r16 ::=             SEQUENCE {

contentionDetected-r16               BOOLEAN                OPTIONAL,

dlRSRPAboveThreshold-r16             BOOLEAN                OPTIONAL,

...

}

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        SEQUENCE {

physCellId-r16                       PhysCellId,

carrierFreq-r16                      ARFCN-ValueNR

}

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    SEQUENCE {

physCellId-r16                   EUTRA-PhysCellId,

carrierFreq-r16                  ARFCN-ValueEUTRA

}

}

},

reconnectCellId-r16                  CHOICE {

nrReconnectCellId-r16                CGI-Info-Logging-r16,

eutraReconnectCellId-r16             CGI-InfoEUTRALogging

}                                                                                        OPTIONAL,

timeUntilReconnection-16             TimeUntilReconnection-16                            OPTIONAL,

reestablishmentCellId-r16            CGI-Info-Logging-r16                                OPTIONAL,

timeConnFailure-r16                  INTEGER (0..1023)                                   OPTIONAL,

timeSinceFailure-r16                 TimeSinceFailure-r16,

connectionFailureType-r16            ENUMERATED {rlf, hof},

rlf-Cause-r16                        ENUMERATED {t310-Expiry, randomAccessProblem, rlc-MaxNumRetx,

beamFailureRecoveryFailure, lbtFailure-r16,

bh-rlfRecoveryFailure, spare2, spare1},

locationInfo-r16                     LocationInfo-r16                                    OPTIONAL,

noSuitableCellFound-r16              ENUMERATED {true}                                   OPTIONAL,

ra-InformationCommon-r16             RA-InformationCommon-r16                            OPTIONAL,

...

},

eutra-RLF-Report-r16                 SEQUENCE {

failedPCellId-EUTRA                  CGI-InfoEUTRALogging,

measResult-RLF-Report-EUTRA-r16      OCTET STRING,

...

}

}

MeasResultList2NR-r16 ::=            SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2NR-r16

MeasResultList2EUTRA-r16 ::=         SEQUENCE(SIZE (1..maxFreq)) OF MeasResult2EUTRA-r16

MeasResult2NR-r16 ::=                SEQUENCE {

ssbFrequency-r16                     ARFCN-ValueNR                                           OPTIONAL,

refFreqCSI-RS-r16                    ARFCN-ValueNR                                           OPTIONAL,

measResultList-r16                   MeasResultListNR

}

MeasResultListLogging2NR-r16 ::=     SEQUENCE(SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16

MeasResultLogging2NR-r16 ::=         SEQUENCE {

carrierFreq-r16                      ARFCN-ValueNR,

measResultListLoggingNR-r16          MeasResultListLoggingNR-r16

}

MeasResultListLoggingNR-r16 ::=      SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16

MeasResultLoggingNR-r16 ::=          SEQUENCE {

physCellId-r16                       PhysCellId,

resultsSSB-Cell-r16                  MeasQuantityResults,

numberOfGoodSSB-r16                  INTEGER (1..maxNrofSSBs-r16) OPTIONAL

}

MeasResult2EUTRA-r16 ::=             SEQUENCE {

carrierFreq-r16                      ARFCN-ValueEUTRA,

measResultList-r16                   MeasResultListEUTRA

}

MeasResultRLFNR-r16 ::=              SEQUENCE {

measResult-r16                       SEQUENCE {

cellResults-r16                      SEQUENCE{

resultsSSB-Cell-r16                  MeasQuantityResults                             OPTIONAL,

resultsCSI-RS-Cell-r16               MeasQuantityResults                             OPTIONAL

},

rsIndexResults-r16                   SEQUENCE{

resultsSSB-Indexes-r16               ResultsPerSSB-IndexList                         OPTIONAL,

ssbRLMConfigBitmap-r16               BIT STRING (SIZE (64))                          OPTIONAL,

resultsCSI-RS-Indexes-r16            ResultsPerCSI-RS-IndexList                      OPTIONAL,

csi-rsRLMConfigBitmap-r16            BIT STRING (SIZE (96))                          OPTIONAL

}                                                                                    OPTIONAL

}

}

TimeSinceFailure-r16 ::= INTEGER (0..172800)

MobilityHistoryReport-r16 ::= VisitedCellInfoList-r16

TimeUntilReconnection-16 ::= INTEGER (0..172800)

-- TAG-UEINFORMATIONRESPONSE-STOP

-- ASN1STOP

UEInformationResponse-IEs field descriptions

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-Report This field is used to provide the list of RA reports that is stored by the UE for the past upto maxRAReport-r16 number of successful random access procedues.

rlf-Report This field is used to indicate the RLF report related contents.

LogMeasReport field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by NR within absoluteTimeInfo.

anyCellSelectionDetected This field is used to indicate the detection of any cell selection state, as defined in TS 38.304 [20]. The UE sets this field when performing the logging of measurement results in RRC_IDLE or RRC_INACTIVE and there is no suitable cell or no acceptable cell.

measResultServingCell This field refers to the log measurement results taken in the Serving cell.

numberOfGoodSSB Indicates the number of good beams (beams that are above absThreshSS-BlocksConsolidation, if configured by the network) associated to the cells within the R value range (which is configured by network for cell reselection) of the highest ranked cell as part of the beam level measurements. If the UE has no SSB of a neighbour cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE does not include numberOfGoodSSB for the corresponding neighbour cell. If the UE has no SSB of the serving cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE shall set the numberOfGoodSSB for the serving cell to one.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

RA-Report field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the raPurpose is set to requestForOtherSI.

csi-RS-Index This field is used to indicate the CSI-RS index corresponding to the random access attempt.

dlRSRPAboveThreshold This field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold (rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery; Otherwise, rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure.

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SS/PBCH block.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRAInfoList This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts.

perRACSI-RSInfoList This field provides detailed information about the successive random access attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, transition from RRC-INACTIVE and the MSG3 based SI request are indicated using the indicator 'accessRelated'. The indicator beamFailureRecovery is used in case of beam failure recovery failure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request.

ra-InformationCommon This field is used to indicate the common random-access related information between RA-report and RLF-report. For RA report, this field is mandatory presented. For RLF-report, this field is optionally included when connectionFailureType is set to 'hof' or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

subcarrierSpacing Subcarrier spacing used in the BWP associated to the random-access resources used by the UE.

RLF-Report field descriptions

connectionFailureType This field is used to indicate whether the connection failure is due to radio link failure or handover failure.

csi-rsRLMConfigBitmap This field is used to indicate the CSI-RS indexes that are also part of the RLM configurations.

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

failedPCellId This field is used to indicate the PCell in which RLF is detected or the target PCell of the failed handover. For intra-NR handover nrFailedPCellId is included and for the handover from NR to EUTRA eutraFailedPCellId is included. The UE sets the ARFCN according to the frequency band used for transmission/ reception when the failure occurred.

failedPCellId-EUTRA This field is used to indicate the PCell in which RLF is detected or the source PCell of the failed handover in an E-UTRA RLF report.

measResultListEUTRA This field refers to the last measurement results taken in the neighboring EUTRA Cells, when the radio link failure or handover failure happened.

measResultListNR This field refers to the last measurement results taken in the neighboring NR Cells, when the radio link failure or handover failure happened.

measResultLastServCell This field refers to the log measurement results taken in the PCell upon detecting radio link failure or the source PCell upon handover failure.

measResult-RLF-Report-EUTRA Includes the E-UTRA RLF-Report-r9 IE as specified in TS 36.331 [10].

noSuitableCellFound This field is set by the UE when the T311 expires.

previousPCellId This field is used to indicate the source PCell of the last handover (source PCell when the last RRCReconfiguration message including reconfigurationWithSync was received). For intra-NR handover nrPreviousCell is included and for the handover from EUTRA to NR eutraPreviousCell is included.

reconnectCellId This field is used to indicate the cell in which the UE comes back to connected after connection failure and after failing to perform reestablishment. If the UE comes back to RRC CONNECTED in an NR cell then nrReconnectCellID is included and if the UE comes back to RRC CONNECTED in an LTE cell then eutraReconnectCellID is included

reestablishmentCellId This field is used to indicate the cell in which the re-establishment attempt was made after connection failure.

rlf-Cause This field is used to indicate the cause of the last radio link failure that was detected. In case of handover failure information reporting (i.e., the connectionFailureType is set to 'hof'), the UE is allowed to set this field to any value.

ssbRLMConfigBitmap This field is used to indicate the SS/PBCH block indexes that are also part of the RLM configurations.

timeConnFailure This field is used to indicate the time elapsed since the last HO initialization until connection failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (radio link or handover) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

timeUntilReconnection This field is used to indicate the time that elapsed between the connection (radio link or handover) failure and the next time the UE comes to RRC CONNECTED in an NR or EUTRA cell. Value in seconds. The maximum value 172800 means 172800s or longer.

End of Modified Subclause