3GPP TSG-RAN WG2 Meeting #113e	R2-2102476

# Electronic Meeting, 25th Jan – 5th Feb 2021

Title:	LS Reply on RRC based BWP switch

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Naveen Palle

E-mail Address:	naveen dot palle at apple dot com

Attachments:

- Overall Description:

RAN2 thanks RAN4 for their enquiry on the topic of RRC based BWP switching and would like to provide the below responses to the questions asked.

- On whether RRC reconfiguration can change any parameter of the already active BWP of an activated SCell or SpCell.

Response: According to the RAN2 specification the NW can change any parameters of an already active BWP of an SPCell or an SCell with the exception that the modification of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id  for an SCell is not allowed (for both Rel-15 and Rel-16).  The modification of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id  for an SCell is allowed by first releasing and subsequently adding the SCell, and this is not considered by RAN2 as a BWP switch.

- On whether the RRC reconfiguration without modification of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id for an activated SCell or SpCell can trigger a BWP switch.

Response: RAN2 confirms that an RRC message with a firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id that is different from the UE’s current BWP ID, results in a BWP switch for an SpCell. And in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible.

According to RAN2 specifications, the reconfiguration of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id as mentioned earlier) of an already active BWP of an SPCell or an Scell is allowed for  both Rel-15 and Rel-16. However, RAN2 has not specified whether this is a BWP switch or not, i.e. it would require further discussion on whether particular parameter change would cause BWP switch or not and RAN2 does not intend to discuss this.

Furthermore, RAN2 would like to inform RAN4 that during the RRCResume/RRCSetup procedure for the PCell, the active BWP parameters change for the UE or the BWP can be switched.

2. To RAN WG2 group.

ACTION: RAN2 requests RAN4 to take the above responses into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG2   Meeting #113-bis-e                   Apr 12 – 20 , 2021	         Online

TSG RAN WG2   Meeting #114-e            	   May 19–  27 , 2021            Online