3GPP TSG RAN WG2 #113-e	draft R2-21xxxxx

Online, 25th Jan – 5th Feb, 2021

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [033][eNPN] IMS voice and emergency services for SNPN

Agenda Item:	8.16.4

Document for:	Discussion and decision

# Introduction

This document is a summary of the following offline discussion:

- [AT113-e][033][eNPN] IMS voice and emergency services for SNPN (Huawei)

Scope: Take into account documents submitted to this section, 1st pass: identify what is required to be supported by AS and determine the RAN2 impact, if possible. Identify common views / potential initial agreements, Identify points that need further discussion. Can also gather comments on the need to ask questions to other group.

Intended outcome: Report with agreeable proposals and discussion points (not too many, preferably < 6) for treatment on-line

Deadline: 1st Deadline for Comments: Friday Jan 29 1000 UTC. Other deadline if needed by rapporteur. Report Ready for treatment on-line Feb 3.

# Discussion

## Broadcasting of parameters

In Rel-16, the IMS voice and emergency services are not supported for SNPNs. In Rel-17, the support of IMS voice and emergency services for SNPN will be specified. SA2’s eNPN WI (S2-2009251) covers the following objective:

- Support of IMS voice and emergency services for SNPN (KI#3) - To enable support for IMS services as per conclusions in clause 8.3. - Use of IMC when USIM or ISIM is not available in UEs accessing IMS via an SNPN - Reuse of USIM credentials for IMS AKA shall be possible when USIM is available in UEs accessing IMS via an SNPN - Functionality for support of voice and emergency services with SNPN, excluding EPS fallback and T-ADS.

And the broadcasting of relevant parameters is in the scope of RAN2 according to the RAN WI (RP-202363):

- Support of IMS voice and emergency services for SNPN [RAN2] - Broadcasting of relevant parameters [RAN2]

To support IMS voice and emergency services in SNPN, the SNPN cell needs to broadcast IMS related parameters so that UEs in limited service state can camp on this cell. The eCallOverIMS-Support and ims-EmergencySupport fields in SIB1 indicate the support of IMS voice and emergency services in the cell, and therefore these two indicators should be extended to SNPN cells.

Q1: Do you agree to extend eCallOverIMS-Support and ims-EmergencySupport fields in SIB1 to SNPN cells?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

In RAN sharing scenarios, RAN2 needs to decide whether the eCallOverIMS-Support and ims-EmergencySupport in R17 should be per physical cell or per network.

Q2: If your answer to Q1 is Yes, do you think the eCallOverIMS-Support and ims-EmergencySupport fields in R17 should be Option A) physical cell specific or Option B) network-specific?

| Company | Option A/B | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option B | Different networks may not have the same support for IMS voice and emergency services |

## Impacts on acceptable cell selection

The current definition of acceptable cells in TS 38.304 is excerpted as follows:

acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2.

In R16, SNPN cells support neither emergency calls nor ETWS/CMAS notifications. In R17, emergency calls will be supported, but the support of ETWS/CMAS is not in the scope of SA2 WID or RAN WID.

Q3 : If emergency calls are supported whereas ETWS/CMAS notifications are not, can the SNPN cells be viewed as acceptable cell ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We think ETWS/CMAS also need to be supported in an acceptable cell. |

Q4 : Do you think RAN2 should send an LS to SA2/SA1 to ask for the support of ETWS/CMAS notifications for SNPN cells ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It’s better to clarify. |

The current definition of reserved cells in TS 38.304 is excerpted as follows :

reserved cell: A cell is reserved if it is so indicated in system information, as specified in TS 38.331 [3]. Following exception to these definitions are applicable for UEs: - if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call. - camped on a cell that belongs to a tracking area that is forbidden for regional provision of service; a cell that belongs to a tracking area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service. - if the UE in RRC_IDLE fulfils the conditions to support NR sidelink communication or V2X sidelink communication in limited service state as specified in TS23.287 [16] clause 5.7, the UE may perform NR sidelink communication or V2X sidelink communication.

Q5: If your answer to Q3 is Yes (SNPN cells supporting emergency calls can be viewed as acceptable cells), do you agree to extend the above highlighted texts to SNPN cells (i.e. For reserved cells specified in TS 38.304, all acceptable cells of an SNPN supporting emergency services are treated as suitable when the UE has an ongoing emergency call) ?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

## Availability of emergency services

If an SNPN cell supports emergency services, it is unclear which kinds of UEs are able to camp on this cell to obtain emergency service:

Case 1) UEs in SNPN Access Mode

Case 2) SNPN-capable UEs that are not in SNPN Access Mode

Case 3) Non-SNPN capable UEs

Similar discussions have been made in R16 for CAG cells that support emergency services.

Q6.a: For an R17 UE, please indicate your understanding of whether the UE is able to camp on an SNPN cell supporting emergency services to obtain emergency services.

| Company | Case 1 (Yes/No) | Case 2 (Yes/No) | Case 3 (Yes/No) | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSilicon | Yes | Yes | Yes | These questions may eventually depend on operators’ preferences. |

Q6.b: For an R16 UE, please indicate your understanding of whether the UE is able to camp on an SNPN cell supporting emergency services to obtain emergency services.

| Company | Case 1 (Yes/No) | Case 2 (Yes/No) | Case 3 (Yes/No) | Comments |
| --- | --- | --- | --- | --- |
| Huawei, HiSilicon | Yes | Yes | Yes |  |

Q6.c: For an R15 UE, please indicate your understanding of whether the UE is able to camp on an SNPN cell supporting emergency services to obtain emergency services.

| Company | Case 3 (Yes/No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

## Other issues

According to eNPN WI (S2-2009251), EPS fallback is not supported for SNPN. Therefore, the SNPN shall not indicate the EPS fallback indicators to UE in RRC release and Mobility from NR failure procedures.

Q7: Do you agree that “The voiceFallbackIndication field in RRCRelease and MobilityFromNRCommand is not applicable to SNPN cells”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Q8: If there’re other critical issues not mentioned in this discussion that need to be treated in this meeting, please elaborate it below (Please note that the scope of this offline should be limited and that the agreeable proposals and discussion points to be treated online should be preferably < 6 as per the Chairman’s guidance):

| Company | Issue | Comments |
| --- | --- | --- |

# Conclusion

To be added.

# Reference

R2-2101003	Discussion on support of IMS voice and emergency services for SNPN	Huawei, HiSilicon, China Telecom	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100279	Discussion on Support of IMS Emergency for SNPN	CATT	discussion	Rel-17

R2-2100364	RAN2 impact on support of IMS emergency call for SNPN	Intel Corporation	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100433	Consideration on the IMS voice and emergency services for SNPN	ZTE Corporation, Sanechips	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100492	Support of IMS voice and emergency services for SNPNs	Ericsson	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100545	Overview of RAN2 impacts to support IMS and emergency services for SNPN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NG_RAN_PRN_enh

R2-2100639	Support of IMS voice and emergency services for SNPN	Qualcomm Incorporated	discussion

R2-2100840	Support of IMS voice and emergency service for  SNPN	vivo	discussion

R2-2101517	Support of IMS voice and emergency services for SNPN	LG Electronics	discussion	Rel-17

R2-2101631	Support of IMS voice and emergency services for SNPN	CMCC	discussion	Rel-17	NG_RAN_PRN_enh