3GPP TSG-RAN WG2 Meeting #113e                                                 draft-R2-2102012

E-Meeting: January 25- February 05, 2021

Agenda item:	8.10.1

Source:	Qualcomm Incorporated

Title:	[AT113-e][102][NTN] Reply LSs to SA2 and RAN3

Document for:	Discussion and Decision

# Introduction

RAN2 has received following LSs from SA2 and RAN3.

1) R2-2100067	AN-PDB and PER targets for satellite access (S2-2009225; contact: Qualcomm)	SA2	LS in	Rel-17	5GSAT_ARCH	To:RAN1, RAN2	Cc:RAN3

2) R2-2011041	Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (R3-207062; contact: Qualcomm)	RAN3	LS in	Rel-17		NR_NTN_solutions-Core, 5GSAT_ARCH	To: SA2, RAN2	Cc: SA3-LI, SA5

In the first LS, SAs is asking expected lower bound and upper bound values for PDB and upper bound of PER when the different RAT types for satellite access is used. In the second LS, RAN2 is asking RAN2 feedback on SIB content corresponding to momentary coverage area of a satellite beam and gNB acquiring UE’s location information. Based on these LSs, following email discussion has been started.

- [AT113-e][102][NTN] Reply LSs to SA2 and RAN3 (Qualcomm)

Scope: Discuss reply LSs for R2-2100067 (AN-PDB and PER targets for satellite access) and R2-2011041 (SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G). Note: Soft/hard TAC update will be discussed separately

Initial intended outcome: rapporteur summary and, if possible, draft reply LSs

Initial deadline (for companies' feedback): Friday 2021-01-29 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2102012): Monday 2021-02-01 23:00 UTC

This document provides summary of the email discussion.

# Discussion

## LS reply to R2-2100067 (AN-PDB and PER targets for satellite access)

SA2 Question 1: SA2 would like to ask RAN1, and RAN2 to indicate what is the expected “lower” and “higher” AN-PDB values when the different RAT types for satellite access is used?

SA2 Question 2: SA2 would like to ask RAN1, and RAN2 to indicate what is the expected upper bound of PER when the different RAT types for satellite access is used?

It may be difficult to calculate the upper bound for packet delay before the packet can be considered successfully transmitted. It depends on how many RLC and HARQ retransmissions are allowed and what is the target packet error rate. For example, as described in TR 38.821, if 4 RLC retransmissions are allowed, then maximum transmission time for a packet could be 3000ms for GEO and 150ms for LEO at 600km.

If no retransmission is considered in the PDB, then the delay could be at least one maximum RTD (541.46 ms for GEO, 25.77 ms for LEO at 600km, 41.77 ms for LEO at 1200km) considering scheduling delay and the target PER would be different than what would be assumed when retransmission is allowed.

For HAPS, as propagation delay is much lower than LEO, the expected PDB can be similar to those used in TN.

- Do you agree to suggest the expected lower bound PDB and/or upper bound PDB to SA2 for the different RAT types for satellite access?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | - We understand that the services and corresponding requirements for delay and reliability supported in various NTN scenarios, e.g. GEO/LEO/HAPS based NTN with transparent payload, should be clarified in SA groups before defining the NTN specific PDB and PER. - From RAN2’s perspective, we can provide some assistance information on the maximum round trip delay for the typical scenarios we identified in the first RAN2 meeting for SA2 to take into account when the decision is made there. - We prefer not to mention the number of allowed RLC or HARQ retransmissions to avoid any unintended limitation to SA groups when the service types and corresponding requirements for NTN are discussed and decided. |
| Thales | Yes | Indeed considering the additional delay experienced in satellite access compared to terrestrial RAT, we recommend to suggest higher upper bound PDB to SA2. We are not sure about the need to suggest specific lower bound PDB for satellite access. |

- If answer of Discussion point 1 is “No”, please elaborate how to reply to SA2 regarding the value of the expected PDB for the different RAT types for satellite access?

| Company | Comments |
| --- | --- |
| ZTE | Our draft reply can be found in R2-2101200: RAN2 understand the services and corresponding requirements for delay and reliability supported in various NTN scenarios, e.g. GEO/LEO/HAPS based NTN with transparent payload, should be clarified in SA groups before defining the NTN specific PDB and PER. Regarding the AN-PDB, RAN2 would like to provide the following assistance information for SA2 to make decision: - The maximum round trip delay between UE and the NTN-GW would be 25.77ms (600km-LEO based NTN with transparent payload) ~ 541.46ms (35786km-GEO based NTN with transparent payload). The case where gNB is co-located at the NTN-GW is considered with priority in RAN2, in which the propagation delay between NTN-GW and the gNB can be ignored. - RAN2 understand the propagation delay in HAPS based NTN with transparent payload would be similar to that in TN. |

- If answer of Discussion point 1 is “Yes”, please elaborate what value to indicate to SA2 for the different RAT types for satellite access?

| Company | Comments |
| --- | --- |
| Thales | Compared to terrestrial networks, the GEO based NR access (NTN’s worst case in terms of round trip delay) will add a further delay of 541.46 ms compared to the RTD of cellular networks (see Table 4.2-2: Reference scenario parameters of TR 38.821) If we assume a maximum of “n” RLC retransmissions, the current upper bound PDB should be increased by 2*(n+1) times the maximum propagation delay (one way) of the satellite access. |

The target PER may be different depending on whether the PDB considers retransmissions. However, Rapporteur thinks RAN2 is not in the position to suggest the value of the expected PER.

- Please elaborate how to respond to SA2 on the expected upper bound of PER when the different RAT types for satellite access is used?

| Company | Comments |
| --- | --- |
| ZTE | Regarding PER, although there have been agreements made in RAN1 and RAN2 on disabling HARQ feedback for downlink, the HARQ retransmission and RLC ARQ would still be supported in NTN to ensure the reliability. Thus, we understand the PER for a certain service, if supported in NTN, would be similar to the corresponding PER in TN. |
| Thales | The upper bound of PER should be provided by RAN1 |

## LS reply to R2-2011041 (SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G)

RAN3 agreed that a Cell ID as used in the User Location Information on the NG/N2 interface corresponds to a fixed geographical area, and the Tracking Area is coupled with geographical area. RAN3 has asked feedback on the following approach (a) and approach (b).

Approach a)	On Uu, SIB content corresponds to momentary coverage area of a satellite beam related to the geographically fixed areas of TAs/Cells - irrespective of whether the beam is fixed or moving.

As mentioned in [1], the SIB content, i.e., broadcast TAC and cell ID correspond to momentary coverage area of a satellite beam. The cell ID is changed as soon as the cell enters a new area. But in [2], the multiple cell IDs can be broadcast, and boundary of each cell can be provided to the UE. The UE can select the cell based on its geographical area. In [3], multiple cell IDs are broadcast and fixed association between tracking area and cell ID is used.

However, in any case, SIB content (cell identity) would need to be changed when a cell enters a new area. This contradicts with the following clarification RAN2 made in [4].

A moving radio cell covers different earth area over time and system information including Cell ID moves with the radio cell.

- Please provide comment on how to respond to the approach (a)?

| Company | Comments |
| --- | --- |
| ZTE | We understand approach a) is not workable for the earth moving scenario as the coverage of the satellite beam change when the satellite sweep on the ground thus it would be quite difficult for NW to broadcast a cell ID correspond to momentary coverage area of a satellite beam. Our understanding on the cell ID broadcast in earth fixed and moving scenario is shown below: |
| Thales | On Uu, the cell Id in SIB should corresponds to a beam foot print or a set of beam footprint. Hence Cells move with the beam footprint on Earth |

Approach b) 	The cell ID used on Uu SIB content (and probably on Xn) are decoupled from cell ID used on NG(N2). The respective mapping is performed in RAN. This requires gNB to acquire the UE’s location information.

In this approach, RAN will construct a fixed cell identity to be provided to AMF and this cell identity may not be same as the cell identity that is being broadcast in cell. Therefore, RAN would need UE location information to create a cell identity that corresponds to a fixed geographical area.

When AS security is not enabled, i.e., when constructing the fixed cell identity, for example to be used in “User Location Information” in “INITIAL UE MESSAGE” after UE sends the Msg5, RAN may consider following information to determine the fixed geographical area.

  - Broadcast PCID/cell ID

  - Broadcast TAC

  - Satellite beam coordinates

  - Time stamp

However, there is a problem when relying only on this information.

Problem: The size of geographical area that RAN determines for the fixed cell ID may be too large. When a satellite beam spills over multiple countries, the geographical area identified from the above-mentioned information may not correspond to the country where UE is physically located. This may bring issues like paging and incorrect charging. The UE registered in country A may access the core network of the same country A (sounds correct!) while the UE is physically located in different country B (but not correct!) if moving NTN cell covers both country A and B. This will be against the following fundamental LI requirements for NTN indicated by SA3-LI in [5].

- Any solution shall support the ability to enforce the use of a Core Network of PLMN in the country where the UE is physically located. The enforcement needs to also include cross-border service continuity scenarios.

- Do you agree that severity of problem of cell coverage spill over multiple countries is worse than what is today for terrestrial network?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | / | This is not related to the content of RAN3 LS and is out of the scope of this offline discussion. |
| Thales | Yes | One should distinguish between 4 most likely NTN scenarios: - GEO providing Earth fixed beams (and cells) of large size spanning across multiple countries to serve “VSAT” type terminal with broadband services => The enforcement of LI requirements will require additional location related information to determine in which country the UE is with sufficient accuracy. - LEO providing Earth fixed beams (and cells) of size comparable to maximum size of terrestrial networks’ cells (e.g. super cells) to serve handset or “VSAT” type terminal with broadband services => The enforcement of LI requirements may raise similar needs in terms of additional location related information as for terrestrial networks - LEO providing Earth moving beams (and cells) of large size spanning across multiple countries to serve IoT devices with narrowband services => Not sure that LI requirements applies to IoT services. - HAPS providing Earth fixed or moving beams (and cells) of size comparable to maximum size of terrestrial networks’ cells (e.g. super cells) to serve handset with broadband services => The enforcement of LI requirements may raise similar needs in terms of additional location related information as for terrestrial networks |

To reduce the severity of the problem, RAN may need additional location related information other than the above information (i.e., cell ID, TAC, beam coordinate and time stamp). For example, the UE specific TA reported by UE in MsgA/Msg5 can help RAN determine whether the UE is close to the beam center.

- Do you agree that RAN requires additional location related information other than the broadcast cell ID, broadcast TAC, beam coordinates and time stamp to reduce the problem of satellite beam spill over multiple countries?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | / | This is not related to the content of RAN3 LS and is out of the scope of this offline discussion. So far, no clear need for RAN to acquire additional UE location info has been identified. |
| Thales | Yes | Yes in accordance with SA2 requirement in agreed CR in S2-2009486, “Selection of CN node by NG-RAN node providing satellite access across multiple countries”, Ericsson |

- If your answer to the discussion point 7 is “Yes”, do you agree to inform RAN3 on possible RAN2 impact of the approach (b)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Thales | Yes |  |

- Please indicate your view how to respond to approach (b) in the RAN3 reply LS? Also indicate which approach (a) or (b) is better.

| Company | Comments |
| --- | --- |
| ZTE | As mentioned under discussion point 5, approach a) is not workable for the earth moving scenario as the coverage of the satellite beam change when the satellite sweep on the ground thus it would be quite difficult for NW to broadcast a cell ID correspond to momentary coverage area of a satellite beam. Thus, approach a) is not preferred. For option b), we understand it is the geographical area information of the cell coverage that is needed for RAN to map this area to a cell ID used on NG interface. But it can be achieved in various ways if the geographical area information of the cell coverage is needed in CN: - Option 1: The RAN node derive the geographical area information of the cell coverage based on the satellite ephemeris, beam information and cell deployment information and map this area to a cell ID used on NG interface. - Option 2: The CN acquire the UE location information to identify the cell coverage. - Option 3: The RAN node acquire the UE location information to identify the cell coverage. We understand that if CN needs the UE location, CN can acquire such information itself. If it is the geographical area information of the cell coverage that matters, the RAN node is able to derive this area information (i.e. option 1) and map to a corresponding cell ID used in NG interface. With the above understanding, we do not see the need for RAN to acquire the UE location information. |
| Thales | The most elegant solution is to define cell Id at Uu which corresponds to beam/set of beams and hence both are moving on Earth. It is up to RAN3 to map Cell Id at Uu with Cell Id at NG. |

# Conclusion

TBD..

# References

[1] R2-2100348, NTN location reporting and network identifiers, Ericsson.

[2]	R2-2100330, Discussion on geographical fixed CGI,	CATT.

[3]	R2-2100582, NR-NTN: Cell ID Handling, Fraunhofer.

[4] R2-2008229, LS Reply on SA WG2 assumptions on architecture aspects for using, Qualcomm Inc.

[5] R2-2006532, Response LS on the “LS OUT on Location of UEs and associated key issues” (S3i200056; contact: Rogers), SA3-LI.