3GPP TSG-RAN WG2 Meeting #113 electronic	                                                                       R2-210xxxx

January 25th – February 5th 2021

Agenda item:	8.13.3.2

Source:	CMCC

Title:	Report of [AT113-e][845] [NR/R17 SON/MDT] Logged MDT part II (CMCC)

Document for:	Discussion, Decision

# Introduction

This is to report the result of the following email discussion at RAN2#113-e meeting:

* [AT113-e][845][NR/R17 SON/MDT]  Logged MDT part II (CMCC)

-     Discussion on 2.3.2 of R2-2102250

Intended outcome: Agreeable WF

Deadline: Thursday 04/02/2021

According to the chair’s guidance, this report will be based on the discussion on 2.3.2 of R2-2102250. Please provide your comments by 04 Feb 0700 UTC so that we have time to prepare the summary.

# Discussion

## Scenarios for UE to record/report the on-demand SI related information

In contributions [1] [2], it is proposed for UE to record the on-demand SI related information in following scenarios:

Proposal 1 of [1]: UE reports the on demand SI related information even when the triggered random access procedure fails.

Proposal 4 of [2]: RAN2 to discuss the cases for UE to record the on-demand SI related information. Options are provided as follows:

- When the maximum attempts of RACH for SI request are reached.

- When cell reselection occurs during the RACH for SI request.

- When the required SI is already broadcast periodically by network

Companies are invited to provide views on the scenarios for UE to record the on-demand SI related information:

Q1: Do you agree that UE records the on demand SI related information for the scenario that the triggered random access procedure fails?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Q2: Which following option(s) do you agree for UE to record the on demand SI related information?

- Option 1: When the maximum attempts of RACH for SI request are reached.

- Option 2: When cell reselection occurs during the RACH for SI request.

- Option 3: When the required SI is already broadcast periodically by network

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

## Information for UE to report for on-demand SI

It is proposed to report following information:

Proposal 2 of [1]: UE reports its requested notBroadcasting SI message to help the network side to optimize the transmission of the SI message.

Q3: Do you agree that UE reports its requested notBroadcasting SI message, especially when the triggered random access procedure fails?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Proposal 9 of [3]:	For on-demand SI optimization, extend RA report to include the following information:

- Time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure

Q4: Do you agree that UE reports Time elapsed since the SI request initiation until the successful SI acquisition or the acquisition failure?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Furthermore, it is proposed to report MSG3 related information:

Proposal 9 of [3]:		For on-demand SI optimization, extend RA report to include the following information:

- Msg3-based SI request related information

Proposal 3 of [1]: One specific raPurpose is introduced for MSG3 based on demand SI request.

Proposal 4 of [1]: UE reports the list of requested SI messages that request by MSG3 to the network.

Companies are invited to provide views on the MSG3 related information for UE to report:

Q5: Do you agree that one specific raPurpose is introduced for MSG3 based on demand SI request?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Q6: Do you agree that UE reports the SI messages which requested by MSG3, especially when the triggered random access procedure fails?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Q7: Is there any other scenario or information for UE to record on-demand SI related information?

| Company | Reasons/Comments/Suggestions |
| --- | --- |

Summary:

# Conclusions

# References

- R2-2101642	MDT enhancement for on-demand SI	CMCC	discussion

- R2-2101960	Consideration of logged MDT enhancements	OPPO	discussion

- R2-2101945	Progressing Logged MDT for R17 concerning MR-DC, IRAT and IDC	Samsung Telecommunications	discussion