3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2102681

Online, April 12 – 16, 2021

Agenda item:	8.12.3.1

Source:	Qualcomm Incorporated

Title:	Discussions on eDRX configurations

WID/SID:	FS_NR_redcap

Document for:	Discussion and Decision

# Introduction

This paper discusses an issue related to eDRX configurations raised in RAN2#113-e: whether or how to coordination eDRX configurations for RRC Idle and RRC Inactive.

# Discussion

In LTE, eDRX cycle is configured by core network for RRC Idle. In NR, DRX cycle for RRC Inactive is configured by RAN. We think the same principles should be adopted to NR, especially for RRC Inactive. As discussed in RAN2#113-e, having RAN decide eDRX configuration of RRC Idle provides more flexibility in the configuration and allows different eDRX cycles between RRC Inactive and RRC Idle.

Observation 1. In LTE, eDRX cycle is configured by core network for RRC Idle. In NR, DRX cycle for RRC Inactive is configured by RAN.

When a UE’s RRC connection is terminated, network may release it to either RRC Idle or RRC Inactive state. Network may choose one particular RRC state instead of another for various reasons. For example, if network does not expect a UE to have new data any time soon, it may release the UE to RRC Idle state, with a long eDRX cycle. On the other hand, if network does expect a UE to have new data arrivals in the near future but still see opportunity for the UE to sleep before the data arrives, it is more power efficient to release the UE to RRC Inactive state. In this case, which we believe is one of the motivations for RRC Inactive, it is important for UE to have a shorter eDRX cycle, so that the UE does not need to suffer from a long delay when paged for the new data. Therefore, we think in general it is both necessary and beneficial to have the flexibility of having different eDRX configurations for RRC Idle and RRX Inactive.

Observation 2. In general, it is both necessary and beneficial to have the flexibility of independent eDRX configurations for different RRC states.

In RAN2#113-e meeting [x], some companies proposed to have a common eDRX configuration for RRC Idle and RRC Inactive. The justification behind this proposal is to avoid potential impacts due to RRC state mismatch between gNB and UE. For example, a gNB may lose the UE context of a UE which is in RRC Inactive state. Therefore, network switches to CN paging instead of RAN paging when it has a page for the UE. However, since the UE is unaware of this mismatch with its network, it considers itself is in RRC Inactive and thus monitors only RAN paging occasions. The proponents of the proposal argue that this mismatch can cause extra delays in paging but the undesired consequence can be avoided by enforcing a common eDRX configuration between the two RRC states.

There might be other causes to RRC state mismatch too. However, in our understanding, they generally are rare events in a reasonably well managed network. In addition, even if UE has a page in the midst of RRC state mismatch, the impact is limited to one eDRX cycle (the longer one between the two states), for the following reasons:

- PF offset and first PDCCH monitoring occasion in PO are configured by RAN. eDRX configuration for CN paging does not involve these two parameters;

- Although RAN paging and CN paging may have different eDRX configurations, current eDRX cycles are multiple of each other. Therefore, RAN paging and CN paging always have some overlapping POs;

- The state mismatch can be corrected once UE receives a page. For example, TS 38.304 specifies that UE shall switch to RRC Idle if it receives a CN paging while in RRC Inactive state.

Observation 3. RRC state mismatch generally is a rare event in reasonably well managed networks and its impact is only temporarily (i.e. can be corrected after UE receives a page).

Since differentiated eDRX configuration for RRC Idle and RRC Inactive are needed in typical use cases, then in our view, it is not a sensible engineering choice to trade that flexibility for a rare event. If for some reason, some network is concerned with the impact of RRC state mismatch, 5GC already has the necessary signaling to allow CN and RAN coordinate their eDRX configurations for RRC Idle and RRC Inactive. For example, according to TS 23.501, during N2 activation with the (new) serving NG-RAN node (i.e. during Registration, Service Request, Handover), AMF provides “RRC Inactive Assistance Information” to assist the NG-RAN's decision. This "RRC Inactive Assistance Information" includes at least UE-specific DRX values and UE-specific extended idle mode DRX values (cycle length and Paging Time Window length). Therefore, if some type of coordination between the eDRX configurations of two RRC states, we think that can be done by network implementation using existing mechanisms. No new spec changes are needed.

Lastly, a reference for the issue is that for eMTC eDRX configurations are already independent for RRC Inactive and RRC Idle, although RRC state mismatch can happen for eMTC too.

Proposal 1. 	eDRX configurations (eDRX cycle and PTW) for RRC Idle and RRC Inactive can be independently configured by CN and RAN, respectively.

Proposal 2. 	CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

Proposal 3. 	Any coordination between eDRX configurations for RRC Idle and RRC Inactive can be left to network implementation.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

Observation 1. In LTE, eDRX cycle is configured by core network for RRC Idle. In NR, DRX cycle for RRC Inactive is configured by RAN.

Observation 2. In general, it is both necessary and beneficial to have the flexibility of independent eDRX configurations for different RRC states.

Observation 3. RRC state mismatch generally is a rare event in reasonably well managed networks and its impact is only temporarily (i.e. can be corrected after UE receives a page).

Proposal 1. 	eDRX configurations for RRC Idle and RRC Inactive can be independently configured by CN and RAN, respectively.

Proposal 2. 	CN paging and RAN paging use the same paging frame offset and first PDCCH monitoring occasion in PO, which are configured by RAN without involvement of CN.

Proposal 3. 	Any coordination between eDRX configurations for RRC Idle and RRC Inactive can be left to network implementation.

# References

- R2_2102019, Summary of email discussion [AT113-e][109][REDCAP] eDRX cycles (CATT), Feb 2021.