3GPP TSG-RAN WG2 Meeting #113bis-e                                            R2-2102727

Electronic, 12 April – 20 April 202‎1

Source:	CATT

Title:	Consideration on topology-wide fairness, multi-hop latency and congestion mitigation

Agenda Item:	8.4.2

Document for:	Discussion and Decision

Introduction

In RAN2-113e meeting, open issues on topology-wide fairness, multi-hop latency and congestion mitigation were agreed.

In this contribution, we will further discuss the open issues on topology-wide fairness, multi-hop latency and congestion ‎mitigation.

Discussion

Topology-wide fairness

This section will focus on the 3 issues captured in chairman’s note as following:

- IF-1: The scheduler of an IAB node does not have all the information needed (e.g. link quality across multiple hops) to make appropriate upstream or downstream scheduling decisions which take into account the overall route link quality (such as e.g. using downstream link quality measurements to adjust the scheduling weights so as to achieve proportional fairness for different bearers/RLC channels across multiple child-IAB nodes) - IF-2: Congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested, thereby leading to some users experiencing longer latency and violating fairness requirement. - IF-4: IAB node cannot give more resource to those BH RLC CHs that aggregate more bearers and/or carry bearers with higher load per bearer (i.e. IAB node cannot give more resource to those BH RLC CHs with higher aggregate load)

IF-1: More information for the scheduler of IAB-node

In RAN2-112e, RAN2 agreed the fairness definition that QoS is met across the topology, regardless of where a UE attaches to the IAB network. Based on the RAN2-113e agreement, RAN2 will not further discuss ways of evaluating success of any fairness mechanisms, beyond the already agreed definition of fairness and its variants. We understand the a variant of fairness definition is that UE with different hops have the same QoS for the same service regardless of where a UE attaches to the IAB network.

In R16 IAB, IAB-node cannot schedule according to the remaining hops. For example, there are two UEs, i.e., UE1 and UE2. The remaining hops number between intermediate IAB-node and UE1 is 5, while the remaining hops number between intermediate IAB-node and UE2 is 3. Since each hop in the network introduces additional latency in the upstream or downstream. Thus, the potential latency of UE1 is larger than UE2. So if IAB-node can schedule according to the remaining hops, it will have benefits for IAB topology fairness.

IF-1 states that IAB-node does not have all the information for scheduling decisions overall route link quality. In order to ensure the topology-wide fairness (UE with different hops have the same QoS for the same service regardless of where a UE attaches to the IAB network). We think IAB-node should be provided‎ with topology information‎, e.g., the number of remaining hops in the upstream or downstream, to prior scheduling the data transmission with larger number of remaining hops.

Observation 1: If IAB-node can schedule according to the remaining hops, it will have benefits for IAB topology fairness.

Proposal 1: IAB-node is provided with topology information, e.g., the number of remaining hops in the upstream or downstream, to achieve the topology-wide fairness in R17.

IF-2: Unbalanced Congestion

IF-2 states that congestion conditions on BH RLC channels carrying UE bearers with same or similar QoS requirements can be unbalanced and some channels may even be congested.

E2E flow control can solve this issue in long term. RAN2 doesn’t need to discuss E2E flow control before RAN3 input. If the congestion arises temporarily, flow control feedback and local rerouting can alleviate the unbalanced QoS satisfaction. Local rerouting in upstream has been specified in Rel-16. For IF-2, local rerouting should be extended in two aspects: 1) local rerouting in downstream; 2) additional triggers for local rerouting.

Proposal 2: Enhanced flow control feedback and local rerouting can be considered for IF-2.

IF -4: More resource for the BH RLC channel with higher load

In F1 specification [2], the IE QoS Flow Level QoS Parameters “defines the QoS to be applied to a QoS flow, to a DRB or to a BH RLC channel”. For a BH RLC channel, this IE reflects QoS characteristics of all aggregated bearers. For example, when GBR QoS Flow Information presents in QoS Flow Level QoS Parameters, Guaranteed Flow Bit Rate Downlink in GBR QoS Flow Information should be the sum of GFBR of all aggregated GBR bearers. With reasonable CU configuration, IF-4 should not be a practical issue.

Observation 2: IF-4 is not a practical issue with reasonable CU configuration..

Proposal 3: RAN2 should deprioritize IF-4.

Multi-hop latency

In this section, we focus on the following issues:

- IL-1: IAB node cannot help ensure that overall or remaining PDB is met for a packet (e.g. by prioritizing bearers with higher number of hops), as it does not have a latency reference for the packets being scheduled, resulting in packets with the same QoS requirement ending up with different latency - IL-2: IAB node may need to report joint buffer status for LCHs which have rather differing QoS requirements, due to the current (Rel-16) limit on the number of LCGs - IL-3: Buffer size calculation for pre-emptive BSR may differ for nodes of different vendors as it is left to implementation in Rel-16 - IL-5: The CU is unable to put bearers with lower PDB on routes with less congestion risk (higher resource efficiency) or which are RLF-free - IL-6: The CU is unable to configure routing based on actual (real-time) latency per BH RLC channel

IL-1: Overall or remaining PDB

Referring to [2], PDB of BH RLC channel in IAB-node is one-hop. Note in TS38.473, it states that “For a BH RLC channel, the Packet Delay Budget included in 5QI defines the upper bound for the time that a packet may be delayed between the gNB-DU and its child IAB-MT”. Donor-CU should split overall PDB to each hop based on the potential latency of each hop. Reasonable PDB allocation can guarantee the overall or remaining PDB basically. But some additional information may help IAB-node to perform fairness scheduling. This issue can be de-prioritized or merged with IF-1.

Proposal 4: IL-1 can be de-prioritized or merged with IF-1.

IL -2: Extend the number of LCGs

LCG identifies the group of logical channel(s) whose buffer status is being reported by BSR. Currently, the length of LCG is 3 bits. IAB-node aggregates bearers with different QoS requirements from different UEs. The number of logical channel IDs for identity of the logical channels has been extended largely in Rel-16. For example, the indexes of eLCID 320 to (2^16 + 319) are extended to identify DL logical channels. Meanwhile, increasing the number of LCGs per BH link can refine the scheduling granularity. Therefore, the nunber of LCGs should be extended accordingly. The length of LCG can be extended to 8 bits.

Proposal 5: The length of LCG can be extended to 8 bits.

IL -3: Pre-emptive BSR

Some companies argued that buffer size calculation for pre-emptive BSR may differ for nodes of different vendors and suggested to standardize the pre-emptive buffer size calculation. Pre-emptive BSR is to request uplink resources for the UL data transmission before it receives actual data. It is not urgent for the parent node to get accurate data volume for future transmission. Currently, pre-emptive BSR could be triggered when the IAB-node received BSR from its child node or allocated UL grant to the child node. There are too many undeterministic factors between pre-emptive BSR trigger and the data arrival for the IAB-node, such as scheduling delay, HARQ retransmissions, and RLC retransmissions etc.

Observation 3: Pre-emptive BSR is not urgent and it is almost impossible to be calculated precisely.

Proposal 6: Leave the calculation of pre-emptive BSR to implementation as Rel-16.

IL-5: CU put bearers with lower PDB on routes with less congestion risk

This issue seems should be solved in RAN3. RAN2 should focus on one-hop issue. CU cannot know the information of immediate IAB-node by HbH signaling. E2E flow control in RAN3 could be used in this issue. RAN2 should deprioritize IL-5.

Proposal 7: RAN2 should deprioritize IL-5.

IL -6: CU configure routing based on actual (real-time) latency

The CU is not only unable to configure routing based on actual (real-time) latency per BH RLC channel, but also cannot configure one-hop PDB precisely. However, packet delay report has been discussed in MDT and captured in TS38.314 [3]. Same L2 measurement report mechanism could be applied between IAB-DU and CU. We can check it further and confirm if it has additional specification impact.

Proposal 8: IL-6 may be resolved by packet delay report in TS38.314.

Congestion mitigation

In this section, we focus on the following issues:

- IC-1: Long-term downstream congestion on a single link cannot be alleviated using existing Rel-16 DL HbH flow control mechanisms, without having to rely on dropping packets - IC-7: CU (not having knowledge of local congestion conditions) cannot update the routing path that is experiencing congestion.

IC-1: Long-term downstream congestion

For IC-1, although E2E flow control can reduce long-term downstream congestion, we can also make some enhancements in RAN2. In Rel-16 DL HbH flow control mechanism, parent IAB-node will reduce the data transmission of its child IAB-node after receiving the DL HbH flow control feedback from child IAB-node. However, it may cause the Long-term congestion of parent IAB-node.

If ancestor IAB-node can know potential congestion of parent IAB-node in advance, ancestor IAB-node may reduce the downstream transmission before the actual congestion of parent IAB-node. This is similar as pre-emptive BSR. Pre-emptive BSR is to apply for the resource for potential data transmission before received the actual data.

Thus we propose that RAN2 support IAB-node forwarding flow control feedback from its child IAB-node to the ancestor IAB-node.

Proposal 9: RAN2 support IAB-node forwarding flow control feedback from its child IAB-node to ancestor IAB-node.

IC-7: CU update the congested routing path

Same as IL-5, this issue seems should be solved in RAN3. RAN2 should focus on one-hop issue. RAN2 should deprioritize IC-7.

Proposal 10: RAN2 should deprioritize IC-7.

Conclusion

In section 2, we made the following observations:

Observation 1: If IAB-node can schedule according to the remaining hops, it will have benefits for IAB topology fairness.

Observation 2: IF-4 is not a practical issue with reasonable CU configuration.

Observation 3: Pre-emptive BSR is not urgent and it is almost impossible to be calculated precisely.

Based on the discussion in section 2, we propose the following:

Proposal 1: IAB-node is provided with topology information, e.g., the number of remaining hops in the upstream or downstream, to achieve the topology-wide fairness in R17.

Proposal 2: Enhanced flow control feedback and local rerouting can be considered for IF-2.

Observation 2: IF-4 is not a practical issue with reasonable CU configuration.

Proposal 3: RAN2 should deprioritize IF-4.

Proposal 4: IL-1 can be de-prioritized or merged with IF-1.

Proposal 5: The length of LCG can be extended to 8 bits.

Proposal 6: Leave the calculation of pre-emptive BSR to implementation as Rel-16.

Proposal 7: RAN2 should deprioritize IL-5.

Proposal 8: IL-6 may be resolved by packet delay report in TS38.314.

Proposal 9: RAN2 support IAB-node forwarding flow control feedback from its child IAB-node to ancestor IAB-node.

Proposal 10: RAN2 should deprioritize IC-7.

Reference

- R2-2101168	Report from email discussion [Post112-e][065][eIAB] Fairness Latency Congestion (Samsung) 	Samsung Electronics GmbH	report

- TS38.473, F1 application protocol (F1AP), V16.4.0

- TS38.314, Layer 2 Measurements, V16.2.0