3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2102839

# Electronic meeting, 12th – 20th Apr 2021

Agenda item:		8.1.2.4

Source:	Intel Corporation

Title:	MBS MAC Layer and Group Scheduling Aspects

Document for:		Discussion and Decision

# Introduction

In NR MBS, group scheduling mechanism is important to be specified to allow UEs to receive multicast and broadcast services. Meanwhile, enhancements to enable simultaneous operation with unicast reception is also included in the WI objection.

In RAN1 #103e meeting, common understanding of PTP/PTM transmission scheme and how they are used in initial transmission and retransmission are discussed in RAN1. Moreover, in RAN1 #103e and #104e meeting, following agreements and working assumptions are captured for SPS group-common PDSCH and reliability enhancement:

RAN1 #103e Agreements: Support SPS group-common PDSCH for MBS for RRC_CONNECTED UEs - FFS: use group-common PDCCH or UE-specific PDCCH for SPS group-common PDSCH activation/deactivation - FFS: whether to support more than one SPS group-common PDSCH configuration per UE - FFS: whether and how uplink feedback could be configured - FFS: retransmission of SPS group-common PDSCH RAN1 #104e Agreements: For RRC_CONNECTED UEs, more than one SPS group-common PDSCH configuration for MBS can be configured per UE subject to UE capability - The total number of SPS configurations supported by a UE currently defined for unicast is not increased due to additionally supporting MBS. - FFS: How to allocate the total SPS configurations between MBS and unicast. For RRC_CONNECTED UEs, support HARQ-ACK feedback for SPS group-common PDSCH for MBS Working assumption: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, - At least group-common PDCCH is supported - FFS: Whether and how to address the missed activation and deactivation - FFS: Whether UE-specific PDCCH is supported for activation/deactivation Agreement: For RRC_CONNECTED UEs, if ACK/NACK based HARQ-ACK feedback is supported for PTM scheme 1, and if initial transmission for multicast is based on PTM transmission scheme 1, support retransmission(s) using PTP transmission. - The HARQ process ID and NDI indicated in DCI is used to associate the PTM scheme 1 and PTP transmitting the same TB.

In this contribution, we first discuss the different understanding regarding to PTP and PTM in RAN1 and RAN2, proposed together with a RAN2 definition. G-RNTI mapping, DRX, SPS, multiplexing/de-multiplexing in MAC layer are then discussed based on two delivery modes agreed in RAN2 and two PTM transmission schemes agreed in RAN1.

# Discussion

## PTP/PTM Definition

NR MBS introduces the support of PTP and PTM transmission in 5GC shared traffic delivery. For the convenience of discussion, it would be good to discuss and clarify the terminology of PTP and PTM from RAN1 and RAN2 point of view.

PTP/PTM transmission definitions are captured in the running CR [1] as:

For multicast service, gNB may deliver MBS data packets using the following methods: - PTP Transmission: gNB individually delivers separate copies of MBS data packets to each UEs independently, i.e. gNB uses UE-specific PDCCH with CRC scrambled by UE-specific RNTI (e.g., C-RNTI) to schedule UE-specific PDSCH which is scrambled with the same UE-specific RNTI. - PTM Transmission: gNB delivers a single copy of MBS data packets to a set of UEs, e.g., gNB uses group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI. Editor’s Note: FFS that RAN1 inputs is needed for the definition of PTP/PTM transmission. A gNB node dynamically decides whether to deliver multicast data by PTM or PTP for a given UE based on the protocol stack defined in section 16.x.3.

However, it is not clear which traffic channel is used for PTP and PTM transmission. From RAN2 point of view, understanding of PTP and PTM may be different from RAN1.

As discussed in [2], MTCH (multicast traffic channel) and MCCH (multicast control channel) are proposed as new logical channels for NR MBS. MBS traffic/control data via PTM transmission can be mapped to MTCH and MCCH (for delivery mode 2), respectively. However, for PTP transmission, since gNB can individually deliver MBS data packets to each UEs independently, dedicated logical channel DTCH and DCCH can be used, so that MBS service can send together with unicast service in the same radio resource.

Hence, to facilitate discussion in RAN2, from traffic channel point of view, the terminology of PTP and PTM is clarified as below:

- PTP transmission: gNB individually delivers separate copies of MBS data packets to each UEs independently via dedicated logical channel DTCH

- PTM transmission: gNB delivers a single copy of MBS data packets to a set of UEs via shared logical channel MTCH

Recalling RAN1 agreements on PTP/PTM transmission scheme, PTP and PTM transmission scheme is separated based on whether MBS traffic data is transmitted over UE-specific PDSCH or group-common PDSCH. From RAN1 perspective, transport channel DL-SCH is used for both PTP and PTM. Similar as LTE SC-PTM, both MTCH and MCCH can be mapped to DL-SCH.

- Understanding of the terminology “PTP” and “PTM” is different in RAN1 and RAN2. In RAN2, PTP transmission use dedicated logical channel, while PTM transmission use shared logical channel.

RAN1 agreed that PTP transmission scheme defined in RAN1 is supported as HARQ retransmission for an initial transmission via PTM transmission scheme. From RAN2 perspective, same logical channel is used for both initial transmission and retransmission. Hence, when PTP transmission scheme defined in RAN1 is used for retransmission, shared logical channel MTCH and MCCH are still used in MAC layer, which will be mapped to UE-specific PDSCH. In summary, PTP and PTM transmission in RAN2 can use UE-specific PDSCH (PTP transmission scheme in RAN1), PTM transmission in RAN2 can use group-common PDSCH (PTM transmission scheme 1/2 in RAN1).

In following contribution, we use (RAN1) and (RAN2) to represent different terminology used for PTP and PTM in RAN1 and RAN2, respectively.

- From RAN2’s perspective, shared logical channel MTCH is mapped to UE-specific PDSCH when PTP (RAN1) is used for HARQ retransmission of an initial transmission via PTM (RAN1).

## Mapping between G-RNTI and MBS session

As summarized in [3], majority companies’ view is using a one-to-one mapping between G-RNTI and MBS session. Additionally, one-to-multiple and multiple-to-one mapping are also proposed in [4], [5] and [6], respectively.

Regarding to one-to-multiple mapping between G-RNTI and MBS session, if a given UE (e.g. UE1) is interested in multiple services, scheduling with the same G-RNTI can be easier for network management. If there’s another UE (e.g. UE2) who is interested in the subset of UE1’s MBS services, UE2 may need to share the same G-RNTI. It is also not possible for UE2 to identify the packets of subset MBS services from other MBS services (which is not UE2’s interest), as packets are multiplexed together with the same G-RNTI. It could be a power waste for UE2.

Regarding multiple-to-one mapping between G-RNTI and MBS session, it introduces more complexity in network side to schedule and manage MBS services.

- Only one-to-one mapping between G-RNTI and MBS session is supported.

## Multiplexing/De-multiplexing

In TR23.757 [7], conclusions for Key Issue #4 “QoS level support for Multicast and Broadcast communication services” includes that the network shall support one or multiple QoS flow for an MBS session. It is possible that each MBS service has one or multiple logical channels.

In PTP transmission (RAN2), as described in WID “enhancements that are required to enabled simultaneous operation with unicast reception” is supported within the scope of WI. It is possible that UE can receive one or more MBS services and unicast services simultaneously in one slot. Different from LTE SC-PTM “Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding and SC-PTM Stop Indication”, different logical channels can be used to separate MBS services and unicast services, since PDSCH for both MBS and unicast service are scrambled by C-RNTI. In this case, UE can receive all data in the same transport block by multiplexing multiple logical channels from both MBS and unicast service, even from multiple MBS services.

- In PTP transmission (RAN2), multiplexing/de-multiplexing of different logical channels from same or different MBS and unicast service is supported in MAC.

Considering MBS service may have multiple logical channels, during PTM transmission (RAN2), MAC layer should multiplex logical channel(s) from the same MBS service into the same TB, so that data from the same MBS service can have similar latency as required.

- In PTM transmission (RAN2), multiplexing/demultiplexing of different logical channels from the same MBS service/session is supported in MAC for NR MBS.

Moreover, interested MBS services of each UE served in the same NG-RAN may be different let alone the number of interested MBS services. For power saving, UEs do not need to receive and demultiplex those MBS services which are not their interest(s). Even if one UE is interested in multiple MBS services, considering each MBS service has its own latency/QoS requirement, scheduling occurrence among MBS services may not be the same, it is hard to multiplex among logical channels from different MBS services. UE can receive multiple configurations from RAN for different MBS services, and receive each subscribed MBS service data according to its configuration.

- In PTM transmission (RAN2), multiplexing/demultiplexing of different logical channels from different MBS services/sessions is not supported in MAC for NR MBS.

Different from MBSFN defined in LTE, it is proposed that MBS and unicast service can share the same LCID space, e.g. Table 6.2.1-1 in TR38.321 [4], as both MTCH and DTCH are mapped to DL-SCH. According to RAN1 agreement in RAN1 #104e meeting, PTP (RAN1) can be used as HARQ retransmission scheme for NR MBS. It is important that LCID values retain the same in initial transmission and HARQ retransmission. Hence, it is proposed that those LCID values defined for MBS services cannot be used by unicast service. Also, as discussed in the companion contribution [8], RLC ARQ is also proposed to use either PTP (RAN2) or PTM (RAN2) for retransmission according to the number of UE’s NACK. Since PTP (RAN2) and PTM (RAN2) are using DTCH and MTCH, respectively, one MBS service should allocate different LCIDs for PTP (RAN2) and PTM (RAN2).

- NG-RAN assign LCID values for MBS logical channels from LCID values for DL-SCH. One MBS service has different LCIDs in PTP (RAN2) and PTM (RAN2).

To be more specific, the MAC structure overview updated with MBS is shown as Figure 1.

Figure 1. MAC structure overview

## DRX

As discussed in companion contribution [9], three types of high QoS multicast services is transmitted via delivery mode 1. To reduce latency caused by RRC state switching, it is considered that UE keep staying in RRC_CONNECTED state for multicast services with high latency requirement, even during the period no data is transmitted. As for multicast services only with high reliability requirement, UE can switch to RRC_IDLE/INACTIVE when there’s no data in transmission. Hence, DRX is used to control UE’s PDCCH monitoring activity and lower UE’s power consumption.

When MBS transmission and retransmission is sent over PTP (RAN2), DRX can reuse the same configuration as what for unicast service, including drx-onDurationTimer, drx-SlotOffset, drx-InactivityTimer, drx-LongCycleStartOffset, drx-RetransmissionTimerDL, drx-HARQ-RTT-TimerDL, etc.

- DRX of MBS services via PTP transmission (RAN2) reuses the same configuration as unicast service.

For PTM transmission (RAN2), as discussed in the companion contribution [9], traffic channel configurations (MTCH configurations) of multicast services via delivery mode 1 is directly signaled via dedicated RRC signaling. Hence, in delivery mode 1, DRX is only used to monitor PDCCH for RNTI value of MTCH.

MBS services may have different scheduling occurrence according to its QoS requirement, a unique set of DRX parameters should be configured for each MBS services. MBS DRX can follow the same mechanism as defined in normal DRX.

During PTM transmission (RAN2), both PDCCH and PDSCH are scrambled with G-RNTI during MBS initial transmission. Each DRX operation controls UE’s PDCCH monitoring activity for each MBS service corresponding G-RNTI, including parameters:

- drx-onDurationTimerMBS: the duration at the beginning of a DRX cycle of a MBS service;

- drx-SlotOffsetMBS: the delay before starting the drx-onDurationTimerMBS of a MBS service;

- drx-InactivityTimerMBS: the duration after the PDCCH occasion in which a PDCCH indicates a new MBS transmission for the MAC entity;

- drx-schedulingPeriodStartOffsetMBS: the SchedulingCycleMBS and drx-StartOffsetMBS which defines the subframe where the MBS DRX cycle starts, where the value of SchedulingCycleMBS is in ms and drx-StartOffsetMBS is in multiples of 1ms.

- DRX configuration drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS and drx-schedulingPeriodStartOffsetMBS is used for MBS initial transmission via PTM (RAN2).

Moreover, HARQ-ACK feedback for RRC_CONNECTED UE is agreed in RAN1 for NR MBS. Considering DL MBS traffic HARQ retransmission may be expected by the MAC entity, other than drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS and drx-schedulingPeriodStartOffsetMBS, comparing with LTE SC-PTM, DL HARQ RTT timer and DL retransmission timer for MBS transmission channel are introduced for NR MBS DRX, e.g. drx-HARQ-RTT-TimerDLMBS and drx-RetransmissionTimerDLMBS, respectively. Following parameters are configured differently for different options:

- drx-RetransmissionTimerDLMBS: the maximum duration until a MBS retransmission is received (per MBS service);

- drx-HARQ-RTT-TimerDLMBS: the minimum duration before a DL assignment for MBS HARQ retransmission is expected by the MAC entity.

However, considering HARQ retransmission has different options, e.g. via PTM scheme 1, PTM scheme 2 and PTP (RAN1). HARQ related DRX timers can be configured differently based on different transmission scheme used for HARQ retransmission.

Since RAN1 has agreed to support retransmission via PTP (RAN1) scheme, MBS DRX HARQ related timers are not suitable to monitor retransmission via UE-specific PDSCH. Normal DRX timer drx-HARQ-RTT-TimerDL starts for the corresponding HARQ process. Another normal DRX timer drx-RetransmissionTimerDL also follows the same procedure as normal DRX.

- For MBS retransmission via PTP (RAN1), HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are configured.

If PTM scheme 1 (RAN1) is used for retransmission, drx-HARQ-RTT-TimerDLMBS starts after UE sends HARQ feedback for MTCH and monitors DL assignment in PDCCH scrambled by G-RNTI. Timer drx-RetransmissionTimerDLMBS starts after the expiry of drx-HARQ-RTT-TimerDLMBS and stops if PDCCH indicates a DL transmission for corresponding HARQ process.

- HARQ related DRX timer drx-HARQ-RTT-TimerDLMBS and drx-RetransmissionTimerDLMBS are configured if PTM scheme 1 (RAN1) is used for MBS retransmission.

If PTP (RAN1) is used for retransmission, MBS DRX HARQ related timers are not suitable to monitor retransmission via UE-specific PDSCH. Normal DRX timer drx-HARQ-RTT-TimerDL starts for the corresponding HARQ process. Another normal DRX timer drx-RetransmissionTimerDL also follows the same procedure as normal DRX.

- HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are configured if PTP (RAN1) is used for MBS retransmission.

In PTM scheme 2 (RAN1), different from above two options, since UE-specific PDCCH with CRC scrambled by C-RNTI, normal DRX timer drx-HARQ-RTT-TimerDL starts after the end of the corresponding transmission carrying the DL MBS HARQ feedback. Then drx-HARQ-RTT-TimerDLMBS starts after the expiry of drx-HARQ-RTT-TimerDL, since UE receives HARQ retransmission via the group-common PDSCH.

- HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDLMBS are configured if PTM scheme 2 (RAN1) is used for MBS retransmission.

Delivery mode 2 is similar as LTE SC-PTM, DRX is configured separately for MCCH and MTCH. For simplicity of transmission for low QoS MBS services, in delivery mode 2, only PTM transmission scheme (RAN2) is considered. HARQ support in RRC_IDLE/INACTIVE UE is still under discussion in RAN1, it is suggested that HARQ related timers are also configured for delivery mode 2 if HARQ is supported for RRC_IDLE/INACTIVE UEs. In this case, DRX for MTCH in delivery mode 2 is also configured per MBS service, including drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS, drx-schedulingPeriodStartOffsetMBS, drx-RetransmissionTimerDLMBS, drx-HARQ-RTT-TimerDLMBS, etc.

- DRX configuration parameters for each MTCH in delivery mode 2 includes: drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS, drx-schedulingPeriodStartOffsetMBS, drx-RetransmissionTimerDLMBS, drx-HARQ-RTT-TimerDLMBS.

Different from delivery mode 1, MCCH is introduced in delivery mode 2 to transmit traffic control information. A new RNTI (MBS-RNTI), similar to SC-RNTI in LTE SC-PTM, is introduced to dynamically schedule NR MBS delivery mode 2 control information. In [Post112-e][069][MBS] “Delivery mode 2” email discussion [7], RAN2 further discuss if multiple MCCH based PTM (RAN2) configuration can be supported for NR MBS delivery mode 2. Regardless how many MCCH is configured, DRX is considered to configure for each MCCH. DRX configuration for MCCH includes timer drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS.

- DRX configuration parameters for each MCCH in delivery mode 2 includes: drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS, drx-schedulingPeriodStartOffsetMBS.

According to QoS requirement of different MBS services, MBS DRX timer durations may vary among services and different from normal DRX.

Normal DRX configuration is independent from MBS services. For a certain MBS service, it is possible that UE can receive PTP (RAN2) and PTM (RAN2) simultaneously. In this case, UE is configured with both PTP (RAN2)/normal DRX parameters and PTM (RAN2) DRX parameters. It is also possible that UE receives one MBS in PTP (RAN2) using normal DRX configuration and another MBS in PTM (RAN2) using PTM (RAN2) DRX configuration.

Beyond PTP (RAN1) scheme used for HARQ retransmission (that has been agreed), RAN1 should further discuss whether HARQ retransmission is supported via PTM scheme 2 (RAN1) and how to decide the switch between different HARQ retransmission schemes. If multiple HARQ retransmission schemes are supported, MBS services can configure either full set of DRX configurations or limited configurations according to the chosen retransmission scheme. For example, as agreed in RAN1, when PTM scheme 1 (RAN1) is used for initial transmission and PTP (RAN1) is used for HARQ retransmission, HARQ related DRX timers can be configured with NULL and replaced by HARQ related DRX timers for unicast service.

## SPS

In LTE SC-PTM, SPS is not supported for multicast and broadcast services. As agreed in RAN1 #103e meeting, SPS group-common PDSCH for MBS is supported for RRC_CONNECTED UEs. Since there’s no uplink data transmission in MBS, only DL SPS is supported. Both dynamic scheduling and DL SPS should be supported for NR MBS.

SPS of PTP transmission (RAN2) in delivery mode 1 should have the same configuration/behavior for unicast service. CS-RNTI is used for DL SPS activation/deactivation/retransmission.

- In PTP transmission (RAN2), CS-RNTI is used for DL SPS activation/deactivation/retransmission.

As discussed in section 2.3, group-common PDSCH is used for both transmission and retransmission in PTM transmission (RAN2). Similar as DRX configured per MBS service, each MBS service has its own DL SPS configuration according to its QoS requirement. A new type of RNTI (e.g. G-CS-RNTI) can be used to indicate configured scheduled MBS transmission.

It is agreed in RAN1 #104e meeting that HARQ-ACK feedback for SPS group-common PDSCH is supported for MBS. In normal SPS, HARQ process ID is derived from CURRENT_slot, numberOfSlotsPerFrame, periodicity, nrofHARQ-Processes and harq-ProcID-Offset. It is possible that MBS DL SPS may be overlapped with normal DL SPS. In order to avoid the initial transmission of DL SPS for MBS and unicast service use the same HARQ process ID when they are configured in the same slot, mbs-nrofHARQ-Processes, mbs-harq-ProcID-Offset and mbs-periodicity are introduced to calculate HARQ process ID of DL SPS for MBS. If DL SPS for MBS and unicast service are overlapped in the same slot, MBS DL SPS can set a different HARQ process ID by using mbs-harq-ProcID-Offset.

Following parameters is configured for MBS DL SPS:

-	g-cs-RNTI: G-CS-RNTI for activation, deactivation, and retransmission;

-	mbs-nrofHARQ-Processes: the number of configured HARQ processes for MBS DL SPS;

-	mbs-harq-ProcID-Offset: Offset of HARQ process for MBS DL SPS;

-	mbs-periodicity: periodicity of configured downlink assignment for MBS DL SPS.

- MBS DL SPS is configured with parameters g-cs-RNTI, mbs-nrofHARQ-Processes, mbs-harq-ProcID-Offset and mbs-periodicity.

- G-CS-RNTI is used for MBS DL SPS activation/deactivation/retransmission if PTM scheme (RAN1) is used for DL SPS initial transmission and DL SPS HARQ retransmission in both delivery mode 1 and delivery mode 2.

When PTP (RAN1) is used for retransmission, both CS-RNTI and G-CS-RNTI are used in MBS DL SPS. It is because that CS-RNTI is used for dynamic scheduling for retransmission in the normal DL SPS. During MBS DL SPS, G-CS-RNTI is used for DL SPS activation/deactivation. CS-RNTI is used to indicate the retransmission from DL SPS, so that the downlink assignment of MBS DL SPS retransmission can be received on the PDCCH which is scrambled with CS-RNTI in PTP (RAN1). The retransmission TB can be identified for MBS by comparing HARQ process ID with initial transmission.

- In delivery mode 1, G-CS-RNTI is used for activation/deactivation, CS-RNTI is used for DL SPS HARQ retransmission when PTM scheme 1 (RAN1) is used for DL SPS initial transmission and PTP (RAN1) is used for DL SPS HARQ retransmission.

# Conclusion

In this contribution, we defined PTP and PTM definition in RAN2 should be based on logical channel used for MBS transmission. Following that, we compared different mapping options between G-RNTI and MBS session. We then analyzed the support of MBS multiplexing, including multiplexing between multiple MBS services, multiplexing between MBS and unicast service. In the end, the configuration parameters for MBS DRX, SPS are discussed for different considerations of initial transmission and retransmission.

We propose the following observations and proposals:

Observation 1:Understanding of the terminology “PTP” and “PTM” is different in RAN1 and RAN2. In RAN2, PTP transmission use dedicated logical channel, while PTM transmission use shared logical channel.

Observation 2:From RAN2’s perspective, shared logical channel MTCH is mapped to UE-specific PDSCH when PTP (RAN1) is used for HARQ retransmission of an initial transmission via PTM (RAN1).

Proposal 1:Only one-to-one mapping between G-RNTI and MBS session is supported.

Proposal 2:In PTP transmission (RAN2), multiplexing/de-multiplexing of different logical channels from same or different MBS and unicast service is supported in MAC.

Proposal 3:In PTM transmission (RAN2), multiplexing/demultiplexing of different logical channels from the same MBS service/session is supported in MAC for NR MBS.

Proposal 4:In PTM transmission (RAN2), multiplexing/demultiplexing of different logical channels from different MBS services/sessions is not supported in MAC for NR MBS.

Proposal 5:NG-RAN assign LCID values for MBS logical channels from LCID values for DL-SCH. One MBS service has different LCIDs in PTP (RAN2) and PTM (RAN2).

Proposal 6:DRX of MBS services via PTP transmission (RAN2) reuses the same configuration as unicast service.

Proposal 7:DRX configuration drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS and drx-schedulingPeriodStartOffsetMBS is used for MBS initial transmission via PTM (RAN2).

Proposal 8:For MBS retransmission via PTP (RAN1), HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are configured.

Proposal 9:HARQ related DRX timer drx-HARQ-RTT-TimerDLMBS and drx-RetransmissionTimerDLMBS are configured if PTM scheme 1 (RAN1) is used for MBS retransmission.

Proposal 10:HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDL are configured if PTP (RAN1) is used for MBS retransmission.

Proposal 11:HARQ related DRX timer drx-HARQ-RTT-TimerDL and drx-RetransmissionTimerDLMBS are configured if PTM scheme 2 (RAN1) is used for MBS retransmission.

Proposal 12:DRX configuration parameters for each MTCH in delivery mode 2 includes: drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS, drx-schedulingPeriodStartOffsetMBS, drx-RetransmissionTimerDLMBS, drx-HARQ-RTT-TimerDLMBS.

Proposal 13:DRX configuration parameters for each MCCH in delivery mode 2 includes: drx-onDurationTimerMBS, drx-SlotOffsetMBS, drx-InactivityTimerMBS, drx-schedulingPeriodStartOffsetMBS.

Proposal 14:In PTP transmission (RAN2), CS-RNTI is used for DL SPS activation/deactivation/retransmission.

Proposal 15:MBS DL SPS is configured with parameters g-cs-RNTI, mbs-nrofHARQ-Processes, mbs-harq-ProcID-Offset and mbs-periodicity.

Proposal 16:G-CS-RNTI is used for MBS DL SPS activation/deactivation/retransmission if PTM scheme (RAN1) is used for DL SPS initial transmission and DL SPS HARQ retransmission in both delivery mode 1 and delivery mode 2.

Proposal 17:In delivery mode 1, G-CS-RNTI is used for activation/deactivation, CS-RNTI is used for DL SPS HARQ retransmission when PTM scheme 1 (RAN1) is used for DL SPS initial transmission and PTP (RAN1) is used for DL SPS HARQ retransmission.

# References

- R2-2102503, 38.300 Running CR for MBS in NR_final, CMCC

- R2-2006952, Consideration on L2 protocol impact by MBS, Intel

- R2-2102249, Summary for MBS Group Scheduling under Agenda Item 8.1.2.4, Vivo

- R2-2101013, High layer aspects for group scheduling, Huawei

- R2-2101173, Aspects of Group Scheduling, Ericsson

- R2-2101860, Discussion on overall architecture of MBS traffic delivery, LG

- TR23.757, Study on architectural enhancement for 5G multicast-broadcast services

- R2-210xxxx, Complexity analysis for reliability enhancement in RLC and PDCP, Intel

- R2-2102838, Discussion on SA2 LS and multicast session activation, Intel