3GPP TSG-RAN WG2 #113bis-e	R2-2102881

E-meeting, April 2021

Agenda Item:	6.2.2

Source:	OPPO, Ericsson, Apple, Nokia, Nokia Shanghai Bell

Title:	Left issue on synchronization of PSSCH vs. PSFCH

Document for:	Discussion, Decision

Introduction

This is to discuss the left issues on synchronization.

Discussion

According to sync procedure of R16 NR-V2X, specified in TS 38.331 section 5.8.6, for two UEs communicating via PC5

Figure 1 Sync procedure for UEs communication via PC5

Each UE (UE1 and UE2) based on the sync source in the proximity (gNB, GNSS and UE), and the sync configuration, to derive the sync reference, of which the sync is used as the reference of Tx sync of its own:

- UE1 to derive the sync for UE1 transmission, e.g., sync-1;

- UE2 to derive the sync for UE2 transmission, e.g., sync-2;

Since besides the GNSS, the gNB/UE sync reference may be different from different UE’s proximity, so the Tx-sync of each UE may be different.

Based on the R16 NR-V2X sync procedure, it is possible that two UEs communicating via PC5 adopt different Tx-sync.

In LTE, since there is no PHY feedback, the transmission of UE1 and UE2 are independent from PHY-layer perspective, i.e., from UE1 perspective

- For the PSCCH/PSSCH transmission from UE1 to UE2, it uses UE1 Tx pool;

- For the PSCCH/PSSCH transmission from UE2 to UE1, it uses UE1 Rx pool;

The two pools are not necessarily sync-ed with each other, so even if sync-1/2 are different, the current specification is able to handle it, and therefore, there is no need to align the sync of the two UEs in a PC5 link.

While for NR, since there is PHY feedback introduced, i.e., PSFCH, there seems a need to align the sync of the two UEs in a PC5 link, i.e. from UE1 perspective .

- For the PSCCH/PSSCH transmission from UE1 to UE2, it uses UE1 Tx pool;

- For the PSFCH transmission from UE2 to UE1, it uses UE1 Tx pool;

But if sync-2 of UE2 is not the same as sync-1 of UE1, the time/frequency resources of UE1 Tx pool is not aligned with sync-2, so the current specification seems to require UE2 to perform PSFCH transmission in a time/frequency resource which is not aligned with its Tx sync (i.e., sync-2)

- For unicast, it means that when UE1 send unicast data to UE2, UE2 shall use the same sync to feedback PSFCH to UE1;

- For groupcast, it means that when UE1 send unicast data to UE2/3/4, UE2/3/4 shall use the same sync to feedback PSFCH to UE1;

However, for the transmission of PSFCH, sync difference between Tx-UE and Rx-UE(s) is not feasible.

Actually, the issue has previously already identified by RAN4, in LS of R4-1912826

RAN4 is discussing the need of requirements for sidelink synchronization when multiple asynchronized sources are presented. RAN4 noticed that RAN1 agreed the following synchronization source priority.

[…]

With the above synchronization source priority, RAN4 identifies the following scenarios in which timing misalignment may exist between UEs communicating on SL

- UE1 and UE2 synced to two different gNBs, but the 2 gNBs are with different timing

- UE1 and UE2 synced to two different eNBs, but the 2 eNBs are with different timing

- UE1 synced to eNB, UE2 synced to gNB, eNB and gNB are with different timing

RAN4 would ask RAN2 to check if there is any signalling available for timing adjustment in the above scenarios where multiple gNB/eNB with different timing are presented.

However, this LS ended up with introducing the offsetDFN, which is actually the tool for another issue, i.e., how to solve the misalignment between SL TX and UL TX, not for the misalignment between SL TX two UEs, as in LS reply by RAN1 in R1-1913696

It is RAN1 understanding that asynchronous deployment scenarios also existed in LTE-V2X. RAN1 believes that the “offsetDFN” parameter used in LTE-V2X is also necessary in NR-V2X for aligning sidelink timing in some scenarios when GNSS is used as high priority synchronization source.

RAN1 has not discussed all details of the scenarios mentioned above and based on the working assumption there is no specific signaling support for timing adjustment in these scenarios.

I.e., this issue was not really solved.

So it is worthwhile to clarify the RAN2 spec

5.8.6.1	General

The purpose of this procedure is to select a synchronisation reference and used when transmitting NR sidelink communication.

When define the sidelink sync procedure, whether / how for UE2 to perform PSFCH transmission is such case (i.e., when UE1 and UE2 are of different sync).

RAN2 to discuss whether the sidelink sync procedure specified in TS 38.331 section 5.8.6 is applicable to PSFCH transmission.

RAN2 discuss whether a LS based on the RAN2 agreement to RAN1 is needed.

Conclusion

We have the following observation:

Observation 1	Based on the R16 NR-V2X sync procedure, it is possible that two UEs communicating via PC5 adopt different Tx-sync.

Observation 2	However, for the transmission of PSFCH, sync difference between Tx-UE and Rx-UE(s) is not feasible.

We have the following proposal:

Proposal 1	RAN2 to discuss whether the sidelink sync procedure specified in TS 38.331 section 5.8.6 is applicable to PSFCH transmission.

Proposal 2	RAN2 discuss whether a LS based on the RAN2 agreement to RAN1 is needed.

Reference

- R2-2008782	Correction on CG maximum retransmission number	OPPO	CR	Rel-16	38.321	16.2.1	0892	-	F	5G_V2X_NRSL-Core