3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-2102926

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Considerations on Positioning for UEs in RRC_INACTIVE state

Agenda Item:	8.11.3

Document for:	Discussion and Decision

Introduction

Based on the revised WID on R17 NR positioning [1], the objective of positioning for UEs in RRC_INACTIVE state is:

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]: - DL NR positioning methods and RAT-independent positioning methods - Support of UE positioning measurements for UEs in RRC_INACTIVE state - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state Note: this work will be coordinated with the SDT WI. - As 2nd priority: - UL and DL+UL NR positioning methods - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

In this contribution, we would like to discuss our considerations on positioning for UEs in RRC_INACTIVE state based on revised WID.

Discussion

- DL NR positioning methods and RAT-independent positioning methods for RRC_INACTIVE UEs

DL NR positioning and RAT-independent positioning procedures with UE-based and UE-assisted mode can be divided into the following procedures:

- Location service request;

- Capability transfer;

- Assistance data transfer;

- Location information Request;

- Measurement triggering;

- UE measurement reporting.

Figure 1 Possible procedures for DL NR positioning and RAT-independent positioning

- Location service request

The serving AMF may receive location service request from 5GC LCS entities or determine a need for location service by itself when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state as the UE is in 5GMM-CONNECTED mode. The AMF may also receive location service request from the UE when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state if the UE is allowed to send signalling within RRC_INACTIVE state with the discussion in R17 SDT WI.

Then the AMF transfers location service request to a selected LMF.

Proposal 1: During DL NR positioning and RAT-independent positioning procedures for UEs, location service request may be tansferred when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state.

- Capability transfer

Before the LMF decides the positioning method/positioning mode, LMF needs to obtain UE positioning capability via LPP capability transfer procedure. Straightforward, capability transfer procedure can happen when the UE is in RRC_CONNECTED. In addition, the UE may piggback UE positioning capability with the location service request. Therefore, similar as location service request, capability can also be provided when the UE is in RRC_INACTIVE state which is in the scope of R17 SDT WI.

Proposal 2: During DL NR positioning and RAT-independent positioning procedures for UEs, capability transfer procedure may happen when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state.

- Assistance data transfer

After the LMF decides positioning method/positioning mode, LMF needs to provide assistance data to the UE. It is straightforward that assistance data transfer can happen when the UE is in RRC_CONNECTED. Due to UE mobility, assistance data may be not valid. In this case, the UE can acquires updated assistance data via broadcast if current serving cell broadcasts positioning assistance data no matter the UE is in RRC_INACTIVE, RRC_IDLE or RRC_CONNECTED state. If the UE cann’t acquire required positioning assistance data via broadcast, the UE can request positioning assisatance data via LPP message. In this case, if the UE is in RRC_CONNECTED state, the UE can send LPP RequestAssistanceData message to the LMF directly. If the UE is in RRC_INACTIVE mode, the UE can transition into RRC_CONNECTED mode first then send the LPP RequestAssistanceData message to the LMF, or the UE can transmit the RequestAssistanceData LPP message to the LMF within RRC_INACTIVE state which is in the scope of R17 SDT WI.

Proposal 3: During DL NR positioning and RAT-independent positioning procedures, assistance data transfer procedure may be in RRC_CONNECTED state, or in RRC_INACTIVE state, or in RRC_IDLE state.

- Location Information Request

After the LMF decides positioning method/positioning mode and provides assistance data to the UE, the LMF can send LPP RequestLocationInformation message to the UE. As there are some procedure exchange between the UE and LMF when RequestLocationInformation message is sent to the UE, it is more suitable for UE to keep in RRC_CONNECTED state. But if the UE piggbacks UE positioning capability with the location service request while the LMF sends RequestLocationInformation message directly to the UE as assistance data can be acquired by the UE via broadcast method, RequestLocationInformation message can also be sent to the UE within RRC_INACTIVE state as a subsequent DL SDT which is in the scope of R17 SDT WI.

Proposal 4: During DL NR positioning and RAT-independent positioning procedures, Location Information Request may be sent when the UE is in RRC_CONNECTED mode or RRC_INACTIVE state.

- Measurement triggering

The LMF sends LPP RequestLocationInformation message to the UE, which contains measurement triggering conditions. For example, in current LPP RequestLocationInformation message, periodical measurement and cell change have already been defined as measurement triggering conditions.

For UE positioning measurements in idle state for NB-IoT, the UE performs requested measurements only when the UE has entered RRC_IDLE state. However, normal UEs are also considered in NR R17 positioning enhancement. We don’t need to have the restriction that the UE performs requested measurements only when the UE is in RRC_INACTIVE state. The UE performs requested measurements whenever the measurement triggering condition is met, no matter the UE is in RRC_INACTIVE, RRC_IDLE or RRC_CONNECTED state.

Proposal 5: During DL NR positioning and RAT-independent positioning procedures, the UE performs requested measurements whenever the measurement triggering condition is met, no matter the UE is in RRC_INACTIVE, RRC_IDLE or RRC_CONNECTED state.

- UE measurement reporting

The UE can report measurement results to the network when the UE is in RRC_CONNECTED mode, or within RRC_INACTIVE mode which is in the scope of R17 SDT WI.

Proposal 6: During DL NR positioning and RAT-independent positioning procedures, UE measurement reporting may happen in RRC_CONNECTED state or in RRC_INACTIVE state.

Procedures and corresponding RRC state are summaried below.

| Procedure | RRC_IDLE | RRC_INACTIVE | RRC_CONNECTED |
| --- | --- | --- | --- |
| Location service request | X | √ | √ |
| Capability transfer | X | √ | √ |
| Assistance data transfer | √(broadcast) | √ | √ |
| Location Information Request | X | √ | √ |
| Measurement triggering | √ | √ | √ |
| UE measurement reporting | X | √ | √ |

- UL NR positioning methods for RRC_INACTIVE UEs

Some UEs who don’t have many service requirements stay in RRC_IDLE mode or RRC_INACTIVE mode in many cases. If the network receives UL signals sent by these UEs in RRC_INACTIVE mode, UE power consumption can be reduced. However, for UL NR positioning methods for RRC_INACTIVE UEs, the following issues need to be discussed:

- During SI stage, two signals for UL NR positioning methods for RRC_INACTIVE UEs were mentioned, including SRS and PRACH. It is up to RAN1 to decide what signal is supported for UL positioning in RRC_INACTIVE.

- It would be nice to have UL time sync when UEs send UL positioning signals in RRC_INACTIVE. But the issue would be better to discuss in RAN1 at first.

RAN2 will discuss further on UL NR positioning procedure for RRC_INACTIVE UEs, e.g. configuration procedure for UL signal, validity of UL signal for positioning in RRC_INACTIVE, measurement procedure, and so on, based on RAN1’s conclusions.  Hence, we propose to wait for RAN1’s conclusion on such as UL NR positioning signals and UL sync issues for UEs in RRC_INACTIVE mode.

Proposal 7: RAN2 waits for RAN1’s conclusion on such as UL NR positioning signals and UL sync issues for UEs in RRC_INACTIVE mode.

Conclusion

In this contribution, we discuss our considerations on positioning for UEs in RRC_INACTIVE state based on revised WID.

For DL NR positioning methods and RAT-independent positioning methods for RRC_INACTIVE UEs, we propose:

Proposal 1: During DL NR positioning and RAT-independent positioning procedures for UEs, location service request may be tansferred when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state.

Proposal 2: During DL NR positioning and RAT-independent positioning procedures for UEs, capability transfer procedure may happen when the UE is in RRC_CONNECTED state or in RRC_INACTIVE state.

Proposal 3: During DL NR positioning and RAT-independent positioning procedures, assistance data transfer procedure may be in RRC_CONNECTED state, or in RRC_INACTIVE state, or in RRC_IDLE state.

Proposal 4: During DL NR positioning and RAT-independent positioning procedures, Location Information Request may be sent when the UE is in RRC_CONNECTED mode or RRC_INACTIVE state.

Proposal 5: During DL NR positioning and RAT-independent positioning procedures, the UE performs requested measurements whenever the measurement triggering condition is met, no matter the UE is in RRC_INACTIVE, RRC_IDLE or RRC_CONNECTED state.

Proposal 6: During DL NR positioning and RAT-independent positioning procedures, UE measurement reporting may happen in RRC_CONNECTED state or in RRC_INACTIVE state.

Procedures and corresponding RRC state are summaried below.

| Procedure | RRC_IDLE | RRC_INACTIVE | RRC_CONNECTED |
| --- | --- | --- | --- |
| Location service request | X | √ | √ |
| Capability transfer | X | √ | √ |
| Assistance data transfer | √(broadcast) | √ | √ |
| Location Information Request | X | √ | √ |
| Measurement triggering | √ | √ | √ |
| UE measurement reporting | X | √ | √ |

For UL NR positioning methods for RRC_INACTIVE UEs, we propose:

Proposal 7: RAN2 waits for RAN1’s conclusion on such as UL NR positioning signals and UL sync issues for UEs in RRC_INACTIVE mode.

Reference

- RP-210903, Revised WID on NR Positioning Enhancements, RAN#91e