3GPP TSG-RAN WG2 #113bis-e	R2-2103002

Electronic meeting, 2021-04-12 – 2021-04-20

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	UP aspects for Layer 2 SL relay

Document for:	Discussion, Decision

# 1	Introduction

Therefore, in this paper, we discuss these key issues and analyze potential RAN2 impacts introduced by these issues.

# 2	Discussion

## 2.1	Adaptation layer

During the SI phase, it has been agreed to support the adaptation layer at the Uu interface (i.e., between relay UE and gNB on the Uu link for UE to network relay). However, RAN2 needs further discussions on whether or not the adaptation layer is also supported at the PC5 interface between remote UE and relay UE.

In our views, it is beneficial to support the adaptation layer at the PC5 interface due to the below reasons

- There may be multiple remote UEs connecting to a same relay UE, by adding the adaption layer at the PC5 interface, it is feasible to support multiplexing multiple remote UEs onto the same PC5 RLC channel.

- By supporting the adaption layer at the PC5 interface, remote UE can exchange control PDUs on link status with relay UE. Otherwise, remote UE and relay UE have to use RRC signaling for indicating link status, which is not efficient in term of signaling overhead and latency.

- Remote UE may also operate as relay UE

Support the adaptation layer at the PC5 interface between remote UE and relay UE.

In addition, the information content carried in adaptation layer header has been agreed.  At least the below information has been proposed for UE to network relay

(1) Identity of the Remote UE known by gNB and Relay UE (e.g. Remote UE ID or a local ID)

(2) Identity of End-to-End Remote UE RB

It is agreed that adaptation layer header contains at least remote UE ID and RB ID.

- Since adaptation layer is below PDCP layer of the remote UE, there will be no security protection for the adaptation layer header, which means that the UE and/or RB ID of remote UE would be disclosed during transmission between remote UE and gNB.

- In this case, a malicious UE may e.g., initiate an attack against a gNB by initiating a fake RRC connection setup or re-establishment (or basically initiating a fake random access procedure). This would cause unnecessary radio resources to be wasted so that another valid UE may be therefore blocked by the gNB since there is no available resource left.

In another case, a malicious UE may transmit some fake data to the relay UE or remote UE by knowing the UE ID, which causes the SL channel to be congested. This would cause the remote UE to be not able to transmit data in time. Unnecessary latency would be introduced to the data transfer.

There is a security risk of UE ID disclosure since there is no security protection mechanism in the adaptation layer.

It is therefore necessary for RAN2 to investigate this security issue. Since security handling is in the SA3 scope, it is suggested that RAN2 should send a LS to SA3 for confirmation of this security issue.

RAN2 sends a LS to SA3 informing SA3 of the security risk on UE ID disclosure.

If the issue is confirmed by SA3, SA3 can further develop mechanisms to address the issue.

When SA3 confirms the issue, SA2 develops mechanisms to address the security issue.

Since adaptation layer is responsible for bearer mapping and packet routing, the adaptation layer may therefore also responsible for maintaining the link. In case there is failure or event occurred on one hop, it is beneficial to inform another hop so that the UE (either remote UE or relay UE) may take proper actions to recover from the failure, or to avoid the failure or event to occur. A UE is able to achieve a fast failure indication to its peer UE using a control PDU. Another choice for the UE is to use RRC signaling (e.g., PC5-RRC signaling) to signal the failure indication to its peer UE. However, this may also increase the RRC signaling overhead.

Define control PDU in the adaptation layer.

## 2.2	Resource allocation

- For a remote UE connecting to RAN via a UE to network relay, the remote UE first transmits its data to the relay UE via sidelink, after that, the relay UE relays the data to the gNB via Uu. Since the remote UE is out of coverage, the remote UE has no direct connection to the gNB, the remote UE can obtain SL grants with SL resource allocation Mode 2, which is sufficient for Rel-17.

It is sufficient for Remote UE out of coverage to obtain SL grants with SL resource allocation Mode 2.

- In addition, for the SL transmission, a relay UE can be configured with either SL resource allocation Mode 1 or SL resource allocation Mode 2.

For SL transmissions, Relay UE can be configured with either SL resource allocation Mode 1 or SL resource allocation Mode 2.

- For the Uu link to the gNB, the relay UE can use either a dynamic grant or a configured grant. The Buffer Size field of a BSR identifies the total amount of data available according to the data volume determined at the RLC layer and the PDCP layer across all logical channels of a logical channel group after the MAC PDU has been built.

However, remote UE’s  PDCP layer and  relay UE’s lower layers including RLC and MAC are located at different UEs, therefore, the data volume of remote UE’s PDCP layer cannot  be informed  to the MAC layer directly, which would lead to a case that a BSR generated at relay UE’s MAC layer cannot contain the data volume of the remote UE’s PDCP layer. When remote UE’s PDCP PDUs are received by relay UE, relay UE sends SR and BSR to gNB for requesting UL grants. This is not delay efficient.

A Uu BSR created by Relay UE may not be able to contain data volume of Remote UE’s Uu PDCP layer in time, which may cause additional scheduling latency for remote UE.

In case remote UE has delay sensitive relay traffic, it is beneficial for remote UE to reduce scheduling latency if relay UE can trigger Uu BSR reflecting data volume of remote UE in advance of reception of the data from remote UE. So that relay UE may formulate a Uu BSR indicating both volume of the pending data (at Uu RLC layer) and coming data from the remote UE (at Uu PDCP layer).  In this way, the relay UE can trigger a pre-emptive BSR to request grants ahead of reception of the data from remote UE.

RAN2 develops solutions upon which relay UE can trigger pre-emptive Uu BSR for remote UE.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	It is agreed that adaptation layer header contains at least remote UE ID and RB ID.

Observation 2	There is a security risk of UE ID disclosure since there is no security protection mechanism in the adaptation layer.

Observation 3	For SL transmissions, Relay UE can be configured with either SL resource allocation Mode 1 or SL resource allocation Mode 2.

Observation 4	A Uu BSR created by Relay UE may not be able to contain data volume of Remote UE’s Uu PDCP layer in time, which may cause additional scheduling latency for remote UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support the adaptation layer at the PC5 interface between remote UE and relay UE.

Proposal 2	RAN2 sends a LS to SA3 informing SA3 of the security risk on UE ID disclosure.

Proposal 3	When SA3 confirms the issue, SA2 develops mechanisms to address the security issue.

Proposal 4	Define control PDU in the adaptation layer.

Proposal 5	It is sufficient for Remote UE out of coverage to obtain SL grants with SL resource allocation Mode 2.

Proposal 6	RAN2 develops solutions upon which relay UE can trigger pre-emptive Uu BSR for remote UE.

# 4 References