3GPP TSG RAN WG2 Meeting #113-bis-e                                                      R2-2103071

# Electronic meeting, Online, April 2021

Agenda item:		8.7.2

Source:			Intel Corporation

Title:		SL Relay Discovery aspects

Document for:	 	Discussion

# Introduction

In RAN#91 e-meeting, a new WI was agreed focusing on L2 and L3 based UE-to-Network relaying to support UE-to-Network coverage extension. The detailed WID is as below [1]:

Work Item objectives on aspects common to both L2 and L3: - Specify mechanisms for U2N relay discovery and (re)selection for L3 and L2 relaying [RAN2, RAN4] - Re-use LTE relay discovery and (re)selection as baseline - Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3] - Re-use LTE as baseline Work Item objectives specific to Layer-2 (L2) relaying: - Specify mechanisms for E2E, i.e. PC5 and Uu, QoS management [RAN2]: - Specify mechanisms for service continuity - Limited to intra-gNB cases [RAN2] - Specify mechanisms for U2N Adaptation layer design [RAN2] - For bearer mapping and Remote UE identification, incl. RAN related security aspects if any - Specify Control Plane procedures for U2N, including RRC connection management, system information delivery, paging mechanism and access control for Remote UE [RAN2, RAN3]

In this contribution, we focus on sidelink discovery aspects common for both L2 and L3 based U2N relay and address some open aspects at the conclusion of the study.

# Discussion

## Aspects pertaining to Relay UE

The following aspects have been captured in the TR regarding SL discovery support for U2N relay UE [2]:

For Relay UE of UE-to-Network Relay, - The Relay UE needs to be within a minimum and a maximum Uu signal strength threshold(s) if provided by gNB before it can transmit discovery message when in RRC_IDLE or in RRC_INACTIVE state. - Relay UE is allowed to transmit discovery message based on NR sidelink communication configuration provided by gNB in all RRC states. - Relay UE supporting L3 UE-to-Network Relay is allowed to transmit discovery message based on at least pre-configuration when it is connected to a gNB which is not capable of sidelink relay operation, in case its serving carrier is not shared with carrier for sidelink operation. - Relay UE supporting L2 UE-to-Network Relay should be always connected to a gNB which is capable of sidelink relay operation including providing configurations for transmission of discovery messages.

From the above, at least two open questions still remain. Firstly, given that RAN2 agreed to consider LTE discovery procedure as baseline and the relay UE shall rely on NR sidelink communication configuration for transmitting discovery messages, we need to discuss what this configuration should comprise of. Based on the conclusion in RAN, we think it is straightforward to use LTE signaling as baseline. In particular, LTE SIB19 contains the sidelink discovery relayed resource configuration information, specifically relayUE-Config containing the relevant link quality thresholds as well as resource pool configuration to be used for transmitting discovery messages. Therefore, as a starting point, we can consider the parameters contained therein to be reused for NR, i.e. the high and low RSRP thresholds and the hysteresis to indicate when the relay UE may use the indicated SL discovery resource pool for transmission of discovery messages. Note that as in LTE, these thresholds can be valid for the Relay UE in RRC_IDLE/INACTIVE or RRC_CONNECTED.

Proposal 1:	The NR sidelink communication configuration for discovery for the Relay UE includes at least the high and low RSRP link quality thresholds (and hysteresis) as in LTE.

In addition, this configuration should also include the SL resource pool configuration for discovery. Since both mode-1 and mode-2 are supported for NR sidelink operation, we assume that the same is applicable for discovery related transmissions as well. That is to say, both network scheduled and UE autonomous resource selection mechanisms shall be supported for NR sidelink discovery transmission:

- For the former, the Remote/Relay UE in RRC_CONNECTED may request and obtain resources via dedicated signaling for announcement of discovery message

- For the latter, the gNB may provide the resource pool configuration in broadcast or dedicated signaling and the UE autonomously selects discovery resources from the configured pool.

Observation 1:	Similar to non-discovery case, both network scheduled and UE autonomous resource selection mechanisms (mode 1 and mode 2) can be applicable for NR sidelink discovery transmission.

Therefore, for UEs in RRC_IDLE/RRC_INACTIVE, the gNB may or may not provide resource pool configuration for discovery message transmission via SIB or indicate that UE needs to request dedicated resources in RRC_CONNECTED. Following the design on NR sidelink non-discovery related communication, we assume that it can be applicable for discovery related communication as well, given that both shared and dedicated resource pool are applicable for relay discovery.

Proposal 2:	The resource pool configuration for transmission of discovery messages is also included within NR sidelink communication configuration for discovery by the Relay UE. The detailed configuration design and UE behaviour can follow LTE/NR operation as baseline.

## Aspects pertaining to Remote UE

As opposed to the relay UE, several open aspects need further discussion and clarification for the remote UE. Before going into further details, it is important to clarify that here we focus on discovery procedure in general, without specifically considering whether it is for new/initial relay UE or reselection of a new relay UE. Since there are additional aspects to consider for the latter case, we focus on the latter aspect in our companion contribution on relay reselection aspects [3].

For Remote UE of UE-to-Network Relay, - The Remote UE in RRC_IDLE and RRC_INACTIVE state is allowed to transmit discovery message if measured signal strength of serving cell is lower than a configured threshold. - Whether Remote UE in RRC_CONNECTED is allowed to transmit discovery is based on configuration provided by serving gNB. The detail of configuration provided by serving gNB can be discussed in WI phase. - No additional network configuration is needed for Uu measurement by Remote UE in RRC_IDLE or RRC_INACTIVE. - Remote UE out of coverage is always allowed to transmit discovery message based on pre-configuration while not connected with network through a Relay UE yet. - Remote UE supporting UE-to-Network Relay is allowed to transmit discovery message based on at least pre-configuration when it is directly connected to a gNB which is not capable of sidelink relay operation, in case its serving carrier is not shared with SL carrier. - For Remote UE supporting L3 UE-to-Network Relay which is out of coverage and connected to a gNB indirectly, it is not feasible for the serving gNB to provide radio configuration to transmit discovery message. - For Remote UE supporting L2 UE-to-Network Relay which is out of coverage and connected to a gNB indirectly, whether it is allowed to transmit discovery message based on configuration provided by the gNB can be discussed in WI phase.

As for the case of relay UE, the first and foremost aspect to consider is the detail of configuration to be provided or pre-configured to enable discovery procedure by the remote UE. It was agreed in the SI phase that the remote UE (at least in IDLE/INACTIVE) can transmit discovery messages if the serving cell signal strength is below a configured threshold. In LTE, SIB19 also contains the discovery related configuration for the remote UE (SL-DiscConfigRemoteUE), including the RSRP threshold and associated hysteresis range, which can be leveraged for NR discovery operation.

Proposal 3:	The NR sidelink communication configuration for initiating relay discovery by the Remote UE includes at least the low RSRP link quality threshold (and hysteresis) as in LTE.

Moreover, once the remote UE has met the conditions to initiate discovery for a suitable Relay UE, it needs to then evaluate the candidate relay UEs and the corresponding configuration needs to be defined. In LTE, the remote UE compares the SD-RSRP with a configured threshold after performing L3 filtered measurement over the sidelink discovery channel [4]. For NR sidelink, we think that the same principle can be applicable (even if there is no dedicated channel for discovery) based on at least the discovery messages (using model A/B). Therefore, the RSRP threshold (and associated hysteresis) for selecting a suitable candidate relay UE can be defined similar to LTE. Note that this does not preclude the use of additional AS layer criteria when selecting a suitable relay UE, as discussed in [3].

Proposal 4:	RAN2 agrees that after initiating relay discovery, the remote UE evaluates candidate relay UEs based on filtered RSRP measurements of the discovery message to select a suitable relay, using LTE as baseline.

Furthermore, the details of remote UE behavior and how the necessary configuration for discovery procedure in case of being out of direct network coverage also need to be discussed. As per the TR, at least four different sub-scenarios can be identified for the out of coverage case [2]:

|  | Connection status for OOC remote UE | Remote UE behavior |
| --- | --- | --- |
| 1 | Not connected to any gNB (directly or indirectly) | Remote UE can rely on pre-configured discovery related configuration for discovery of relay UE |
| 2 | Connected indirectly to non-relay capable gNB | Remote UE can rely on pre-configured discovery related configuration for discovery of relay UE, assuming the serving carrier is not shared with SL carrier |
| 3 | Connected indirectly to gNB via L3-relay | Not feasible for the gNB to provide configuration for discovery |
| 4 | Connected indirectly to gNB via L2-relay | FFS whether UE is allowed to perform discovery based on configuration provided by gNB |

At least the following can be gleaned from the table above:

For scenario 3, given that the gNB cannot provide the configuration for discovery, there is no other way for the remote UE to be able to obtain this configuration for discovering/re-selecting a new Relay UE other than to rely on pre-configuration. This is similar to the case in scenarios 1 and 2 as well.

For scenario 4, on the one hand, it seems more flexible to allow the gNB to explicitly provide resource pool and related configuration for discovery to the remote UE and in doing so, more directly control the remote UE behavior in this case. On the other hand, it may be preferrable to have a common behavior with the three scenarios above, i.e. relying on pre-configuration. In our view, both options can be supported, i.e. the gNB can provide the discovery configuration to an (indirectly) connected L2 remote UE, via SIB and/or dedicated signaling. Note that since SIB forwarding is supported as per the SI conclusion, it can leverage to provide this configuration to the connected remote UE. In case it is not provided, same behavior as in scenario 2 can be supported, i.e. the remote UE relies on pre-configuration for SL discovery, assuming the serving carrier is not shared with the SL carrier.

Proposal 5:	For OOC Remote UE connected to L3 UE-to-Network Relay and connected to a gNB indirectly, it can rely on pre-configuration to obtain discovery related configuration.

Proposal 6:	For L2 based relay, an out of coverage Remote UE indirectly connected to the network via a Relay UE can obtain SL discovery related configuration from the network for transmission of discovery message. In case it is not provided, the remote UE can rely on pre-configuration assuming the serving carrier is not shared with the SL carrier

# Conclusion

This contribution discusses the sidelink relay discovery related aspects and makes the following observations and proposals:

Observation 1:	Similar to non-discovery case, both network scheduled and UE autonomous resource selection mechanisms (mode 1 and mode 2) can be applicable for NR sidelink discovery transmission.

Proposal 1:	The NR sidelink communication configuration for discovery for the Relay UE includes at least the high and low RSRP link quality thresholds (and hysteresis) as in LTE.

Proposal 2:	The resource pool configuration for transmission of discovery messages is also included within NR sidelink communication configuration for discovery by the Relay UE. The detailed configuration design and UE behaviour can follow LTE/NR operation as baseline.

Proposal 3:	The NR sidelink communication configuration for initiating relay discovery by the Remote UE includes at least the low RSRP link quality threshold (and hysteresis) as in LTE.

Proposal 4:	RAN2 agrees that after initiating relay discovery, the remote UE evaluates candidate relay UEs based on filtered RSRP measurements of the discovery message to select a suitable relay, using LTE as baseline.

Proposal 5:	For OOC Remote UE connected to L3 UE-to-Network Relay and connected to a gNB indirectly, it can rely on pre-configuration to obtain discovery related configuration.

Proposal 6:	For L2 based relay, an out of coverage Remote UE indirectly connected to the network via a Relay UE can obtain SL discovery related configuration from the network for transmission of discovery message. In case it is not provided, the remote UE can rely on pre-configuration assuming the serving carrier is not shared with the SL carrier.

# References

| [1] | "RP-210904, New WID on NR Sidelink Relay, OPPO/Ericsson". |
| --- | --- |
| [2] | "TR 38.836, Study on NR Sidelink Relay (Release 17)". |
| [3] | "R2-2103739, Discussion on SL Relay (re)selection, Intel Corporation". |
| [4] | "TS 36.331, LTE; Radio Resource Control (RRC); Protocol specification". |