3GPP TSG-RAN WG2 Meeting #113bis-e                                             R2-2103092

Electronic, 12th - 20th April, 2021

Source:	CATT

Title:	TX Resource (Re-)selection with HARQ FB Consideration

Agenda Item:	6.2.3

Document for:	Discussion and Decision

Introduction

RAN1 has agreed that the minimum time gap between any two selected resources for a given TB transmission is ensured only when HARQ-ACK feedback is enabled for the TB transmission. However, TS 38.321 captured it as ‘ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources.  Some company proposed CR to change the condition from PSFCH is configured to sl-HARQ-FeedbackEnabled has been set to enabled for logical channel(s). This issue has been discussed in email discussion in last meeting [1], two options have been proposed:

Proposal 6: RAN2 to make a decision which following option should be adopted for WF - Option 1: Keep the current specification and send a LS to RAN1 to explain the technical concern if we capture their agreement in MAC. Proposed CR in R2-2102260 is not pursued. FFS the exact content of the LS. - Option 3: Agree “in case that sl-HARQ-FeedbackEnabled has been set to enabled for the logical channel” for both single MAC PDU and multiple MAC PDUs, without changing the existing LCP. And this issue is closed.

In this contribution, which one of the above two options, option 1 or option 3 should be adopted  will be further discussed.

Discussion

Current section 5.22.1.1 in MAC specification, when UE selects resources, it should first decide whether create a selected sidelink grant corresponding to transmissions of single MAC PDU or multiple MAC PDUs.,

The following analysis for this issue is performed separately for single and multiple MAC PDUs.

- Single MAC PDU

Before UE selects resources, it selects a resource pool allowed for the logical channel, if sl-HARQ-FeedbackEnabled is set to enabled for the logical channel, UE shall select any pool of resources configured with PSFCH resources among the available pools; else, it selects any pool among the available pools. In the combination of HARQ FB attribute of logical channels and whether the PSFCH is configure for resource pools, only the following three cases are supported which are shown in the following Table-1:

Table-1 Combination of LCH HARQ FB attribute and resource pool PSFCH configuration

|  | PSFCH is configured | PSFCH is not configured |
| --- | --- | --- |
| LCH HARQ FB enable | Support | Do not Support |
| LCH HARQ FB disable | Support | Support |

For single MAC PDU, the performance of the two options mentioned on the last RAN2 meeting is same for the following two cases (which were marked with yellow in Table-1):

- when PSFCH is configured and sl-HARQ-FeedbackEnabled is set to enabled for the logical channel; or

- when PSFCH is not configured and sl-HARQ-FeedbackEnabled is set to disabled for the logical channel;

But the performance of the two options is different for the cases marked with purple. In case of PSFCH is configured and sl-HARQ-FeedbackEnabled is set to disabled for the logical channel. If Option 1 is used, it will introduce an unnecessary minimum gap between two selected resources. This can be avoided by Option 3.

The above analysis can be summarized in the following Table-2:

Table-2 Comparison for the two options for single MAC PDU

| Resource Pool | HARQ FB attribution of the logical channel | Option1 | Option3 |
| --- | --- | --- | --- |
| PSFCH is configured | Enable | Work well | Work well |
|  | Disable | Work Unnecessary minimum gap will be introduced. | Work well |
| PSFCH is not configured | disable | Work well | Work well |

- Multiple MAC PDUs

Figure-1 Resource reservation for multiple MAC PDUs

For Multiple MAC PDU transmission, we can assume this happens in periodical resource reservation when there is already data transmission ongoing for one logical channel with periodical traffic pattern. If new data arrival of another logical channel arrives, e.g., arrives before grant #3, the newly arrived data may be multiplexed with the legacy data in grant #3 and transmitted on these selected resources.

To compare the above two options, the following 5 cases may be considered:

- Case1: PSFCH is configured; the HARQ FB attributes of the first and the subsequent MAC PDUs are both set to enabled.

- Case2: PSFCH is configured; the HARQ FB attributes of the first MAC PDU is set to enabled and the subsequent is set to disabled.

- Case3: PSFCH is configured; the HARQ FB attributes of the first and the subsequent MAC PDUs are both set to disabled.

- Case4: PSFCH is configured; the HARQ FB attributes of the first MAC PDU is set to disabled and the subsequent is set to enabled.

- Case5: PSFCH is not configured; the HARQ FB attributes of the first and the subsequent MAC PDU are both set to disabled.

Note: First MAC PDU refers the MAC PDU sent in grant #1 and grant #2 and subsequent MAC PDUs refers to the MAC PDU sent in grant# 3 and consecutive grant after grant #3.

Similar as the analysis in single MAC PDU, for case 1, case 3 and case5, both of the proposed option 1 and option 3 in the last meeting can work. But for case2, option 1 would introduce unnecessary minimum time gap.  And for case 4, if option 1 is used, nothing needs to be enhanced; while if option 3 is used, resource reselection should be performed. The above analysis can be summarized in Table-3:

Table-3 Comparison for the two options for multiple MAC PDUs

| Resource Pool | The HARQ FB of the first MAC PDU | The HARQ FB of subsequent MAC PDU | Option1 | Option 3 | Case # |
| --- | --- | --- | --- | --- | --- |
| PSFCH is configured | Enable | Enable | Work well | Work well | 1 |
|  |  | Disable | Work Unnecessary minimum gap will be introduced | Work well | 2 |
|  | Disable | Disable | Work Unnecessary minimum gap will always be introduced | Work well | 3 |
|  |  | Enable | Work well | Resource reselection should be triggered | 4 |
| PSFCH is not configured | Disable | Disable | Work well | Work well | 5 |

According to the above Table-2 and Table-3, both option 1 and option 3 can work and both have disadvantages:

- The disadvantage of option 1 is that it may introduce some latency in case of the HARQ feedback attribute of the MAC PDU is changed from enabled to disabled in case of PSFCH is configured;

- The disadvantage of option 3 is that resource reselection should be triggered if the HARQ feedback attribute of the MAC PDU is changed from disable to enabled.

Considering the Rel-16 is frozen, since option 1 has no impact on RAN2 spec and latency is not a big issue, while option 3 will introduce new resource triggers in certain cases. Hence, it had better keep the current RAN2 text as it is.

Proposal 1: Keep the current specification as it is, that is UE ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for the selected resource pool.

Proposal2: Send LS to RAN1 includes:

- Resource selection based on HARQ FB attribute of TB will introduce new trigger to resource reselection.

- Ask RAN1 if they could change their agreement.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Keep the current specification as it is, that is UE ensuring the minimum time gap between any two selected resources in case that PSFCH is configured for the selected resource pool.

Proposal2: Send LS to RAN1 includes:

- Resource selection based on HARQ FB attribute of TB will introduce new trigger to resource reselection.

- Ask RAN1 if they could change their agreement.

Reference

- R2-2102193	 Summary of [AT113-e][713][V2X/SL] TX resource (re)selection w/ HARQ feedback consideration  vivo