3GPP TSG-RAN WG2 Meeting #113bis electronic                                            R2-2103106

Online, April 12 – April 20, 2021

Source:	CATT

Title:	Discussion on Deactivation of SCG

Agenda Item:	8.2.2.1

Document for:	Discussion and Decision

Introduction

In RAN2#113e meeting, the SCG activation and deactivation has been discussed, the following agreements were made [1]:

- Agreements

- 1a 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 1b 	SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

- 3 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

- Agreements

- 1	NW-triggered SCG activation is indicated to the UE via the MCG.

- 9	NW-triggered SCG deactivation can be indicated to the UE via the MCG. FFS via SCG.

In this contribution, we will discuss on SCG deactivation in detail including how MN/SN request for SCG deactivation works and whether the request can be rejected.

Discussion

In RAN2#113e meeting, the agreements were made that SCG deactivation can be requested by MN/SN and RRC signaling is defined for the interaction between UE/MN and MN/SN in SCG deactivation. That is the SCG deactivation command can be indicated to the UE via the MCG. However, the details of the SCG deactivation procedure have not been discussed. It was agreed the SCG state can be set to be activated/deactivated at the PSCell addition/change, RRC resume and HO, how MN and SN request the SCG deactivation and the interaction between MN and SN can be discussed based on different scenarios.

2.1 MN/SN request for SCG deactivation

- MN requests SCG deactivation during SN addition procedure

Currently, when MN adds a secondary node, the legacy state of the new added SCG is activated state. When the MN adds a new SCG, considering there is no data transmission ongoing at SCG side at this moment, MN can decide to set the SCG to be deactivated state. In this case, there seems no reason for SN to reject the SCG deactivation request. Even though data transmission arrives on the SN, SN can request to active the SCG after the completion of the SN addition procedure. So for SN addition procedure, it is no need to support rejecting the SCG deactivation request for SN.

Observation 1: For SN addition procedure, there is no data transmission ongoing on SN side. Hence there is no need for SN to reject the SN deactivation request. If data transmission arrival on SN occurs during the SN addition procedure, SN can request to active SCG after complete the SN addition procedure.

Proposal 1: SN cannot reject the SCG deactivation request during SN addition procedure.

- MN requests SCG deactivation during SN change procedure

In current TS 37.340 spec, both MN and SN can initiate the SN change procedure, MN will send the SN addition request message to the target SN, then target SN will reply a SN addition response message to the MN. During SN change procedure, the MN can request the target SN to deactivate SCG. .

When a UE is configured MRDC, the UE can transfer data at both MCG side and SCG side. There are three possible cases when execute the SN change procedure:

Case1: no data is being transmitted at SCG RLC bearer, source SCG is in deactivated state;

Case2: no data is being transmitted at SCG RLC bearer, source SCG is in activated state;

Case3: data is being transmitted at SCG RLC bearer, source SCG is in activated state;

For case1, there is no data being transmitted at SCG bearer while SN change and the source SCG is deactivated SCG, there seems no reason to reject the deactivated SCG request for target SN. For case2, there is no data being transmitted at SCG bearer while SN change and source SCG is activated SCG, considering no data transmission is ongoing on target SN, similarly with SN addition procedure, it seems to be no problem setting the new added SCG to be deactivated SCG. However, for case3, when data is being transmitted at SCG RLC bearer, the data forwarding may be executed if the SN change procedure is initiated. Firstly we can assume the NW can make smart decision on this case. Even the NW still set the target SN to be deactivated state with some DRB transmission on target SN, the target SN can request to active the SCG after the completion of SN change procedure. So based on the above discussion, there is no need to allow the SN reject the deactivated SCG state upon SN change procedure.

Proposal 2: The target SN cannot reject the SCG deactivation request during SN change procedure.

- MN/SN request SCG deactivation without SN addition/change

In this scenario, both MN and SN can request the SCG deactivation. MN does not know whether data is being transmitted or waiting for transmission on SCG side for the DRB which has a PDCP located in SCG. When MN sends SCG deactivation indication to SN and there is data transmission ongoing on SCG side at the moment, if SN accepts deactivation SCG, the data transmission will be interrupted which could cause the loss of transmission data, however, if SN rejects the deactivation command, it seems to may go against the intention of the MN making the decision e.g. for power saving.

Observation 2: When MN sends SCG deactivation indication to SN and there is data transmission ongoing on SCG side at the moment, if SN accepts deactivation SCG, the data transmission will be interrupted which could cause the loss of transmission data, however, if SN rejects the command, it seems to may go against the intention of the MN making the decision e.g. for power saving

We think the following two methods may solve the question mentioned above:

- Method 1: Inform each other of the data transmission status upon without/with data transmission on SN side

- Method 2: SN provides assistant information about SCG data transmission to MN responded to the deactivation SCG request

Based on method 1, when the SN finishes the data transmission on SN side for the DRB which has PDCP located in SN, the SN informs MN about the data transmission end.  Similarly, when the SN starts data transmission on SCG bearer again, the SN informs MN about the data transmission on SCG bearer started. In this way, MN can get acknowledge of the data transmission state of on SN side and make smart decision on deactivate SCG. MN can decide whether to deactivate the SCG based on the information from SN and the data transmission status of DRB which has PDCP located in MN and has RLC bearer in SN.

Method 2 may need a negotiation between MN and SN. When MN sends SCG deactivation request to SN, if data transmission is ongoing on SCG side, SN can send the assistant information such as the DRB status information, SCG status suggestion to MN upon reception of the SCG deactivation request. When MN receives the assistant information from SN, the MN can make decision whether to postpone the SCG deactivation or reconfigure the radio bearer and continue the SCG deactivation procedure.

Based on the above discussion, we can see there is also no need to reject the SCG deactivation request for SN. As to resolve the problem of data transmission in ongoing on SN side, the above 2 methods can be taken into consideration to help the network make further decision.

Proposal 3: The target SN cannot reject the SCG deactivation request from MN without the SN addition/change.

Similarly, if the SN deactivation is initiated by SN, above 1 method can also be taken into consideration, MN provides the data transmission state to SN for the DRB which has PDCP located in the MN and has RLC bearer configured in SN to assistant the SN makes the decision. Or method 2 is used to negotiate with SN for the decision of SCG deactivation.

Proposal 4: RAN2 should discuss which method to be adopted to help the MN/SN determine to deactivate SCG:

- Method 1: Inform each other of the data transmission status upon without/with data transmission on SN side

- Method 2: SN provides assistant information about SCG data transmission to MN responded to the deactivation SCG request

2.2 Whether SCG deactivation can be indicated to the UE via the SCG

In RAN2#113e meeting, it was agreed that the NW-trigged SCG deactivation can be indicated to the UE via the MCG. Whether via SCG is FFS. The advantage of sending SCG deactivation command to UE via SCG is not obvious. On one hand, the SN always needs to inform MN when the SN sends SCG deactivation command to UE. It doesn’t seem to be much faster than the deactivation command sent to UE by MN. On the other hand, the scenario is limited for SCG deactivation command sent to UE via SN. It only applies to all the PDCP associated with the SCG RLC bearers located in SN sides and the SRB3 is configured. Therefore, it is more simply and uniform that MN makes the decision and the SCG deactivation command can only be sent to UE by MN.

Proposal 5: NW-trigged SCG deactivation cannot be indicated to the UE via the SCG.

Conclusion

In this contribution, we discuss the deactivation of a SCG, and propose:

Observation 1: For SN addition procedure, there is no data transmission ongoing on SN side. Hence there is no need for SN to reject the SN deactivation request. If data transmission arrival on SN occurs during the SN addition procedure, SN can request to active SCG after complete the SN addition procedure.

Observation 2: When MN sends SCG deactivation indication to SN and there is data transmission ongoing on SCG side at the moment, if SN accepts deactivation SCG, the data transmission will be interrupted which could cause the loss of transmission data, however, if SN rejects the command, it seems to may go against the intention of the MN making the decision e.g. for power saving

Proposal 1: SN cannot reject the SCG deactivation request during SN addition procedure.

Proposal 2: The target SN cannot reject the SCG deactivation request during SN change procedure.

Proposal 3: The target SN cannot reject the SCG deactivation request from MN without the SN addition/change.

Proposal 4: RAN2 should discuss which method to be adopted to help the MN/SN determine to deactivate SCG:

- Method 1: Inform each other of the data transmission status upon without/with data transmission on SN side

- Method 2: SN provides assistant information about SCG data transmission to MN responded to the deactivation SCG request

Proposal 5: NW-trigged SCG deactivation cannot be indicated to the UE via the SCG.

Reference

- RAN2-113e LTE DCCA Mobility RAN slicing and Multi-SIM (Tero)_2021_02_05_EOM;