3GPP TSG-RAN WG2 Meeting #113 bis electronic                      	             R2-2103166

Online, April 12 – April 20, 2021		revision of R2-2100701

Agenda Item:	8.13.3.1

Source: 	vivo

Title:         	Support of immediate MDT in MR-DC

Document for: 	Discussion and Decision

- Introduction

At the first meeting (RAN2#111-e) of Rel-17, the following was agreed [1]:

=> Study the support of logged and Immediate MDT in MR-DC scenario. For M5/M6/M7, it is proposed to apply them for EN-DC/MR-DC cases with different bear types. FFS on details.

At RAN2#113-e meeting, the support of M5~M7 in MR-DC scenarios made the following progress:

- In case split bearer data goes through Xn/X2 interface, the delay over Xn/X2 interface should be taken into account in M6 for split bearers. - D3 is re-used to reflect the DL delay on F1-U/X2/Xn, D2.3 is re-used to reflect the UL delay on F1-U/X2/Xn, LS to RAN3 for further confirmation. - The delay over Xn/X2/F1-U interface should be taken into account in M6 for MN terminated SCG bearers and SN terminated MCG bearers. - For QoS monitoring related delay reporting to CN, the minimum value between two legs is defined as the total delay measurement M6 over MCG/SCG for split bearers WITH PDCP duplication. - For QoS monitoring related delay reporting to CN, the delay estimation coordination (forwarding) between MN and SN is needed for split bearers. - For QoS monitoring related delay reporting to CN, the delay estimation coordination (forwarding) between MN and SN is needed for MN terminated SCG bearers and SN terminated MCG bearers.

But the support of immediate MDT in MR-DC scenario has not been discussed thoroughly. In this contribution, we would like to focus on this issue.

- Discussion

Immediate MDT in EN-DC is supported in Rel-16, UE can be configured with immediate MDT in CONNECTED state, where the configurations are independently configured by MN and SN. As far as we are concerned, no issues are foreseen if RAN2 to support immediate MDT in MR-DC with the same principle applied for EN-DC.

The principle applied for immediate MDT in EN-DC can be reused for the support in other MR-DC cases.

- For signalling-based and management-based immediate MDT in EN-DC, the configurations are provided by MME and OAM respectively as described in TS 37.320 [2].

In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR). In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT.

The same principle for configuration distribution can also be applied for MR-DC. For example, in signalling-based immediate MDT, AMF provides MDT configurations for both MN and SN; in management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently.

However, an issue is identified for the MDT configurations of signalling-based immediate MDT. In EN-DC, MME provides two different MDT configurations for MN and SN, respectively. The issue is, in NR-DC, whether the AMF should follow the legacy procedure to deliver two different configurations for both MN and SN, or only deliver the same configuration for both MN and SN. From our perspective, this scenario is similar to the measurement configurations sent to UE in MR-DC, without the optimization purposes for SON. Both MN and SN will configure an independent measurement configuration to UE even that the MN and SN are of the same RAT. Consequently, we believe it is still necessary for the AMF to deliver two MDT configurations for both MN and SN in the case of NR-DC, NE-DC and NGEN-DC.

RAN2 to agree the following text proposal for the support of signalling-based and management-based immediate MDT in MR-DC.

*<Text Proposal for NR-DC, NE-DC and NGEN-DC>* In signalling based immediate MDT, AMF provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration. MN then forwards the NR MDT configuration towards SN. In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT.

Since RAN3 is also involved in the configuration process, an LS should be sent to RAN3 for further confirmation on this issue, therefore we propose:

LS to RAN3 that RAN2 agrees to support immediate MDT in MR-DC and confirm the solution for the support of signalling-based and management-based immediate MDT in MR-DC.

- Conclusion

In this paper, the following observations and proposal are given:

Proposal 1	The principle applied for immediate MDT in EN-DC can be reused for the support in other MR-DC cases.

Proposal 2	RAN2 to agree the following text proposal for the support of signalling-based and management-based immediate MDT in MR-DC.

Proposal 3	LS to RAN3 that RAN2 agrees to support immediate MDT in MR-DC and confirm the solution for the support of signalling-based and management-based immediate MDT in MR-DC.

- References

- Chairman notes, 3GPP WG2 Meeting #111-e, Electronic meeting, August 17th – 28th, 2020.

- 3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA), Evolved Universal Terrestrial Radio Access (E-UTRA) and New Radio (NR); Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; Stage 2".