3GPP TSG-RAN WG2 Meeting #113 bis electronic				R2-2103264
Online, April 12 – April 20, 2021

Agenda item:		8.6.4

Source:			Asia Pacific Telecom, FGI

Title:		PDCCH monitoring in subsequent data transmission period

Document for:	 	Discussion and decision

Introduction

From previous agreements, it is known that the UE can keep in the RRC_INACTIVE state to transmit the small UL/DL data after successful completion of the RA procedure in the SDT procedure. The period between RA completion to the end of the RA-SDT procedure can be referred to as the subsequent data transmission period. In the subsequent data transmission period, the UE should monitor the PDCCH to receive the potential scheduling from NW. However, how to monitor the PDCCH in the subsequent data transmission period is unclear. Thus, we provide some options and our views on this issue.

Agreement in RAN2#111 - UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported - When UE is in RRC_INACTIVE, it should be possible to send multiple UL and DL packets as part of the same SDT mechanism and without transitioning to RRC_CONNECTED on dedicated grant. FFS on details and whether any indication to network is needed. Agreement in RAN2#112 - For RACH based solutions, upon successful completion of contention resolution, the UE shall monitor the C-RNTI. Agreement in RAN2#113 - RAN2 design assumes that RRCRelease message is sent at the end to terminate the SDT procedure from RRC point of view. The RRCRelease sent at the end of the SDT may contain the CG resource (as per previous agreement). Write an LS to SA3 to explain SDT procedure and agreement.

Discussion

RA-SDT procedure

Figure 1 illustrates a flow for the RA-SDT procedure. When the UE in RRC_INACTIVE has UL small data available for transmission, and the criteria of CG-SDT are not satisfied, the UE will initiate a RA procedure in the SDT procedure. The UE transmits the preamble/PRACH resource for SDT, via Msg1/MsgA. Then the UE transmits an RRC resume request message, through Msg3/MsgA PUSCH. Once the MSG3/MSGA has been transmitted, the UE should monitor the PDCCH to receive the Msg4/MsgB for contention resolution. After successful completion of the RA procedure, the UE should further monitor PDCCH to receive the possible scheduling for subsequent data transmission if the SDT procedure hasn’t been terminated. In the end, the NW can send an RRC release message to terminate the SDT procedure. The period between RA completion to the end of the RA-SDT procedure can be referred to as the subsequent data transmission period. However, how to monitor the PDCCH in the subsequent data transmission period is unclear.

Observation 1: How to monitor the PDCCH in the subsequent data transmission period is unclear.

Figure 1: RA-SDT procedure

PDCCH monitoring in subsequent data transmission period

In the subsequent data transmission period, for PDCCH monitoring, three options can be taken into account.

Option 1 is to keep monitoring the PDCCH in the subsequent data transmission period, i.e., the UE should continuously monitor the PDCCH after successful completion of the RA procedure until the end of the SDT procedure. The benefit of this option is to increase the scheduling opportunity. The NW can schedule the UE with UL and/or DL data at any moment without limitation. However, the shortcoming is power consumption due to constant monitoring and blind decoding.

Option 2 is to monitor the PDCCH based on a timer or window. For example, the UE can start a timer upon successful completion of the RA procedure. When the UE receives a PDCCH or performs a DL/UL transmission, the UE can restart the timer. While the timer is running, the UE should monitor the PDCCH. The timer is stopped if the SDT procedure is terminated. Option 2 is similar to option 1 since the UE will only monitor the PDCCH while the timer is running. The NW may need to keep scheduling the UE to prolong the running timer. Otherwise, if the timer expires, the UE will stop monitoring PDCCH, then the NW cannot send the RRC release to terminate the SDT procedure. As a result, the UE can only terminate the SDT procedure by the implicit way, e.g., new/legacy T319 expiry.

Option 3 is to introduce the DRX mechanism in SDT. For example, the concept of DRX Active Time can be reused.  Since the scheduling behaviors in the subsequent data transmission period is somewhat similar to the normal scheduling in the RRC_CONNECTED state, which DRX timers, e.g., drx-ondurationTimer, drx-inactivityTimer, drx-retransmisionTimer, and/or drx-HARQ-RTTTimer, is needed for SDT can be further studied. In option 3, the NW can be aware of when the UE is in Active Time, thus the NW can send the RRC release message to terminate the SDT procedure while the UE is in Active Time. The main advantage of this option is power efficiency.

Figure 2: PDCCH monitoring in subsequent data transmission period

Proposal 1: RAN2 down selects one of the following options for PDCCH monitoring in the subsequent data transmission period (i.e., after successful completion of the RA procedure until the end of the SDT procedure):

- Option 1: UE continuously monitors the PDCCH after successful completion of the RA procedure until the end of the SDT procedure.

- Option 2: UE monitors the PDCCH while a timer is running.

  - The timer is started upon successful completion of the RA procedure.

  - The timer is restarted when the UE receives a PDCCH or performs a DL/UL transmission.

  - The timer is stopped if the SDT procedure is terminated.

- Option 3: UE monitors the PDCCH based on DRX mechanism. FFS how to specify the DRX mechanism for SDT.

Conclusion

Based on the above description, we have the following observation as well as a proposal.

Observation 1: How to monitor the PDCCH in the subsequent data transmission period is unclear.

Proposal 1: RAN2 down selects one of the following options for PDCCH monitoring in the subsequent data transmission period (i.e., after successful completion of the RA procedure until the end of the SDT procedure):

- Option 1: UE continuously monitors the PDCCH after successful completion of the RA procedure until the end of the SDT procedure.

- Option 2: UE monitors the PDCCH while a timer is running.

  - The timer is started upon successful completion of the RA procedure.

  - The timer is restarted when the UE receives a PDCCH or performs a DL/UL transmission.

  - The timer is stopped if the SDT procedure is terminated.

- Option 3: UE monitors the PDCCH based on DRX mechanism. FFS how to specify the DRX mechanism for SDT.