3GPP TSG-RAN WG2 Meeting #113bis-e		R2-2103313

Electronic, 12th Apr. – 20th Apr. 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | xxxx | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.306 for the capability of supporting txDiversity-r16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN4#98e meeting, an LS on signalling scheme of transparent TxD was agreed and sent to RAN2 in R2-2102646. In the LS, RAN4 has agreed to introduce a new per-band capability signaling in Rel-16 for FR1 UEs supporting transparent TxD. Thus, RAN2 should introduce the corresponding capability according to RAN4 conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the section of BandNR parameters, introduce a new Rel-16 capability of txDiversity-r16 and the corresponding description. Based on RAN4 conclusion, it should be per-band capability. And the current conclusion is only applied to FR1. Impact analysis Impacted 5G architecture options: (NG)EN-DC, NR SA, NE-DC, NR-DC Impacted functionality: PC2 Inter-operability: 1. If the network is implemented according to the CR and the UE is not, the UE cannot report the transparent txDiversity capability. 2. If the UE is implemented according to the CR and the network is not, there is no compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no txDiversity capability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR xxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ----Omit----- |  |  |  |  |
| txDiversity-r16 Indicates whether the UE supports transparent transmitter diversity (as specified in TS 38.101-1 [2]). | Band | No | N/A | FR1 only |
| ----Omit----- |  |  |  |  |

End of change