3GPP TSG-RAN WG2 Meeting #113-bis electronic	R2-2103392

Online, April 12th – 20th, 2021

Title: 	Discussion on DAPS for IAB network

Source: 	Lenovo, Motorola Mobility

Agenda Item:	8.4.3

Document for:	Discussion and decision

- Introduction

In the RAN3 110 and 111 e-meetings, DASP-like solutions were discussed by RAN3 and following agreements were achieved [1][2].

Agreement of RAN3 110 e-meeting: Discuss how to support simultaneous connectivity with 2 donors, to reduce service interruption; potential solutions may include dual-protocol-stack solutions (“DAPS-like”); FFS whether the same solution also applies to descendant nodes The simultaneous connectivity dual-protocol-stack solutions (“DAPS-like”) of an IAB node should allow at least DL simultaneous transmission of BH traffic carried on BH RLC channels, on the paths to both donors. Agreement of RAN3 111 e-meeting: RAN3 further studies “DAPS-like” solution after RAN2 has conclusions

And from the RAN2 point of view, DAPS-like solutions for IAB were also analyzed [3] responding to the Liaison from RAN3. However, there is no evident progress till last RAN2 and RAN3 e-meetings. In this contribution, DAPS-like mechanism for IAB is further discussed, including the motivation, protocol stack, and UE impacts.

- Discussion

In R16, DAPS was introduced for UE handover in order for reduction of service interruption. And in case of DAPS handover, UE maintains the source gNB connection after reception of RRC message for handover and until releasing the source cell after successful random access to the target gNB. Which means UE has the connections to both gNBs and can at least receive user data from the two gNBs simultaneously.

Observation 1: DAPS for UE handover is introduced for reduction of service interruption, at least for DL.

Before discussion for the detail IAB DAPS solutions, we need to firstly clarify the use case for IAB DAPS and the difference between IAB DAPS and IAB topology redundancy.

Like the UE DAPS, DAPS for IAB is introduced during the discussion for IAB migration. From the motivation point of view, IAB DAPS is similar to UE, which focus on the service interruption reduction during handover. While load balance and robustness are the main characters for dual connectivity. In addition, DAPS is the short-term state during handover, while topology redundancy may maintain for a long period.

Proposal 1: IAB DAPS is introduced for service interruption reduction during IAB migration, rather than load balance and robustness.

As for the protocol stack for the IAB DAPS, since PDCP is terminated at UE and IAB-donor, there is no PDCP sublayer for an IAB-node. To use UE DAPS as reference, two separate sets of RLC/MAC/PHYs are needed and they are toward to source and target parent node respectively. While for the BAP sublayer, common BAP and separate BAPs are both logically feasible, but common BAP may be preferred with fewer impacts on TS 38.473. An example of simplified protocol stack for IAB DAPS, where GTP-U/UDP/IP are omitted, is shown in Figure 1.

Proposal 2: For the protocol stack of IAB DAPS, two separate sets of RLC/MAC/PHYs and common BAP sublayer are used for the DAPS IAB-node.

Figure 1. An example of simplified protocol stack for IAB DAPS

For UE DAPS, common PDCP layer is used for both gNBs but the PDCP entity is configured with two sets of security functions and keys and each set is associated with one gNB. For DAPS bearers, the PDCP entity shall perform the ciphering or deciphering for the PDCP SDU using the ciphering algorithm and key either configured for the source cell or configured for the target cell, based on to/from which cell the PDCP SDU is transmitted/received.

Since IAB-node in Figure 1 has the DAPS toward source and target IAB-donors, descendant UE of the IAB-node may receive the DL data packets from both source and target IAB-donors, or transmit the UL data packets to both IAB-donors. Therefore, the UE needs to be configured with two sets of security functions so that it can process the data packets from/to both IAB-donors.

Protocol 3: Descendant UE of the DAPS IAB-node needs to be configured with two sets of security functions so that it can process the data packets from/to both IAB-donors.

# Conclusion

This contribution aims to analyze the DAPS-like mechanism for IAB. And following observations and proposals are concluded.

Observation 1: DAPS for UE handover is introduced for reduction of service interruption, at least for DL.

Proposal 1: IAB DAPS is introduced for service interruption reduction during IAB migration, rather than load balance and robustness.

Proposal 2: For the protocol stack of IAB DAPS, two separate sets of RLC/MAC/PHYs and common BAP sublayer are used for the DAPS IAB-node.

Protocol 3: Descendant UE of the DAPS IAB-node needs to be configured with two sets of security functions so that it can process the data packets from/to both IAB-donors.

# Reference

- Chairman notes of RAN3 110 e-meeting.

- Chairman notes of RAN3 111 e-meeting.

- R2-2102364. Reply LS on DAPS-like solution for service interruption reduction.