3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103413

Online, April 12th – 20th 2021

Agenda item:	   8.1.2.1

Source:		Lenovo, Motorola Mobility

Title:		Issues on MBS reliability

Document for:		Discussion and Decision

1	Introduction

In this paper we discuss the issues related to NR MBS reliability.

2	Discussion

2.1	HARQ for MBS

For MBS service, at least ACK/NACK based HARQ feedback is supported for PTM multicast transmission/reception according to RAN1 agreement in the last meeting.

RAN1#104e: Agreements: For RRC_CONNECTED UEs receiving multicast, support the following: - ACK/NACK based HARQ-ACK feedback for multicast, - It is up to network to configure orthogonal PUCCH resources among UEs within the same group. - FFS: NACK-only based HARQ-ACK feedback for multicast, - It is up to network to configure the PUCCH resources and the PUCCH resources can be shared among UEs within the same group. - FFS details. For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, - Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback. - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled - FFS details on RRC signalling and DCI indicating. - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. - FFS whether enabling or disabling the feedback is the default mode. - Option 2: RRC indicates enabling/disabling. - FFS: whether down-selection between option 3 and option 2 is needed or support the both options. - FFS: enabling/disabling by MAC-CE.

It can be seen that for RRC_CONNECTED UE, HARQ-ACK feedback is supported for multicast, and RAN1 will further work on the details. Then, the next question is what are possible RAN2 impacts.

From configuration perspective, UE shall be indicated whether HARQ-ACK feedback is enabled, and whether it is ACK/NACK based or NACK-only based. According to the progress in RAN1, there are three different ways on the table to enable/disable HARQ-ACK feedback:

- RRC signalling configures whether DCI indicates the enabling/disabling of HARQ-ACK feedback. Then, UE will understand if DCI carries the indication to enable/disable HARQ-ACK feedback, and act accordingly.

- RRC signalling directly configures whether HARQ-ACK feedback is enabled or disabled.

- MAC CE indicates whether HARQ-ACK feedback is enabled or disabled.

Due to the lack of details from RAN1 perspective, RAN2 is suggested to wait for RAN1 conclusion on HARQ-ACK feedback enabling/disabling before discussing the impact on RRC/MAC CE.

RAN 1 supports HARQ feedback for multicast, e.g. UE specific ACK/NACK based or NACK-only based.

RAN2 waits for RAN1 progress before discussing whether/how to provide multicast HARQ configuration for enabling/disabling HARQ-ACK feedback.

On the other hand, RAN2 is suggested to discuss the HARQ process design for PTM transmission in specific. In our understanding there are three possible ways:

Option a: As in legacy leave it upon NW implementation to identify common HARQ process available among all relevant UEs.

Considering the initial transmission or retransmission per PTM to a group of UEs is associated with the same HARQ process (i.e. indicated by the HARQ process ID in the DCI for PTM transmission), following legacy HARQ process design gNB has to make sure the same HARQ process is available or used in the same way among all relevant UEs. This might be hard to achieve in practice considering different UEs might be in prior configured differently, with respect to HARQ process usage. For example, UE#1’s HARQ process#1 is in prior configured for downlink SPS scheduling, while HARQ process#1 is the only available HARQ process in UE#2, then to use HARQ process ID = 1 for multicast MBS TB transmission, gNB needs to reconfigure UE#1 and make HARQ process#1 available first. It will become more difficult if the number of UEs becomes high.

Option b: Support the case that gNB may assign an MBS packet and a non-MBS packet to the same HARQ process in DL, and extra indicator is needed to distinguish different TBs assigned to the same HARQ process.

To reduce the scheduling complexity in option a, mechanism can be investigated to resolve the scenario that gNB may assign an MBS packet to a HARQ process which has been occupied by a non-MBS packet. One possible way out would be to introduce an extra indicator in e.g. DCI to distinguish the MBS packet from a non-MBS packet, so that UE can by implementation store them in two different HARQ buffers. In this way, the UE can avoid storing the MBS packet and non-MBS packet into the same HARQ buffer.

Option c: Introduce MBS dedicated HARQ process(es) to avoid HARQ process collision against non-MBS services.

To reduce the scheduling complexity in option a, RAN2 can introduce MBS dedicated HARQ processes. Then, a certain number of HARQ processes will be reserved only for MBS services. The HARQ process collision between unicast service and MBS services can be avoided.

RAN2 considers the following alternatives for HARQ process design for MBS PTM transmission

  - As in legacy leave it upon implementation to identify common HARQ process available among all relevant UEs

  - Support the case that gNB may assign an MBS packet and a non-MBS packet to the same HARQ process in DL, and extra indicator is needed to distinguish different TBs assigned to the same HARQ process.

  - Introduce MBS dedicated HARQ process(es) to avoid HARQ process collision against non-MBS services.

Among the three options listed above, we slightly prefer option a to leave the scheduling complexity to NW implementation, which does not need any modification to the specification. Option b and c in proposal 2 might require some UE capability enhancement or DCI enhancement, thus RAN2 is suggested to consult RAN1 before agreeing on option b or c.

RAN2 is suggested to agree on option a, otherwise RAN2 shall consult RAN1 before agreeing option b or option c.

2.2	CA and CA based Duplication for MBS

Another issue we would like to discuss is whether to support carrier aggregation and CA based duplication for MBS services. In the legacy, CA and CA based duplication is used to enhance the reliability and data rate for transmission per unicast. For MBS transmission, we believe CA and CA based duplication could be also beneficial for those MBS services demanding high reliability or high data rate.

For PTP transmission, it’s similar as unicast transmission, and thus we don’t see any issue to adopt CA and CA based duplication just like unicast in legacy.

For PTM transmission, when applicable, gNB can transmit MBS packet via any common serving cell(s) among the relevant the group of UEs. Similarly, gNB may decide to adopt CA or CA based duplication to transmit packets to the relevant group of UEs.

Considering the limited time in WI, RAN2 is suggested to at least support CA and CA-based duplication for PTP mode as in legacy unicast, and can FFS CA and CA-based duplication for PTM if time allows.

RAN2 is suggested to at least support CA and CA-based duplication for PTP mode as in legacy unicast.

FFS CA and CA-based duplication for PTM if time allows.

3	Conclusion

Based on the discussion above, we observe:

- RAN 1 supports HARQ feedback for multicast, e.g. UE specific ACK/NACK based or NACK-only based.

Based on the discussion above, we propose:

- RAN2 waits for RAN1 progress before discussing whether/how to provide multicast HARQ configuration for enabling/disabling HARQ-ACK feedback.

RAN2 considers the following alternatives for HARQ process design for MBS PTM transmission

  - As in legacy leave it upon implementation to identify common HARQ process available among all relevant UEs

  - Support the case that gNB may assign an MBS packet and a non-MBS packet to the same HARQ process in DL, and extra indicator is needed to distinguish different TBs assigned to the same HARQ process.

  - Introduce MBS dedicated HARQ process(es) to avoid HARQ process collision against non-MBS services.

RAN2 is suggested to agree on option a, otherwise RAN2 shall consult RAN1 before agreeing option b or option c.

RAN2 is suggested to at least support CA and CA-based duplication for PTP mode as in legacy unicast.

FFS CA and CA-based duplication for PTM if time allows.